| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting: Chengdu, CN, 14–21 October 2016 | Document: JCTVC-Y0034 |
| --- | --- |

| Title: | AHG13: Further results for luma sample adjustment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne, V. Kolesnikov Building A, The Park Estate 5 Talavera Road Macquarie Park NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
| Source: | Canon Information Systems Research Australia |  |  |

_____________________________

# Abstract

The ‘Draft 2 of Conversion and Coding Practices for HDR/WCG Y′CbCr 4:2:0 Video with PQ Transfer Characteristics’ (JCTVC-X1017) describes an iterative method known as ‘luma sample adjustment’ to address chroma leakage that my occur in the non-constant luminance (NCL) representation. Luma sample adjustment compensates for a shift in luminance that occurs e.g. when downsampling from 4:4:4 to 4:2:0 using NCL and YCbCr. The described method performs an iterative search to determine each luma sample value, which is asserted to be overly complex for real time implementation. This contribution shows a method for performing luma sample adjustment that replaces an iterative search with a fixed-complexity linearization method which is applied to predict the adjusted luma sample. This method is asserted to be an alternative to the ‘linearisation’ approach to luma sample adjustment presented in JCTVC-X0072 which does not require the values of EOTF derivative f ' (X).

T he proposed method achieves the following tPSNR results of 53.80dB, 69.70dB, and 43.08dB in X, Y, and Z components, respectively. Note that in the Y component, this result is 0.07dB below that achieved by the iterative method of luma sample adjustment.

# Introduction

When generating Y’CbCr data from R’G’B’ using non-constant luminance and downsampling to 4:2:0, luminance after conversion back to RGB 4:4:4 is distorted compared to the input RGB luminance. This problem is described in [2].

A luma sample adjustment process, described by Ericsson in [3], was proposed to compensate for distortions introduced by these stages. Figure 1 shows how this process operates in the context of encoder-side processing, as used in the MPEG HDR/WCG CfE.

Figure 1. Luma sample adjustment process.

Within the ‘luma adjustment’ block above the following steps are performed:

- Youtput is initialised from Y’initial.

- Compute R’G’B’ from Y’outputCbCr.

- Convert R’G’B’ to RGB.

- Compute Ylinear from RGB.

- Compare Ylinear to Ytarget and adjust Youtput according to a binary search.

- While Youtput differs from Ytarget repeat steps 2-5.

- Compute and output Y’ from Youtput.

Contribution JCTVC-X0054 Error: Reference source not found presented an alternative method whereby a linear approximation was used to derive the luma value Y from the input YlinearCbCr.

# Method description

The method takes as input two values: a value of initial unadjusted Yprime (point A) and target luminance value (point F) and outputs an adjusted Yprime value (point D). The method applies the following steps:

1. Select two approximating Yprime values (points B and C) on both sides of the initial Yprime point A using fixed offsets. The authors used an offset of half-unit (i.e. half of the codeword luma step size at the considered point on the PQ curve), giving one unit of difference between the approximating points B and C, but other values tested by authors did not influence the result significantly.

2. Calculate target function values for the selected approximating Yprime values. That is, for points B and C find corresponding luminance points G and E by applying the inverse PQ transform to the points B and C.

3. Calculate approximation line slope coefficients from the values of points B, C, G, E.

4. Derive adjusted Yprime point D from target Ylinear point F using the approximation line.

As seen from the algorithm description, the method does not require calculation of PQ’.

Also, as seen in the results section below, the performance of this method under the no-compression simulation is 0.07dB less than the iterative method for tPSNR_Y described in [3]. The worst case loss was 0.22dB for tPSNR_Y, seen in BikeSparklers_cut1. No loss was seen in t_PSNR_X/Z.

# Results

Table 1 presents results for the iterative method

Table 2 presents results for the presented method

Table 1. Results for the iterative method (TF LUT is on)

|  | tPSNR X | tPSNR Y | tPSNR Z | tPSNR XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL 100 | Time (Sec) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FireEaterClip4000r1 | 63.18 | 70.21 | 52.23 | 56.60 | 55.01 | 49.16 | 24.73 | 58.38 | 1424.96 |
| Market3Clip4000r2 | 55.60 | 69.61 | 45.96 | 50.27 | 49.89 | 36.79 | 21.87 | 47.89 | 3122.08 |
| SunRise | 59.67 | 69.61 | 50.92 | 55.09 | 54.57 | 37.88 | 24.22 | 47.94 | 1361.44 |
| BikeSparklers cut 1 | 47.54 | 69.78 | 37.04 | 41.44 | 39.83 | 34.19 | 21.82 | 51.48 | 841.39 |
| BikeSparklers cut 2 | 48.61 | 69.76 | 38.42 | 42.79 | 41.63 | 34.52 | 21.77 | 51.08 | 1006.74 |
| GarageExit | 56.37 | 69.73 | 47.09 | 51.35 | 50.05 | 38.10 | 22.85 | 49.78 | 2392.14 |
| ShowGirl2Teaser | 53.44 | 69.24 | 44.97 | 49.12 | 47.85 | 39.29 | 22.50 | 52.90 | 2626.53 |
| StEM_MagicHour cut 1 | 54.05 | 69.83 | 39.98 | 44.58 | 42.05 | 36.57 | 24.96 | 52.16 | 642.89 |
| StEM_MagicHour cut 2 | 51.52 | 69.89 | 38.72 | 43.27 | 40.81 | 36.00 | 24.71 | 52.68 | 1530.4 |
| StEM_MagicHour cut 3 | 50.92 | 69.94 | 37.51 | 42.08 | 39.66 | 36.07 | 24.28 | 52.94 | 655.22 |
| StEM_WarmNight cut 1 | 52.58 | 69.94 | 38.46 | 43.07 | 40.65 | 36.67 | 25.11 | 52.94 | 1511.87 |
| StEM_WarmNight cut 2 | 54.20 | 70.18 | 39.14 | 43.78 | 41.72 | 37.06 | 23.35 | 52.81 | 1236.74 |
| BalloonFestival | 53.73 | 69.59 | 45.06 | 49.27 | 51.73 | 40.68 | 21.15 | 48.24 | 1859.1 |
| EBU_04_Start | 54.15 | 69.59 | 46.67 | 50.71 | 49.18 | 36.87 | 22.23 | 47.34 | 4089.13 |
| EBU_06_Hurdles | 51.23 | 69.60 | 44.07 | 48.07 | 47.75 | 36.12 | 21.28 | 47.71 | 3517.16 |
| Average | 53.79 | 69.77 | 43.08 | 47.43 | 46.16 | 37.73 | 23.12 | 51.08 | 1854.52 |

Table 2. Results for the proposed method (TF LUT is on)

|  | tPSNR X | tPSNR Y | tPSNR Z | tPSNR XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL 100 | Time (Sec) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FireEaterClip4000r1 | 63.16 | 70.198 | 52.236 | 56.605 | 55.015 | 49.152 | 24.723 | 58.365 | 731.17 |
| Market3Clip4000r2 | 55.64 | 69.54 | 45.956 | 50.266 | 49.887 | 36.788 | 21.874 | 47.886 | 1510.49 |
| SunRise | 59.69 | 69.604 | 50.91 | 55.089 | 54.571 | 37.879 | 24.217 | 47.935 | 733.99 |
| BikeSparklers cut 1 | 47.565 | 69.56 | 37.034 | 41.433 | 39.83 | 34.186 | 21.823 | 51.436 | 494.41 |
| BikeSparklers cut 2 | 48.637 | 69.562 | 38.406 | 42.781 | 41.624 | 34.521 | 21.772 | 51.04 | 569.61 |
| GarageExit | 56.384 | 69.725 | 47.091 | 51.354 | 50.048 | 38.097 | 22.852 | 49.78 | 1341.76 |
| ShowGirl2Teaser | 53.457 | 69.191 | 44.963 | 49.119 | 47.847 | 39.286 | 22.504 | 52.88 | 1467.68 |
| StEM_MagicHour cut 1 | 54.055 | 69.822 | 39.98 | 44.58 | 42.046 | 36.569 | 24.955 | 52.16 | 395.26 |
| StEM_MagicHour cut 2 | 51.527 | 69.871 | 38.723 | 43.269 | 40.808 | 35.998 | 24.706 | 52.675 | 920.14 |
| StEM_MagicHour cut 3 | 50.943 | 69.842 | 37.513 | 42.088 | 39.663 | 36.071 | 24.282 | 52.934 | 404.29 |
| StEM_WarmNight cut 1 | 52.619 | 69.85 | 38.465 | 43.069 | 40.651 | 36.674 | 25.107 | 52.934 | 953.49 |
| StEM_WarmNight cut 2 | 54.219 | 70.146 | 39.144 | 43.778 | 41.716 | 37.058 | 23.354 | 52.809 | 773.22 |
| BalloonFestival | 53.756 | 69.559 | 45.05 | 49.259 | 51.722 | 40.679 | 21.151 | 48.237 | 880 |
| EBU_04_Start | 54.168 | 69.574 | 46.669 | 50.708 | 49.176 | 36.865 | 22.234 | 47.34 | 1883.66 |
| EBU_06_Hurdles | 51.25 | 69.481 | 44.073 | 48.071 | 47.746 | 36.115 | 21.276 | 47.702 | 2143.86 |
| Average | 53.80 | 69.70 | 43.08 | 47.43 | 46.16 | 37.73 | 23.12 | 51.07 | 1013.535 |

# References

- E. François, P. Yin, J. Sole, “Common Test Conditions for HDR/WCG Video Coding Experiments”, Moving Pictures Experts Group, w15793, Geneva, October, 2015.

- J. Strom, “Investigation of HDR Color Subsampling”, Moving Pictures Experts Group, m35841, Geneva, February, 2015.

- J. Strom, J. Samuelsson, M. Pettersson, K. Andersson, P. Wennersten, R. Sjöberg, “Ericsson’s response to CfE for HDR and WCG”, Moving Pictures Experts Group, m36184, Warsaw, June, 2015.

- A. Norkin, “On closed form HDR 4:2:0 chroma subsampling (AHG13 related)”, Joint Collaborative Team on Video Coding (JCT-VC), JCTVC-X0072, Geneva, CH, June 2016.

- C. Rosewarne, V. Kolesnikov “AHG13: Further results for LUT-based luma sample adjustment”, Joint Collaborative Team on Video Coding (JCT-VC), JCTVC-X0054, Geneva, CH, June 2016.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).