| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting: Chengdu, CN, 14–21 October 2016 | Document: JCTVC-Y0038 |
| --- | --- |

| Title: | Closed GOP Support for the Random Access Common Conditions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Y.T. Peng, A.M. Tourapis, Y. Su, D. Singer 1 Infinite Loop Cupertino, CA 95014 USA | Tel: Email: | +1-408-228-7983 atourapis@apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

This contribution requests the consideration of using Closed Group of Pictures (GOP) configurations instead of, or at least in addition to, Open GOP configurations for any future experiments within the context of the JVET activity. The necessary modifications to the JEM software for supporting this configuration, while enabling other features such as splicing and finer control of coding parameters, are also presented.

# Introduction

In recent years, video standardization activities have focused primarily on three different coding structure configuration modes, an Intra only coding mode, a low delay configuration, and a random access configuration, each targeting different functionalities and applications. Intra only coding, in particular, targets low delay and low complexity applications that might not permit the use of any temporal prediction, such as live, high end camera capture, editing and archiving, among others. Similarly, low delay configurations target applications that may require low end-to-end delay but also high compression, such as video-conferencing and surveillance. These configurations strongly couple the coding with the display order.

On the other hand, random access configurations try to maximize compression efficiency by permitting a more flexible coding structure, which may allow better exploiting temporal redundancies and prediction. Such configurations commonly use deep hierarchical coding structures and make effective use of bi-prediction and, when used, weighted prediction. Delay is not as an important factor, however, random access, i.e. the ability of accessing any frame within the bitstream using a “limited” number of processing steps, is commonly desirable. Such functionality can permit and enable the use of trick modes, such as fast-forward or rewind, channel change, adaptive bitrate switching for streaming applications, and error robustness among others.

Both Closed and Open Group of Pictures (GOP) are commonly used in random access configurations. Closed GOPs (Figure 1-2 and Figure 1-3) commonly utilize fully independent coding segments, commonly separated using Instantaneous Decoder Refresh (IDR) pictures. On the other hand Open GOPs  (Figure 1-1) may result in some coding segments that may share some prediction information between adjacent GOPs and commonly use Clean Random Access (CRA) pictures. This characteristic allows Open GOPs to have commonly better coding performance than Closed GOPs, while at the same time the shared segments can ease the transitioning between two Closed GOPs. This can be achieved in HEVC with the use of Random Access Skipped Leading (RASL) pictures. On the other hand, during a trick mode request, they may either result in the loss of decoded pictures, i.e. all RASL pictures may have to be discarded, or result in decoding artifacts due to the unavailability of the appropriate reference pictures and their associated information during decoding.

Figure 1-1. Open GOP Support in the HM and JEM software

Figure 1-2. Closed GOP Support in the HM and JEM software

Figure 1-3. Proposed/Traditional Closed GOP

Encoder implementations may also be impacted by the use of Open GOPs. In particular, encoder parallelization is highly desirable by many applications. A common strategy employed is the use of splicing where a video is split into several, commonly equal length segments, and each segment is encoded independently given certain rate control parameters, such as an initial decoder buffer fullness and a final rate target. Although this is quite easy to achieve in the Closed GOP case since each segment is completely independent and the only constraints are that all segments need to satisfy the exact initial decoder buffer fullness, this is not the case for Open GOPs. Buffer behaviour may need to be established not only for the key frames but also at the start for any trailing pictures after the key frame to ensure proper rate control behaviour. This process also implies that the CRA picture may have been encoded twice, once in each parallel instantiation, with exactly identical parameters, and only one bitstream instantiation was retained and appropriately multiplexed in the final bitstream. An alternative would be to properly pipeline the parallel GOP encodings, i.e. only initiate the encoding of the next GOP when all needed pictures from the current GOP have been encoded (i.e. the last picture in display order of GOP 0 in Figure 1-1). In general, such approaches imply that a somewhat more elaborate control process is needed for real implementations of Closed GOPs with Splicing/GOP level parallelization support.

Another major issue that exists when using Open GOPs is the inability to cleanly switch between streams in the context of Adaptive Streaming. This is because when switching from one stream of a certain quality to another stream of different quality () the Shared Pictures across two GOPs likely do not anymore use their intended references. This may result in some “uncontrolled” artifacts that may be undesirable during playback. Commonly, streaming services, for these as well as other reasons prefer the use of Closed GOPs instead of Open GOPs. On the other hand, Open GOPs might be preferred for applications involving Fixed Media or Broadcast Applications.

Figure 1-4. Bitstream switching in the context of Open GOPs. The Shared Pictures do not, anymore, use the same reference points. This may cause some “uncontrolled” artifacts during the playback of that segment.

HEVC and currently JVET test conditions have utilized the Open GOP configuration, using also an Intra period of ~1 second. Although this may cater for some applications, it seems to ignore the importance of streaming applications, which have and are more likely to adopt new video coding formats such as HEVC and the JVET codec, when that is finalized. The perceived advantage also of the temporal filtering may, in fact, for standardization development purposes be a disadvantage since it may end hiding artifacts that could appear in practice. Standardization development should not only assume the best possible scenarios but also keep the worst possible scenarios in mind, and ensure that quality is consistent in all cases.

Unfortunately, although a Closed GOP implementation does exist in the HM and JVET software this tries to mimic the Open GOP implementation, and for any subsequent GOP other than the first leading pictures are introduced after an IDR frame is encoded. This can have an impact on coding quality, but also again has implications in parallel encoding, at least using the HM and JVET reference software. Unfortunately, using this configuration and because of the presence of the leading pictures, splicing using these two software implementations cannot be achieved since the software cannot recreate, without any further modifications, the GOPs other than the first.  Enabling the use of Closed GOPs, although desirable, cannot be effectively achieved with the current software.

# Augmented Implementation of Closed GOPs in the HM and JEM

We have augmented the HM and JEM software to support a more “conventional” Closed GOP structure as seen in Figure 1-3. In particular, we have allowed the use of Closed GOPs without any leading pictures. This configuration is commonly used by many different systems, especially for streaming applications, and allows the use of splicing. In our implementation, some additional features have been enabled:

- Enabling writing the various high level syntax parameter sets for every IDR. This is quite essential for being able to “exactly” match performance of both splicing and serial encodings (WriteParamSetsIDRFlag=1).

Figure 2-5. GOP Cycle QP/Offset parameter adjustments using QPoffsetCycle/ QPFactorCycle

- Having the ability to adjust QP and lambda parameters for different GOPs in a periodic manner. In particular, we may wish to lower the QP value for a periodic GOP after a certain number of GOPs, so as to boost Quality from that point onward. This gives us basically finer control in the software. In an example, we may wish to lower the QP, i.e. by 1, for the first frame of the third GOP after the intra picture, as well as adjust its lambda parameter to 0.3 (Figure 2-5). We have found also that disabling this feature for the last GOP prior to the next Intra Period may also be advantageous. An extension of this could be made, if desirable,  that allows us to control the quality of that last GOP explicitly. This can be done easily done by adjusting the following parameters and the newly introduced GOP coding parameters highlighted  below.

QPoffsetCycle                 : 3

QPFactorCycle                 : 3

QPCycleBoundaryOffFlag        : 1

| # | T | P | QPoff | GiQPoff | QPfactor | GQPfactor | tcOff | betaOffset | t_id | #ref_pics | #ref_pics | Ref. pictures | predict | deltaRPS | #ref_idcs | Ref. idcs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame1: | B | 16 | 1 | -1 | 0.6 | 0.3 | 0 | 0 | 0 | 2 | 3 | -72 | 0 |  |  |  |
| Frame2: | B | 8 | 2 | 0 | 0.2 | 0.2 | 0 | 0 | 1 | 2 | 3 | -8 -16 8 | 1 | 8 | 4 | 1 1 0 1 |
| Frame3: | B | 4 | 3 | 0 | 0.33 | 0.33 | 0 | 0 | 2 | 2 | 4 | -4 -12 4 12 | 1 | 4 | 4 | 1 1 1 1 |
| Frame4: | B | 2 | 4 | 0 | 0.33 | 0.33 | 0 | 0 | 3 | 2 | 5 | -2 -10 2 6 14 | 1 | 2 | 5 | 1 1 1 1 1 |
| Frame5: | B | 1 | 5 | 0 | 0.4 | 0.4 | 0 | 0 | 4 | 2 | 5 | -1 1 3 7 15 | 1 | 1 | 6 | 1 0 1 1 1 1 |
| Frame6: | B | 3 | 5 | 0 | 0.4 | 0.4 | 0 | 0 | 4 | 2 | 5 | -1 -3 1 5 13 | 1 | -2 | 6 | 1 1 1 1 1 0 |
| Frame7: | B | 6 | 4 | 0 | 0.33 | 0.33 | 0 | 0 | 3 | 2 | 4 | -2 -6 2 10 | 1 | -3 | 6 | 0 1 1 1 1 0 |
| Frame8: | B | 5 | 5 | 0 | 0.4 | 0.4 | 0 | 0 | 4 | 2 | 5 | -1 -5 1 3 11 | 1 | 1 | 5 | 1 1 1 1 1 |
| Frame9: | B | 7 | 5 | 0 | 0.4 | 0.4 | 0 | 0 | 4 | 2 | 5 | -1 -3 -7 1 9 | 1 | -2 | 6 | 1 1 1 1 1 0 |
| Frame10: | B | 12 | 3 | 0 | 0.33 | 0.33 | 0 | 0 | 2 | 2 | 3 | -4 -12 4 | 1 | -5 | 6 | 0 0 1 1 1 0 |
| Frame11: | B | 10 | 4 | 0 | 0.33 | 0.33 | 0 | 0 | 3 | 2 | 4 | -2 -10 2 6 | 1 | 2 | 4 | 1 1 1 1 |
| Frame12: | B | 9 | 5 | 0 | 0.4 | 0.4 | 0 | 0 | 4 | 2 | 5 | -1 -9 1 3 7 | 1 | 1 | 5 | 1 1 1 1 1 |
| Frame13: | B | 11 | 5 | 0 | 0.4 | 0.4 | 0 | 0 | 4 | 2 | 5 | -1 -3 -11 1 5 | 1 | -2 | 6 | 1 1 1 1 1 0 |
| Frame14: | B | 14 | 4 | 0 | 0.33 | 0.33 | 0 | 0 | 3 | 2 | 4 | -2 -6 -14 2 | 1 | -3 | 6 | 0 1 1 1 1 0 |
| Frame15: | B | 13 | 5 | 0 | 0.4 | 0.4 | 0 | 0 | 4 | 2 | 5 | -1 -5 -13 1 3 | 1 | 1 | 5 | 1 1 1 1 1 |
| Frame16: | B | 15 | 5 | 0 | 0.4 | 0.4 | 0 | 0 | 4 | 2 | 5 | -1 -3 -7 -15 1 | 1 | -2 | 6 | 1 1 1 1 1 0 |

# Simulation results

We have tested performance using this new implementation of the Closed GOP coding structure vs the Open GOP coding structure in the HM and JVET software for the JVET common conditions, with the following, however, modification. JVET and HEVC common conditions dictate the use of 1 second GOPs. Even though this setting might be common for broadcast applications or file based playback systems, a larger intra period is commonly used for streaming applications. In particular, a value of 2s or even higher, e.g. 4 or 10, is recommended by many streaming systems. We have settled with a value of ~2s for our simulations so as to not deviate too much from the performance and characteristics of the defined common conditions. The periodic refresh was set to 1 second to match the CRA test conditions.

Summary of the performance results is shown in Table 3-1 (HM), Table 3-2 (JEM), and Table 3-3 (HM vs JEM). There are definitely changes in performance vs the Open GOP configuration for both codecs, however performance benefits of the JEM vs the HM software are retained while providing for an appropriate test for, especially, streaming applications.

Table 3-1. Performance of the 2s Closed GOP vs the 1s Open GOP in the HM reference software

|  | Piecewise Cubic |  |  | Cubic |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | Y | U | V |
| A1 | -1.49% | -1.29% | -1.28% | -1.48% | -1.28% | -1.32% |
| A2 |  |  |  |  |  |  |
| B | -0.74% | 0.93% | 1.10% | -0.75% | 0.91% | 1.07% |
| C | -1.70% | 0.15% | -0.53% | -1.71% | 0.07% | -0.54% |
| D | -0.92% | 0.32% | 0.04% | -0.92% | 0.31% | -0.03% |
| E |  |  |  |  |  |  |
| F | -12.19% | -10.67% | -11.10% | -12.19% | -10.72% | -11.07% |
| Overall |  |  |  |  |  |  |

Table 3-2. Performance of the 2s Closed GOP vs the 1s Open GOP in the JVET reference software

|  | Piecewise Cubic |  |  | Cubic |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | Y | U | V |
| A1 |  |  |  |  |  |  |
| A2 |  |  |  |  |  |  |
| B |  |  |  |  |  |  |
| C | -1.14% | 6.48% | 5.85% | -1.14% | 6.31% | 5.77% |
| D | -0.62% | 7.60% | 7.57% | -0.62% | 7.54% | 7.60% |
| E |  |  |  |  |  |  |
| F | -11.68% | -6.13% | -6.66% | -11.67% | -6.19% | -6.83% |
| Overall |  |  |  |  |  |  |

Table 3-3. Performance of the 2s Closed GOP HM vs 2s Closed GOP JVET codecs

|  | Piecewise Cubic |  |  | Cubic |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | Y | U | V |
| A1 |  |  |  |  |  |  |
| A2 |  |  |  |  |  |  |
| B |  |  |  |  |  |  |
| C | -26.27% | -29.99% | -32.52% | -26.27% | -29.72% | -32.26% |
| D | -26.49% | -28.23% | -30.20% | -26.52% | -27.86% | -29.86% |
| E |  |  |  |  |  |  |
| F | -21.25% | -28.34% | -28.31% | -21.25% | -27.81% | -28.03% |
| Overall |  |  |  |  |  |  |

Given the performance, the practicality of this configuration, as well as the ability to run proper splicing tests if those are desired, it is recommended that the Random Access Conditions are either replaced or at least augmented using this new Closed GOP configuration.

# Conclusion/Recommendations

This contribution provides for an augmented implementation of the Closed GOP coding structure in both the HM and JEM software. It is suggested that the Random Access Conditions are either replaced or at least augmented using this new Closed GOP configuration. This new implementation still results in gains of over 20% for the JEM software over the HM software.

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

That feature can seen as a temporal filterimg

In many instances, decoding systems may not reset their decoder reference buffer when encountering Open GOPs and may assume that pictures that are already in the decoder buffer are ok to use for prediction as an alternative to the original references.