| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting: Chengdu, CN, 14–21 October 2016 | Document: JCTVC-Y0040r1 |
| --- | --- |

| Title: | Content colour volume SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexis Michael Tourapis Yeping Su David Singer Hyun Mook Oh Jang Won Choi Jong-Yeul Suh Adarsh K. Ramasubramonian Done Bugdayci Sansli Joel Sole Dmytro Rusanovskyy Marta Karczewicz | Tel: Email: | atourapis@apple.com hyunmook.oh@lge.com aramasub@qti.qualcomm.com |
| Source: | Apple Inc., LG Electronics Inc., Qualcomm |  |  |

_____________________________

# Abstract

This document proposes an SEI message that can describe the colour volume of the video content. The colour volume is specified using several different methods that are characterized by different degrees of complexity and precision. This information may be utilized by decoder-side entities and displays when processing and/or displaying the content.

In revision 1 of this document, a few clarifications to the semantics of the SEI message are provided along with aligning the representation of certain sytnax elements.

# Introduction

The characteristics of HDR video content are different from that of what is known traditionally as SDR content. HDR displays, on the other hand, have specific capabilities that exceed their SDR counterparts. Since the characteristics of the content, or those of the mastering display (e.g. peak luminance and the colour gamut), are quite often different from, and in cases where they exceed, the capabilities supported by the receiver-side display, displays may have to appropriately modify the content so as to make it consistent on and appropriate for the respective display. Display manufacturers commonly provide a variety of algorithms onto their displays that enable them to adjust and adapt the content based on their characteristics and those of the display. In the absence of other metadata that describes content adaptation onto a specific display, the display that is unaware of the full content characteristics has to resort to compacting the content, to appropriately clip the colour volume of the content so as to fit on the colour volume of the display, or to potentially analyze the content so as to determine its characteristics prior to any more advanced type of processing. Such steps, may either result in less efficient, quality-wise, conversion, while they may also be computationally expensive and could impact the power consumption of the device. On the other hand, an efficient metadata mechanism that could sufficiently describe the content characteristics, i.e. its colour volume, could enable reduced complexity processing as well as improved display rendering on a target device. It should be noted that such metadata are complementary to the metadata specified by specifications such as SMPTE ST 2094 [2], since they do not provide information on how to adapt the content for a display, but instead provide a concise description of the content’s colour volume.

With the above motivation, a proposal [3] was initially presented at the 22nd JCT-VC meeting that provided provisions for the description of such metadata. Further extensions were presented during the 23rd [4][5] and 24th [6][7][8] JCT-VC meetings. JCTVC-V0038 [3], JCTVC-W0098 [5], and JCTVC-X0052 [7], in particular, introduced a rather extensive SEI message that enabled the description of the effective colour volume message using a multitude of representations. This proposal permitted the signaling of a rather accurate description of the colour volume of the content for a variety of applications. The document also provided a description of the potential benefits of such representations. JCTVC-W0057[4] and JCTVC-X0040 [6] was build on similar principles, but suggested the use of a simpler SEI message with a more limited scope. Finally, JCTVC-X0069 [8], extended upon this work by providing additional means for explicitly describing the colour volume occupied by the content. At the parent-body meeting, it was agreed that this functionality was indeed desirable and the proponents were asked to come up with a combined syntax solution for describing the colour volume of the content. This document presents the combined syntax of such a method.

# Proposal

## General SEI syntax structure

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| … |  |
| else if( payloadType = = 149 ) |  |
| content_colour_volume( payloadSize ) |  |
| … |  |
| …. |  |
| else if( payloadType = = 181 ) |  |
| alternative_depth_info( payloadSize ) /* specified in Annex Error: Reference source not found */ |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| … |  |
| } |  |
| } |  |

## Content colour volume SEI message syntax

| content_colour_volume( payloadSize ) { | Descriptor |
| --- | --- |
| ccv_cancel_flag | u(1) |
| if( !ccv_cancel_flag ) { |  |
| ccv_persistence_flag | u(1) |
| ccv_mode_type_flags | u(8) |
| if( CcvModeTypeFlag[ 0 ] == 1 ) { |  |
| ccv_gamut_type | u(8) |
| if( ccv_gamut_type = = 255 ) { |  |
| number_of_primaries_minus3 | u(4) |
| for( c = 0; c < number_of_primaries_minus3 + 3; c++ ) { |  |
| ccv_primary_x[ c ] | u(16) |
| ccv_primary_y[ c ] | u(16) |
| } |  |
| } |  |
| ccv_min_lum_value | u(32) |
| ccv_max_lum_value | u(32) |
| } |  |
| if( CcvModeTypeFlag[ 1 ] == 1 ) { |  |
| ccv_ctb_partitions_y_minus1 | ue(v) |
| ccv_ctb_partitions_x_minus1 | ue(v) |
| for( m = 0; m < ccv_ctb_partitions_y_minus1 + 1; m++ ) { |  |
| for( n = 0; n < ccv_ctb_partitions_x_minus1 + 1; n++ ) { |  |
| ccv_num_implicit_repn_minus1[ m ][ n ] | u(8) |
| for( i = 0; i <= ccv_num_implicit_repn_minus1[ m ][ n ]; i++ ) { |  |
| ccv_implicit_repn_type[ m ][ n ][ i ] | u(8) |
| for( j = 0; j < NumValsRepn[ m ][ n ][ i ]; j++ ) { |  |
| if ( PrimariesPresentFlag[ m ][ n ][ i ] ) { |  |
| ccv_impl_repn_primary_x[ m ][ n ][ i ][ j ] | i(24) |
| ccv_impl_repn_primary_y[ m ][ n ][ i ][ j ] | i(24) |
| } |  |
| ccv_impl_repn_primary_min[ m ][ n ][ i ][ j ] | i(32) |
| ccv_impl_repn_primary_max[ m ][ n ][ i ][ j ] | i(32) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( CcvModeTypeFlag[ 2 ] == 1 ) { |  |
| ccv_num_explicit_repn_minus1 | u(8) |
| for( i = 0; i <= ccv_num_explicit_repn_minus1; i++ ) { |  |
| ccv_explicit_repn_type[ i ] | u(8) |
| ccv_expl_num_ranges_minus2[ i ] | ue(v) |
| if( ccv_expl_num_ranges_minus2[ i ] > 0 ) |  |
| ccv_expl_ranges_idc[ i ] | u(2) |
| for( j = 0; j <= CcvExplNumRanges[ i ]; j++ ) { |  |
| if( !ccv_expl_ranges_idc[ i ] ¦¦ j = = 0 ¦¦ j = = CcvExplNumRanges[ i ] ) |  |
| ccv_expl_val[ i ][ j ] | i(32) |
| ccv_expl_num_coord[ i ][ j ] | ue(v) |
| for( k = 0; k < ccv_expl_num_coord[ i ][ j ]; k++ ) { |  |
| ccv_expl_coord_1[ i ][ j ][ k ] | i(24) |
| ccv_expl_coord_2[ i ][ j ][ k ] | i(24) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Semantics of content colour volume SEI message

The content colour volume SEI message provides information about the colour volume characteristics of the associated picture using a variety of colour spaces and representations. This SEI describes the colour volume characteristics of the associated picture in terms of a nominal range and deviations from this range may occur. This SEI message requires knowledge of the colour description information of the signal, such as the colour_primaries, transfer_charactreristics, and matrix_coeffs in the VUI parameters syntax. This SEI message shall not be present when any of these parameters is unspecified or reserved. When the value of transfer_characteristics is equal to 1, 6, 7, 14, or 15, then the BT.1886 transfer function should be used to convert the signal back to its linear representation. When the value of transfer_characteristics is equal to 18, the EOTF specified in BT.2100 when using the HLG transfer function shall be used. Otherwise, the exact inverse of the transfer function specified in Table E.4 (Transfer Characteristics) shall be used to convert the non-linear signal to a linear representation.

ccv_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous content colour volume SEI message in output order that is associated with one or more primary picture layers to which this SEI applies. ccv_cancel_flag equal to 0 indicates that content colour volume information follows.

ccv_persistence_flag specifies the persistence of the content colour volume SEI message.

ccv_persistence_flag equal to 0 specifies that the content colour volume applies to the current decoded picture only.

Let picA be the current picture. ccv_persistence_flag equal to 1 specifies that the content colour volume SEI message persists for the current layer in output order until any of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing an content colour volume SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for the picture order count of picB.

ccv_mode_type_flags specifies the representations used to indicate the content colour volume. The value of ccv_mode_type_flags shall not be equal to 0.

The array CcvModeTypeFlag[ ] for i in the range of 0 to 7, inclusive, is derived as follows:

tempFlagVals = ccv_mode_type_flags
for( i = 0; i < 8; i++ ) {
	CcvModeTypeFlag[ i ] = tempFlagVals & 0x01
	tempFlagVals = tempFlagVals  >>  1
}

The value of CcvModeTypeFlag[ i ] equal to 1 for i in the range of 3 to 7, inclusive, is reserved for future use by ITU-T | ISO/IEC.

ccv_gamut_type specifies the type of colour gamut representation. ccv_gamut_type has the same semantics as specified for the colour_primaries syntax element, except that ccv_gamut_type specifies the chromaticity coordinates that specify a colour gamut within which the video content is restricted. When ccv_gamut_type is equal to 255, the chromaticity coordinates that specify the colour gamut are specified by the syntax elements ccv_primary_x[ ] and ccv_primary_y[ ].

number_of_primaries_minus3 plus 3 specifies the total number of colour primaries that are used to describe the content colour gamut boundary. The value of number_of_primaries_minus3 shall be in the range of 0 to 15, inclusive.

ccv_primary_x[ c ] and ccv_primary_y[ c ] specify the normalized x and y chromaticity coordinates, respectively, of the colour primary component c, for c in the range of 0 to 2, inclusive, of the content gamut in increments of 0.00002, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). For describing content gamut using red, green, and blue colour primaries, it is suggested that index value c equal to 0 should correspond to the green primary, c equal to 1 should correspond to the blue primary, and c equal to 2 should correspond to the red colour primary (see also Annex Error: Reference source not found and Error: Reference source not found). The values of ccv_primaries_x[ c ] and ccv_primaries_y[ c ] shall be in the range of 0 to 50 000, inclusive. When the value of number_of_primaries_minus3 is greater than 0, the order of ccv_primary_x[ c ] and ccv_primary_y[ c ] should be in the order of the points that consist the convex hull of the content color gamut.

ccv_min_lum_value specifies the normalized minimum luminance value, according to CIE 1931, that is expected to be present in the content, where values are normalized to Lo or Lc as specified in Table E.4 for the transfer characteristics of the signal.trans,T the values of ccv_min_cv_min_lum_value are in unitsnorements of 00000001 where the values shall be in the  of 0 to 1000010 000 000, inclusive, in t

ccv_max_lum_value specifies the maximum luminance value, according to CIE 1931, that is expected to be present in the content, where values are normalized to Lo or Lc as specified in Table E.4 for the transfer characteristics of the signal.trans, Tthe values of ccv_max_cv_max_lum_value are in unitsnorements of 00000001 where the values shall be in the  of 0 to 1000010 000 000, inclusive in t

ccv_ctb_partitions_y_minus1 specifies the number of vertical partitions for which colour volume information can be signalled.

ccv_ctb_partitions_x_minus1 specifies the number of horizontal partitions for which colour volume information can be signalled.

The height of each partition ccvPartition[ m ][ n ] at index (m,n), for m in the range of 0 to ccv_ctb_partitions_y_minus1, inclusive, and n in the range of 0 to ccv_ctb_partitions_x_minus1, inclusive, is computed as follows:

numYParts = ccv_ctb_partitions_y_minus1 + 1
ccvPartHeight[ m ][ n ] = Round( (m + 1) * PicHeightInCtbsY / numYParts ) 
					 − Round( m * PicHeightInCtbsY / numYParts )

The width of each partition ccvPartition[ m ][ n ] at index (m,n), for m in the range of 0 to ccv_ctb_partitions_y_minus1, inclusive, and n in the range of 0 to ccv_ctb_partitions_x_minus1, inclusive, is computed as follows:

numXParts = ccv_ctb_partitions_x_minus1 + 1
ccvPartWidth[ m ][ n ] = Round( (n + 1) * PicWidthInCtbsY / numXParts ) 
					− Round( n * PicWidthInCtbsY / numXParts )

ccv_num_implicit_repn_minus1[ m ][ n ] plus 1 specifies the number of implicit volume representations of the partition ccvPartition[ m ][ n ] of the content specified in the SEI message. The value of ccv_num_implicit_repn_minus1 shall be in the range of 0 to 6, inclusive. The value of ccv_num_implicit_repn_minus1 in the range of 7 to 255, inclusive, are reserved for future use by ITU-T | ISO/IEC.

ccv_implicit_repn_type[ m ][ n ][ i ] specifies the interpretation, as described in Table 2Table 1, of the syntax elements impl_vol_repn_primary_x[ m ][ n ][ i ][ j ], impl_vol_repn_primary_y[ m ][ n ][ i ][ j ], impl_vol_repn_primary_min[ m ][ n ][ i ][ j ], and impl_vol_repn_primary_max[ m ][ n ][ i ][ j ] and the variables NumValsRepn[ m ][ n ][ i ] and PrimariesPresentFlag[ m ][ n ][ i ]. The value of ccv_implicit_repn_type[ m ][ n ][ i ] shall be in the range of 0 to 6, inclusive. Value of ccv_implicit_repn_type[ m ][ n ][ i ] in the range of 7 to 255, inclusive, are reserved for future use by ITU-T | ISO/IEC.

ccv_impl_repn_primary_x[ m ][ n ][ i ][ j ] and ccv_impl_repn_primary_y[ m ][ n ][ i ][ j ] specify the normalized x and y chromaticity coordinates, respectively, of the colour primary component c for the primary colour volume representation of the partition ccvPartition[ m ][ n ] as interpreted from Table 1 in increments of 0.00002, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). The interpretation of index value j to each primary is as specified in Table 1. The values of ccv_impl_repn_primary_x[ m ][ n ][ i ][ j ] and ccv_impl_repn_primary_y[ m ][ n ][ i ][ j ] shall be in the range of-−  00  00  to 5 000 000, inclusive.

ccv_impl_primary_max[ m ][ n ][ i ][ j ] and ccv_impl_primary_min[ m ][ n ][ i ][ j ] specify the nominal maximum and minimum values, respectively, of the normalized signals of the partition ccvPartition[ m ][ n ] after conversion into the corresponding colour space, in normalized increments of 0.00000001 units, where the colour space is specified in n. When present, ccv_impl_primary_min[ m ][ n ][ i ][ j ] shall be less than ccv_impl_primary_max[ m ][ n ][ i ][ j ].

Table 2: Interpretation of ccv_implicit_repn_type[ ][ ]

| ccv_implicit_ repn_type[ m ][ n ] | Representation space | NumValsRepn[ m ][ n ][ ] | PrimariesPresentFlag[ m ][ n ] | Index correspondence |
| --- | --- | --- | --- | --- |
| 0 | RGB | 3 | 1 | 0: green 1: blue 2: red |
| 1 | RYGCB | 5 | 1 | 0: green 1: cyan 2: blue 3: red 4: yellow |
| 2 | RYGCBM | 6 | 1 | 0: green 1: cyan 2: blue 3: magenta 4: red 5: yellow |
| 3 | YCbCr | 3 | 0 | 0: luma 1: Cb 2: Cr |
| 4 | XYZ | 3 | 0 | 0: X 1: Y 2: Z |
| 5 | HSV | 3 | 0 | 0: H 1: S 2: V |
| 6 | CIECAM02 LMS | 3 | 0 | 0: long (L) 1: medium (M) 2: short (S) |
| 7 – 255 | Reserved for future use by ITU-T/ISO-IEC |  |  |  |

Note X: The YCbCr representation space corresponds to the colour representation utilized by the encoded content.

ccv_num_explicit_repn_minus1 plus 1 specifies the number of explicit colour volume representations of the content specified in the SEI message. The value of ccv_num_explicit_repn_minus1 shall be in the range of 0 to 1, inclusive. The value of ccv_num_explicit_repn_minus1 in the range of 2 to 255, inclusive, are reserved for future use by ITU-T | ISO/IEC.

ccv_explicit_repn_type[ i ] specifies the interpretation, as described in Table 3Error: Reference source not found, of the syntax elements ccv_expl_num_coord_1[ i ][ j ][ k ], ccv_expl_num_coord_2[ i ][ j ][ k ], and ccv_expl_range_value[ i][ j ]. The value of ccv_explicit_repn_type[ i ] shall be in the range of 0 to 2, inclusive. The value of ccv_explicit_repn_type[ i ] in the range of 3 to 255, inclusive, are reserved for future use by ITU-T | ISO/IEC.

ccv_expl_num_ranges_minus1[ i ] plus 1 specifies the number of ranges used in the i-th explicit representation to describe the cross-section of content volume in a colour space as specified by Error: Reference source not found. The value of ccv_explicit_num_ranges_minus1[ i ] shall be in the range of 0 to 255, inclusive. When not present, the value of ccv_explicit_num_ranges_minus1[ i ] is inferred to be equal to 0.

The variable CcvExplNumRanges[ i ] is set equal to ccv_explicit_num_ranges_minus1[ i ] + 1.

ccv_expl_ranges_idc[ i ] equal to 0 specifies that the syntax elements ccv_expl_range_value[ i ][ j ] are explicitly signalled. ccv_expl_ranges_idc[ i ] equal to 1 or 2 specifies that the syntax element ccv_expl_range_value[ i ][ j ] for j in the range of 1 to CcvExplNumRanges[ i ] – 1, inclusive, are not explicitly signalled. When ccv_expl_num_ranges_minus2 is equal to 0, the value of ccv_expl_ranges_idc[ i ] is inferred to be equal to 1. The value of ccv_expl_ranges_idc[ i ] shall be in the range of 0 to 2, inclusive. The value of ccv_expl_ranges_idc[ i ] equal to 3 is reserved for future use by ITU-T | ISO/IEC.

ccv_expl_val[ i ][ j ] is used to derive the j-th value of the component with index 0, as described by the semantics of ccv_explicit_repn_type[ i ] and Error: Reference source not found, which is used to specify the colour volume of the content. The value of ccv_expl_val[ i ][ j ] is in normalized increments of 0.00000001 units as specified for the component with index 0 in .

The variable CcvExplVal[ i ][ j ], for j in the range of 0 to CcvExplNumRanges[ i ], inclusive, is derived as follows:

CcvExplVal[ i ][ 0 ] = ccv_expl_val[ i ][ 0 ]
lumDiffVal = ccv_expl_val[ i ][ CcvExplNumRanges[ i ] ] – ccv_expl_val[ i ][ 0 ]
for( j = 1; j < CcvExplNumRanges[ j ]; i++ )
	if( ccv_expl_ranges_idc  = =  0 )
		CcvExplVal[ i ] = ccv_expl_val[ i ][ j ]
	else if(ccv_expl_ranges_idc = =  1 ) {
		diffVal = lumDiffVal ÷ CcvExplNumRanges[ i ] 
		CcvExplVal[ i ][ j ] = CcvExplVal[ i ][ j − 1 ] + diffVal
	}
	else if(ccv_expl_ranges_idc  = =  2 ) {
		logDiffVal = Log10( lumDiffVal ) ÷ CcvExplNumRanges[ i ]
		CcvExplVal[ i ][ j ] = CcvExplVal[ i ][ 0 ] + 										10( j*logDiffVal )
	}
CcvExplVal[ i ][ CcvExplNumRanges[ i ] ] = expl_vol_val[ i ][ CcvExplNumRanges[ i ] ]

ccv_expl_num_coord[ i ][ j ] specifies the number of points associated with the j-th value of the component with index 0, as described in Error: Reference source not found, of the i-th explicit representation of the colour volume of the content. The value of ccv_expl_num_coord[ i ][ j ] shall be in the range of 0 to 15, inclusive.

ccv_expl_coord_1[ i ][ j ][ k ] and ccv_expl_coord_2[ i ][ j ][ k ] specify the coordinate components with index value 1 and 2, respectively, of the k-th coordinate corresponding to the j-th value of the component with index 0 that is used to derive an estimate of the colour volume in increments of 0.00002. The values of ccv_expl_coord_1[ i ][ j ][ k ] and ccv_expl_coord_2[ i ][ j ][ k ] shall be in the range of −5 000 000 to 5 000 000, inclusive.

For each value of j in the range of 0 to CcvExplNumRanges[ i ], inclusive, let convexRegion[ i ][ j ] refer to the two-dimensional convex hull of the coordinates ccv_expl_coord_1[ i ][ j ][ k ] and ccv_expl_coord_2[ i ][ j ][ k ] for k in the range of 0 to ccv_expl_num_coord[ i ] – 1, inclusive.

The colour volume of the content, colourVolumeBoundingRegion, is defined as the union of colourVolumeRegions[ i ][ j ] for j in the range of 0 to CcvExplNumRanges[ i ] − 1, inclusive, where colourVolumeRegions[ i ][ j ] is defined as the three-dimensional convex hull of the points convexRegion[ i ][ j ] and convexRegion[ i ][ j + 1 ].

Table 3 Interpretation of ccv_explicit_repn_type[ ][ ]

| ccv_explicit_repn_type[ i ] | Representation space |
| --- | --- |
| 0 | xyY; index correspondence 0: Y, 1:x, 2:y |
| 1 | Lab; index correspondence 0: L, 1: a, 2: b |
| 2 | YCbCr: index correspondence 0: Y, 1:Cb, 2: Cr |
| 3 – 255 | Reserved for future use by ITU-T/ISO-IEC |

Table D.1 – Persistence scope of SEI messages (informative)

| - SEI message | - Persistence scope |
| --- | --- |
| Buffering period | - The remainder of the bitstream |
| Picture timing | - The access unit containing the SEI message |
| Pan-scan rectangle | - Specified by the syntax of the SEI message |
| Filler payload | - The access unit containing the SEI message |
| User data registered by Rec. ITU-T T.35 | - Unspecified |
| User data unregistered | - Unspecified |
| Recovery point | - Specified by the syntax of the SEI message |
| Scene information | - The access unit containing the SEI message and up to but not including the next access unit, in decoding order, that contains a scene information SEI message |
| Picture snapshot | - The access unit containing the SEI message |
| Progressive refinement segment start | - Specified by the syntax of the SEI message |
| Progressive refinement segment end | - The access unit containing the SEI message |
| Film grain characteristics | - Specified by the syntax of the SEI message |
| Post-filter hint | - The access unit containing the SEI message |
| Tone mapping information | - Specified by the syntax of the SEI message |
| Frame packing arrangement | - Specified by the syntax of the SEI message |
| Display orientation | - Specified by the syntax of the SEI message |
| Structure of pictures information | - The set of pictures in the CLVS that correspond to entries listed in the SEI message |
| Decoded picture hash | - The access unit containing the SEI message |
| Active parameter sets | - The CVS containing the SEI message |
| Decoding unit information | - The decoding unit containing the SEI message |
| Temporal sub-layer zero index | - The access unit containing the SEI message |
| Scalable nesting | - Depending on the nested SEI messages. Each nested SEI message has the same persistence scope as if the SEI message was not nested |
| Region refresh information | - The set of VCL NAL units within the access unit starting from the VCL NAL unit following the SEI message up to but not including the VCL NAL unit following the next SEI NAL unit containing a region refresh information SEI message (if any) |
| No display | - The access unit containing the SEI message |
| Time code | - The access unit containing the SEI message |
| Mastering display colour volume | - The CLVS containing the SEI message |
| Segmented rectangular frame packing arrangement | - Specified by the syntax of the SEI message |
| Temporal motion-constrained tile sets | - The CLVS containing the SEI message |
| Chroma resampling filter hint | - The CLVS containing the SEI message |
| Knee function information | - Specified by the syntax of the SEI message |
| Colour remapping information | - Specified by the syntax of the SEI message |
| Deinterlaced field identification | - One or more pictures associated with the access unit containing the SEI message |
| Content light level information | - The CLVS containing the SEI message |
| Dependent RAP indication | - The access unit containing the SEI message |
| Coded region completion | - The current slice segment associated with the SEI message |
| Alternative transfer characteristics information | - The CLVS containing the SEI message |
| Ambient viewing environment | - The CLVS containing the SEI message |
| Content Colour Volume | - Specified by the syntax of the SEI message |

# References

- ISO/IEC 23008:2016 Information technology -- High efficiency coding and media delivery in heterogeneous environments -- Part 2: High efficiency video coding, Second Edition

- SMPTE ST 2094-1:2016: Dynamic Metadata for Color Volume Transform – Core Components

- A.M. Tourapis, Y. Su, and D. Singer, “Effective Colour Volume SEI”, JCTVC document JCTVC-V0038, Geneva, SW, Oct. 2015

- Hyun Mook Oh, Jangwon Choi, and Jong-Yeul Suh, “Content colour gamut SEI message”, JCTVC document JCTVC-W0057, San Diego, USA, Feb. 2016

- A.M. Tourapis, Y. Su, and D. Singer, “Effective Colour Volume SEI”, JCTVC document JCTVC-W0098, San Diego, USA, Feb. 2016

- Hyun Mook Oh, Jangwon Choi, and Jong-Yeul Suh, “Content colour gamut SEI message”, JCTVC document JCTVC-X0040, Geneva, SW, Jun. 2016

- A.M. Tourapis, Y. Su, and D. Singer, “Improvements to the proposed effective colour volume SEI message”, JCTVC document JCTVC-X0052, Geneva, SW, Jun. 2016

- A. K. Ramasubramonian, D. B. Sansli, J. Sole, D. Rusanovskyy, and M. Karczewicz, “Content colour volume SEI message”, JCTVC document JCTVC-X0069, Geneva, SW, Jun. 2016

- C. Fogg, J. Helman, M. Smith, and M. Zink, “Content light level information SEI”, MPEG/JCTVC document M35567/JCTVC-T0101, Switzerland, Feb. 2015

- Recommendation ITU-R BT.2020, “Parameter values for ultra-high definition television systems for production and international programme exchange” (2012).

- Recommendation ITU-R BT.709-5, “Parameter values for the HDTV standards for production and international programme exchange”

- SMPTE ST 2084:2014, “High Dynamic Range Electro-Optical Transfer Function of Mastering Reference Displays”, 2014

- Society of Motion Picture and Television Engineers EG 432-1 (2010), Digital Source Processing – Color Processing for D-cinema

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).