| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting: Chengdu, CN, 14–21 October 2016 | Document: JCTVC-Y0051 |
| --- | --- |

| Title: | BoG report on Content Colour Volume SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | BoG meeting report |  |  |
| Author(s) or Contact(s): | E. François | Tel: Email: | edouard.francois@technicolor.com |
| Source: | Break out Group |  |  |

_____________________________

# Abstract

This document provides a report of the BoG on Content Colour Volume SEI message.

# Introduction

A Break-out group (BoG) meeting on “Content Colour Volume SEI message” has taken place on Oct. 16 2016 from 10.00 to 12.05.

The mandate of the BoG was to further analyze the proposed technologies (Y0032 and Y0040), to clarify the use cases, and to recommend possible next steps.

- Participants:

- Adarsh Ramasubramonian

- Ajay Luthra

- Alexis Tourapis

- Edouard François

- Hyun Mook Oh

- Peng Yin

# Proposals summary

- A short review of the different solutions was made.

- Y0032

- 3 primaries in xyY + min, max and average Luminance + optional 3 cross-sections, with 3 primaries per cross-section (1 section for min lum, 1 for max lum, 1 for avg lum)

  - Worst case max size 	 8 + 3*32 + 96 + 3*(3*32)  500 bits (30kbps at 60fps)

- Y0040

- 3 non-exclusive modes

- Mode 1 (CcvModeTypeFlag[ 0 ]) – 3 to 18 primaries in xyY (polygon) + min and max Luminance

  - Worst case max size 	 18*32 + 64  640-650 bits (40kbps at 60fps)

- Mode 2 (CcvModeTypeFlag[ 1 ]) – picture divided in partitions – for each partition, up to 7 colour volume representations can be used – for each color volume, 0 to 6 xyY primaries signaled + min and max Luminance, depending on representation space (RGB, RYGCB, RYGCBM, YCbCr, XYZ, HSV, CIECAM02 LMS)

  - Worst case max size 	 ( nbPartitions * 7 * 6 * (32+48) ) + 16 + X bits
			 16+X+ nbPartitions*3360 bits

  - nbPartitions coded on X bits (2 ue(v)), no max value specified

    - if 16 partitions  53770 bits (3000kbps at 60fps)

  - can send 7 versions of the 6 primary case, using 6 different primary values; allows to create different volumes and define the final volume as the cross-section (even with that same representation).

- Mode 3 (CcvModeTypeFlag[ 2 ]) – up to 2 colour volumes – for each colour volume, 1 representation (xyY, Lab, YCbCr) – up to 256 ranges (cross-section of colour volume) per colour volume – up to 16 primaries (polygon) per cross-section

  - Worst case max size 	 8 + 2 * (8 + 256*(32+16*48)) bits
			 409624 bits (25000kbps at 60fps)

  - Proponent does not believe that worst case complexity (metadata bit rate) may be exercised by a reasonable encoder, and indicates that the ranges were put large due to what proponent thought was common in JCT-VC (8 cross-sections, 8 primaries per section was judged as a reasonable maximum).

  - Expected max size	 8 + 2 * (8 + 8*(32+8*48)) bits
			 6680 bits (400kbps at 60fps)

- It was asked why 3 exclusive modes were needed

- Proponents explained that Mode1 is needed, as the simplest signalling. Mode3 is an extension of mode1, and is required for finer representation. Mode2 is an alternative to mode3.

# Use cases discussion

Use cases requiring such a new SEI message were discussed. The goal was to clarify what gain and how much gain would be obtained from having a precise colour volume description.

- Alexis presentation (based on previous contribution JCTVC-V0038)

  - Content often comes in limited colour gamut, limited brightness, even if larger gamut container is used to represent the content

  - Currently, there is no precise description of the content colour volume; the current syntax can indicate the container colour volume (VUI) or limited description of the content (MDCV and CLL SEIs)

  - An accurate color gamut description is relevant for processing applications such as colour volume mapping (incl. gamut mapping, display adaptation), adaptive clipping, adaptive precision processing

    - An example of adaptive Clipping using cross-section based description is shown (Cb/Cr min/max depending on Y value)

  - For hardware implementations, limited bit-depth is required (e.g. 10bits). It would be beneficial to know the actual content gamut boundaries to better use this limited bit-depth representation (typically by applying reshaping of the actual content to this bit-depth), for more accurate processing

- Regarding spatial partitioning feature

  - The proponent indicates that the benefits are to allow parallel processing, precise process per region; this could allow control of localized power management on the display (e.g. LED regions)

  - It was indicated that this feature is actually not tested, and the gain is hypothetical

  - It was noted that the spatial partitioning concept can apply to the 3 modes of Y0040, or to Y0032

  - It was also noted that spatial partitioning concept is also used in ST 2094 (but this is for color grading/ production side)

- Gain from cross-section based description?

  - Example of adaptive clipping mentioned by Alexis shows potential interest of such a description

  - It was indicated that the typical application of this is gamut mapping

- There is a consensus among BoG participants that a content colour volume information is needed in addition to existing SEIs/VUI signalling – Question is: what level of accuracy is needed

- 3 main questions to answer

  - Do we need spatial partitioning

  - Do we need accurate color volume description

    - If yes, what type of accuracy do we need (e.g. do we need cross-section based representation)

  - What impact on complexity in pre-/post-processing

- Tests would be needed to answer these questions  it was suggested to set-up a verification experiment to address these questions

  - Start with a light SEI with single partition signalling, 3 primaries RGB, min/max luminance as anchor

  - Goal: show any benefit of additional information in the SEI – if benefit is established, then propose an enhanced SEI message

  - No consensus on this point, 3 proponents agreed to have a verification experiment, 1 proponent objected that such tests would oblige companies to disclose their proprietary processing

  - One discussed option was: test could be done with only reporting results of processing, without describing the processing

  - Details of potential verification experiment to be discussed (test processing chain, test content, usage of compression or not)

# Recommendations

It is recommended to discuss in JCT-VC the possibility to set-up a verification test to evaluate the benefits of using spatial/luminance/precise primaries partitioning for content colour volume description.

If JCT-VC decides to set-up this verification test, it is further recommended:

- to perform experiments from the 25th to 26th JCT-VC meeting;

- to report verification experiment results at the 26th JCT-VC meeting, with conclusions regarding the data required to be signalled in the proposed content colour volume SEI.