| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting: Chengdu, CN, 14–21 October 2016 | Document: JCTVC-Y0051r1 |
| --- | --- |

| Title: | BoG report on Content Colour Volume SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | BoG meeting report |  |  |
| Author(s) or Contact(s): | E. François | Tel: Email: | edouard.francois@technicolor.com |
| Source: | Break out Group |  |  |

_____________________________

# Abstract

This document provides a report of the BoG meetings on Content Colour Volume SEI message.

has taken place on O 10 to 12am, a s on Oct. 6

Introduction

A first break-out group (BoG) meeting on “Content Colour Volume SEI message” has taken place on Oct. 15 2016 from 10.00 to 12.05.

The mandate of the BoG was to further analyze the proposed technologies (Y0032 and Y0040), to clarify the use cases, and to recommend possible next steps.

- Participants:

- Adarsh Ramasubramonian

- Ajay Luthra

- Alexis Tourapis

- Edouard François

- Hyun Mook Oh

- Peng Yin

- Section 2 to 4 relate to this first BoG meeting.

A second BoG meeting on “Content Colour Volume SEI message” has taken place on Oct. 16 2016 from 2 to 6 p5.30 am.

The mandate of the BoG was to draft the content colour volume SEI used as starting point, and to draft an initial verification experiment description.

- Participants:

- Adarsh Ramasubramonian

- Alexis Tourapis

- Edouard François

- Hyun Mook Oh

- Peng Yin

- Walt Husak

Section 5 relate to this second BoG meeting.

# Proposals summary

- A short review of the different solutions was made.

- Y0032

- 3 primaries in xyY + min, max and average Luminance + optional 3 cross-sections, with 3 primaries per cross-section (1 section for min lum, 1 for max lum, 1 for avg lum)

  - Worst case max size 	 8 + 3*32 + 96 + 3*(3*32)  500 bits (30kbps at 60fps)

- Y0040

- 3 non-exclusive modes

- Mode 1 (CcvModeTypeFlag[ 0 ]) – 3 to 18 primaries in xyY (polygon) + min and max Luminance

  - Worst case max size 	 18*32 + 64  640-650 bits (40kbps at 60fps)

- Mode 2 (CcvModeTypeFlag[ 1 ]) – picture divided in partitions – for each partition, up to 7 colour volume representations can be used – for each color volume, 0 to 6 xyY primaries signaled + min and max Luminance, depending on representation space (RGB, RYGCB, RYGCBM, YCbCr, XYZ, HSV, CIECAM02 LMS)

  - Worst case max size 	 ( nbPartitions * 7 * 6 * (32+48) ) + 16 + X bits
			 16+X+ nbPartitions*3360 bits

  - nbPartitions coded on X bits (2 ue(v)), no max value specified

    - if 16 partitions  53770 bits (3000kbps at 60fps)

  - can send 7 versions of the 6 primary case, using 6 different primary values; allows to create different volumes and define the final volume as the cross-section (even with that same representation).

- Mode 3 (CcvModeTypeFlag[ 2 ]) – up to 2 colour volumes – for each colour volume, 1 representation (xyY, Lab, YCbCr) – up to 256 ranges (cross-section of colour volume) per colour volume – up to 16 primaries (polygon) per cross-section

  - Worst case max size 	 8 + 2 * (8 + 256*(32+16*48)) bits
			 409624 bits (25000kbps at 60fps)

  - Proponent does not believe that worst case complexity (metadata bit rate) may be exercised by a reasonable encoder, and indicates that the ranges were put large due to what proponent thought was common in JCT-VC (8 cross-sections, 8 primaries per section was judged as a reasonable maximum).

  - Expected max size	 8 + 2 * (8 + 8*(32+8*48)) bits
			 6680 bits (400kbps at 60fps)

- It was asked why 3 exclusive modes were needed

- Proponents explained that Mode1 is needed, as the simplest signalling. Mode3 is an extension of mode1, and is required for finer representation. Mode2 is an alternative to mode3.

# Use cases discussion

Use cases requiring such a new SEI message were discussed. The goal was to clarify what gain and how much gain would be obtained from having a precise colour volume description.

- Alexis presentation (based on previous contribution JCTVC-V0038)

  - Content often comes in limited colour gamut, limited brightness, even if larger gamut container is used to represent the content

  - Currently, there is no precise description of the content colour volume; the current syntax can indicate the container colour volume (VUI) or limited description of the content (MDCV and CLL SEIs)

  - An accurate color gamut description is relevant for processing applications such as colour volume mapping (incl. gamut mapping, display adaptation), adaptive clipping, adaptive precision processing

    - An example of adaptive Clipping using cross-section based description is shown (Cb/Cr min/max depending on Y value)

  - For hardware implementations, limited bit-depth is required (e.g. 10bits). It would be beneficial to know the actual content gamut boundaries to better use this limited bit-depth representation (typically by applying reshaping of the actual content to this bit-depth), for more accurate processing

- Regarding spatial partitioning feature

  - The proponent indicates that the benefits are to allow parallel processing, precise process per region; this could allow control of localized power management on the display (e.g. LED regions)

  - It was indicated that this feature is actually not tested, and the gain is hypothetical

  - It was noted that the spatial partitioning concept can apply to the 3 modes of Y0040, or to Y0032

  - It was also noted that spatial partitioning concept is also used in ST 2094 (but this is for color grading/ production side)

- Gain from cross-section based description?

  - Example of adaptive clipping mentioned by Alexis shows potential interest of such a description

  - It was indicated that the typical application of this is gamut mapping

- There is a consensus among BoG participants that a content colour volume information is needed in addition to existing SEIs/VUI signalling – Question is: what level of accuracy is needed

- 3 main questions to answer

  - Do we need spatial partitioning

  - Do we need accurate color volume description

    - If yes, what type of accuracy do we need (e.g. do we need cross-section based representation)

  - What impact on complexity in pre-/post-processing

- Tests would be needed to answer these questions  it was suggested to set-up a verification experiment to address these questions

  - Start with a light SEI without spatial partition signalling, 3 colour primaries expressed in CIE1931 xyY color representation, min/max luminance as anchor

  - Goal: show any benefit of additional information in the SEI – if benefit is established, then propose an enhanced SEI message

  - No consensus on this point, 3 proponents agreed to have a verification experiment, 1 proponent objected that such tests would oblige companies to disclose their proprietary processing

  - One discussed option was: test could be done with only reporting results of processing, without describing the processing

  - Details of potential verification experiment to be discussed (test processing chain, test content, usage of compression or not)

# Recommendations

It is recommended to discuss in JCT-VC the possibility to set-up a verification test to evaluate the benefits of using spatial/luminance/precise primaries partitioning for content colour volume description.

If JCT-VC decides to set-up this verification test, it is further recommended:

- to perform experiments from the 25th to 26th JCT-VC meeting;

- to report verification experiment results at the 26th JCT-VC meeting, with conclusions regarding the data required to be signalled in the proposed content colour volume SEI.

# Verification experiment description discussion (2nd BoG meeting)

## Purpose

- The goal of the verification experiment is:

- to show any benefit of specifying a basic (minimum) content colour volume SEI message, noted reference SEI, as defined in section xx below;

- to show any benefit of specifying additional information in the SEI;

- if benefit is established, to propose an enhanced SEI message syntax.

- The concept of container colour volume, mastering display colour volume and content colour volume is illustrated in Figure 1 (from JCTVC-X0040).

Figure 1. Illustration on ‘Market3’ BT709 test sequence of concept of different colour gamut (left, ‘Market3’ picture, right, actual distribution of colour samples and boundary of different colour gamuts) – from JCTVC-X0040

## Reference SEI message syntax and semantics

### General SEI syntax structure

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| … |  |
| else if( payloadType = = 149 ) |  |
| content_colour_volume( payloadSize ) |  |
| … |  |
| } |  |

### Content colour volume SEI message syntax

| content_colour_volume( payloadSize ) { | Descriptor |
| --- | --- |
| ccv_cancel_flag | u(1) |
| if( !ccv_cancel_flag ) { |  |
| ccv_persistence_flag | u(1) |
| for( c = 0; c < 3; c++ ) { |  |
| ccv_primaries_x[ c ] | u(16) |
| ccv_primaries_y[ c ] | u(16) |
| } |  |
| ccv_min_lum_value | u(32) |
| ccv_max_lum_value | u(32) |
| } |  |
| } |  |

### Semantics of content colour volume SEI message

The content colour volume SEI message provides information about the colour volume characteristics of the associated picture. This SEI describes the colour volume characteristics of the associated picture in terms of a nominal range and deviations from this range may occur. This SEI message requires knowledge of the colour description information of the signal, such as the colour_primaries, transfer_characteristics, and matrix_coeffs in the VUI parameters syntax. This SEI message shall not be present when any of these parameters is unspecified or reserved. When the value of transfer_characteristics is equal to 1, 6, 7, 14, or 15, then the BT.1886 transfer function should be used to convert the signal back to its linear representation. When the value of transfer_characteristics is equal to 18, the EOTF specified in BT.2100 when using the HLG transfer function shall be used. Otherwise, the exact inverse of the transfer function specified in Table E.4 (Transfer Characteristics) shall be used to convert the non-linear signal to a linear representation.

ccv_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous content colour volume SEI message in output order that is associated with one or more primary picture layers to which this SEI applies. ccv_cancel_flag equal to 0 indicates that content colour volume information follows.

ccv_persistence_flag specifies the persistence of the content colour volume SEI message.

ccv_persistence_flag equal to 0 specifies that the content colour volume applies to the current decoded picture only.

Let picA be the current picture. ccv_persistence_flag equal to 1 specifies that the content colour volume SEI message persists for the current layer in output order until any of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing an content colour volume SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for the picture order count of picB.

ccv_primaries_x[ c ] and ccv_primariesyy[ c ] specify the normalized x and y chromaticity coordinates, respectively, of the colour primary component c, for c in the range of 0 to 2, inclusive, of the content gamut in increments of 0.00002, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). For describing content gamut using red, green, and blue colour primaries, it is suggested that index value c equal to 0 should correspond to the green primary, c equal to 1 should correspond to the blue primary, and c equal to 2 should correspond to the red colour primary (see also Annex E 	and Table E.3).

The values of ccv_primariesix[ c ] and ccv_primariesyiesy ] shall be in the range of 0 to 50 000, inclusive.

The values of ccv_primaries_x[ c ] and ccv_primariesyy[ c ] shall be in the range of −5 000 000 to 5 000 000, inclusive.

ccv_min_lum_value specifies the normalized minimum luminance value, according to CIE 1931, that is expected to be present in the content, where values are normalized to Lo or Lc as specified in Table E.4 for the transfer characteristics of the signal. The values of ccv_min_lum_value are in normalized increments of 0.00000001 in the linear representation.

ccv_max_lum_value specifies the maximum luminance value, according to CIE 1931, that is expected to be present in the content, where values are normalized to Lo or Lc as specified in Table E.4 for the transfer characteristics of the signal. The values of ccv_max_lum_value are in normalized increments of 0.00000001 inclusive in the linear representation.

## S showcase  reference SEI

- The goal is to assess the benefit of using the reference content colour volume SEI message.

TEST1 - BT.709 content in YCbCr 4:2:0 BT.2020 container, represented in YCbCr 4:2:0 10b, to be displayed on BT.709 display

- Participant: LG

- TEST1-Ref – use only VUI information, apply gamut compression (colour gamut tailoring) from BT.2020 to BT.709 before rendering on BT.709 display

- TEST1 – use reference SEI, convert to BT.709 with clipping in RGB domain before rendering on BT.709 display

- Test content: BT.709 SDR versions, converted in BT.2020 container, of the JCTVC HDR test content

- Check:

  - Objective metrics (PSNR) – test 709 SDR vs source 709 SDR (both in 709 container)

  - Visual test – ,omparing content resulting from TEST1-Ref condered ing from TE SDR display, to content resulting from TEST1 rendered on a same display content rendering from TEST1,

Figure 2. TEST1 test chain.

- Note: does not assess the need for the syntax elements min/max luminance.

Figure 3. Comparison of display process using and without using content colour gamut. (left, colour gamut clipping result when content and display colour gamut are matched, right, gamut mapping result using container colour gamut only) – from JCTVC-X0040

## Enhanced tests

- The goal is to demonstrate the benefit of using an enhanced SEI message, compared to using the reference content colour volume SEI message. The following enhanced tests are considered.

- TEST32 – BT.709 or P3D65 content in BT.2020 container, represented in YCbCr 4:2:0 10b, with compression, to be displayed on BT.709 display

- Partcipant: DLB

- TEST2-Ref – use reference SEI, tone map (same as TEST2, but using only min/max lum) and convert to BT.709 with clipping in RGB domain, then display

- TEST2 – enhanced mapping using enhanced SEI and colour volume mapping (note: this test is designed to see if additional content luminance statistics is needed; so colour volume mapping might probably mainly focus on tone mapping. For gamut mapping, simple clipping might be considered).gamut mapping

  - Note: DLB does not plan to disclose the gamut mapping algo source or exe

- Concerns: gain could also come from gamut mapping algorithm, how can we differentiate gains from the algo and gains from enhanced SEI description

  - TEST2a – using the DLB colour volume mapping algogath additional source content luminance statistics signaled in the enhanced SEI)

  - TEST2b – using the DLB colour volume mapping algogath the same statistics as in Test3a, derived from the decoded content – can become the reference if it is better that TEST2-Ref?

  - Proponent will provide source content luminance statistics and luminance statistics estimated from the decoded content, and if possible, numbers about complexity to derive these statistics at decoder side

- Test content: JCTVC BT.709 HDR test content in BT.2020 container, P3D65 HDR test content in BT.2020 container from DLB (e.g. the ones used for MPEG CfE)

    - TEST2-Ref and TEST2?

    - If BT.709 SDR display used, a tone mapping HDR-to-SDR isired lgorithm?ffor both TEST2-Ref and TEST2

- Check: Visual test – comparing content resulting from TEST2-Ref rendered on a BT.709 SDR display, to content resulting from TEST2 rendered on a same displaycomparing content rendering from TEST2-Ref on a BT.709 SDR display, to content rendering from TEST2, on a BT.709 SDR display – check visual conformity with comparing the source HDR content rendereding on a BT.2020 HDR display, the converted content rendering from TEST2-Ref on a BT.709 display, the converted content rendering from TEST2a on a BT.709 display, the converted content rendering from TEST2b on a BT.709 display

Figure 4. TEST2 test chain.

- TEST43 – Degraded BT.709 signal in BT.2020 container, to be displayed on BT.2020 display

- Degrded by 4:4:4 to 4:2:0 then go back to 4:4:4, and/or by Compression

- Participant: APPLE

- TEST3-RefeRefereneference SEI, convert degraded content to BT.709 with clipping in RGB domain, go back to BT.2020 container, then display

- TEST3 – same concept as reference (convert degraded content to BT.709 with clipping, go back to BT.2020 container, then display), exploiting partitioning and additional volume description

- Test content: JCTVC HDR in BT.2ntent

- Check:

  - Objective results (e.g. metrics from HDRMetrics)

  - Visual test – comparing the source HDR rendering on a BT.2020 HDR display, the reconstructed content rrom TEST3-Ref rendered on same display, the reconstructed content rendering frendered on same display

- TEST4 (tentative) – adaptive precision processing

- Participant: APPLE

- TBD

- TEST5 (tentative) – benefit from more than one luminance cross-section

- Participant: QCM

- TBD

- Tests above are not exclusive. The proponents can bring any additional result to demonstrate the benefits of their proposal.

## Test content

It is recommended to use the JCTVC HDR content described  u HDR CTCs (JCTVC-X1020), and when needed, theit SDR vsion (the provided SDR versions are in BT.709 container; when needed by the test, a conversion into BT.2020 container will be maderepresente

For Test2, P3D65 HDR test content in BT.2020 container from DLB used for MPEG CfE is also planned to be used.

It was further suggested that new content could be generated using existing MPEG material with different properties. For example, a montage sequence combining an SDR and an HDR clip, both in BT.2100 representation could be used for the tests.

## Timeline

4 weeks editing after 25th meeting

Report results in 26th meeting – visual sessions to be planed (HDR BT.2020 monitor + SDR BT.709 monitor)