| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0003 |
| --- | --- |

| Title: | JCT-VC AHG report: HEVC HM and HDRTools software development and software technical evaluation (AHG3) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Ad-hoc group report |  |  |
| Author(s) or Contact(s): | Karsten Suehring (chair) Karl Sharman Alexis Tourapis | Email: | karsten.suehring@hhi.fraunhofer.de karl.sharman@eu.sony.com atourapis@apple.com |
| Source: | AHG chairs |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AhG on HEVC HM and HDRTools software development and software technical evaluation that have taken place between the 25th and 26th JCT-VC meetings. Activities focused on integration of software adoptions and software maintenance, i.e. code tidying and fixing bugs.

# Introduction

The mandates given to the AhG are as follows:

•	Coordinate development of the HM and HDRTools software and its distribution.

•	Produce documentation of software usage for distribution with the software.

•	Prepare and deliver HM 16.x “point” software versions according to JCTVC-X1100 and X1020 common conditions.

•	Suggest configuration files for additional testing of tools.

•	Investigate how to minimize the number of separate codebases maintained for group reference software.

•	Coordinate with AHG2 on HEVC test model editing and errata reporting to identify any mismatches between software and text.

A brief summary of activities related to each mandate is given below. In particular, for the HM, the following activities were performed:

- HM16.13 and HM 16.14 released after the previous meeting.

- Changed configuration files to match the meeting decisions (search range, QP adaptation).

- Updated the common test conditions document to fix an error in the 4:2:0 CTC Excel sheet.

- In addition, some minor bug fixes and cleanups were addressed. The distribution of the software was made available through the SVN server set up at HHI, as announced on the JCT-VC email reflector, and http://hevc.info has been updated.

- There are a number of reported software bugs that should be fixed.

For HDRTools, the following activities were performed:

- V0.13 was released immediately after the previous meeting.

- The configuration conversion files were modified to match the current recommended practices document  and common test conditions documents.

- A new development branch v0.14-dev was created with the following changes compared to V0.13.

  - Added support for non-linearly encoded EXR video data

  - Added support for generalized scaling using Lanczos filters

  - Added display mapping modules based on ITU-R BT.2390

  - Extended the power law transfer function to now support controllable dark level and peak brightness parameters.

  - Fixed I/O bugs relating to RGB raw input files

  - Fixed bugs in some distortion metrics supported, such as regional PSNR and xPSNR.

  - Added option to forcibly clip the input data based on the representation range

  - Inclusion of the GamutTest tool for color gamut analysis

  - Support of a single input source in the HDRMontage tool

  - Fixed several other bugs and performed cleanups in the software, mostly related to I/O issues.

- A beta version, v0.14-beta was released to a few interested individuals. A new release is expected to become publicly available during this meeting.

# History of HM revisions

## HM16.13

HM16.13 was tagged on 8 November and announced on the reflector. It included the following modifications:

- Enable adaptive QP setting (JCTVC-X0038)

- Fix an uninitialized memory access issue which could cause different encoding results on different platforms and compiler settings

- Enable JCTVC-X0038 in CTC configuration files.

The table below summarizes the changes relative to HM16.12:

|  | Random Access Main |  |  | Random Access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1,4% | -8,9% | -10,1% | -1,6% | -9,4% | -10,1% |
| Class B | -2,2% | -5,8% | -7,7% | -2,4% | -6,2% | -7,8% |
| Class C | -3,3% | -7,2% | -7,7% | -3,3% | -7,2% | -7,5% |
| Class D | -2,6% | -7,4% | -7,6% | -2,7% | -7,7% | -7,9% |
| Class E |  |  |  |  |  |  |
| Overall | -2,3% | -7,3% | -8,2% | -2,5% | -7,5% | -8,3% |
|  | -2,7% | -7,1% | -8,0% | -2,9% | -7,3% | -8,1% |
| Class F | -3,0% | -4,2% | -4,4% | -3,1% | -4,6% | -4,2% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

## HM16.14

HM16.14 was tagged on 28 November. The changes were

- Addition of the ability to repeat parameter sets in IRAP pictures.

- Enable repetition of IRAP pictures in CTC configuration files.

  - This has primarily impacted small intra and random-acccess sequences at higher QPs (this does not affect low-delay configurations).

- Increase of search range from 64 to 256 to match JEM (see section 5).

  - This has had negligible effect on coding efficiency, but has increased run-time by 15%.

The table below summarizes the changes relative to HM16.13:

|  | Random Access Main |  |  | Random Access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0,3% | -0,8% | -1,2% | -0,3% | -1,1% | -2,0% |
| Class B | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% | -0,1% |
| Class C | 0,1% | 0,0% | 0,1% | 0,1% | 0,0% | 0,0% |
| Class D | 0,3% | 0,3% | 0,3% | 0,3% | 0,3% | 0,3% |
| Class E |  |  |  |  |  |  |
| Overall | 0,0% | -0,2% | -0,2% | 0,0% | -0,2% | -0,4% |
|  | 0,0% | -0,2% | -0,2% | 0,0% | -0,2% | -0,4% |
| Class F | -0,3% | -0,4% | -0,4% | -0,4% | -0,4% | -0,4% |
| Enc Time[%] | 115% |  |  | 115% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Towards HM16.15

There are a number of agreed modifications still to be included:

- The adopted changes in JCTVC-Y0038 that include changes in the GOP settings, which require coordination with JVET for JEM development.

- The Main 10 Still Picture Profile discussed in JCTVC-Y0043, but as JCTVC-Y1000 indicates during the joint meeting, there is no specific signalling defined yet.

- The cross-component peak signal to noise ratio calculation, as discussed in JCTVC-Y0037.

# History of HDRTools revisions

## HDRTools v0.13

Version 0.13 of HDRTools was tagged on November 4th, 2016

## Towards HDRTools v0.14

A new branch v0.14-dev was created for the continuing development of the HDRTools software. Currently this new branch contains the following modifications versus v0.13:

- Added support for non-linearly encoded EXR video data

- Added support for generalized scaling using Lanczos filters

- Added display mapping modules based on ITU-R BT.2390

- Extended the power law transfer function to now support controllable dark level and peak brightness parameters.

- Fixed I/O bugs relating to RGB raw input files

- Fixed bugs in some distortion metrics supported, such as regional PSNR and xPSNR.

- Added option to forcibly clip the input data based on the representation range

- Inclusion of the GamutTest tool for color gamut analysis

- Support of a single input source in the HDRMontage tool

- Fixed several other bugs and performed cleanups in the software, mostly related to I/O issues.

Generalized scaling has only been tested for EXR input data and on linear representation data. Tests are planned after the upcoming release for other input sources, including YCbCr 4:2:0 data, and if software problems are determined, to be fixed before any subsequent release (after v0.14). Support of also non-linear, fixed precision scaling is also planned to be supported and tested. Other display mapping mechanisms may also be added if there is sufficient interest by the MPEG community.

# CTC 4:2:0 Excel sheet update

A bug was found in the 4:2:0 CTC Excel template provided in JCTVC-X1100-v1, which caused incorrect summary reporting. Fortunately this document was not widely used.

In future contributions only the templates provided in JCTVC-X1100-v2 should be used.

# CTC Changes

At the last meeting the motion estimation search range was increased from 64 to 256 based on improvements that were found with JEM under JVET CTC. However, during the development of HM16.14, similar improvements could not be found under the new JCT-VC CTCs.

Further analysis revealed that in JVET CTC the gains are originating purely from the sequences in the new classes A1 and A2. These sequences seem to have increased motion activity, which is not present in the JCT-VC test-set.

This explains why no significant improvement in BD-rate performance was noticed using the JCT-VC CTCs. However, the increased search range increases coding time by 15%, which seems unnecessary based on the coding results.

# 360 degree library

The 360 degree library has been reviewed. It was not included at this point as it was seen as too invasive to the core of HEVC. The coordinators have prepared, but not yet shared and discussed, an alternative patch to demonstrate how this library could be included with less impact on HM and its various branches.

# Conformance software

A patch has been prepared by the coordinators and is being reviewed that optionally checks the ranges of values decoded in the decoder, warning or exiting if the values do not conform to the indicated profile/level/tier settings. This also includes checks that the access unit fits within the coded picture buffer.

# Screen content coding model

There has been an interest by the JCT-VC to integrate the SCM into the main HM branch – in affect making SCM the new trunk from which the other branches (eg. 3D-HEVC, MV-HEVC, SHM) are then based upon.

After initial comments between the HM coordinators to the SCM coordinators during the previous meeting cycle, no other action has been reported.

# Further work

The following are persistent bug reports where study is encouraged:

- High level picture types: IRAP, RASL, RADL, STSA:

Tickets #1096, #1101, #1333, #1334, #1346.

- Rate-control and QP selection – numerous problems with multiple slices:
		Tickets #1314, #1338, #1339.

- Field-coding:
		Tickets #1145, #1153.

- Decoder picture buffer:

Tickets #1277, #1286, #1287, #1304.

- NoOutputOfPriorPicture processing:

Tickets #1335, #1336, #1393.

- Additional decoder checks:

Tickets #1367, #1383.

As described to the community at the last four JCT-VC meetings, alterations to remove the unused software hierarchy in the entropy coding sections of the code, and to remove terms such as CAVLC is being considered. However, this will now need to also consider the impact on the JEM branch.

Further testing and possibly extensions of the newly added scaling support in HDRTools, as well integration of other display mapping mechanisms, is currently in progress.

# Recommendations

- Continue to develop reference software based on HM version 16.14 and HDRTools version 0.14 and improve their quality.

- Revert change in search range

- Test reference software more extensively outside of common test conditions.

- Add more conformance checks to the decoder to more easily identify non-conforming bit-streams, especially for profile and level constraints.

- Encourage people who are implementing HEVC based products to report all (potential) bugs that they are finding in that process.

- Encourage people to submit bit-streams that trigger bugs in the HM. Such bit-streams may also be useful for the conformance specification.

- Continue to investigate the merging of branches with the other software coordinators.