| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0026 |
| --- | --- |

| Title: | SEI message for signaling of 360-degree video information |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sejin Oh Hyunmook Oh 19, Yangjae-daero 11gil Seocho-gu, Seoul 137-130, South Korea | Email: | sjin.oh@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document suggests a SEI message for signaling of 360-degree video information in video bistreams. In particular, the proposed SEI message is used to indicate projection format describing how a spherical video is mapped to a 2D planer video and to specify the spherical surface area to which the coded picture is projected. It also specifies cube face packing parameters for the content that the cube map projection is applied.

# Introduction

Most of capturing equipment for 360-degree video use two fish-eye cameras or multiple cameras to capture the scene. The captured multiple images from cameras are stitched, and then 360-degree video are generated. Depending on applications, some of 360-degree videos only capture and include a partial range, e.g., 180 x 180 degree, of the sphere. The 180-degree video is more used for sports events in a fixed setup, such as a stadium, whereas the 360-view is more used for documentary, news and music video clips. Omnidirectional media application format (OMAF) have considered how to support a requirement “OMAF shall support the storage, delivery (not new mechanism) and rendering of omnidirectional (up to 360° panoramic) coded (with specific configuration) image/video (monoscopic and stereoscopic)” [1].

Figure 1 describes stereoscopic 180 video formats that one of views covers 180 x 180 degrees video and two views are packed to one image frame.

Figure 1. Example of stereoscopic 180 video [2]

Figure 2 describes cases when same resolution of coded pictures covers different surface area of the sphere. Even though the coded picture is coded as same resolution, same 2D point in a coded picture can be projected to a different point onto the spherical surface according to covering area of the sphere by the coded picture. Thus, in order to enable a device to correctly project regions in coded pictures to area onto the sphere, we need to consider a way to indicate spherical surface area to which the coded picture is projected.

| (a) a full spherical (360x180) video | (b) a non-full spherical (180x180) video |
| --- | --- |

Figure 2. Examples of the coded pictures covering different surface area of the sphere

As one of projection schemes to indicate how a 360-degree video maps to a 2D image plane, the cube map projection has been discussed to map certain area of a spherical video to a specific cube face. The below figure 3 describes one of woring examples of how the cube map projection is applied. During a 360-degree video is projected to a 3D cube geometry, the certain area on the spherical surface is mapped to a specific cube face, e.g., cube_front. These cube faces are mapped to one or more iamge frames. The below figure 3(c) describe an example when all cube faces are unfolded and mapped to one 2D image plane by correspondence between cube faces and regions of a coded picture.

Figure 3. Flow of cube map projection from a 360-degree video to a 2D image plane

When the cube map projection is applied, there could be various different ways to map cube faces to regions of a coded picture. Figure 4 describes examples of cube face packing arrangements, 3x2 and 4x3 layout.

(a) 3x2 layout

(b) 4x3 layout

Figure 4.  Cube face packing arrangment

Even with the same layout, i.e., same number of rows and columns is applied, there can be different mapping between the region of coded picture and cube faces as shown in Figure 4(b). In order to enable a device to correctly project from regions in coded picture to the cube geometry, the following cube face packing parameters are needed to indicate how regions of a coded picture are projected from certain cube surface.

- Cube map arrangement describing the number of rows and columns of cube faces in the coded picture

- Correspondence between regions of a coded picture and cube faces

- Optionally the rotation information if it is rotated when the cube face is projected to the region in the coded picture.

Hence, this document suggests a VR information SEI message for signaling of 360-degree video information in video bitstreams. This proposed SEI message indicates following properties of 360-degree video information:

- Projection format indicator describing how a spherical video is mapped to a 2D planer video

- Indicator whether the picture is coded mono or stereo

- The spherical surface area to which the coded picture is projected.

- Cube face packing parameters when the cube map projection is applied

# Proposal

## Proposed VR information SEI syntax

| vr_info (payloadSize) { | Descriptor |
| --- | --- |
| vr_info_id | ue(v) |
| vr_info_cancel_flag | u(1) |
| if( !vr_info_cancel_flag ) { |  |
| vr_projection_format | ue(v) |
| vr_stereo_flag | u(1) |
| full_spherical_flag | u(1) |
| if (!full_spherical_flag ) { |  |
| vr_area_min_yaw | se(v) |
| vr_area_max_yaw | se(v) |
| vr_area_min_pitch | se(v) |
| vr_area_max_pitch | se(v) |
| } |  |
| if (vr_projection_format == 2) { |  |
| cube_face_packing_arrangement | ue(v) |
| num_regions_minus1 | ue(v) |
| for( i = 0; i <= num_regions_minus1; i++ ) { |  |
| region_left_offset[i] | ue(v) |
| region_top_offset[i] | ue(v) |
| region_width[i] | ue(v) |
| region_height[i] | ue(v) |
| cmp_face_id[i] | u(3) |
| cmp_face_rotation[i] | u(16) |
| } |  |
| } |  |
| vr_info_persistence_flag | u(1) |
| } |  |
| } |  |

## Proposed VR information SEI semantics

vr_info_id contains an identifying number that may be used to identify the usage of the proposed VR information SEI message.

vr_info_cancel_flag equal to 1 indicates that the VR information SEI message cancels the persistence of any previous VR information SEI message. vr_info_cancel_flag equal to 0 indicates that 360-degree video information follows.

vr_projection_format specifies how the 360-degree video is projected to coded pictures. It can be extended by including other projection formats.

Table 1. Definition of vr_projection_format

| Value | Interpretation |
| --- | --- |
| 0 | Equirectangular projection |
| 1 | Cube map projection |
| 2-15 | reserved |

vr_stereo_flag indicates whether the picture is coded mono or stereo. vr_stereo_flag equal to 1 indicates that the picture is coded stereo, and maybe frame_packing_arrangement_SEI and other indicators provide information for stereo view presentation, e.g., stereoscopic 3D layout.

full_spherical_flag equal to 1 indicates that the coded picture intends to represent a full spherical 360 x 180 video. full_spherical_flag equal to 0 indicates that the coded picture intends to represent the non-full spherical video, e.g., 180 x 180.

vr_area_min_yaw and vr_are_max_yaw specify the minimum and maximum yaw angle, in degrees, of points on the spherical surface to which the coded picture is projected.

vr_area_min_pitch and vr_are_max_pitch specify the minimum and maximum pitch angle, in degrees, of points on the spherical surface to which the coded picture is projected

cube_face_packing_arrangement specifies the type of packing arrangement describing the number of rows and columns of cube faces in the coded picture when the cube map projection is applied.

Table 2. Definition of cube_face_packing_arrangement

| Value | Interpretation |
| --- | --- |
| 0 | 3 columns and 2 rows are used (3x2 layout) described in Figure 4(a) |
| 1 | 4 columns and 3 rows are used (4x3 layout) described in Figure 4(b) |
| 2-15 | reserved |

num_regions_minus1 specifies the number of regions that cube faces are mapped to in the coded picture.

region_left_offset and region_top_offset specify respectively the horizontal and vertical offset of the top-left pixel of the rectangular region in the coded picture.

region_width and region_height specify respectively the width and height of the region in the coded picture.

NOTE: cube faces can be packed into the same coded picture with an emphasis (e.g. greater spatial area) on the certain viewport. For example, the cube front face may be sampled with a higher resolution compared to other cube faces. In this case, since each face may have different resolutions, the region corresponding to each face is needed to indicate by the region_left_offset, region_top_offset, region_width and region_height.

cube_face_id specifies the identifier of cube face that is mapped to the region indicate by the region_left_offset, region_top_offset, region_width and region_height.

Table 3. Definition of cube_face_id

| Value | Interpretation |
| --- | --- |
| 0 | cube_front |
| 1 | cube_top |
| 2 | cube_bottom |
| 3 | cube_left |
| 4 | cube_right |
| 5 | cube_back |
| 6-7 | reserved |

cube_face_rotation specifies the amount of rotation, in degrees, that has been applied to map the cube face to the corresponded region in the coded picture. The cube face region may be rotated by +/- 90 or 180 degrees, or any other value.

vr_info_persistence_flag specifies the persistence of the VR information SEI message.

# References

- ISO/IEC JTC1/SC29/WG11/N16143, "Requirements for OMAF", MPEG 114, Feb. 2016.

- Lucidcam, https://lucidcam.com/

- C. Fogg, “Essential metadata to support virtual reality in AVC and HEVC elementary video streams”, JCTVC-Y0023, Chengdu, October 2016

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).