| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0027 |
| --- | --- |

| Title: | AHG7: Test report on Content Colour Volume SEI message – TEST1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyun Mook Oh Soo Yeon Lee Jong-Yeul Suh | Tel: Email: | hyunmook.oh@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

This document provides test results with regards to Content Colour Volume SEI message. Among the test plans established in 25th JCT-VC meeting, this report covers TEST1 which aims to assess the benefits of content colour gamut in terms of image quality. For this purpose, BT.2020 to BT.709 colour gamut mapping is performed using container colour primaries in VUI or content colour gamut in Content Colour Volume SEI message draft document JCTVC-Y1005. In the subjective and objective comparisons of those two outputs, it is shown that Content Colour Volume SEI message helps to improve the displayed image quality in terms of preserving intended colorfulness.

# Backgrounds

With the technology development for HDR (high dynamic range) and WCG (wide colour gamut), image quality is considered as one of the important factors in variety of applications, such as broadcasting, video streaming, and OTT services, etc. However, video tone and colorfulness intended at productions and post-productions is not guaranteed to be delivered to viewers due to the different capability of devices in end-to-end image processing chains. Moreover, different image quality could be produced due to the wide range of display devices with different HDR and/or WCG capabilities.

Figure 1. The concept of content colour gamut

As an approach to preserve creative intent, a content colour and luminance description method, called content colour volume, have been discussed. In [3,4,5], the range of pixel values in different colour representations was proposed to describe colour volume in different spatial regions. By focusing on the description of actual colour distribution in a content, content colour gamut was proposed in [6,7]. In Figure 1, the concept of content colour gamut is described with example colour distribution of an image, and illustrated with previously defined colour gamut, such as mastering display colour gamut and container colour gamut. In [8], it was proposed to use multiple luminance slices with colour boundary descriptions at each luminance plane to describe content colour volume precisely. As an alternative approach to the luminance plane selection, an average luminance value was proposed in addition to the minimum and maximum luminance values [9].

With those proposals, it was discussed that content colour volume provides benefits in image processing methods, such as colour gamut tailoring, tone mapping, colour volume mapping, since it describes actual boundary of colour and luminance values which currently does not distributed in the full range defined in HDR/WCG standards [10,11]. This is anticipated more beneficial when the gap between container (or mastering display information) and the content is large, e.g. BT.709 content is delivered through BT.2020 WCG service or SDR content is delivered through HDR mastering format. In addition, it is discussed that this information could improve data fidelity by providing actual pixel ranges.

Based on the discussion and support to define the content oriented colour and luminance description, it was tried to combine and merge the proposals. As a result of 24th JCT-VC meeting, an AHG was established to compromise three approaches [5,7,8] which results in an integrated content colour volume description [12]. In the consideration of essential and effective description for content colour volume, a BoG was made during 25th JCT-VC meeting to produce a compromised approach of four proposals [2]. Also test plans were set to examine effectiveness of common information as well as the potential additional information [1].

- In this document, one of the test results, called TEST1 in BoG report [1], is described to show the benefits of content colour volume in terms of improving image colorfulness. Among the content colour volume information depicted in previous proposals, this document focuses on the content colour gamut, which represents the boundary of actual colour distribution of a content.

# Test description

In this test, two different outputs are produced by using different colour boundary information to examine the effect of information. In the assumption of SDR content delivered via BT.2020 mastering format, SDR BT.709 contents contained in BT.2020 format are used as input signals. Before it is displayed on a SDR BT.709 monitor, the signal is converted to be mapped into the display colour gamut. As described in Figure 2, the output signals are produced by using colour primaries given by Content Colour Volume SEI message [2], i.e. BT.709 original content colour gamut, or VUI, i.e. BT.2020 container colour gamut. Followings are the detailed plan of this test defined in BoG report [1].

- TEST1 - BT.709 content in BT.2020 container, represented in YCbCr 4:2:0 10b, to be displayed on BT.709 display

- Participant: LG

- TEST1-Ref – use only VUI information, apply gamut compression (colour gamut tailoring) from BT.2020 to BT.709 before rendering on BT.709 display

- TEST1 – use reference SEI, convert to BT.709 with clipping in RGB domain before rendering on BT.709 display

- Test content: BT.709 SDR versions, converted in BT.2020 container, of the JCTVC HDR test content: Market SDR image was used.

- Check:

  - Objective metrics (PSNR) – test 709 SDR vs source 709 SDR (both in 709 container)

  - Visual test – comparing content resulting from TEST1-Ref rendered on a BT.709 SDR display, to content resulting from TEST1 rendered on a same display

- Note: does not assess the need for the syntax elements min/max luminance.

Figure 2. Test chain for TEST1

# Test results and discussions.

- In this test, the colour gamut mapping outputs using different colour gamut information are compared in subjective and objective methods. As a test input, SDR BT.709 version of JCTVC HDR test contents are selected and colour re-sampled to be represented in BT.2020 container. BT.2020 to BT.709 gamut mapping is produced to display on a SDR BT.709 display device. In the following, TEST1 and TEST1-ref are used to represent gamut mapping outputs using colour primaries in Content Colour Volume SEI message and VUI, respectively.

- In Figure 3, output images of TEST1-ref and TEST1 are compared for different image sequences: Market, FireEater, Tibul, and BalloonFestival. As shown in the images, TEST1 results have vivid colours while the TEST1-ref results lose colorfulness and suffer from colour distortion. Since the original content is BT.709 and there is no valid information out of content colour gamut, clipping to BT.709 boundary is the best practice to recover original colorfulness. However, if there is no information about content colour distribution, BT.2020 to BT.709 colour gamut mapping, e.g. colour gamut compression used in TEST1-ref, might be a choice to keep relative difference between colours while sacrificing colorfulness.

- As an objective comparison, the resultant images are compared with the original SDR BT.709 content in terms of PSNR and SSIM metrics. The PSNR and SSIM metrics are calculated for each channel through entire sequence represented in BT.709 container format. In Tables 1 and 2, TEST1 produces higher PSNR values in all channels of all sequences. To reflect the visual differences in the objective metrics, the results are compared in terms of SSIM logarithmic values. Similar to the PSNR values, TEST1 in Table 3 shows higher SSIM values than TEST1-ref in Table 4 in all channels of all sequences.

Figure 3. Experimental results: (a) TEST1-ref using container colour gamut (b) TEST1 using content colour gamut

Table 1. PSNR result for TEST1-ref output

| - PSNR |  | - Y | - U | - V |
| --- | --- | --- | --- | --- |
| - Market3Clip4000r2_1920x1080p_50_10_709 | - Avg | - 46.65738 | - 40.956841 | - 31.409702 |
|  | - Min | - 45.674431 | - 39.848068 | - 31.037537 |
|  | - Max | - 47.167015 | - 41.424632 | - 31.675765 |
| - FireEater2Clip4000r1_1920x1080p_25_10_709 | - Avg | - 37.018278 | - 47.265306 | - 31.798007 |
|  | - Min | - 35.466959 | - 39.08706 | - 30.468732 |
|  | - Max | - 38.634734 | - 54.755521 | - 33.072607 |
| - Tibul2Clip4000r1_1920x1080p_30_10_709 | - Avg | - 28.163742 | - 42.374461 | - 24.298126 |
|  | - Min | - 26.396454 | - 38.042284 | - 22.373204 |
|  | - Max | - 30.038667 | - 47.969664 | - 26.913631 |
| - BalloonFestival_1920x1080p_24_10_709 | - Avg | - 37.012964 | - 32.439633 | - 24.767937 |
|  | - Min | - 36.870474 | - 32.30181 | - 24.74388 |
|  | - Max | - 37.112844 | - 32.552405 | - 24.782331 |

Table 2. PSNR result for TEST1 output

| - PSNR |  | - Y | - U | - V |
| --- | --- | --- | --- | --- |
| - Market3Clip4000r2_1920x1080p_50_10_709 | - Avg | - 58.179117 | - 50.627756 | - 52.077287 |
|  | - Min | - 57.605339 | - 47.547194 | - 51.53749 |
|  | - Max | - 58.771723 | - 51.783533 | - 52.880606 |
| - FireEater2Clip4000r1_1920x1080p_25_10_709 | - Avg | - 58.104911 | - 60.411548 | - 58.58184 |
|  | - Min | - 56.834409 | - 55.954334 | - 56.654572 |
|  | - Max | - 59.6803 | - 65.194771 | - 60.761129 |
| - Tibul2Clip4000r1_1920x1080p_30_10_709 | - Avg | - 49.299266 | - 51.545731 | - 53.959736 |
|  | - Min | - 48.285429 | - 50.461096 | - 52.82802 |
|  | - Max | - 51.076399 | - 52.552265 | - 54.961134 |
| - BalloonFestival_1920x1080p_24_10_709 | - Avg | - 59.394312 | - 54.507954 | - 59.60341 |
|  | - Min | - 59.252011 | - 54.145414 | - 59.143364 |
|  | - Max | - 59.58679 | - 54.736472 | - 60.057939 |

Table 3. SSIM result for TEST1-ref output

| - SSIM (log) |  | - Y | - U | - V |
| --- | --- | --- | --- | --- |
| - Market3Clip4000r2_1920x1080p_50_10_709 | - Avg | - 38.414 | - 37.95086 | - 26.76274 |
|  | - Min | - 37.04745 | - 37.0949 | - 26.30994 |
|  | - Max | - 39.14456 | - 38.33366 | - 27.01586 |
| - FireEater2Clip4000r1_1920x1080p_25_10_709 | - Avg | - 29.00545 | - 40.44223 | - 30.88643 |
|  | - Min | - 27.1925 | - 32.70599 | - 29.46175 |
|  | - Max | - 31.4551 | - 47.88261 | - 32.40121 |
| - Tibul2Clip4000r1_1920x1080p_30_10_709 | - Avg | - 16.82135 | - 36.01451 | - 23.48362 |
|  | - Min | - 15.96628 | - 31.11565 | - 21.83852 |
|  | - Max | - 17.55488 | - 42.32198 | - 25.66452 |
| - BalloonFestival_1920x1080p_24_10_709 | - Avg | - 32.0507 | - 26.18553 | - 20.09724 |
|  | - Min | - 31.82613 | - 26.0454 | - 20.06464 |
|  | - Max | - 32.1815 | - 26.3085 | - 20.11881 |

Table 4. SSIM result for TEST1 output

| - SSIM(log) |  | - Y | - U | - V |
| --- | --- | --- | --- | --- |
| - Market3Clip4000r2_1920x1080p_50_10_709 | - Avg | - 50.43427 | - 45.22403 | - 44.97026 |
|  | - Min | - 49.91528 | - 43.75737 | - 44.24023 |
|  | - Max | - 51.10029 | - 45.76937 | - 46.13026 |
| - FireEater2Clip4000r1_1920x1080p_25_10_709 | - Avg | - 47.72616 | - 50.63842 | - 49.30816 |
|  | - Min | - 46.56691 | - 46.43077 | - 47.34938 |
|  | - Max | - 48.87585 | - 55.1569 | - 51.34914 |
| - Tibul2Clip4000r1_1920x1080p_30_10_709 | - Avg | - 36.10553 | - 41.95967 | - 44.89285 |
|  | - Min | - 35.17919 | - 40.92786 | - 43.95464 |
|  | - Max | - 38.14959 | - 42.86672 | - 45.83817 |
| - BalloonFestival_1920x1080p_24_10_709 | - Avg | - 49.89516 | - 44.66823 | - 49.84034 |
|  | - Min | - 49.78256 | - 44.3412 | - 49.41235 |
|  | - Max | - 50.04977 | - 44.88227 | - 50.24788 |

# Conclusion

In this document, a test procedure and its results are described to examine the effect of Content Colour Volume SEI message. The benefit was assessed by comparing two colour gamut mapping results using content colour gamut given by current version of Content Colour Volume SEI message and container colour gamut which is widely used and given in VUI information. Based on the subjective and objective metrics, it is shown that the use of content colour gamut improves the image quality of colour gamut mapping output in terms of colorfulness. From the test results, it is anticipated that the content colour gamut information defined in Content Colour Volume SEI message provides a benefit in improving display adaptation process for HDR and/or WCG displays, and provide more improved image quality to viewers.

# References

- E. François, “BoG report on Content Colour Volume SEI message”, JCTVC document JCTVC-Y0051, Chengdu, CN, Oct. 2016

- A.M. Tourapis, E. Francois, H.M. Oh, A.K. Ramasubramonian, P. Yin, “Content Colour Volume SEI Message Draft 1” JCTVC document JCTVC-Y1005, Chengdu, CN, Oct. 2016

- A.M. Tourapis, Y. Su, and D. Singer, “Effective Colour Volume SEI”, JCTVC document JCTVC-V0038, Geneva, SW, Oct. 2015

- A.M. Tourapis, Y. Su, and D. Singer, “Effective Colour Volume SEI”, JCTVC document JCTVC-W0098, San Diego, USA, Feb. 2016

- A.M. Tourapis, Y. Su, and D. Singer, “Improvements to the proposed effective colour volume SEI message”, JCTVC document JCTVC-X0052, Geneva, SW, Jun. 2016

- H.M. Oh, J. Choi, and J.-Y. Suh, “Content colour gamut SEI message”, JCTVC document JCTVC-W0057, San Diego, USA, Feb. 2016

- H.M. Oh, J. Choi, and J.-Y. Suh, “Content colour gamut SEI message”, JCTVC document JCTVC-X0040, Geneva, SW, Jun. 2016

- A. K. Ramasubramonian, D. B. Sansli, J. Sole, D. Rusanovskyy, and M. Karczewicz, “Content colour volume SEI message”, JCTVC document JCTVC-X0069, Geneva, SW, Jun. 2016

- T. Chen, P. Yin, T. Lu, F. Pu, W. Husak, “AHG 7: On Content Colour Volume SEI message”, JCTVC document JCTVC-Y0032, Chengdu, CN, Oct. 2016

- Recommendation ITU-R BT.2020, “Parameter values for ultra-high definition television systems for production and international programme exchange” (2015).

- Recommendation ITU-R BT.2100, “Image parameter values for high dynamic range television for use in production and international programme exchange” (2016)

- A.M. Tourapis, Y. Su, D. Singer, H.M. Oh, J. Choi, J.-Y. Suh, A.K. Ramasubramonian, D.B. Sansli, J. Sole, D. Rusanovskyy, M. Karczewicz “Content colour volume SEI message”, JCTVC document JCTVC-Y0040, Chengdu, CN, Oct. 2016

# Patent rights declaration

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).