| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0028 |
| --- | --- |

| Title: | AHG7: On content colour volume SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taoran Lu, Fangjun Pu, Tao Chen, Peng Yin, Walt Husak 432 Lakeside Drive, Sunnyvale, CA 94085, USA | Tel: Email: | 408-330-3251 pyin@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

JCTVC-Y1005 provides the draft text for Content Colour Volume (CCV) SEI message. The draft includes the minimal set of CCV SEI message: colour primaries, minimum luminance value and maximum luminance value. This contribution proposes to further add the signaling of the average luminance value. Verification tests are carried out to illustrate the benefit of using the proposed syntax for colour volume mapping.

# Introduction

In last meeting, JCTVC-Y0032 [1] and JCTVC-Y0040 [2] were submitted for potential modifications to the text of the HEVC standard to include a content colour volume (CCV) SEI message. Y0032 aims at providing a compact set of metadata to signal three color primaries in (x, y, Y) description and min, max and average Luminance, and optionally signal three cross-sections with three primaries per cross-section. Y0040 included three non-exclusive modes that provide more features at the cost of added complexity and SEI size. A break-out group (BoG) meeting on CCV SEI message was held during the meeting to identify the most essential and common metadata set among the proposals. Consensus were reached among BoG participants that a content colour volume information is needed in addition to existing SEIs/VUI signalling while the remain question is what level of accuracy is needed [3]. It was suggested to set-up a verification experiment to address the question.

The test plan is to start with a “minimal set” light CCV SEI. The agreed minimal set includes three colour primaries expressed in CIE1931 (x, y, Y) color representation, as well as minimum and maximum luminance. The text of this minimal set has been made available in JCTVC-Y1005 [4]. Other metadata proposed in Y0032 and Y0040, such as average luminance, signalling of spatial partition or cross-sections are not included as their benefit is pending to be verified by verification test.

This contribution proposes to add one additional signalling of average luminance on top of the minimal set and provides verification test results to justify the benefit of the added syntax.

# Proposed Syntax and Semantics

shows the proposed syntax of content colour volume SEI message. The changes over the minimal set [4] are highlighted in yellow.

Table 1 Proposed syntax of CCV SEI message

| content_colour_volume( payloadSize ) { | Descriptor |
| --- | --- |
| ccv_cancel_flag | u(1) |
| if( !ccv_cancel_flag ) { |  |
| ccv_persistence_flag | u(1) |
| for( c = 0; c < 3; c++ ) { |  |
| ccv_primaries_x[ c ] | i(16) |
| ccv_primaries_y[ c ] | i(16) |
| } |  |
| ccv_min_lum_value | u(32) |
| ccv_avg_lum_value | u(32) |
| ccv_max_lum_value | u(32) |
| } |  |
| } |  |

ccv_avg_lum_value specifies the normalized average luminance value, according to CIE 1931, that is expected to be present in the content, where values are normalized to Lo or Lc as specified in Table E.4 and according to the transfer characteristics of the signal. The values of ccv_avg_lum_value are in normalized increments of 0.00000001 in the linear representation.

# Verification Test

The importance of average luminance information for display mapping or colour volume mapping is known. ST 2094 Applications #1, #2 and #4, which apply parametric mapping model, all use average luminance information in some variance.

The verification test is designed to show the benefit of average luminance information in addition of the minimal set for colour volume mapping. Colour volume mapping can be roughly split into two parts: tone mapping and gamut mapping. Tone mapping algorithm is based on Luminance Transform Curve described in Annex B.2 of ST 2094-10 [5]. For gamut mapping, colour/gamut clipping using the colour primaries information in CCV is applied. The high level block diagram of the test chain is shown in Figure 1.

Figure 1 verification test chain.

## Tone mapping Algorithm

ST. 2094-10 Annex B.2 [5] defines a sigmoid tone curve as

| Lm(L)=c1+c2×L1+c3×L | (1) |
| --- | --- |

where:

𝐿𝑚 = mapped (transformed) luminance in units of cd/m2

𝐿 = input luminance in units of cd/m2

𝑐𝟷, 𝑐𝟸, 𝑐𝟹 = computed coefficients

The tone curve can be determined as long as the following luminance parameters are known:

𝑥𝟷 = input image essence minimum luminance value in units of cd/m2

𝑥𝟸 = input image essence average luminance value in units of cd/m2

𝑥𝟹 = input image essence maximum luminance value in units of cd/m2

𝑦𝟷 = targeted system display minimum luminance in units of cd/m2

𝑦𝟸 = derived targeted system display adaptation point in units of cd/m2

𝑦𝟹 = targeted system display maximum luminance in units of cd/m2

The targeted system display adaptation point luminance 𝑦𝟸 is chosen to be the geometric mean of targeted system display minimum luminance, targeted system display maximum luminance and input image essence average luminance value:

| y2=x2×y3×y1 | (2) |
| --- | --- |

The three constants and  in the sigmoidal tone mapping curve equation (1) can be solved by:

| c1c2c3=1αx2x3(y2−y3)x1x3(y3−y1)x1x2(y1−y2)x3y3−x2y2x1y1−x3y3x2y2−x1y1x3−x2x1−x3x2−x1y1y2y3 | (3) |
| --- | --- |
| where α=x3y3(x1−x2)+x2y2(x3−x1)+x1y1(x2−x3) | (4) |

Figure 2 ST.2094-10 tone mapping curve examples: the three cases share same parameters (source max = 4000nits, source min = 0.005 nits, target max = 100 nits, target min = 0.005nits) except for the average source luminance (blue: source avg = 1 nits, red: source avg = 20 nits, green: source avg = 500nits), but the derived tone mapping characteristics are significantly different.

Figure 2 shows an example of the three tone curves which has the same content min/max luminance value but different average luminance value. The resulted tone mapping curve are significant different. It proves that average luminance value is very crucial to the resulted tone mapping curve.

## Test Results

The experiment is based on Figure 1.

For case of Reference, two test references are designed:

- Reference 1: since only content max (x3) and min (x1) luminance values are available in minimal CCV SEI, one reference is to perform linear tone mapping which can be solely determined by these two control points. The linear model is:

| Lm(L)=c1×(L−x1)+c2c1 = (y3−y1) / (x3−x1)c2 = y1 | (5) |
| --- | --- |

- Reference 2: the other reference is to perform the ST. 2094-10 sigmoid tone mapping, i.e., the model in equation (1), which is controlled by three control points, and to derive the average luminance value as the average of the signaled maximum and minimum luminance value, i.e., 𝑥𝟸 = (𝑥1 + 𝑥3) / 2.

For Proposed CCV SEI case (Reference CCV SEI + avg. content luminance), the ST. 2094-10 sigmoid tone mapping model in equation (1) is used as in Reference 2. The only difference is to use the proposed additional signalling of true content average luminance value 𝑥𝟸 instead of the derived value to compute the tone curve parameters.

The two clips: “ShowGirl” and “StEM_MagicHour” from the JCTVC HDR Common Test Condition dataset are selected. Both of them contain multiple scene changes and have very dark and bright scenes. They can represent very realistic content to test display mapping. Content luminance statistics are computed at scene basis at encoder side. The YCbCr 4:2:0 10bit conversion only and compression results (Anchor v3.2) are used as input to perform display mapping. For gamut mapping, simple RGB color conversion from BT. 2020 to BT. 709 and clipping is performed.

Table 2 showed the computed content luminance statistics (scene-based) of the test clips. It can be observed that the content average luminance values are highly content-dependent, and are very far away from the average of max and min luminance. It is hard to have any common formula to derive the content average luminance using only the min and max. For several scenes (ShowGirl scene1, ShowGirl scene2, MagicHour scene2), even though they all have very similar max (close to 4000nits) and min (close to 0 nits) luminance values, their real average luminance values differ significantly from 8 nits to 522 nits.

Table 2 content luminance statistics (in units of cd/m2) of test data

| Content | min lum | max lum | avg. lum | (min+max)/2 |
| --- | --- | --- | --- | --- |
| ShowGirl_scene1 | 0.00 | 3863.14 | 26.16 | 1931.57 |
| ShowGirl_scene2 | 0.00 | 3995.94 | 521.74 | 1997.97 |
| MagicHour_scene1 | 0.00 | 1791.30 | 12.56 | 895.65 |
| MagicHour_scene2 | 0.00 | 3767.13 | 8.50 | 1883.57 |
| MagicHour_scene3 | 0.00 | 2530.98 | 9.16 | 1265.49 |

Based on the content luminance statistics and the tone mapping model to be used (linear or ST.2094 sigmoid), the corresponding tone curve for each scene in the test clips for reference and proposal cases are illustrated in Figure 3. It can be seen that the resulted tone mapping characteristics differ significantly with or without the true content average luminance value. Consequently, the mapped image quality differs significantly. Figure 4 and Figure 5 show the comparison of the mapping results. It is our observation that the quality of mapped images are always reasonable for all the tested scenes when the proposed CCV SEI is in use. Without using the real content average luminance, the mapped image quality can become unacceptable for many scenes no matter linear tone mapping or sigmoid tone mapping models are used.

A subjective assessment has been performed to compare the visual conformity of the mapped SDR to the original HDR content before display mapping. The mapped SDR is rendered on a BT. 709, 100 nits reference display and the original HDR content is rendered on a BT. 2020, 4000 nits HDR display. The two displays are placed side-by-side. The mapped SDR generated by the proposed CCV SEI conforms much better than that of the two reference cases.

The complete dataset of the display mapped results (direct mapping of uncompressed content and mapping of all four compressed rate points of each sequence) for the two reference cases and the proposed CCV SEI will be made available during the 26th JCTVC meeting.

Figure 3 Comparison of derived tone mapping curves using reference and proposed CCV SEI, for each scene of tested clips.

(a) CCV SEI Reference1         (b) CCV SEI Reference2         (c) CCV SEI Proposed

Figure 4 Examples of display mapped (to BT.709 100 nits SDR) images (ShowGirl compressed QP=19) using: (a) minimal CCV SEI with linear tone mapping; (b) minimal CCV SEI with ST.2094-10 tone mapping; (c) CCV SEI + avg. luminance with ST.2094-10 tone mapping.

(a) CCV SEI Reference1        (b) CCV SEI Reference2         (c) CCV SEI Proposed

Figure 5 Examples of display mapped (to BT.709 100 nits SDR) images (StEM_MagicHour compressed QP=18) using: (a) minimal CCV SEI with linear tone mapping; (b) minimal CCV SEI with ST.2094-10 tone mapping; (c) CCV SEI + avg. luminance with ST.2094-10 tone mapping.

# Conclusion

This contribution justified the benefit of adding the signaling of the content average luminance value in addition to the minimal set of content colour volume SEI message defined in JCTVC-Y1005. We propose to adopt the syntax of the average luminance value in CCV SEI message.

# References

- “AHG 7: On Content Colour Volume SEI message”, JCTVC-Y0032, Oct. 2016, Chengdu, CN

- “Content colour volume SEI message”, JCTVC-Y0040, Oct. 2016, Chengdu, CN

- “BoG report on Content Colour Volume SEI message”, JCTVC-Y0051, Oct. 2016, Chengdu, CN

- “Content Colour Volume SEI Message Draft 1”, JCTVC-Y1005, Oct. 2016, Chengdu, CN

- SMPTE ST 2094-10:2016: "Dynamic Metadata for Color Volume Transform – Application #1"

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).