| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0029 |
| --- | --- |

| Title: | Full range video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chad Fogg | Email: | chadfogg@gmail.com |
| Source: | Motion Picture Laboratories |  |  |

_____________________________

# Abstract

BT.2100 (High Dynamic Range / Wide Color Gamut) is the first television exchange signal standard in the ITU-R BT series to include a definition for a full range video. The commonly observed BT.601, BT.709, and BT.2020 specifications have never included a concept for full range code representation. Applications such as JFIF (JPEG), HDMI, MPEG / ITU-T H-series have on their own described full range with essentially the same formulae, where the lowest nominal intensity signal level is mapped to the first digital code level, with an index of 0, and the highest intended, nominal intensity signal level is mapped to the last code level, 2^BitDepth -1. The first edition of BT.2100, published in July 2016, defines full range scaling slightly differently from the traditional, canonical full range scaling formula applied in digital video applications. In BT.2100, real video signals, often represented in floating point data types, are scaled to 10-bit unsigned integer code levels by a constant 1024, instead of the traditional 1023. Likewise, for 12-bit video samples, the new scale factor is 2048 instead of the traditional 2047. This document reports side observations made during an investigation of a pending change to traditional scaling in BT.2100 that may be included in a second edition, estimated to be published in the second half of 2017.  Some of the historical, background information that motivated the original full range scaling may be unexpected to users. Several trade-offs were made in full range formula created by informal industrial groups around the year 1992 that create corner conditions, such as the clipping of pure red and green in Y'CbCr signals, that may have been acceptable for limited distribution applications then, but may not be desired in the greater, broader user community, including contribution and archive users. Y'CbCr has a slightly greater natural signal range than the R'G'B' signal it is derived from due to the mathematical sign introduced in the color difference equations.  If all code levels are used for the zero to nominal peak excursion, then it is not possible to represent both -0.5 and +0.5 with unique code words. One end must be clipped. A slightly reduced scale factor, such as 2^BitDepth-2 or smaller will avoid this clipping. The Serial Digital Interface (SDI), over which video signals are commonly exchanged between equipment in studio environments, also nominally clamps 4 code levels at each end of both full range R'G'B and full range Y'CbCr signals. Thus an even smaller scale factor would be necessary to avoid clamping. Neither the "1024" or the traditional "1023" scaling for 10-bit video avoid these clipping issues. Among the conclusions of this investigation is that narrow range signals avoid all such clippings, and provide room for temporary expansion (overshoot and undershoot) of the video signal created transform coefficient quantization noise, motion compensation filter ringing (overshoot, undershoot), or filter ringing introduced in pre-processing stages such as scaling operations, including 4:4:4 to 4:2:0 chroma format conversion.

# Introduction

The first use of full range scaling appears to have originated as an outcome of an informal, industrial group in a year 1992 draft of the "JFIF" JPEG file format specification. The print and photographic industry drove most requirements in the development of JPEG up until that time. JFIF served as the first bridge between those communities and the early adopters of consumer digital video.  Print, photo, and computer graphics users were accustomed to using all 256 code levels in their applications, while by 1992 only narrow range video signals were practiced in high-end television equipment, which used code levels 16 through 235 for R'G'B' and Y', and 16 through 240 for CbCr.  Since full range Y'CbCr has only existed in JPEG applications until now, full range JPEG had been considered only as consumer distribution format where the noise introduced by compression (transform coefficient quantization) overshadowed any small clipping loss in the conversion from R'G'B' to Y'CbCr. Archive applications would preserve video or still images in R'G'B' or native scanning pixel domains, such as C'M'Y'.

Before BT.2100, full range scaling had not been described in television signal standards. The year 2014 publication of SMPTE RP-2077 documented for the first time the common practice conversion between full range and narrow range signals in studio file exchange.

HEVC is perhaps the first consumer distribution video coding representation that can efficiently represent smooth picture areas that have colors at those signal container boundaries otherwise clipped in full range CbCr signals. In modern digital cinema practice, where the video distribution signal is coded as narrow range JPEG 2000, precise reconstruction of flat areas such as infinite black or pure patches of cyan, magenta, red, blue, etc. is now an artistic intent that is expected to be reconstructed at the distribution end display.

JFIF’s choice of full range scaling was motivated to provide a code word that, when converted back to a real-value (floating point number between [0.0, 1.0] for R'G'B' or Y', and [-0.5 to 0.5] for Cb,Cr), reconstructs to 0.0.  This is the key desired property of “mid-tread” quantizer, especially important in low-bitrate video applications where large number of blocks are necessary to have zero residuals. Mid-rise quantizers on the other hand do not have a code level that creates zero real-value reconstruction.  However, mid-tread quantizers are supposed to only be applied to a range consisting of an odd number of quantization levels.  By forcing use of a binary power of two number representation scheme to utilize all code levels, this odd-number rule would naturally conflict with an even number of quantization levels produced by 2^BitDepth.

# Suggestion

All future standards document editions, that have descriptions of full range video signals (such as the VUI semantic section for the Y'CbCr matrix_coeff code points), should include a NOTE that informs readers that today’s color difference equations clips pure colors.