| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0030r1 |
| --- | --- |

| Title: | Elementary bitstream omni-directional video indicators for web applications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg | Email: | chadfogg@gmail.com |
| Source: | Motion Picture Laboratories |  |  |

_____________________________

# Abstract

This proposal document requests that JCT draft an SEI to carry the W3 Spherical Video RFC v2 metadata set for omni-directional video. The RFC specification currently only defines carriage of its metadata in MP4 boxes or MKV headers. To facilitate a more general pass-through of such metadata in elementary AVC and HEVC bitstreams, two possible solutions for a specific, native  SEI carriage mechanism are suggested in this proposal: (1) embed the RFC metadata boxes into the payload of a newly defined SEI so that direct payload copy is possible between MP4 and SEI; (2) translate the syntax and semantics of the RFC to have equivalent meaning in the common SEI language of AVC & HEVC.; (3)ind of separate S the AVC anpecificay community employ either a user_data_registere_t35 SEuser_dataistere. The RFC was created by the G inserted into MP4 headers by large web services such as YouTube, and is applied in virtual reality (VR) players from various manufacturers. An example VR player that fovideo output per the RFC is the open source VLC player program. A second version of this proposal includes a few coons and improvements suggested by Google. Alternatively to dedicated SEI, virtual reality developers could employ either a user_data_registered_itu_t_t35 SEI  or  a user_data_unregistered SEI to carry the Spherical Video RFC payload in elementary bistreams.

# Introduction

The spatial media project [1][2] is considered by several virtual reality services and vendors to be the default metadata specification for virtual reality / omni-directional / panoramic / 360 degree spherical video coding (to name just a few synonyms). Tools [3] for inserting RFC metadata into MP4 and MKV headers are provided by YouTube. This proposal translates the Spherical Video RFC v2 industry specification into common syntax language of AVC, HEVC so that the metadata can be generated by encoders and passed through equipment that accepts traditional MPEG-2 Transport Stream (MPEG-2 TS) and other bitstream containers. Like Video Usability Information (VUI) and frame packing arrangement SEI elements, the RFC metadata is essential to interpreting the intended presentation format of pictures output by the decoder.

A subset of the Spherical Video RFC v2 is proposed for inclusion in an AVC and HEVC SEI.  Note that the antiquated spherical video RFC v1 has an optional user_data_unregistered SEI convention, identified by uuid_iso_iec_11578 value equal to the hexadecimal byte array: { 0xff, 0xcc, 0x82, 0x63, 0xf8, 0x55, 0x4a, 0x93, 0x88, 0x14, 0x58, 0x7a, 0x02, 0x52, 0x1f, 0xdd }.

The –v2 zip archive of JCTVC-Z0030 includes the current Spatial Media github depositry in the subdirectory “spatial-media-master” (approximately 2 Mbyte).

# Proposal

## Syntax

The following alternative syntax suggestions are proposed.

### Embedding MP4 st3d and sv3d boxes into SEI payload

| spherical_rfc2_sv3d(payloadSize) { | Descriptor |
| --- | --- |
| i = 0 |  |
| do { |  |
| spherical_rfc2_sv3d_byte | b(8) |
| i++ |  |
| } while (i < payloadSize ) |  |
| } |  |

| spherical_rfc2_st3d(payloadSize) { | Descriptor |
| --- | --- |
| i = 0 |  |
| do { |  |
| spherical_rfc2_st3d_byte | b(8) |
| i++ |  |
| } while (i < payloadSize ) |  |
| } |  |

| _sv3d(payloadS{ |  |
| --- | --- |
| sp2_sv3d_byte | ) |
| + |  |

### RFC translated into specialized SEI syntax

| spherical_rfc2 (payloadSize) { | Descriptor |
| --- | --- |
| spherical_rfc2_id | u(7) |
| spherical_rfc2_cancel_flag | u(1) |
| if( ! spherical_rfc2_cancel_flag ) { |  |
| spherical_rfc2_persistence_flag | u(1) |
| number_of_spherical_rfc2_boxes | u(7) |
| i = number_of_spherical_rfc2_boxes |  |
| while (i>0){ |  |
| spherical_rfc2_box_type | u(8) |
| spherical_rfc2_box_byte_count | u(8) |
| j = spherical_rfc2_box_byte_count |  |
| if ( spherical_rfc2_box_type == 1 ) { |  |
| st3d_stereo_mode | u(8) |
| j = j - 1 |  |
| } else if ( spherical_rfc2_box_type == 2 ) { |  |
| prhd _version | u(32) |
| prhd _flags | u(32) |
| prhd_pose_yaw_degrees | i(32) |
| prhd_pose_pitch_degrees | i(32) |
| prhd_pose_roll_degrees | i(32) |
| j = j - 20 |  |
| } else if ( spherical_rfc2_box_type == 3 ) { |  |
| cbmp_version | u(32) |
| cbmp_flags | u(32) |
| cbmp_layout | u(32) |
| cbmp_padding | u(32) |
| j = j - 16 |  |
| } else if ( spherical_rfc2_box_type == 4 ) { |  |
| equi_projection_version | u(32) |
| equi_projection_flags | u(32) |
| equi_projection_bounds_top | u(32) |
| equi_projection_bounds_bottom | u(32) |
| equi_projection_bounds_left | u(32) |
| equi_projection_bounds_right | u(32) |
| j = j - 24 |  |
| } |  |
| while(j>0){ |  |
| spherical_rfc2_byte | u(8) |
| j = j – 1 |  |
| } |  |
| i = i - 1 |  |
| } |  |
| } |  |
| } |  |

Alternative, non-looping structure:

| spherical_rfc2 (payloadSize) { | Descriptor |
| --- | --- |
| spherical_rfc2_id | u(7) |
| spherical_rfc2_cancel_flag | u(1) |
| if( ! spherical_rfc2_cancel_flag ) { |  |
| spherical_rfc2_persistence_flag | u(1) |
| spherical_rfc2_st3d_stereo_extension_flag | u(1) |
| spherical_rfc2_sv3d_prhd_extension_flag | u(1) |
| spherical_rfc2_sv3d_cbmp_extension_flag | u(1) |
| spherical_rfc2_sv3d_equi_extension_flag | u(1) |
| reserved_bits | u(3) |
| if ( spherical_rfc2_st3d_stereo_extension_flag ) { |  |
| st3d_stereo_mode | u(8) |
| } |  |
| if ( spherical_rfc2_sv3d_prhd_extension_flag) { |  |
| prhd _version | u(32) |
| prhd _flags | u(32) |
| prhd_pose_yaw_degrees | i(32) |
| prhd_pose_pitch_degrees | i(32) |
| prhd_pose_roll_degrees | i(32) |
| } |  |
| if (spherical_rfc2_sv3d_cbmp_extension_flag ) { |  |
| cbmp_version | u(32) |
| cbmp_flags | u(32) |
| cbmp_layout | u(32) |
| cbmp_padding | u(32) |
| } |  |
| if (spherical_rfc2_sv3d_equi_extension_flag ) { |  |
| equi_projection_version | u(32) |
| equi_projection_flags | u(32) |
| equi_projection_bounds_top | u(32) |
| equi_projection_bounds_bottom | u(32) |
| equi_projection_bounds_left | u(32) |
| equi_projection_bounds_right | u(32) |
| } |  |
| } |  |
| } |  |

## Semantics

### Embedding MP4 st3d and sv3d boxes into SEI payload

spherical_rfc2_sv3d_byte shall be a byte containing spherical RFC v2 st3d video metadata.

spherical_rfc2_st3d_byte shall be a byte containing spherical RFC v2 sv3d video metadata.

rfc2_sv3d_shalle containna.

### RFC translated into specialized SEI semantics

spherical_rfc2_id is the identifier of the current spherical RFC v2 SEI message.

spherical_rfc2_cancel_flag equal to 1 indicates that the spherical RFC v2 SEI message cancels the persistence of any previous spherical RFC v2 SEI message. spherical_rfc2_cancel_flag equal to 0 indicates that spherical RFC v2 information follows.

spherical_rfc2_persistence_flag specifies the persistence of the spherical media RFC v2 SEI message.

number_of_spherical_rfc2_boxes specifies the number of boxes contained in the current spherical RFC v2 SEI message.

spherical_rfc2_box_type specifies the type of the current spherical RFC v2 SEI box.

spherical_rfc2_box_type equal to 3 or 4 shall be present when spherical_rfc2_cancel_flag is equal to 0.

spherical_rfc2_box_byte_count specifies the number of bytes of the current spherical RFC v2 SEI box.

st3d_stereo_mode specifies the stereo frame layout. The values 0 to 255 are reserved for current and future layouts. The following values are defined:

| st3d_stereo_mode | Stereo Mode Description |
| --- | --- |
| 0 | Monoscopic: Indicates the video frame contains a single monoscopic view. |
| 1 | Stereoscopic Top-Bottom: Indicates the video frame contains a stereoscopic view storing the left eye on top half of the frame and right eye at the bottom half of the frame. |
| 2 | Stereoscopic Left-Right: Indicates the video frame contains a stereoscopic view storing the left eye on left half of the frame and right eye on the right half of the frame. |

prhd _version specifies the version of the Spherical Media RFC v2 projection header (prhd) box. Only value 0 shall be used in this SEI.

prhd_flags is for future use in the Spherical Media RFC v2 projection header (prhd) box pro. Only value 0 shall be used in this SEI.

Pose values are 16.16 fixed point values measuring rotation in degrees. These rotations transform the projection as follows:

prhd_pose_yaw_degrees specifies the counter-clockwise rotation in degrees around the up vector, restricted to -180.0 to 180.0. If prhd_pose_yaw_degrees is not present in the bitstream, a value of 0 shall be assumed.

prhd_pose_pitch_degrees specifies the counter-clockwise rotation in degrees around the right vector post yaw transform, restricted to -90.0 to 90.0. If prhd_pose_pitch_degrees is not present in the bitstream, a value of 0 shall be assumed.

prhd_pose_roll_degrees specifies the clockwise-rotation in degrees around the forward vector post yaw and pitch transform, restricted to -180.0 to 180.0. If prhd_pose_roll_degrees is not present in the bitstream, a value of 0 shall be assumed.

cbmp_version specifies the version of the Spherical Media RFC v2 cubemap projection box. Only value 0 shall be used in this SEI.

cbmp_flags is for future use in the Spherical Media RFC v2 cubemap projection box. Only value 0 shall be used in this SEI.

cbmp_layout is a 32-bit unsigned integer describing the layout of cube faces. The values 0 to 255 are reserved for current and future layouts.

a value of 0 corresponds to a grid with 3 columns and 2 rows. Faces are oriented upwards for the front, left, right, and back faces. The up face is oriented so the top of the face is forwards and the down face is oriented so the top of the face is to the back.

| right face | left face | up face |
| --- | --- | --- |
| down face | front face | back face |

cbmp_padding specifies the number of pixels to pad from the edge of each cube face.

equi_projection_version specifies the version of the Spherical Media RFC v2 equirectangular projection box. Only value 0 shall be used in this SEI.

equi_projection_flags is for future use in the Spherical Media RFC v2 equirectangular projection box. Only value 0 shall be used in this SEI.

The projection bounds use 0.32 fixed point values. These values represent the proportion of projection cropped from each edge not covered by the video frame. For an uncropped frame all values are 0

equi_projection_bounds_top is the amount from the top of the frame to crop

equi_projection_bounds_bottom is the amount from the bottom of the frame to crop; must be less than 0xFFFFFFFF - projection_bounds_top

equi_projection_bounds_left is the amount from the left of the frame to crop

equi_projection_bounds_right is the amount from the right of the frame to crop; must be less than 0xFFFFFFFF - projection_bounds_left

spherical_rfc2_byte shall be parsed from the bitstream until the number of bytes identified by spherical_rfc2_byte_count is 0

# References

[1] https://github.com/google/spatial-media

[2] http://www.w3.org/1999/02/22-rdf-syntax-ns#"

[3] https://support.google.com/youtube/answer/6316263?hl=en&ref_topic=2888648

# Patent rights declaration(s)

MovieLabs does not have any current or pending patent rights relating to the technology described in this contribution.