| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0032 |
| --- | --- |

| Title: | AHG12: MCTS extraction usage scenario and showcase |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | R. Skupin, Y. Sanchez, C. Hellge, T. Schierl T. Wrede, T. Christophory | Tel: Email: | +49 30 31002 185 robert.skupin@hhi.fraunhofer.de thomas.wrede@ses.com |
| Source: | Fraunhofer HHI SES S.A. |  |  |

_____________________________

# Abstract

This informational document describes usage scenario and showcase for MCTS extraction as drafted in JCTVC-Y1008. The usage scenario is based on a Fraunhofer HHI collaboration with European satellite operator SES S.A. in which HEVC coded panoramic content beyond UHD resolution is broadcasted via satellite to a lower resolution end device that presents a switchable picture subsection. This document further provides information on how MCTS extraction allows to reduce decoder requirements in the given usage scenario.

Revision 1 adds SES S.A. as co-author and a slideset.

# Introduction

Fraunhofer HHI, in cooperation with SES S.A., first demonstrated the transmission of a panoramic video signal via satellite to various devices at the International Broadcast Convention (IBC) 2016. The transmitted content consists of two scenes shot with Fraunhofer HHI's OmniCam camera at 10K x 2K resolution that are HEVC encoded using Fraunhofer HHI’s software broadcast encoder at around 20 Mbps. The transmission side relies on SES's ASTRA 19.2°E satellites reaching around 100 million households in 35 European countries and the signal is still broadcasted over a publicly receivable test channel as of January 2017. The demonstration entailed satellite signal reception, video decoding and displaying a switchable picture subsection on an UHD TV with the user being able to interact through a remote control. The whole 10K x 2K picture is decoded in this demonstration by a standard HEVC software decoder without tiling functionality in order to gather the desired picture subsection.

This usage scenario serves as a showcase for use of MCTS (Motion Constrained Tile Sets), specifically MCTS extraction as drafted in JCTVC-Y1008 [1]. MCTS extraction would allow reduction on decoder requirements in the above-mentioned scenario, as a decoder would only need to decode whichever subsection of the picture the viewer is interested in.

Figure 1: Illustration of HEVC tiled encoding of panoramic content beyond UHD.

An example is given in Figure 1, that shows a screenshot of one of the sequences used for the above-mentioned demonstration along with illustrations of HEVC tiling (white lines) and the relevant picture subsection at three consecutive time instances t0, t1 and t2 (colored overlays) after user interaction.

The HEVC bitstream in this example would employ tiling (7 columns x 4 rows) together with Temporal motion-constrained tile sets SEI messages for each 2x2 set of tiles. The MCTS guarantees coding-wise independence from neighboring tiles and associate unique values of mcts_id to the available tile picture subsections.

Each RAP of the video bitstream would also include a MCTS extraction info set SEI message as drafted in JCTVC-Y1008 that carries substitute parameter sets associated with the various values of mcts_id. Execution of the process drafted in the MCTS extraction information set SEI message semantics in JCTVC-Y1008 would result in a conformant HEVC bitstream that includes only the NAL units associated with the desired mcts_id for a given time span between two RAPs, e.g. as illustrated in Figure 1: the red subsection from t0 until t1, the blue subsection between t1 and t2 and the green subsection from t2 on. Therefore, the number of samples the video decoder would have to process in this case is reduced and hence the video decoder requirement compared to the above-mentioned demonstration.

Additionally, the MCTS extraction information nesting SEI message described in JCTVC-Y1008 would allow to replace further SEI messages that may be invalidated through MCTS extraction, e.g. Pan-Scan rectangle SEI message, Picture snapshot SEI message, Frame packing arrangement SEI message, Decoded Picture Hash SEI message.

# References

# R. Skupin, “Motion-Constrained Tile Sets Extraction Information SEI Messages Draft 1”, Document of Joint Collaborative Team on Video Coding, JCTVC-Y1008, m39592, Chengdu, CN, October 2016.