| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0034 |
| --- | --- |

| Title: | Spherical viewport SEI for HEVC and AVC 360 video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce, Qian Xu | Tel: Email: | +1 503 712 2977 jill.boyce@intel.com qian.z.xu@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

An SEI message is proposed for HEVC and AVC to indicate two different but related spherical viewport modes for 360 degree video. The first proposed mode (“ROI mode”) allows the encoder to indicate a “director’s view,” which is a recommended rectangular viewport in a rectilinear projection from the coded spherical video, similar to being considered for inclusion in the systems layer by the OMAF AHG on ROI. The second proposed mode (“viewport mode”) is to indicate that the bitstream contains a rectangular region in a rectilinear projection format that has already been extracted from a spherical video and (re-)encoded. Because of the relationship of the two modes and the similar required syntax, both modes are proposed to be supported with a single SEI message.

# Proposal

360 degree video can be coded using existing HEVC and AVC profiles, using a projection format to map spherical video data to a rectangular format used for coding. Only a portion of the 360 x 180 degree coded video is typically viewed at a given time, with the particular position viewed determined at the client, and then projecting the coded data from its coding projection (such as ERP) to a sphere, and then from the sphere to a rectilinear viewport. If a head mounted display (HMD) is used, the position of the HMD is used to determine which region is used for the viewport. If a 2D display is used, such as on a computer, an input interface is provided for the user to indicate the desired viewport.

The MPEG OMAF group on ROI is studying indications of regions of interest. One of the formats under discussion is to provide metadata to indicate the size and position of a rectangular rectilinear region. A rectangular region in a rectilinear projection does not correspond to a rectangle in the ERP format. OMAF is considering incorporating this metadata in the systems layer. In this contribution, it is proposed to include this information in HEVC and AVC SEI messages, regardless of whether or not it is ultimately included in the systems layer.  This first mode of the proposed SEI message is the “ROI mode.”

An additional mode is also proposed to be supported by the same SEI message, “viewport mode.” A Media Aware Network Element (MANE) may be used as a transcoder in an omnidirectional video distribution system, to offload decoding computing resources from the client and to reduce network bandwidth. The MANE receives upstream information of a client HMD position, decodes its input video, extracts a region which includes the expected display viewport based on the client HMD position and maps it to a rectilinear projection, and re-encodes the video. The SEI message indicates the size and position of the extracted region, to enable display of the desired viewport on the client HMD.

| SEI mode | Coded video contents (yaw x pitch angles) | Projection format | Metadata indicates |
| --- | --- | --- | --- |
| ROI mode | 360º x 180º | Many options – ERP, CMP, ISP, OSP, SSP, etc. | Content creator’s recommended viewing region (“Director’s view”) |
| Viewport | FOV, i.e. 75º x 75º or 90º x 90º, which was previously extracted from a sphere | Rectilinear | Region that was extracted from a sphere, based on communicated client HMD position |

## ROI mode

When viewing omnidirectional video, the viewer typically views only a small portion of the full 360º x 180º video at a given time. A viewer may miss the “most interesting” area of the video. The content creator/director may provide information indicating the suggested viewing angle and size, which can be sent as metadata in an SEI message. The client viewer may choose to follow the recommendation or to override it.

In discussion in the OMAF AHG on ROI, it was suggested that for “director’s view” type of ROI indication, the most appropriate shaped region to indicate is a rectangular region on a rectilinear viewport, as that is what will ultimately be displayed for viewing. (It was suggested for some other ROI use cases that a rectangular region in the ERP format would be a more appropriate region to indicate.)

Figure 1 was provided by Emmanuel Thomas on the email reflector of the OMAF AHG on ROI, to illustrate a rectangular region of a rectilinear projection as mapped to a sphere. The region is bounded by two horizontal “great circles” and two vertical “great circles,” where a great circle is a diameter of the sphere.

Figure 1. Rectangular region of a rectilinear projection mapped to a sphere

[Source: Emmanuel Thomas]

This region can be described by a center position represented by (yaw, pitch, roll), and a size represented by a range of yaw angles and pitch angles.

Because the recommended viewing region is used by the decoding client after the decode process, it seems appropriate to include this metadata in an SEI message, regardless of whether or not it is included in the systems layer by the OMAF group.

## Viewport mode

An example use case for the proposed SEI message in viewport mode is for a Media Aware Network Element (MANE) used as a real-time transcoder in an omnidirectional video distribution system. The MANE offloads decoding computing resources from the client and reduces network bandwidth, by sending a lower resolution video to the client, by utilizing upstream information from the client indicating the instantaneous HMD position/orientation. Figure 2 illustrates an example MANE-assisted omnidirectional video distribution system.

Figure 2. MANE-assisted omnidirectional video distribution system

The MANE decodes a high resolution video which represents the entire 360º x 180º spherical video, in any projection format (ERP, CMP, ISP, OSP, etc.) Based upon instantaneous HMD position data received from the client, and possibly the size of the field of view of the client display, the MANE then extracts a region of the spherical video and maps it to a rectangular region in a rectilinear projection format. The extracted region contains the viewport that would be viewed on the HMD for that position, possibly with a buffer region around it. That extracted region is re-encoded, and the proposed SEI message in “viewport mode” is used to indicate the position and size of the extracted region.

At the HMD, it is necessary to have information about the position and size of the extracted region in order to properly display the appropriate viewport, because the HMD position may change continuously. If the extracted region contains a buffer around the received HMD position data, allowing for some small movement during the period of latency, the displayed viewport may be cropped from the decoded picture.

The same metadata is needed for this mode as for the ROI mode, namely the spherical center position (yaw, pitch, roll) and the range of yaw and pitch angles of a rectangular region in a rectilinear projection of a sphere. Hence the two modes may share an SEI message.

# Proposed syntax and semantics

## Syntax

| spherical_viewport( payloadSize ) { | Descriptor |
| --- | --- |
| spherical_ viewport _cancel_flag | u(1) |
| if( !spherical_viewport_cancel_flag ) { |  |
| spherical_viewport_mode | u(1) |
| spherical_viewport_precision | ue(v) |
| spherical_viewport_yaw | se(v) |
| spherical_viewport_pitch | se(v) |
| spherical_viewport_roll | se(v) |
| spherical_viewport_range_yaw | se(v) |
| spherical_viewport_range_pitch | se(v) |
| spherical_viewport_persistence_flag | u(1) |
| } |  |
| } |  |

## Semantics

When the associated picture has PicOutputFlag equal to 1, the spherical viewport SEI message informs the decoder of a mode which indicates either that a region of interest of a spherical projection is recommended to be displayed, or that the decoded picture corresponds to a region previously extracted from a spherical projection, bounded by two great circles and two small circles on the sphere.

spherical_viewport_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous spherical viewport SEI message in output order. spherical_viewport_cancel_flag equal to 0 indicates that spherical viewport information follows.

spherical_viewport_mode equal to 0 indicates that a region of interest is recommended for display. spherical_viewport_mode equal to 1 indicates that the decoded picture represents a specific region that had been extracted from a spherical projection, bounded by two horizontal great circles and two vertical great circles.

spherical_viewport_precision specifies the precision of spherical_viewport_yaw, spherical_viewport_pitch, spherical_viewport_roll, spherical_viewport_range_yaw, and spherical_viewport_range_pitch. spherical_viewport_precision shall be in the range of 0 to 15 inclusive.

spherical_viewport_yaw, when spherical_viewport_mode equal to 0, specifies the center of a recommended viewing region of the projection mapped decoded picture in 180* spherical_orientation_yaw ÷ 2spherical_viewport_precision degrees along the Z-axis. spherical_viewport_yaw, when spherical_viewport_mode equal to 1, specifies the Z-axis position of the center of a region on a sphere from which the current picture was extracted.

spherical_viewport_pitch, when spherical_viewport_mode equal to 0, specifies the center of a recommended viewing region of the projection mapped decoded picture in 180* spherical_orientation_pitch ÷ 2spherical_viewport_precision degrees along the Y-axis. spherical_viewport_pitch, when spherical_viewport_mode equal to 1, specifies the Y-axis position of the center of a region on a sphere from which the current picture was extracted.

spherical_viewport_roll, when spherical_viewport_mode equal to 0, specifies the center of a recommended viewing region of the projection mapped decoded picture in 180* spherical_orientation_roll ÷ 2spherical_viewport_precision degrees along the X-axis. spherical_viewport_roll, when spherical_viewport_mode equal to 1, specifies the X-axis position of the center of a region on a sphere from which the current picture was extracted.

spherical_viewport_range_yaw, when spherical_viewport_mode equal to 0, specifies the size of a recommended viewing region of the projection mapped decoded picture in 180* spherical_orientation_range_yaw ÷ 2spherical_viewport_precision degrees along the Z-axis. spherical_viewport_range_yaw, when spherical_viewport_mode equal to 1, specifies the Z-axis size of the region on a sphere from which the current picture was extracted.

spherical_viewport_range_pitch, when spherical_viewport_mode equal to 0, specifies the size of a recommended viewing region of the projection mapped decoded picture in 180* spherical_orientation_range_pitch ÷ 2spherical_viewport_precision degrees along the Y-axis. spherical_viewport_range_pitch, when spherical_viewport_mode equal to 1, specifies the Y-axis size of the region on a sphere from which the current picture was extracted.

spherical_viewport_persistence_flag specifies the persistence of the spherical viewport SEI message for the current layer.

spherical_viewport_persistence_flag equal to 0 specifies that the spherical viewport SEI message applies to the current decoded picture only.

Let picA be the current picture. spherical_viewport_persistence_flag equal to 1 specifies that the spherical viewport SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a spherical viewport SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).