| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0035 |
| --- | --- |

| Title: | Showcase of the content colour volume SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Joel Sole Dmytro Rusanovskyy | Email: | joels@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This document demonstrates a case of the content colour volume SEI message proposed in JCTVC-Y0040. Specifically, an example of frame processing using the third mode of JCTVC-Y0040 is presented.

# Introduction

JCTVC-V0038 presented at the 22nd JCT-VC meeting provided provisions for the description of content colour volume (CCV) metadata. Further extensions were presented during the 23rd and 24th JCT-VC meetings. JCTVC-V0038, JCTVC-W0098, and JCTVC-X0052, in particular, introduced a rather extensive SEI message that enabled the description of the effective colour volume message using a multitude of representations. This last proposal permitted the signaling of a rather accurate description of the colour volume of the content for a variety of applications. The document also provided a description of the potential benefits of such representations. JCTVC-W0057 and JCTVC-X0040 were built on similar principles, but suggested the use of a simpler SEI message with a more limited scope. Finally, JCTVC-X0069 extended upon this work by providing additional means for explicitly describing the colour volume occupied by the content. At the parent-body meeting, it was agreed that this functionality was indeed desirable and the proponents were asked to come up with a combined syntax solution for describing the colour volume of the content.

Document JCTVC-Y0040 presented the combined syntax at the 25th JCTVC meeting. At the meeting, it was decided to include in a specification draft the starting point of the CCV SEI message, which consists of a light SEI message without spatial partition signalling, 3 component colour primaries coordinates corresponding to a triangle (CIE 1931 x, y, Y coordinates), negative x and y coordinates allowed, and min/max luminance. The draft text is documented in JCTVC-Y1005. It was also requested to show any benefit of additional information in the SEI message, that is, to “showcase” what can be achieved with some more elaborate CCV representation.

This document showcases mode 3 in JCTVC-Y0040 in section 2. In section 3, the draft text in JCTVC-Y0040 is provided with some modifications discussed at the last meeting.

# Showcase

A specific example of usage of the CCV SEI for colour gamut mapping is showcased. The example shows the benefits of mode 3 on the top of the base CCV SEI design in JCTVC-Y1005 (mode 1).

The MagicHour sequence is taken as an example. The sequence is in the BT.2020 container, but its colour gamut is P3. The reference for comparison purposes is the base CCV SEI, which signals minimum luminance, maximum luminance, and three colour coordinates that represent the content colour gamut (in this example, these coordinates form a triangle that is contained within the P3 content as the content does not occupy the full P3 content). Meanwhile, mode 3 uses more accurate color volume description with the description of eight luma cross-section gamut with three coordinates each. The overhead of signalling this every IRAP picture is around 1.25 kbps.

Gamut mapping is applied taken as an input the information provided by the reference (Figure 1) and the information provided by mode 3 (Figure 2), the syntax corresponding to which is highlighted in Section 3.1. As can be observed in the blue inset box, having multiple luminance cross-section enables better reproduction of the colour of the lamp when compared to single luminance cross-section. The red colour in the lamp is less saturated when only one luminance cross-section is signalled. As a comparison, a simple gamut conversion to 709 with clipping is also shown in Figure 3; the clipping clearly affects the reproduction of the lamp and does not preserve the structure of the bulb.

Figure 1: Mapping with minimum and maximum luminance, and one cross-section of colour coordinates

Figure 2: Mapping with minimum and maximum luminance, and multiple cross-section of colour coordinate

Figure 3: Simple container conversion with clipping

# Draft text

Draft text in JCTVC-Y0040: to be merged with the latest version of JCTVC-Y1005.

## General SEI syntax structure

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| … |  |
| else if( payloadType = = 149 ) |  |
| content_colour_volume( payloadSize ) |  |
| … |  |
| …. |  |
| else if( payloadType = = 181 ) |  |
| alternative_depth_info( payloadSize ) /* specified in Annex I */ |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| … |  |
| } |  |
| } |  |

## Content colour volume SEI message syntax

| content_colour_volume( payloadSize ) { | Descriptor |
| --- | --- |
| ccv_cancel_flag | u(1) |
| if( !ccv_cancel_flag ) { |  |
| ccv_persistence_flag | u(1) |
| for( c = 0; c < 3; c++ ) { |  |
| ccv_primaries_x[ c ] | i(16) |
| ccv_primaries_y[ c ] | i(16) |
| } |  |
| ccv_min_lum_value | u(32) |
| ccv_max_lum_value | u(32) |
| /* Mode 2 */ |  |
| … |  |
| /* Mode 3 – implicit representation */ |  |
| ccv_num_explicit_repn | ue(v) |
| for( i = 0; i < ccv_num_explicit_repn; i++ ) { |  |
| ccv_explicit_repn_type[ i ] | u(8) |
| ccv_expl_num_ranges_minus1[ i ] | ue(v) |
| if( ccv_expl_num_ranges_minus1[ i ] > 0 ) |  |
| ccv_expl_ranges_idc[ i ] | u(2) |
| for( j = 0; j <= CcvExplNumRanges[ i ]; j++ ) { |  |
| if( !ccv_expl_ranges_idc[ i ] ¦¦ j = = 0 ¦¦ j = = CcvExplNumRanges[ i ] ) |  |
| ccv_expl_val[ i ][ j ] | i(32) |
| ccv_expl_num_coord[ i ][ j ] | ue(v) |
| for( k = 0; k < ccv_expl_num_coord[ i ][ j ]; k++ ) { |  |
| ccv_expl_coord_1[ i ][ j ][ k ] | i(16) |
| ccv_expl_coord_2[ i ][ j ][ k ] | i(16) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Semantics of content colour volume SEI message

The content colour volume SEI message provides information about the colour volume characteristics of the associated picture. This SEI describes the colour volume characteristics of the associated picture in terms of a nominal range and deviations from this range may occur. This SEI message requires knowledge of the colour description information of the signal, such as the colour_primaries, transfer_characteristics, and matrix_coeffs in the VUI parameters syntax. This SEI message shall not be present when any of these parameters is unspecified or reserved. When the value of transfer_characteristics is equal to 1, 6, 7, 14, or 15, then the BT.1886 transfer function should be used to convert the signal back to its linear representation. [Ed. (AR): Check to keep BT.1886 or inverse of BT.709.] When the value of transfer_characteristics is equal to 18, the EOTF specified in BT.2100 when using the HLG transfer function shall be used. [Ed. (AR): Check to remove this sentence on HLG transfer function. Ed. (AT): What also happens when the alternative transfer characteristics SEI message is present may need to be clarified.] Otherwise, the exact inverse of the transfer function specified in Table E.4 (Transfer Characteristics) shall be used to convert the non-linear signal to a linear representation.

ccv_cancel_flag equal to 1 indicates that the content colour volume SEI message cancels the persistence of any previous content colour volume SEI message in output order that applies to the current layer. ccv_cancel_flag equal to 0 indicates that content colour volume information follows.

ccv_persistence_flag specifies the persistence of the content colour volume SEI message for the current layer.

ccv_persistence_flag equal to 0 specifies that the content colour volume applies to the current decoded picture only.

Let picA be the current picture. ccv_persistence_flag equal to 1 specifies that the content colour volume SEI message persists for the current layer in output order until any of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a content colour volume SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for the picture order count of picB.

ccv_primaries_x[ c ] and ccv_primaries_y[ c ] specify the normalized x and y chromaticity coordinates, respectively, of the colour primary component c of the nominal content colour volume in increments of 0.00002, according to the CIE 1931 definition of x and y as specified in ISO 11664-1 (see also ISO 11664-3 and CIE 15). For describing colour volumes that use red, green, and blue colour primaries, it is suggested that index value c equal to 0 should correspond to the green primary, c equal to 1 should correspond to the blue primary, and c equal to 2 should correspond to the red colour primary (see also Annex E and Table E.3).

The values of ccv_primaries_x[ c ] and ccv_primaries_y[ c ] shall be in the range of −5 000 000 to 5 000 000, inclusive.

ccv_min_lum_value specifies the normalized minimum luminance value, according to CIE 1931, that is expected to be present in the content, where values are normalized to Lo or Lc as specified in Table E.4 and according to the transfer characteristics of the signal. The values of ccv_min_lum_value are in normalized increments of 0.00000001 in the linear representation.

NOTE 1 – The resulting domain from this conversion process may or may not represent light in a source or display domain – it is merely a gamut representation domain rather than necessarily being a representation of actual light in either the scene or display domain. Therefore, the values corresponding to ccv_min_lum_value might not necessarily correspond to a true luminance value.

[Ed. (AT): It should be noted that even if normalized, the value should not be constrained to the range of [0,1]. Transfer characteristics functions such as xvYCC permit values larger than 1. The current definition basically allows us to specify a value up to L = 42.94. We could maybe change the increment to be equal to 0.000001 instead of the current value, thus permitting even larger ranges. Why/How was the initial increment selected? The same comments apply to the max. Of note also, this only limits luminance, we do not know what the limits are for color/chroma or for R, G, and B separately. It should be noted, as an example, that Blue could in fact be rather large given its contribution to luminance/luma.].

ccv_max_lum_value specifies the maximum luminance value, according to CIE 1931, that is expected to be present in the content, where values are normalized to Lo or Lc as specified in Table E.4 and according to the transfer characteristics of the signal. The values of ccv_max_lum_value are in normalized increments of 0.00000001 inclusive in the linear representation.

NOTE 2 – The resulting domain from this conversion process may or may not represent light in a source or display domain – it is merely a gamut representation domain rather than necessarily being a representation of actual light in either the scene or display domain. Therefore, the values corresponding to ccv_max_lum_value might not necessarily correspond to a true luminance value.

…

…

…

ccv_num_explicit_repn specifies the number of explicit colour volume representations of the content specified in the SEI message. The value of ccv_num_explicit_repn shall be in the range of 0 to 3 inclusive. The value of ccv_num_explicit_repn in the range of 4 to 255, inclusive, are reserved for future use by ITU-T | ISO/IEC.

ccv_explicit_repn_type[ i ] specifies the interpretation, as described in Table , of the syntax elements ccv_expl_num_coord_1[ i ][ j ][ k ], ccv_expl_num_coord_2[ i ][ j ][ k ], and ccv_expl_range_value[ i][ j ]. The value of ccv_explicit_repn_type[ i ] shall be in the range of 0 to 2, inclusive. The value of ccv_explicit_repn_type[ i ] in the range of 3 to 255, inclusive, are reserved for future use by ITU-T | ISO/IEC. When not present, the value of ccv_explicit_repn_type[ i ] is inferred to be equal to 0.

ccv_expl_num_ranges_minus1[ i ] plus 1 specifies the number of ranges used in the i-th explicit representation to describe the cross-section of content volume in a colour space as specified by Table 1 Interpretation of ccv_explicit_repn_type[ ][ ]. The value of ccv_explicit_num_ranges_minus1[ i ] shall be in the range of 0 to 15, inclusive. When not present, the value of ccv_explicit_num_ranges_minus1[ i ] is inferred to be equal to 0.

The variable CcvExplNumRanges[ i ] is set equal to ccv_explicit_num_ranges_minus1[ i ] + 1.

ccv_expl_ranges_idc[ i ] equal to 0 specifies that the syntax elements ccv_expl_range_value[ i ][ j ] are explicitly signalled. ccv_expl_ranges_idc[ i ] equal to 1 or 2 specifies that the syntax element ccv_expl_range_value[ i ][ j ] for j in the range of 1 to CcvExplNumRanges[ i ] – 1, inclusive, are not explicitly signalled. When ccv_expl_num_ranges_minus1 is equal to 0, the value of ccv_expl_ranges_idc[ i ] is inferred to be equal to 1. The value of ccv_expl_ranges_idc[ i ] shall be in the range of 0 to 2, inclusive. The value of ccv_expl_ranges_idc[ i ] equal to 3 is reserved for future use by ITU-T | ISO/IEC.

ccv_expl_val[ i ][ j ] is used to derive the j-th value of the component with index 0, as described by the semantics of ccv_explicit_repn_type[ i ] and Table 1 Interpretation of ccv_explicit_repn_type[ ][ ], which is used to specify the colour volume of the content. The value of ccv_expl_val[ i ][ j ] is in normalized increments of 0.00000001 units as specified for the component with index 0 in Table 1 Interpretation of ccv_explicit_repn_type[ ][ ].

The variable CcvExplVal[ i ][ j ], for j in the range of 0 to CcvExplNumRanges[ i ], inclusive, is derived as follows:

CcvExplVal[ i ][ 0 ] = ccv_expl_val[ i ][ 0 ]
lumDiffVal = ccv_expl_val[ i ][ CcvExplNumRanges[ i ] ] – ccv_expl_val[ i ][ 0 ]
for( j = 1; j < CcvExplNumRanges[ j ]; i++ )
	if( ccv_expl_ranges_idc  = =  0 )
		CcvExplVal[ i ] = ccv_expl_val[ i ][ j ]
	else if(ccv_expl_ranges_idc = =  1 ) {
		diffVal = lumDiffVal ÷ CcvExplNumRanges[ i ] 
		CcvExplVal[ i ][ j ] = CcvExplVal[ i ][ j − 1 ] + diffVal
	}
	else if(ccv_expl_ranges_idc  = =  2 ) {
		logDiffVal = Log10( lumDiffVal ) ÷ CcvExplNumRanges[ i ]
		CcvExplVal[ i ][ j ] = CcvExplVal[ i ][ 0 ] + 										10( j*logDiffVal )
	}
CcvExplVal[ i ][ CcvExplNumRanges[ i ] ] = expl_vol_val[ i ][ CcvExplNumRanges[ i ] ]

When not present, the value of ccv_expl_val[ 0 ][ 0 ] and ccv_expl_val[ 0 ][ 1 ] is inferred to be equal to ccv_min_lum_value and ccv_max_lum_value, respectively.

ccv_expl_num_coord[ i ][ j ] specifies the number of points associated with the j-th value of the component with index 0, as described in  Table 1 Interpretation of ccv_explicit_repn_type[ ][ ], of the i-th explicit representation of the colour volume of the content. The value of ccv_expl_num_coord[ i ][ j ] shall be in the range of 0 to 8, inclusive. When not present, the value of ccv_expl_num_coord[ 0 ][ 0 ] is inferred to be equal to 3.

ccv_expl_coord_1[ i ][ j ][ k ] and ccv_expl_coord_2[ i ][ j ][ k ] specify the coordinate components with index value 1 and 2, respectively, of the k-th coordinate corresponding to the j-th value of the component with index 0 that is used to derive an estimate of the colour volume in increments of 0.00004. The values of ccv_expl_coord_1[ i ][ j ][ k ] and ccv_expl_coord_2[ i ][ j ][ k ] shall be in the range of − 25 000 to 25 000, inclusive.

For each value of j in the range of 0 to CcvExplNumRanges[ i ], inclusive, let convexRegion[ i ][ j ] refer to the two-dimensional convex hull of the coordinates ccv_expl_coord_1[ i ][ j ][ k ] and ccv_expl_coord_2[ i ][ j ][ k ] for k in the range of 0 to ccv_expl_num_coord[ i ] – 1, inclusive.

The colour volume of the content, colourVolumeBoundingRegion, is defined as the union of colourVolumeRegions[ i ][ j ] for j in the range of 0 to CcvExplNumRanges[ i ] − 1, inclusive, where colourVolumeRegions[ i ][ j ] is defined as the three-dimensional convex hull of the points convexRegion[ i ][ j ] and convexRegion[ i ][ j + 1 ].

Table 1 Interpretation of ccv_explicit_repn_type[ ][ ]

| ccv_explicit_repn_type[ i ] | Representation space |
| --- | --- |
| 0 | xyY; index correspondence 0: Y, 1:x, 2:y |
| 1 | Lab; index correspondence 0: L, 1: a, 2: b |
| 2 | YCbCr: index correspondence 0: Y, 1:Cb, 2: Cr |
| 3 – 255 | Reserved for future use by ITU-T/ISO-IEC |

Table D.1 – Persistence scope of SEI messages (informative)

| - SEI message | - Persistence scope |
| --- | --- |
| Buffering period | - The remainder of the bitstream |
| …. | - …. |
| Ambient viewing environment | - The CLVS containing the SEI message |
| Content Colour Volume | - Specified by the syntax of the SEI message |

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).