| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0036 |
| --- | --- |

| Title: | Suggested draft text for omni-directional projection indication SEI message with equirectangular projection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposed Draft |  |  |
| Author(s) or Contact(s): | Chad Fogg Jill Boyce Gary Sullivan | Email: | chadfogg@gmail.com jill.boyce@intel.com garysull@microsoft.com |
| Source: | Motion Picture Laboratories, Intel, Microsoft |  |  |

_____________________________

# Abstract

The draft SEI message text proposed in this document is proposed toward satisfying the mandates for Ad-hoc group 5 established at the end of the 25th meeting of JCT-VC in Chengdu, October 2016. The first mandate is to "develop formulae to project samples of an equirectangular format picture to 360°/omnidirectional spherical space." The second mandate is to "prepare and propose draft text for specification of a code point identification to indicate the use of the equirectangular projection mapping." The third mandate is to "prepare and propose draft text for an SEI message to carry the projection map type indicator."

# Proposal

- In Annex D, add omnidirectional_projection_indication to the sei_payload() syntax table in section D.2.1:

- [ payloadType number to be assigned by the AVC and HEVC specification editors ]

- In Annex D, add the following syntax table to section D.2:

| omnidirectional_projection_indication( payloadSize ) { | Descriptor |
| --- | --- |
| geometry_type | ue(v) |
| projection_type | ue(v) |
| } |  |

- In Annex D, add the following semantic definition to section D.3:

- D.3.X Omnidirectional projection indication

- The omnidirectional projection indication SEI message provides information to enable remapping of the colour samples of the output decoded pictures onto an alternative coordinate space for use in panoramic video applications.

- When an omnidirectional projection indication SEI message is present for any picture of a CLVS of a particular layer, an omnidirectional projection indication SEI message shall be present for the first picture of the CLVS. The omnidirectional projection SEI message persists for the current layer in decoding order from the current picture until the end of the CLVS.

The luma dimensions of the cropped output picture are defined by the following equations:

croppedWidthY = pic_width_in_luma_samples − SubWidthC * ( conf_win_right_offset + conf_win_left_offset )

croppedHeightY = pic_height_in_luma_samples − SubHeightC * ( conf_win_bottom_offset + conf_win_top_offset )

croppedWidthC = croppedWidthY / SubWidthC

croppedHeightC = croppedHeightY / SubHeightC

[Ed. These equations are included here for the sake of completeness of presentation, although they may be redundant with equations already found in the HEVC text.]

Figure D.X-1 cropped output of decoded picture luma samples

- geometry type indicates the mathematical convention where points within a space can be uniquely identified by a location in one or more dimensions. When geometry_type is equal to 1, the projection indicator is given in spherical coordinates, where  is the azimuth (longitude) and  is the elevation (latitude) as depicted in Figure D.X-2.

Figure D.X-2 Spherical surface coordinates , 

projection_type indicates the particular mapping of the rectangular decoder picture output samples onto the coordinate system specified by geometry_type.

When projection_type is equal to 1 and geometry_type is equal to 1, cropped output picture samples at locations (i, j) are indicated as corresponding to angular coordinates. For i equal to SubWidthC * conf_win_left_offset to SubWidthC * conf_win_left_offset + croppedWidthC − 1 and j equal to SubHeightC * conf_win_top_offset to SubHeightC * conf_win_top_offset − 1, the corresponding angular coordinates (, ) for the luma sample locations, in radians, are given by the following equirectangular mapping equations:

 = ( ( i′ + 0.5 ) ÷ croppedWidthY − 0.5 ) * 2π				[Eq. X1]

 = − ( ( j′ + 0.5 ) ÷ croppedHeightY − 0.5 ) * π				[Eq. X2]

where

i′ = i − SubWidthC * conf_win_left_offset				[Eq. X3]

j′ = j − SubHeightC * conf_win_top_offset				[Eq. X4]

When chroma_format_idc is not equal to 0 (monochrome), for i equal to conf_win_left_offset to conf_win_left_offset + croppedWidthC − 1 and j equal to conf_win_top_offset to conf_win_top_offset + croppedHeightC − 1, the corresponding angular coordinates (, ) for the chroma sample locations, in radians, are given by

 = ( ( i′ + CenterLeftOffsetC ) ÷ croppedWidthC − 0.5 ) * 2π		[Eq. X5]

 = − ( ( j′ + CenterTopOffsetC ) ÷ croppedHeightC − 0.5 ) * π		[Eq. X6]

where

i′ = i − conf_win_left_offset						[Eq. X7]

j′ = j − conf_win_top_offset						[Eq. X8]

and CenterLeftOffsetC and CenterTopOffsetC are given by Table XX as a function of chroma_format_idc and ChromaLocType, which is set as follows:

- If j % 2 is equal to 0, ChromaLocType is equal to chroma_sample_loc_type_top_field

- Otherwise, ChromaLocType is equal to chroma_sample_loc_type_bottom_field

Table XX – Derivation of CenterLeftOffsetC and CenterTopOffsetC

| chroma_format_idc | ChromaLocType | CenterLeftOffsetC | CenterTopOffsetC |
| --- | --- | --- | --- |
| 1 (4:2:0) | 0 | 0.125 | 0.25 |
| 1 (4:2:0) | 1 | 0.25 | 0.25 |
| 1 (4:2:0) | 2 | 0.125 | 0.125 |
| 1 (4:2:0) | 3 | 0.25 | 0.125 |
| 1 (4:2:0) | 4 | 0.125 | 0.375 |
| 1 (4:2:0) | 5 | 0.25 | 0.375 |
| 2 (4:2:2) | – | 0.125 | 0.5 |
| 3 (4:4:4) | – | 0.5 | 0.5 |

- When an omnidirectional projection indication SEI message is present for any picture of a CLVS of a particular layer and projection_type is equal to 1 and geometry_type is equal to 1, no frame packing arrangement SEI messages and segmented rectangular frame packing arrangement SEI messages shall be present in the CLVS.

Figure D-X-3  Equirectangular mapping between cropped output picture and spherical coordinates

# Patent rights declaration(s)

MovieLabs, Intel, and Microsoft do not have any current or pending patent rights relating to the technology described in this contribution (to the extent of the personal awareness of the authors).