| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0037 |
| --- | --- |

| Title: | On Motion-Constrained Tile Sets Extraction Information Set SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd., Camas, WA 98607, USA | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

In this document modifications and bug-fixes are proposed for Motion-Constrained Tile Sets Extraction Information Set SEI Message.

Following is proposed:

- Signaling of Picture Parameters Set (PPS) temporal identifier information for replacement Picture Parameter Sets.

- Specification text for creation of parameter set NAL Units (VPS, SPS, PPS NUT) including specifying creation of NAL unit headers for parameters sets.

- Modification of sub-bitstream MCTS extraction process to include extraction for target temporal sub-layers.

- A rule is provided specifying assignment of MCTS extraction information set identifier values.

The r1 revision make editorial changes suggested and agreed during presentation.

# Introduction

JCTVC-Y1008 [1] describes Motion-Constrained Tile Sets Extraction Information SEI Message Draft 1. In this document modifications and bug-fixes are proposed for Motion-Constrained Tile Sets Extraction Information Set SEI Message.

Following is proposed:

- Signaling of Picture Parameters Set (PPS) temporal identifier information for replacement Picture Parameter Sets. 
Current signaling of MCTS extraction information set SEI message [1] provides information which consists of a number of extraction information sets, each containing identifiers of the motion-constrained tile sets to which the extraction information set applies. Each extraction information set contains RBSP bytes of replacement video parameter sets, sequence parameter sets and picture parameter sets to be used during the sub-bitstream MCTS extraction process. However during the creation of PPS_NUT from the signalled PPS RBSP data that is signalled in MCTS extraction information SET SEI message, additional information is needed to populate the appropriate fields of PPS_NUT NAL Unit Header. For this we propose signalling of Picture Parameters Set (PPS) temporal identifier information for replacement Picture Parameter Sets.

- Specification text for creation of parameter set NAL Units (VPS, SPS, PPS NUT) including specifying creation of NAL unit headers for parameters sets. 
Current MCTS extraction set SEI message [1] provides information for VPS RBSP, SPS RBSP and PPS RBSP. A process is also described for creation of replacement VPS_NUTs, SPS_NUTs, and PPS_NUTs. However the description about how NAL unit header fields should be created for these replacement Parameter Set NUTs is missing.  We propose specification text for creation of parameter set NAL Units (VPS, SPS, PPS NUT) including specifying creation of NAL unit headers for parameters sets.

- Modification of sub-bitsream MCTS extraction process to include extraction for target temporal sub-layers. 
Currently defined sub-bitstream MCTS extraction process utilizing MCTS extraction information set SEI message does not allow bitstream extraction based on target temporal sub-layers. We propose a modification to the sub-bitstream MCTS extraction process to allow extraction of bitstream for a target highest TemporalID value – in addition to a target MCTS identifier and target MCTS extraction information set identifier.

- A missing rule is provided specifying assignment of MCTS extraction information set identifier values.

# Proposal

The specification text for the above proposals are shown highlighted compared to JCTVC-Y1008 [1].

D.2.34 MCTS extraction information set SEI message syntax

| mcts_extraction_info_set( ) { | Descriptor |
| --- | --- |
| num_extraction_info_sets_minus1 | ue(v) |
| for( i = 0; i <= num_extraction_information_sets_minus1; i++ ) { |  |
| num_associated_tile_set_identifiers_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_associated_tile_set_identifiers_minus1[ i ]; j++ ) |  |
| mcts_identifier[ i ][ j ] | ue(v) |
| num_vps_in_extraction_info_set_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_vps_in_extraction_set_minus1[ i ]; j++ ) |  |
| vps_rbsp_data_length[ i ][ j ] | ue(v) |
| num_sps_in_extraction_info_set_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_sps_in_extraction_set_minus1[ i ]; j++ ) |  |
| sps_rbsp_data_length[ i ][ j ] | ue(v) |
| num_pps_in_extraction_info_set_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_pps_in_extraction_set_minus1[ i ]; j++ ) { |  |
| pps_nuh_temporal_id_plus1[ i ][ j ] | u(3) |
| pps_rbsp_data_length[ i ][ j ] | ue(v) |
| } |  |
| while( !byte_aligned() ) |  |
| mcts_alignment_bit_equal_to_zero | f(1) |
| for( j = 0; j <= num_vps_in_extraction_set_minus1[ i ]; j++ ) |  |
| for( k = 0; k <= vps_rbsp_data_length[ i ][ j ]; k++ ) |  |
| vps_rbsp_data_bytes[ i ][ j ][ k ] | u(8) |
| for( j = 0; j <= num_sps_in_extraction_set_minus1[ i ]; j++ ) |  |
| for( k = 0; k <= sps_rbsp_data_length[ i ][ j ]; k++ ) |  |
| sps_rbsp_data_bytes[ i ][ j ][ k ] | u(8) |
| for( j = 0; j <= num_pps_in_extraction_set_minus1[ i ]; j++ ) |  |
| for( k = 0; k <= pps_rbsp_data_length[ i ][ j ]; k++ ) |  |
| pps_rbsp_data_bytes[ i ][ j ][ k ] | u(8) |
| } |  |
| } |  |

D.3.34 MCTS extraction information set SEI message semantics

An MCTS extraction information set SEI message provides supplemental information to carry out sub-bitstream MCTS extraction as specified below to derive a HEVC conformant bitstream from a motion-constrained tile set. The information consists of a number of extraction information sets, each containing identifiers of the motion-constrained tile sets to which the extraction information set applies. Each extraction information set contains RBSP bytes of replacement video parameter sets, sequence parameter sets and picture parameter sets to be used during the sub-bitstream MCTS extraction process.

Let a set of pictures associatedPicSet be the pictures from the access unit containing the MCTS extraction information set SEI message, inclusive, up to but not including the first of any of the following in decoding order:

–	The next access unit, in decoding order, that contains an MCTS extraction information set SEI message.

–	The next IRAP picture with NoRaslOutputFlag equal to 1, in decoding order.

–	The next IRAP access unit, in decoding order, with NoClrasOutputFlag equal to 1.

The scope of the MCTS extraction information set SEI message is the set of pictures associatedPicSet.

When an MCTS extraction information set tile sets SEI message is present for any picture in associatedPicSet, a temporal motion-constrained tile set SEI message shall be present for the first picture of associatedPicSet in decoding order and may also be present for other pictures of associatedPicSet. The temporal motion-constrained tile sets SEI message shall have with mcts_id[ ] equal to mcts_identifer[ ] for all pictures in associatedPicSet.

When an MCTS extraction information set tile sets SEI message is present for any picture in associatedPicSet, an MCTS extraction information set SEI message shall be present for the first picture of associatedPicSet in decoding order and may also be present for other pictures of associatedPicSet.

The MCTS extraction information set SEI message shall not be present for any picture in associatedPicSet when tiles_enabled_flag is equal to 0 for any PPS that is active for any picture in associatedPicSet.

The MCTS extraction information set SEI message shall not be present for any picture in associatedPicSet unless every PPS that is active for any picture in associatedPicSet has the same values of the syntax elements num_tile_columns_minus1, num_tile_rows_minus1, uniform_spacing_flag, column_width_minus1[ i ], and row_height_minus1[ i ].

NOTE 1 – This constraint is similar to the constraint associated with tiles_fixed_structure_flag equal to 1, and it may be desirable for tiles_fixed_structure_flag to be equal to 1 when the MCTS extraction information set SEI message is present (although this is not required).

When more than one MCTS extraction information set SEI message is present for the pictures of associatedPicSet, they shall contain identical content.

NAL units that contain tiles belonging to tile set tileSetA shall not contain tiles that do not belong to tile set tileSetA.

The number of MCTS extraction information set SEI messages in each access unit shall not exceed 5.

num_extraction_info_sets_minus1 plus 1 indicates the number of extraction information sets contained in the MCTS extraction information set SEI message to be applied in the mcts extraction process. The value of num_extraction_info_sets_minus1 shall be in the range of 0 to 232 − 2, inclusive.

The i-th extraction information set is assigned a MCTS extraction information set identifier value equal to i.

num_associated_tile_set_identifiers_minus1[ i ] plus 1 indicates the number of values of mcts_id of the tile sets in the i-th extraction information set. The value of num_extraction_info_sets_minus1[ i ] shall be in the range of 0 to 232 − 2, inclusive.

mcts_identifier[ i ][ j ] identifies the j-th tile set with mcts_id equal to mcts_identifier[ i ][ k ] associated to the i-th extraction information set. The value of mcts_identifier[ i ][ j ] shall be in the range of 0 to 232 − 2, inclusive.

num_vps_in_extraction_info_set_minus1[ i ] plus 1 indicates the number of replacement video parameter sets in the i-th extraction information set. The value of num_vps_in_extraction_info_set_minus1[ i ] shall be in the range of 0 to 15, inclusive.

vps_rbsp_data_length[ i ][ j ] indicates the number of bytes vps_rbsp_data_bytes[ i ][ j ][ k ] of the following j-th replacement video parameter set in the i-th extraction information set.

num_sps_in_extraction_info_set_minus1[ i ] plus 1 indicates the number of replacement sequence parameter sets in the i-th extraction information set. The value of num_sps_in_extraction_info_set_minus1[ i ] shall be in the range of 0 to 15, inclusive.

sps_rbsp_data_length[ i ][ j ] indicates the number of bytes sps_rbsp_data_bytes[ i ][ j ][ k ] of the following j-th replacement sequence parameter set in the i-th extraction information set.

num_pps_in_extraction_info_set_minus1[ i ] plus 1 indicates the number of replacement picture parameter sets in the i-th extraction information set. The value of num_pps_in_extraction_info_set_minus1[ i ] shall be in the range of 0 to 63, inclusive.

pps_nuh_temporal_id_plus1[ i ][ j ] specifies a temporal identifier for generating PPS NAL unit associated with PPS data specified in PPS RBSP specified by pps_rbsp_data_bytes[ i ][ j ][ ] for the j-th replacement picture parameter set for the i-th extraction information set.

pps_rbsp_data_length[ i ][ j ] indicates the number of bytes pps_rbsp_data_bytes[ i ][ j ][ k ] of the following j-th replacement picture parameter set in the i-th extraction information set.

mcts_alignment_bit_equal_to_zero shall be equal to 0.

vps_rbsp_data_bytes[ i ][ j ][ k ] contains the k-th byte of the RBSP of the following j-th replacement video parameter set in the i-th extraction information set.

sps_rbsp_data_bytes[ i ][ j ][ k ] contains the k-th byte of the RBSP of the following j-th replacement sequence parameter set in the i-th extraction information set.

pps_rbsp_data_bytes[ i ][ j ][ k ] contains the k-th byte of the RBSP of the following j-th replacement picture parameter set in the i-th extraction information set.

The sub-bistream MCTS extraction process is applied as follows:

Let a bitstream inBitstream, a target MCTS identifier mctsIdTarget and target MCTS extraction information set identifier mctsEISIdTarget, a target highest TemporalId value mctsTIdTarget be the inputs to the sub-bistream MCTS extraction process.

The output of the sub-bistream MCTS extraction process is a sub-bitstream outBitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that is the output of the process specified in this clause with the bitstream shall be a conforming bitstream.

The output sub-bitstream is derived as follows:

–	The bitstream outBitstream is set to be identical to the bitstream inBitstream.

–	The lists ausWithVPS, ausWithSPS and ausWithPPS are set to consist of all access units within outBitstream containing VCL NAL units with types VPS_NUT, SPS_NUT and PPS_NUT.

–	Remove all SEI NAL units that have nuh_layer_id equal to 0 and that contain non-nested SEI messages.

NOTE 2 – A "smart" bitstream extractor may include appropriate non-nested SEI messages in the extracted sub-bitstream, provided that the SEI messages applicable to the sub-bitstream were present as nested SEI messages in the mcts_extraction_info_nesting ( ) in the original bitstream.

- Remove from outBitstream all NAL units with types:

- VCL NAL units that contain tiles not belonging to the tile set with mcts_id[ i ] equal to mctsIdTarget,

- non-VCL NAL units with types VPS_NUT, SPS_NUT or PPS_NUT.

- Insert into all access units within the list ausWithVPS in outBitstream num_vps_in_extraction_info_minus1[ mctsEISIdTarget ] plus 1 NAL units with type VPS_NUT generated from the VPS RBSP data in the mctsEISIdTarget-th MCTS extraction information set, i.e. vps_rbsp_data_bytes[ mctsEISIdTarget ][ j ][ ] for all values of j in the range of 0 to num_vps_in_extraction_info_minus1[ mctsEISIdTarget ], inclusive. For each VPS_NUT that is generated the nuh_layer_id is set equal to 0 and nuh_temporal_id_plus1 is set equal to 1.

- Insert into all access units within the list ausWithSPS in outBitstream num_sps_in_extraction_info_minus1[ mctsEISIdTarget ] plus 1 NAL units with type SPS_NUT generated from the SPS RBSP data in the mctsEISIdTarget-th MCTS extraction information set, i.e. sps_rbsp_data_bytes[ mctsEISIdTarget ][ j ][ ] for all values of j in the range of 0 to num_sps_in_extraction_info_minus1[ mctsEISIdTarget ], inclusive. For each SPS_NUT that is generated the nuh_layer_id is set equal to 0 and nuh_temporal_id_plus1 is set equal to 1.

- Insert into all access units within the list ausWithPPS in outBitstream num_vps_in_extraction_info_minus1[ mctsEISIdTarget ] plus 1 NAL units with type PPS_NUT generated from the PPS RBSP data in the mctsEISIdTarget-th MCTS extraction information set, i.e. pps_rbsp_data_bytes[ mctsEISIdTarget ][ j ][ ] for all values of j in the range of 0 to num_pps_in_extraction_info_minus1[ mctsEISIdTarget ], inclusive for which   pps_nuh_temporal_id_plus1[ mctsEISIdTarget ][ j ]  is less than or equal to mctsTIdTarget. For each PPS_NUT that is generated the nuh_layer_id is set equal to 0 and nuh_temporal_id_plus1 is set equal to pps_nuh_temporal_id_plus1[ mctsEISIdTarget ][ j ] for all values of j in the range of 0 to num_pps_in_extraction_info_minus1[ mctsEISIdTarget ], inclusive for which   pps_nuh_temporal_id_plus1[ mctsEISIdTarget ][ j ]  is less than or equal to mctsTIdTarget.

- Remove from outBitstream all NAL units with TemporalId greater than mctsTIdTarget.

–	For each remaining VCL NAL units in outBitstream, adjust the slice segment header as follows:

–	For the first VCL NAL unit within each access unit, set the value of first_slice_segment_in_pic_flag equal to 1, otherwise 0.

–	Set the value of slice_segment_address according to the tile setup defined in the PPS with pps_pic_parameter_set_id equal to slice_pic_parameter_set_id.

# Conclusion

In this document modifications and bug-fixes are proposed for Motion-Constrained Tile Sets Extraction Information Set SEI Message.

We propose to adopt all the proposals in this document into a revision of Motion-Constrained Tile Sets Extraction Information SEI Message.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

# References

[1] JCTVC-Y1008, “Motion-Constrained Tile Sets Extraction Information SEI Messages Draft I,” October 2016, Chengdu