| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0038 |
| --- | --- |

| Title: | Bit Rate Measurement in CTC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Karsten Suehring Tung Nguyen | Email: | karsten.suehring@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This input document describes and reports bit rate results when accounting the start codes bytes into the measurement. A notable difference in terms of BD-rate has been observed for the lower operation points (e.g., for the class E sequences) with 0.4% for the RA configuration and 0.6% for the LDB/LDP configurations, respectively. A discussion is suggested on unifying the reported bit rates and setting up rules accordingly in common test conditions.

# Introduction

There is a mismatch between the summary bit rate (1) results that are reported by the HM encoder and the final bitstream size. A reason for using the reported summary bit rate values of the HM encoder is, that SEI messages are not included in the bit rate calculation, as it was previously decided.

That rule was especially important when enabling the Checksum SEI, which is quite large, but useful for debugging.

But with all SEI messages disabled, there is still a difference: The summary value (1) does not count the start codes of the Annex B byte stream format.

# Problems

For the lower operation points, such as sequences of class E, these differences are up to 0.4% in RA and 0.6% in LDB/LDP HM configurations (see attached Excel sheet for full 4:2:0 CTC results for HM 16.14). Thus, for a fair comparison, candidate and reference should use the same type of measurement.

Arguments for using the “Summary” value (1)

- Annex B byte stream is not required

- SEI messages are not counted, even if enabled

Arguments for using the “Bytes written to file” value (2)

- Can be verified with the file size on disk

- File size is also available for other encoders, e.g. in a CfE or CfP

- Annex B is the most lightweight file format that we came up with

Generally, it seems desirable to be able to verify the bit rate with a tool other than the encoder itself. For the known bitstream formats, it is possible to write such a tool. But it is not possible to create such a tool for unknown bitstream formats.

# Possible Solutions

A unified bit rate reporting can be achieved by using either of the reported bit rates.

If we decide, that file format bytes should not be reported, for bitstream size verification in the context of a CfE or CfP we would need either:

- A bitstream parser, which can distinguish between coded bytes and file format overhead (in source code).

or:

- A requirement to use Annex B byte format.

If we decide to include file format bytes, the HM encoder can easily be patched to report the start code bytes in (1).

Note regarding JEM: The start code bytes are also not reported per frame. The example scripts for segment-wise encoding parse these per frame lines to add up the proper frames of all segments. Thus, these lines would also need to be modified.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. does not have any current or pending patent rights relating to the technology described in this contribution.