| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0039 |
| --- | --- |

| Title: | Clarification of HEVC specification text for deblocking filter and chroma position indication for ITU-R BT.2020 and ITU-R BT.2100 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposed draft text clarification |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Microsoft Corp. 1 Microsoft Way Redmond WA 98052 USA | Tel: Email: | +1 (425) 703-5308 garysull@microsoft.com |
| Source: | Microsoft |  |  |

_____________________________

# Abstract

This contribution proposes draft text for clarification of two aspects of the HEVC text specification: 1) the general introductory description of the deblocking filter process, and 2) the chroma position indications for ITU-R BT.2020 and ITU-R BT.2020. The latter aspect is also proposed for AVC. These issues are suggested not to be an especially high priority matter, but the proposed changes are proposed to be included in some upcoming revision when feasible.

# Deblocking filter text clarification

In 8.7.2.1, there is a paragraph that says this:

The deblocking filter process is applied to all prediction block edges and transform block edges of a picture, except the edges that are at the boundary of the picture, for which the deblocking filter process is disabled by slice_deblocking_filter_disabled_flag, that coincide with tile boundaries when loop_filter_across_tiles_enabled_flag is equal to 0, or that coincide with upper or left slice boundaries of slices with slice_loop_filter_across_slices_enabled_flag equal to 0. For the transform units and prediction units with block edges less than 8 samples in either the vertical or horizontal direction, only the edges lying on the 8x8 sample grid of the considered component are filtered.

We suggest that this paragraph is (very) hard to read and understand, although the issue does not seem so critical because the paragraph is redundant with detailed specifications in the subsequent subclauses 8.7.2.2 through 8.7.2.4. We suggest replacing this paragraph with the following:

The deblocking filter process is applied to all prediction block edges and transform block edges of a picture, except the following types of edges:

- edges that are at the boundary of the picture

- edges that coincide with tile boundaries when loop_filter_across_tiles_enabled_flag is equal to 0

- edges that coincide with upper or left boundaries of slices with slice_loop_filter_across_slices_enabled_flag equal to 0 or slice_deblocking_filter_disabled_flag equal to 1

- edges within slices with slice_deblocking_filter_disabled_flag equal to 1

- edges that do not correspond with 8x8 sample grid boundaries of the considered component

# Chroma position indication text clarification

Recent efforts have been taken to include support for ITU-R BT.2020 and ITU-R BT.2100 in the HEVC specification. Historically, such specifications for raw video formats have not included the specification of 4:2:0 sampling. However, these two new ITU-R standards have departed from this past practice by including such a specification. Moreover, they have specified a 4:2:0 sampling convention that does not correspond with the default behaviour expressed in HEVC when chroma_loc_info_present_flag is equal to 0. To encourage alignment between the data in the encoded bitstream and the intended interpretation of the decoded video content, we suggest to add a statement such as the following in E.3.1 of the HEVC specification (at the end of the semantics specification for chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field).

When chroma_format_idc is equal to 1 and the decoded video content is intended for interpretation according to Rec. ITU-R BT.2020 or Rec. ITU-R BT.2100, chroma_loc_info_present_flag should be equal to 1 and chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field should be equal to 2.