| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0044 |
| --- | --- |

| Title: | SEI messages for omnidirectional video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Justin Ridge | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes specification of the following two SEI messages for both AVC and HEVC

- Omnidirectional projection SEI message. This should refer to the omnidirectional projection format enumerations in CICP.

- Region-wise packing SEI message. This SEI message should have the same capability as the region-wise packing indications in the ISO base media file format.

It is claimed that this would greatly benefit the file encapsulation process in common workflows.

The contribution further notes that region-wise packing is the latest example of a spatial arrangement, and suggests that an extensible approach be followed that can accommodate new types of spatial arrangement in the future.  To this end, the contribution proposes a spatial arrangement nesting SEI message, which is suggested to be used when more than one of omnidirectional projection, frame packing arrangement, and region-wise packing applies to the content.

# Background

## Omnidirectional media processing flow

Figure 1 shows a typical content flow process for an omnidirectional media application.

Figure 1: Content flow process for omnidirectional media [source: OMAF working draft, MPEG N16439]

The “image stitching, projection, and mapping” process for monoscopic content is broken down in Figure 2 and described as follows. Input images (Bi) are stitched and projected onto a three-dimensional projection structure, such as a sphere or a cube. The image data on the projection structure is further arranged onto a two-dimensional projected frame (C), the format of which is indicated with the projection format indicator defined in CICP or OMAF. Optional region-wise packing is then applied to map regions of the projected frame onto one or more packed frames. If the region-wise packing is not applied, the packed frame is identical to the projected frame. Otherwise, regions of the projected frame are mapped onto a packed frame (D) by indicating the location, shape, and size of each region in the packed frame. In practice, the input images may be converted to a packed frame in one process without intermediate steps.

Figure 2: Image stitching, projection, and region-wise packing for monoscopic content

For stereoscopic content (Figure 3), two projections are made onto separate three-dimensional projection structures, one for each eye. The image data on each projection structure is further arranged onto a two-dimensional projected frame (CL for left eye, CR for right eye). Frame packing is applied to pack the left view frame and right view frame onto the same projected frame. Optional region-wise packing is then applied to pack projected frame onto one or more packed frames. If the region-wise packing is not applied, the packed frame is identical to the projected frame.

Figure 3: Image stitching, projection, and region-wise packing for stereoscopic content

A file encapsulator takes the encoded media content including video, image, and audio as input and encapsulates them into a container file. Metadata such as projection and mapping information to assist in rendering the decoded packed frames is included in the file. Inputs to the file encapsulator hence include the coded bitstreams and may also include the projection, stereoscopic frame packing, and region-wise packing information applied prior to encoding.

## Example of projection and region-wise packing

Figure 4 illustrates an equirectangular panorama projected (ERP) frame that is packed region-wise before encoding. The polar areas of the ERP frame are horizontally stretched. Hence horizontal downsampling top and bottom stripes of an ERP frame may be advantageous for rate-distortion performance. Such downsampling is utilized in the example given in Figure 4.

Figure 4. Example of defining regions and region-wise packing to form a packed frame.

# Problem statements

## Lack of standardized interface for encapsulation

The file encapsulation process lacks a single standardized interface.  This can result in the following issues:

- The file encapsulation process may not be provided with the metadata in a usable form.  For example, the file encapsulation process may run as a completely separate entity from the encoding process; the configuration file used for encoding may not be available to the file encapsulator (or may not be suitable).  The only input to the encapsulator may be the video bitstream itself.

- Metadata describing the projection and region-wise packing arrangement can be lost when the encapsulation is converted.  For example, when a MPEG-2 transport stream over DVB broadcast is converted to an ISO base media file format for transmission over MPEG-DASH.

One approach to solving this problem is to define suitable SEI messages, as these can be included in the track header in the ISO base media file format (see MPEG N16424 "Text of ISO/IEC 14496-15:2014 PDAM 1 Handling of Unspecified NAL Unit Types and other improvements"). Consequently, SEI messages encapsulated in the track header are easily accessible by file parsers and are received by DASH clients within initialization segments.

## Extensibility and optionality

In general, the omnidirectional projection format, frame packing arrangement, and region-wise packing all describe how the source content has been spatially arranged prior to encoding, and conversely how decoded pictures must be processed prior to display. In our opinion, it is likely that new types of spatial arrangement will be developed in the future, and therefore it is desirable to have an extensible mechanism that accommodates such new spatial arrangement operations. The mechanism should be such that accidental displaying of the decoded content without appropriate processing is prevented. In other words, if a decoder or a player does not recognize or process a particular spatial arrangement type, it should not display the decoded content.

It is noted that different combinations of omnidirectional projection format, frame packing arrangement, and region-wise packing are possible, for example:

- When all three are present, the decoded frames have region-wise packed, stereoscopic, and of a particular omnidirectional projection format.

- When omnidirectional projection format and frame packing arrangement are present but region-wise packing is absent, the decoded frames are stereoscopic and of a particular projection format, e.g. top-bottom frame-packed stereoscopic equirectangular panoramas.

- When omnidirectional projection format and region-wise packing are present and frame packing arrangement is absent, the decoded frames are monoscopic and have been region-wise packed, and represent a certain omnidirectional projection format.

- When frame packing arrangement and region-wise packing are present but omnidirectional projection format is absent, the decoded frames are rectilinear, stereoscopic, and have been region-wise packed.

# Proposals

## General

It is proposed that JCT-VC establishes work to specify the following two SEI messages for both AVC and HEVC

- Omnidirectional projection SEI message. This should refer to the omnidirectional projection format enumerations in CICP.

- Region-wise packing SEI message. This SEI message should have the same capability as the region-wise packing indications in the ISO base media file format.

It is expected that major parts of the SEI messages are identical in AVC and in HEVC. It is therefore proposed that JCT-VC also prepares, at least informally, the design for the AVC version of the SEI messages. The approval process of the AVC version of the SEI message naturally takes place in both parent bodies.

## Spatial arrangement nesting SEI message

An extensible spatial arrangement nesting SEI message is proposed.  This should be used when more than one of omnidirectional projection, frame packing arrangement, and region-wise packing applies to the content. The nesting SEI message may contain other types of spatial arrangement SEI messages in the future.

### Syntax

| spatial_arrangement_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| num_seis_in_san_minus1 | u(8) |
| for( i = 0; i <= num_seis_in_san_minus1; i++ ) |  |
| sei_message( ) |  |
| } |  |

### Semantics

The spatial arrangement nesting SEI message provides a mechanism to indicate which spatial arrangements have been applied to pictures prior to encoding. The order of the contained SEI messages indicates the applied processing order of the spatial arrangement operations.

num_seis_in_san_minus1 plus 1 specifies the number of sei_message( ) syntax structures contained in the spatial_arrangement_nesting( ) syntax structure. The value of num_seis_in_san_minus1 shall be in the range of 0 to 254, inclusive. Value 255 is reserved for future use by ITU-T | ISO/IEC.

In bitstreams conforming to this version of this Specification, each sei_message( ) shall be among the following:

- omnidirectional_projection( ) [Ed. (MH): the syntax structure name not yet determined]

- frame_packing_arrangement( )

- segmented_rect_frame_packing_arrangement( )

- region_wise_packing( ) [Ed. (MH): the syntax structure name not yet determined]

However, decoders conforming to this version of this Specification shall allow other SEI messages to be present. When SEI messages within a spatial arrangement nesting SEI message cannot be decoded or processed in the displaying process, the decoded content should not be displayed.

When an omnidirectional projection SEI message is present within a spatial arrangement nesting SEI message, it shall be the first nested SEI message.

At most one frame packing arrangement or segmented rectangular frame packing SEI message shall be present in a spatial arrangement SEI message. The absence of both the frame packing arrangement and segmented rectangular frame packing SEI message indicates that the encoded content is monoscopic.

When a frame packing arrangement or segmented rectangular frame packing SEI message is present within a spatial arrangement SEI message, it shall precede a region-wise packing SEI message, if any, in the same spatial arrangement SEI message.

# Patent rights declaration(s)

Nokia Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).