| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0047 |
| --- | --- |

| Title: | AHG7: Consideration on the inverse transfer function |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyun Mook Oh Jong-Yeul Suh | Tel: Email: | hyunmook.oh@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

In this contribution, selection of suitable inverse transfer function for a decoded signal when using content characteristics described by the content colour volume SEI message is proposed. Among two different inverse functions written in JCTVC-Y1005, some issues with using EOTF, instead of inverse of OETF, are addressed: mismatch between the reproduced signal and the information that describes the original signal, and also change of signals dependent on OETF type. To avoid the problems stated, inverse of the coding transfer function is proposed to be used to recover a signal when using content colour volume SEI message.

# Backgrounds

In the 25th JCT-VC meeting, content colour volume SEI message was discussed and a draft text for the SEI message was developed based on the simple but essential information for colour volume description [1]. One of the discussion issues was the inverse function used to reproduce a signal whose characteristics the SEI message describes. In JCTVC-Y1005, it is tentatively recommended to use corresponding EOTF (electro-optical transfer function) for BT.709 and HLG (hybrid log-gamma) [2] transfer functions while the other transfer functions are recommended to use the exact inverse of the transfer function. However, the EOTF does not produce the signal which content colour volume is describing, while the inverse OETF (opto-electronic transfer function) restores the signal. To help the selection of suitable inverse function to match the restored signal with the given colour volume information, the problems of using EOTF with regard to the content colour volume SEI message will be studied in this contribution.

# EOTF vs. inverse OETF for content colour volume SEI message

The purpose of the content colour volume SEI message is to provide information about the colour volume characteristics of the associated picture [1]. In the encoder point of view depicted in Figure 1, the SEI message describes information about a signal after content generation, rendering, mixing, or mastering processes. As the encoder input signal is produced by transforming into colour encoded digital values, the inverse of the colour encoding process should be performed from the decoder output. The issue is that which inverse function should be used to restore the signal that content colour volume SEI message is describing.

In JCTVC-Y1005, two different inverse functions are recommended according to the OETF types used for colour encoding: EOTF is recommended for BT.709 and HLG transfer functions and inverse of OETF is recommended for other transfer functions [1]. It is obvious that the use of the inverse function could

Figure 1. Encoding and decoding process with content colour volume SEI message.

restore the original source, i.e. the input signal in Figure 1. When EOTF is used for inverse transfer function instead of inverse OETF, the resultant signal will be a modified signal affected by OOTF (opto-optical transfer function) which is defined by the consecutive transfer functions as follows [2].

OOTF=OETF⊗EOTF,EOTF=OETF−1⊗OOTF,OETF=OOTF⊗EOTF−1,

where ⊗ and −1 represents the concatenation of functions and the inverse function, respectively.

In ITU-R Rec. BT.2100, the role of the OOTF is defined as a function applying the “rendering intent”. OETF defined in ITU-R Rec. BT.709 and EOTF defined in ITU-R Rec. BT.1886 are a good example set that produces this rendering intent. When a system applies BT.709 EOTF and then applies corresponding BT.1886 OETF, the entire system gamma modifies the signal to be slightly dimmer lightness with deeper colours. Another example is HLG system where OOTF is designed to reflect the changes of viewing environments, such as display peak luminance and ambient light condition.

The problem of introducing this OOTF to content colour volume SEI message interpretation is that the reproduced signal is not same with the signal described by the information in the SEI message. This causes mismatch between colour boundaries described by the SEI message and the actual colour distribution of the reproduced signal. In addition, a consistent understanding of given information is not guaranteed for HLG based devices. As mentioned about HLG OOTF, it is designed to be changed dependent on the display peak luminance and the ambient light condition, and therefore the amount of modification on a signal will be different from device to device. Also, the interpretation of given information could change depending on the choice of OETF. As an example, consider an HDR source which contains content colour volume SEI message and then encoded with PQ and HLG system, respectively. When using inverse OETF for PQ and EOTF for HLG systems as recommended in JCTVC-Y1005, the output signal will be different while they are refer to the same information provided by the SEI message.

In ITU-R Rec. BT.2087, the choice of transfer function is addressed with regards to the colour conversion from BT.709 to BT.2020 [3]. In applications such as pre-produced content conversion, the goal is to preserve colours seen on a BT.2020 display as similar as that seen on a BT.709 display. In order to preserve the rendering intent on BT.709 display, EOTF is recommended to be used for this application. In another case of matching BT.709 and BT.2020 camera outputs, the goal is to preserve colour regardless of signal representation. Therefore, it is recommended to use the inverse OETF in colour conversion which produces different representation without altering the original colour. Since the content colour volume SEI message describes not the displayed colour but the colour characteristics of a signal, the second case addressed in ITU-R Rec. BT.2087 is suitable for content colour volume SEI message usage.

# Proposal

Based on the issues with using different inverse functions and further study of ITU-R Rec. BT.2087, the alternative text is proposed for the introductory part of the semantics in the draft text specification document, JCTVC-Y1005, to recommend the exact inverse of encoding transfer function when restore the signal related to the SEI message. (The proposed alternative texts are yellow highlighted.)

The content colour volume SEI message provides information about the colour volume characteristics of the associated picture. This SEI message describes the colour volume characteristics of the associated picture in terms of a nominal range and deviations from this range may occur. This SEI message requires knowledge of the colour description information of the signal, such as the colour_primaries, transfer_characteristics, and matrix_coeffs in the VUI parameters syntax. This SEI message shall not be present when any of these parameters is unspecified or reserved. When alternative transfer characteristics SEI message is present, the transfer_characteristics in the SEI message is precedent to the corresponding parameter in VUI. In all values of transfer_characteristics, the exact inverse of the transfer function specified in Table E.4 (Transfer Characteristics) shall be used to convert the non-linear signal to a linear representation.

# Conclusion

In this contribution, two approaches for the inverse transfer functions were studied to determine a suitable inverse transfer function to restore the signal described by the content colour volume SEI message. In comparison with the inverse OETF, the problems of using EOTF for BT.709 and HLG transfer functions were addressed with introduction of OOTF which modifies the output signal. Also, the considerations on the inverse transfer function in colour conversion applications were reviewed which used the inverse OETF to preserve the characteristic of a signal. Based on those studies, the alternative text for the draft specification document was proposed, which recommends the exact inverse of encoding transfer function for all cases.

# References

- A.M. Tourapis, E. Francois, H.M. Oh, A.K. Ramasubramonian, P. Yin, “Content Colour Volume SEI Message Draft 1” JCTVC document JCTVC-Y1005, Chengdu, CN, Oct. 2016

- Recommendation ITU-R BT.2100, “Image parameter values for high dynamic range television for use in production and international programme exchange” (2016)

- Recommendation ITU-R BT.2087, “Colour conversion from Recommendation ITU-R BT.709 to Recommendation ITU-R BT.2020” (2015)

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).