| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0049 |
| --- | --- |

| Title: | AHG3: On the coding efficiency and run-time of search range and adaptive search range |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Karl Sharman Sony Europe Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@sony.com |
| Source: | Sony Europe Ltd. |  |  |

_____________________________

# Abstract

This contribution provides results of simulations using the JVET test conditions in combination with HM16.14, varying the random-access search range and the enabling of the (previously disabled) adaptive search range and the minimum search window parameter associated with the adaptive search range.

# Introduction

During the review of JCTVC-Z0003 AHG3 report on “HEVC HM and HDRTools software development and software technical evaluation”, questions were raised regarding the recently increased search range and the possibility of using an adaptive search range function.

Prior to HM16.14, the inter search range was 64; for HM16.14, the search range was increased to 256 for random-access configurations (it remained at 64 for low-delay configurations). This increased the run time with a small change in BD-rate:

HM16.14 vs HM16.13 results (from JCTVC-Z0003).

|  | Random Access Main 10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.33% | -1.14% | -1.97% |
| Class B | 0.01% | -0.01% | -0.14% |
| Class C | 0.13% | 0.00% | -0.05% |
| Class D | 0.35% | 0.27% | 0.30% |
| Class E |  |  |  |
| Overall | 0.04% | -0.21% | -0.45% |
|  | 0.04% | -0.20% | -0.43% |
| Class F | -0.37% | -0.35% | -0.43% |
| Enc Time[%] | 115% |  |  |
| Dec Time[%] | 100% |  |  |

The exact BD-rate change of this single modification is unknown since in the transition from HM16.13 to HM16.14, parameter sets were output with every IRAP, leading to some loss.

The key questions raised by the JCT-VC were:

- Does Adaptive Search Range work (and what does it do)?

- What are the effects on the coding performance of HM in combination with Adaptive Search Range for random-access simulations using the JVET test sequences?

- What are the effects on encoder run-time of HM in combination with Adaptive Search Range for random-access simulations using the JVET test sequences?

This contribution provides results of simulations that vary the search range and the enabling of the (previously disabled) adaptive search range and the minimum search window parameter associated with the adaptive search range.

In these simulations:

- HM16.14 was used;

- the JVET test sequences were used, as specified in the JVET test conditions;

- the HM 10-bit random-access 4:2:0 common test conditions were used (which is exactly the same as the random-access HM configuration as provided with the JVET software package);

- the random-access encoding was split into intra-period length GOPs (the ‘parallel’ method used in JVET);

- the search range is controlled by the configuration parameter “--SearchRange=<range>”;

- the adaptive search range is enabled using “--ASR=1”;

- the minimum search range is specified using “--MinSearchWindow=<value>”.

Note that the screen-content coding software model (SCM) AHG08 report (JCTVC-Z0008) indicated a more significant increase in encoding run time (up to 757%) due to the change in search range than reported in the HM software model report (JCTVC-Z0003). Therefore the adaptive search range probably should also be evaluated with the SCM model.

Also note that there are additional encoder search options. For example, with bi-prediction, HM only uses one iteration during the search for motion vectors. This is due to the setting of “--FEN” (fast encoder) to 1 in the default configuration. There are also options for “--FastSearch” that could be evaluated. Please refer to “High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description”, JCTVC-Y1002 (and JCTVC-U0040).

# The HM adaptive search range function

HM includes the ability to adaptively adjust the search range using the configuration parameters --ASR and --MinSearchWindow.

When ASR is enabled, the function TEncSlice::setSearchRange is called for each non-I-slice during encoding. This iterates over the reference frames, calculating the distance in POC between the current POC (POCcurr) and the respective reference frame (POCref) as follows:

w=maxwmin,minwmax,roundwmax×k×POCcurr−POCrefGOPsize

where:

wmin is the minimum search window range, as specified using --MinSearchWindow

wmax is the maximum search window range, as specified using --SearchRange

k is a constant ‘ADAPT_SR_SCALE’, currently defined as 1 in CommonDef.h

GOPsize is the number of pictures in the GOP.

This sets up the array TEncSearch::m_aaiAdaptSR. This array is always used during TEncSearch::xMotionEstimation to initialize TEncSearch::m_iSearchRange. For bi-prediction frames, there are two TComMVs that are configured with the maximum search range value as specified by SearchRange; the HM is best examined to understand the exact purpose of these.

To summarize, the adaptive search range linearly scales the search range between the current picture and reference pictures according to the POC distance. There is no current method in HM to link the search window range to the picture size.

# Results

## Effects of changing the search range between 256 and 64

The table below shows the effect of reversing the changes made in HM16.14, i.e. in using a search range of 64 instead of a search range of 256. For completeness, it also illustrates the inverse of these results, i.e. what happened when HM16.14 test conditions were changed from a search range of 64 to a search range of 256.

The reason for providing both sets of results is that if there are any changes in test conditions, it is necessary to note what is being lost in terms of coding efficiency against the best system (which is commonly that with a search range of 256); however, in terms of encoding run time, seeing the increase relative to the original search range of 64 is potentially useful.

It is interesting to note the degradation of classes C and D with the larger search range: having a search range that, for blocks in the centre of the image, would cover the entire reference frame is potentially leading to poor block choices.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Reducing HM16.14 Search Range from 256 (anchor) to 64 (tested) |  |  |  |  | Increasing HM-16.14 Search Range from 64 (anchor) to 256 (tested) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.67% | 0.55% | 73% | 100% | -0.15% | -0.66% | -0.54% | 137% | 100% |
| Class A2 | 2.42% | 2.86% | 2.90% | 83% | 100% | -2.31% | -2.74% | -2.77% | 120% | 100% |
| Class B | 0.03% | 0.14% | 0.16% | 86% | 100% | -0.03% | -0.14% | -0.16% | 117% | 100% |
| Class C | -0.05% | -0.01% | 0.15% | 85% | 100% | 0.05% | 0.01% | -0.15% | 117% | 100% |
| Class D | -0.08% | -0.01% | -0.14% | 92% | 100% | 0.08% | 0.01% | 0.14% | 108% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 0.47% | 0.70% | 0.70% | 84% | 100% | -0.45% | -0.68% | -0.67% | 119% | 100% |
| Class F (optional) | 0.50% | 0.47% | 0.58% | 86% | 100% | -0.50% | -0.47% | -0.57% | 116% | 100% |
| Sequences with significant (magnitude ≥ 0.2%) BD-rate changes | A1: Tango +0.47% A2: TrafficFlow +3.09% A2: Rollercoaster +5.71% A2: DaylightRoad +0.95% D: RaceHorses -0.22% F: SlideEditing +1.25% F: SlideShow +0.74% |  |  |  |  | A1: Tango -0.47% A2: TrafficFlow -3.00% A2: Rollercoaster -5.40% A2: DaylightRoad -0.95% D: RaceHorses +0.22% F: SlideEditing -1.24% F: SlideShow -0.73% |  |  |  |  |

## Effects of using ASR with search range of 256 and min window of 8

The table below shows the effect of using adaptive search range. Anchor 1 is HM16.14 with a search range of 256; anchor 2 is HM16.14 with a search range of 64. The tested case is HM16.14 with a search range of 256, adaptive search range enabled with a minimum search window of 8.

Note that because the GOP size is 16, the minimum range that will be used will be 256/16 = 16. Hence the minimum window is not actually clipping the range.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HM-16.14 search range 256, ASR enabled, min search window 8 (tested) |  |  |  |  |  |  |  |  |  |
|  | Over HM-16.14 search range 256 (anchor) |  |  |  |  | Over HM-16.14 search range 64 (anchor) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | 0.36% | 0.50% | 78% | 100% | 0.02% | -0.30% | -0.04% | 106% | 100% |
| Class A2 | 0.64% | 0.70% | 0.73% | 88% | 100% | -1.71% | -2.07% | -2.07% | 105% | 100% |
| Class B | 0.01% | -0.01% | 0.00% | 89% | 100% | -0.02% | -0.15% | -0.16% | 104% | 100% |
| Class C | -0.09% | -0.11% | -0.17% | 89% | 100% | -0.04% | -0.10% | -0.32% | 104% | 100% |
| Class D | -0.08% | -0.08% | -0.02% | 95% | 100% | 0.00% | -0.07% | 0.12% | 102% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 0.12% | 0.16% | 0.20% | 87% | 100% | -0.33% | -0.52% | -0.48% | 104% | 100% |
| Class F (optional) | 0.79% | 0.76% | 0.83% | 88% | 100% | 0.28% | 0.29% | 0.25% | 103% | 100% |
| Sequences with significant (magnitude ≥ 0.2%) BD-rate changes | A1: Tango +0.41% A1: Campfire +0.22% A2: TrafficFlow +0.40% A2: DaylightRoad +0.65% A2: RollerCoaster +1.63% F: ChinaSpeed +0.24% F: SlideEditing +1.21% F: SlideShow +1.75% |  |  |  |  | A2: TrafficFlow -2.62% A2: DaylightRoad -0.30% A2: RollerCoaster -3.86% B: Cactus +0.21% F: ChinaSpeed +0.25% F: SlideShow +1.01% |  |  |  |  |

## Effects of using ASR with search range of 256 and min window of 16

The results of this experiment are included in this contribution. However, since the GOP size is 16, the lowest the range will go is 256/16 = 16. Hence the results are identical to that of the above configuration with a minimum search window of 8.

## Effects of using ASR with search range of 256 and min window of 32

The table below shows the effect of using adaptive search range. Anchor 1 is HM16.14 with a search range of 256; anchor 2 is HM16.14 with a search range of 64. The tested case is HM16.14 with a search range of 256, adaptive search range enabled with a minimum search window of 32.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HM-16.14 search range 256, ASR enabled, min search window 32 (tested) |  |  |  |  |  |  |  |  |  |
|  | Over HM-16.14 search range 256 (anchor) |  |  |  |  | Over HM-16.14 search range 64 (anchor) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.22% | 0.33% | 78% | 100% | -0.04% | -0.45% | -0.21% | 107% | 100% |
| Class A2 | 0.55% | 0.62% | 0.64% | 88% | 100% | -1.79% | -2.15% | -2.17% | 105% | 100% |
| Class B | -0.02% | -0.02% | -0.04% | 89% | 100% | -0.05% | -0.16% | -0.20% | 104% | 100% |
| Class C | -0.10% | -0.13% | -0.17% | 89% | 100% | -0.06% | -0.12% | -0.32% | 104% | 100% |
| Class D | -0.08% | -0.08% | -0.04% | 95% | 100% | 0.00% | -0.08% | 0.10% | 102% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 0.09% | 0.12% | 0.13% | 88% | 100% | -0.37% | -0.57% | -0.54% | 104% | 100% |
| Class F (optional) | 0.57% | 0.56% | 0.61% | 88% | 100% | 0.07% | 0.08% | 0.03% | 103% | 100% |
| Sequences with significant (magnitude ≥ 0.2%) BD-rate changes | A1: Tango +0.34% A2: TrafficFlow +0.37% A2: DaylightRoad +0.54% A2: RollerCoaster +1.40% F: SlideEditing +0.94% F: SlideShow +1.34% |  |  |  |  | A2: TrafficFlow -2.65% A2: DaylightRoad -0.40% A2: RollerCoaster -4.08% F: SlideEditing -0.31% F: SlideShow +0.60% |  |  |  |  |

## Effects of using ASR with search range of 256 and min window of 64

The table below shows the effect of using adaptive search range. Anchor 1 is HM16.14 with a search range of 256; anchor 2 is HM16.14 with a search range of 64. The tested case is HM16.14 with a search range of 256, adaptive search range enabled with a minimum search window of 64.

Class D is again slightly degraded relative to the search range of 64.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HM-16.14 search range 256, ASR enabled, min search window 64 (tested) |  |  |  |  |  |  |  |  |  |
|  | Over HM-16.14 search range 256 (anchor) |  |  |  |  | Over HM-16.14 search range 64 (anchor) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.08% | 0.13% | 78% | 100% | -0.10% | -0.59% | -0.41% | 107% | 100% |
| Class A2 | 0.38% | 0.42% | 0.43% | 88% | 100% | -1.95% | -2.34% | -2.36% | 105% | 100% |
| Class B | -0.05% | -0.07% | -0.06% | 89% | 100% | -0.08% | -0.21% | -0.22% | 104% | 100% |
| Class C | -0.09% | -0.11% | -0.17% | 89% | 100% | -0.04% | -0.10% | -0.32% | 104% | 100% |
| Class D | -0.05% | -0.05% | -0.02% | 95% | 100% | 0.03% | -0.04% | 0.11% | 103% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 0.04% | 0.05% | 0.06% | 88% | 100% | -0.41% | -0.64% | -0.62% | 105% | 100% |
| Class F (optional) | 0.28% | 0.22% | 0.26% | 89% | 100% | -0.21% | -0.25% | -0.31% | 103% | 100% |
| Sequences with significant (magnitude ≥ 0.2%) BD-rate changes | A1: Tango +0.20% A2: TrafficFlow +0.28% A2: DaylightRoad +0.34% A2: RollerCoaster +1.00% F: SlideEditing +0.67% F: SlideShow +0.59% |  |  |  |  | A1: Tango -0.27% A2: TrafficFlow -2.73% A2: DaylightRoad -0.60% A2: RollerCoaster -4.46% F: SlideEditing -0.57% |  |  |  |  |

# Conclusion

This contribution provides results with various combinations of search range, adaptive search range, and minimum search window using HM16.14 random-access configuration with the JVET sequences.

The adaptive search range scales the search range by the POC distance between the current picture and the reference picture; it does not link range with the picture size.

It would be potentially useful for information on the screen content coding test conditions to also be evaluated with these parameters.