| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting: Geneva, CH, 12–20 January 2017 | Document: JCTVC-Z0051_v2 |
| --- | --- |

| Title: | Suggested draft text for omni-directional projection indication SEI message with equirectangular projection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposed Draft |  |  |
| Author(s) or Contact(s): | Chad Fogg Jill Boyce Gary Sullivan Alexis Tourapis | Email: | chadfogg@gmail.com jill.boyce@intel.com garysull@microsoft.com atourapis@apple.com |
| Source: | Motion Picture Laboratories, Intel, Microsoft, Apple |  |  |

_____________________________

# Abstract

The draft SEI message text proposed in this document is proposed toward satisfying the mandates for Ad-hoc group 5 established at the end of the 25th meeting of JCT-VC in Chengdu, October 2016. The first mandate is to "develop formulae to project samples of an equirectangular format picture to 360°/omnidirectional spherical space." The second mandate is to "prepare and propose draft text for specification of a code point identification to indicate the use of the equirectangular projection mapping." The third mandate is to "prepare and propose draft text for an SEI message to carry the projection map type indicator."

The v2 version of this document has revised syntax order and modified semantics, including equations.

# Proposal

- In Annex D, add omnidirectional_projection_indication to the sei_payload() syntax table in section D.2.1:

- [ payloadType number to be assigned by the AVC and HEVC specification editors ]

- In Annex D, add the following syntax table to section D.2:

| omnidirectional_projection_indication( payloadSize ) { | Descriptor |
| --- | --- |
| omnidirectional_projection_indication_cancel_flag | u(1) |
| if( ! omnidirectional_projection_indication_cancel_flag) { |  |
| omnidirectional_projection_information_persistence_flag | u(1) |
| geometry_type | u(5) |
| sub_geometry_flag | u(1) |
| projection_type | u(8) |
| if (geometry_type = = 1 && sub_geometry_flag == 1 && projection_type = = 1) { |  |
| yaw_center | i(16) |
| pitch_center | i(16) |
| roll_center | i(16) |
| yaw_range | u(16) |
| pitch_range | u(16) |
| } |  |
| } |  |
| } |  |

- In Annex D, add the following semantic definition to section D.3:

- D.3.X Omnidirectional projection indication

- The omnidirectional projection indication SEI message provides information to enable remapping of the colour samples of the output decoded pictures onto an alternative coordinate space for use in panoramic video applications.

- When an omnidirectional projection indication SEI message is present for any picture of a CLVS of a particular layer, an omnidirectional projection indication SEI message shall be present for the first picture of the CLVS. The omnidirectional projection SEI message persists for the current layer in decoding order from the current picture until the end of the CLVS.

The luma dimensions of the cropped output picture are defined by the following equations:

croppedWidthY = pic_width_in_luma_samples − SubWidthC * ( conf_win_right_offset + conf_win_left_offset )

croppedHeightY = pic_height_in_luma_samples − SubHeightC * ( conf_win_bottom_offset + conf_win_top_offset )

croppedWidthC = croppedWidthY / SubWidthC

croppedHeightC = croppedHeightY / SubHeightC

[Ed. These equations are included here for the sake of completeness of presentation, although they may be redundant with equations already found in the HEVC text.]

Figure D.X-1 cropped output of decoded picture luma samples

omnidirectional_projection_information_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous spherical orientation SEI message in output order. omnidirectional_projection_information_cancel_flag equal to 0 indicates that spherical orientation information follows.

omnidirectional_projection_information_persistence_flag specifies the persistence of the spherical orientation SEI message for the current layer.

omnidirectional_projection_information_persistence_flag equal to 0 specifies that the spherical orientation SEI message applies to the current decoded picture only.

Let picA be the current picture. omnidirectional_projection_information_persistence_flag equal to 1 specifies that the spherical orientation SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a spherical orientation SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

- geometry type indicates the mathematical convention where points within a space can be uniquely identified by a location in one or more dimensions. When geometry_type is equal to 1, the projection indicator is given in spherical coordinates, where  is the azimuth (longitude) and  is the elevation (latitude) as depicted in Figure D.X-2.

| (a) region with yaw and pitch offsets | (b) region with yaw, pitch, and roll offsets |
| --- | --- |

- Figure D.X-2 Spherical surface coordinates ,  with yaw, pitch, and roll of the projection image relative to equator and 0 meridian. Example yaw and pitch range occupy ¼ of the sphere in each dimension. [Ed: plot equator and 0 meridian on the sphere above, though implicit from the arrow and the XYZ axis in the this draft figure.  Add neighboring figure to the right of the above that shows a yaw_center, pitch_center, and roll_center that are non-zero]

sub_geometry_flag indicates the presence of sub geometry elements that further describe or limit the geometry of this SEI message.

projection_type indicates the particular mapping of the rectangular decoder picture output samples onto the coordinate system specified by geometry_type.

yaw_center specifies the value of yaw that corresponds to the center in the projection image, in increments of 0.01 degrees. The values of yaw_center shall be in the range of -18000 and 17999. When not present, the value of yaw_center is inferred to be equal to 0.

pitch_center specifies the value of pitch that corresponds to the center in the projection image, in increments of 0.01 degrees. The values of pitch_center shall be in the range of -9000 and 9000. When not present, the value of pitch_center is inferred to be equal to 0.

roll_center specifies the value of roll that corresponds to the center in the projection image, in increments of 0.01 degrees. The values of roll_center shall be in the range of -18000 and 18000. When not present, the value of roll_center is inferred to be equal to 0.

yaw_range specifies the range of values of yaw that are present in the projection image, in increments of 0.01 degrees. The values of yaw_range shall be in the range of 1 and 36000. When not present, the value of yaw_range is inferred to be equal to 36000.

pitch_range specifies the range of values of pitch that are present in the projection image, in increments of 0.01 degrees. The values of pitch_range shall be in the range of 1 and 18000. When not present, the value of pitch range is inferred to be equal to 18000.

When projection_type is equal to 1, sub_geometry_flag is equal to 1, and geometry_type is equal to 1, cropped output picture samples at locations (i, j) are indicated as corresponding to angular coordinates. For i equal to SubWidthC * conf_win_left_offset to SubWidthC * conf_win_left_offset + croppedWidthC − 1 and j equal to SubHeightC * conf_win_top_offset to SubHeightC * conf_win_top_offset − 1, the corresponding angular coordinates (, ) for the luma sample locations, in radians, are given by the following equirectangular mapping equations:

min = (yaw_center – yaw_range ÷ 2) * 0.01 * π ÷ 180				[Eq. X1]

max = (yaw_center + yaw_range ÷ 2) * 0.01 * π ÷ 180				[Eq. X2]

min = (pitch_center – pitch_range ÷ 2) * 0.01 * π ÷ 180				[Eq. X3]

max = (pitch_center + pitch_range ÷ 2) * 0.01 * π ÷ 180				[Eq. X4]

 = ( i′ + 0.5 ) * (max – min) ÷ croppedWidthY  + min				[Eq. X5]

 = ( j′ + 0.5 ) * (min – max) ÷ croppedHeightY - min				[Eq. X6]

 = cos()* + sin()* 							[Eq. X7]

 = cos()* – sin()* 							[Eq. X8]

where

i′ = i − SubWidthC * conf_win_left_offset					[Eq. X9]

j′ = j − SubHeightC * conf_win_top_offset					[Eq. X10]

 = roll_center * 0.01 *  π ÷ 180						[Eq. X11]

- When chroma_format_idc is not equal to 0 (monochrome), for i equal to conf_win_left_offset to conf_win_left_offset + croppedWidthC − 1 and j equal to conf_win_top_offset to conf_win_top_offset + croppedHeightC − 1, the corresponding angular coordinates (, ) for the chroma sample locations, in radians, are given by

′ = ( i′ + CenterLeftOffsetC )* (max – min)  ÷ croppedWidthC+ min		[Eq. X12]

′ = ( j′ + CenterTopOffsetC )* (min – max) ÷ croppedHeightC - min		[Eq. X13]

′ = cos()*′ + sin()*′ 							[Eq. X14]

′ = cos()*′ – sin()*′ 							[Eq. X15]

where

i′ = i − conf_win_left_offset							[Eq. X14]

j′ = j − conf_win_top_offset							[Eq. X15]

and CenterLeftOffsetC and CenterTopOffsetC are given by Table XX as a function of chroma_format_idc and ChromaLocType, which is set as follows:

- If j % 2 is equal to 0, ChromaLocType is equal to chroma_sample_loc_type_top_field

- Otherwise, ChromaLocType is equal to chroma_sample_loc_type_bottom_field

Table XX – Derivation of CenterLeftOffsetC and CenterTopOffsetC

| chroma_format_idc | ChromaLocType | CenterLeftOffsetC | CenterTopOffsetC |
| --- | --- | --- | --- |
| 1 (4:2:0) | 0 | 0.125 | 0.25 |
| 1 (4:2:0) | 1 | 0.25 | 0.25 |
| 1 (4:2:0) | 2 | 0.125 | 0.125 |
| 1 (4:2:0) | 3 | 0.25 | 0.125 |
| 1 (4:2:0) | 4 | 0.125 | 0.375 |
| 1 (4:2:0) | 5 | 0.25 | 0.375 |
| 2 (4:2:2) | – | 0.125 | 0.5 |
| 3 (4:4:4) | – | 0.5 | 0.5 |

- When an omnidirectional projection indication SEI message is present for any picture of a CLVS of a particular layer and projection_type is equal to 1 and geometry_type is equal to 1, no frame packing arrangement SEI messages and segmented rectangular frame packing arrangement SEI messages shall be present in the CLVS.

Figure D-X-3  Equirectangular mapping between cropped output picture luma samples and spherical coordinates

# Patent rights declaration(s)

MovieLabs, Intel, Microsoft and Apple do not have any current or pending patent rights relating to the technology described in this contribution (to the extent of the personal awareness of the authors).