| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0002 |
| --- | --- |

| Title: | 3DV-ATM HP high-level syntax: HRD parameters for texture sub-bitstreams |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

It is asserted that the semantics of the MVC video usability information extension and the MVC scalable nesting SEI message used to carry picture timing and buffering period SEI messages rely on the sub-bitstream extraction process specified in subclause H.8.5.3, which treats NAL units of nal_unit_type equal to 21 as non-VCL NAL units and does not perform temporal_id and view_id based extraction for them. Hence, it is asserted that no proper HRD parameters can be conveyed for sub-bitstreams consisting of texture views only and therefore the contribution proposes a mechanism to convey HRD parameters of VUI as well as picture timing and buffering period SEI messages for the sub-bitstream that consists only of texture views while the depth views are removed by the sub-bitstream extraction process of subclause I.8.5.3. It is proposed that the texture sub-bitstream HRD parameters are conveyed in a second instance of mvc_vui_parameters_extension( ) in a 3DVC sequence parameter set and picture timing and buffering period SEI messages are conveyed in a specific 3DVC texture sub-bitstream HRD nesting SEI message. If a texture sub-bitstream is extracted using the sub-bitstream extraction process, these nested HRD parameters and SEI messages replace the respective MVC ones, which, as stated above, assume the presence of NAL units of nal_unit_type 21 as non-VCL NAL units.

In addition, many related smaller changes are proposed including the following:

- Annex C (Hypothetical Reference Decoder) did not refer to Annex I or subclauses of Annex I. Proper references are to Annex I are now added.

- A clarification of the definition of term view, which proposed to be defined as: unless explicitly limited to either texture view or depth view, a texture view and a depth view of the same value of view_id.

- The use of SEI messages with and without the various nesting SEI message is clarified in subclause I.13.2.

- sei_op_view_applicability_flag[ i ] is removed from 3DV scalable nesting SEI message, because the 3DV operation points always include both texture and depth as specified in I.8.

- A clarification of the definition of term base view, and an introduction of two new related terms base texture view and base depth view.

- Proposed Specification Text Changes

The proposed specification text changes are indicated with yellow highlights relative to ISO/IEC 14496-10:2012/PDAM 2 “MVC extensions for inclusion of depth maps”, MPEG N12731.

<< CHANGE 1 >>

In 7.3.2.1.3, replace the syntax table with:

| subset_seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| seq_parameter_set_data( ) | 0 |  |
| if( profile_idc = = 83 ¦ ¦ profile_idc = = 86 ) { |  |  |
| seq_parameter_set_svc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| svc_vui_parameters_present_flag | 0 | u(1) |
| if( svc_vui_parameters_present_flag = = 1 ) |  |  |
| svc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } else if( profile_idc = = 118 ¦ ¦ profile_idc = = 128 ¦ ¦ profile_idc = = 138 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| mvc_vui_parameters_present_flag | 0 | u(1) |
| if( mvc_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } |  |  |
| if( profile_idc = = 138 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| seq_parameter_set_3dvc_extension( ) | 0 |  |
| 3dvc_vui_parameters_present_flag | 0 | u(1) |
| if( 3dvc_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) | 0 |  |
| texture_vui_parameters_present_flag | 0 | u(1) |
| if( texture_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) | 0 |  |
| } |  |  |
| additional_extension3_flag | 0 | u(1) |
| if( additional_extension3_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| additional_extension3_data_flag | 0 | u(1) |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

...

- I.7.4.2.1.3	Subset sequence parameter set RBSP semantics

The semantics specified in subclause Error: Reference source not found apply with the following additions.

3dvc_vui_parameters_present_flag equal to 0 specifies that the syntax structure mvc_vui_parameters_extension( ) corresponding to 3DVC VUI parameters extension is not present. 3dvc_vui_parameters_present_flag equal to 1 specifies that the syntax structure mvc_vui_parameters_extension( ) is present and referred to as 3DVC VUI parameters extension.

texture_vui_parameters_present_flag equal to 0 specifies that the syntax structure mvc_vui_parameters_extension( ) corresponding to 3DVC texture sub-bitstream VUI parameters extension is not present. texture_vui_parameters_present_flag equal to 1 specifies that the syntax structure mvc_vui_parameters_extension( ) is present and referred to as 3DVC texture sub-bitstream VUI parameters extension.

<< END OF CHANGE 1 >>

<< CHANGE 2 >>

In Table D.1, add the following after “display_orientation (payloadSize)”:

| else if( payloadType = = 48 ) |  |  |
| --- | --- | --- |
| 3dv_scalable_nesting( payloadSize ) |  |  |
| else if ( payloadType = = 49 ) |  |  |
| 3d_view_scalability_info( payloadSize ) |  |  |
| else if (payloadType = = 50 ) |  |  |
| depth_acquisition_info ( payloadSize ) |  |  |
| else if( payloadType = = 51 ) |  |  |
| 3dvc_texture_subbitstream_hrd_nesting( payloadSize ) |  |  |

<< END OF CHANGE 2 >>

<< CHANGE 3 >>

- Annex C

Hypothetical reference decoder

(This annex forms an integral part of this Recommendation | International Standard)

…

The syntax elements of non-VCL NAL units (or their default values for some of the syntax elements), required for the HRD, are specified in the semantic subclauses of clause Error: Reference source not found, Annexes Error: Reference source not found and Error: Reference source not found, and subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, I.7, I.13 and I.14.

Two types of HRD parameter sets (NAL HRD parameters and VCL HRD parameters) are used. The HRD parameter sets are signalled as follows:

–	When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found is applied, the HRD parameter sets are signalled through video usability information as specified in subclauses Error: Reference source not found and Error: Reference source not found, which is part of the sequence parameter set syntax structure.

–	When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied, the HRD parameter sets are signalled through the SVC video usability information extension as specified in subclauses Error: Reference source not found and Error: Reference source not found, which is part of the subset sequence parameter set syntax structure.

NOTE 1 – For coded video sequences that conform to both, one or more of the profiles specified in Annex Error: Reference source not found and one or more of the profiles specified in Annex Error: Reference source not found, the signalling of the applicable HRD parameter sets is depending on whether the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found or the decoding process specified in Annex Error: Reference source not found is applied.

–	When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied, the HRD parameter sets are signalled through the MVC video usability information extension as specified in subclauses Error: Reference source not found and Error: Reference source not found, which is part of the subset sequence parameter set syntax structure.

NOTE 2 – For coded video sequences that conform to both, one or more of the profiles specified in Annex Error: Reference source not found and one or more of the profiles specified in Annex Error: Reference source not found, the signalling of the applicable HRD parameter sets is depending on whether the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found or the decoding process specified in Annex Error: Reference source not found is applied.

–	When the coded video sequence conforms to one or more of the profiles specified in Annex I, the HRD parameter sets are signalled through the 3DVC video usability information extension, which is part of the subset sequence parameter set syntax structure.

NOTE 3 – For coded video sequences that conform to both, one or more of the profiles specified in Annex Error: Reference source not found and one or more of the profiles specified in Annex Error: Reference source not found, the signalling of the applicable HRD parameter sets is depending on whether the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found or the decoding process specified in Annex Error: Reference source not found is applied.

...

In Annexes , Error: Reference source not found, and Error: Reference source not found and subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, I.12, I.13 and I.14, the specification for "presence" of non-VCL NAL units is also satisfied when those NAL units (or just some of them) are conveyed to decoders (or to the HRD) by other means not specified by this Recommendation | International Standard. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

…

The CPB size (number of bits) is CpbSize[ SchedSelIdx ]. The DPB size (number of frame buffers) is Max( 1, max_dec_frame_buffering ). When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found or Annex I and the decoding process specified in Annex Error: Reference source not found or Annex I is applied, the DPB size is specified in units of view components.

…

For each picture in the bitstream, the variable OutputFlag for the decoded picture and, when applicable, the reference base picture is set as follows:

–	If the coded video sequence containing the picture conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found is applied, OutputFlag is set equal to 1.

–	Otherwise, if the coded video sequence containing the picture conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied, the following applies:

–	For a reference base picture, OutputFlag is set equal to 0.

–	For a decoded picture, OutputFlag is set equal to the value of the output_flag syntax element of the target layer representation.

–	Otherwise (the coded video sequence containing the picture conforms to one or more of the profiles specified in Annex Error: Reference source not found or Annex I and the decoding process specified in Annex Error: Reference source not found or Annex I is applied), the following applies:

–	For the decoded view components of the target output views, OutputFlag is set equal to 1.

–	For the decoded view components of other views, OutputFlag is set equal to 0.

The operation of the CPB is specified in subclause Error: Reference source not found. The instantaneous decoder operation is specified in clauses Error: Reference source not found-Error: Reference source not found (for coded video sequences conforming to one or more of the profiles specified in Annex Error: Reference source not found) and in Annex Error: Reference source not found (for coded video sequences conforming to one or more of the profiles specified in Annex Error: Reference source not found) and in Annex Error: Reference source not found (for coded video sequences conforming to one or more of the profiles specified in Annex Error: Reference source not found) and in Annex I (for coded video sequences conforming to one or more of the profiles specified in Annex I). The operation of the DPB is specified in subclause Error: Reference source not found. The output cropping is specified in subclause Error: Reference source not found.

NOTE 5 – Coded video sequences that conform to both, one or more of the profiles specified in Annex Error: Reference source not found and one or more of the profiles specified in Annex Error: Reference source not found, can be decoded either by the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found or by the decoding process specified in Annex Error: Reference source not found. The decoding result and the HRD operation may be depending on which of the decoding processes is applied.

NOTE 6 – Coded video sequences that conform to both, one or more of the profiles specified in Annex Error: Reference source not found and one or more of the profiles specified in Annex Error: Reference source not found, can be decoded either by the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found or by the decoding process specified in Annex Error: Reference source not found. The decoding result and the HRD operation may be depending on which of the decoding processes is applied.

NOTE 7 – Coded video sequences that conform to all three of, one or more of the profiles specified in Annex Error: Reference source not found and one or more of the profiles specified in Annex Error: Reference source not found and one or more of the profiles specified in Annex I, can be decoded either by any of the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found or the decoding process specified in Annex Error: Reference source not found or the decoding process specified in Annex I. The decoding result and the HRD operation may be depending on which of the decoding processes is applied.

HSS and HRD information concerning the number of enumerated delivery schedules and their associated bit rates and buffer sizes is specified in subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found and I.14. The HRD is initialised as specified by the buffering period SEI message as specified in subclauses Error: Reference source not found and Error: Reference source not found. The removal timing of access units from the CPB and output timing from the DPB are specified in the picture timing SEI message as specified in subclauses Error: Reference source not found and Error: Reference source not found. All timing information relating to a specific access unit shall arrive prior to the CPB removal time of the access unit.

…

When the coded video sequence conforms to one or more of the profiles specified in Annex I and the decoding process specified in Annex I is applied, the following is specified:

- When an access unit contains one or more buffering period SEI messages that are included in 3DVC scalable nesting SEI messages, the buffering period SEI message that is associated with the operation point being decoded is the buffering period SEI message that initialises the HRD. Let hrdVId[ i ] be equal to sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, and let hrdTId be the value of sei_op_temporal_id, that are associated with the 3DVC scalable nesting SEI message containing the buffering period SEI message that initialises the HRD.

- The picture timing SEI messages that specify the removal timing of access units from the CPB and output timing from the DPB are the picture timing SEI messages that are included in 3DVC scalable nesting SEI messages associated with values of sei_op_view_id[ i ] equal to hrdVId[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, and sei_temporal_id equal to hrdTId.

- The HRD parameter sets that are used for conformance checking are the HRD parameter sets, included in the 3DVC video usability information extension of the active 3DVC sequence parameter set, that are associated with values of vui_mvc_view_id[ i ][ j ] for all j in the range of 0 to vui_mvc_num_target_output_views_minus1[ i ], inclusive, equal to hrdVId[ j ], and the value of vui_mvc_temporal_id[ i ] equal to hrdTId. For the specification in this annex, num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag are substituted with the values of vui_mvc_num_units_in_tick[ i ], vui_mvc_time_scale[ i ], vui_mvc_fixed_frame_rate_flag[ i ], vui_mvc_nal_hrd_parameters_present_flag[ i ], vui_mvc_vcl_hrd_parameters_present_flag[ i ], vui_mvc_low_delay_hrd_flag[ i ], and vui_mvc_pic_struct_present_flag[ i ], respectively, with i being the value for which vui_mvc_view_id[ i ] is equal to hrdVId[ j ] for all j in the range of 0 to vui_mvc_num_traget_output_views_minus1[ i ], inclusive, and vui_mvc_temporal_id[ i ] equal to hrdTId.

<< END OF CHANGE 3 >>

<< CHANGE 4 >>

- I.3	Definitions

...

I.3.X	view: a texture view and a depth view of the same value of view_id, unless explicitly limited to either texture view or depth view.

I.3.X	view component pair: A texture view component and a depth view component of the same view within the same access unit.

<< END OF CHANGE 4 >>

<< CHANGE 5 >>

- I.7.4.1.2.3	Order of NAL units and coded pictures and association to access units

The specification of subclause H.Error: Reference source not found applies with the following modifications.

- NOTE – Some bitstreams that conform to one or more profiles specified in this annex do not conform to any profile specified in Annex Error: Reference source not found (prior to operation of the base view extraction process specified in subclause I.8.5.4). As specified in subclauses Error: Reference source not found and Error: Reference source not found, for the profiles specified in Annex Error: Reference source not found, NAL units with nal_unit_type equal to 21 are classified as non-VCL NAL units that must be preceded within each access unit by at least one NAL unit with nal_unit_type in the range of 1 to 5, inclusive. For this reason, any bitstream that conforms to one or more profiles specified in this annex does not conform to any profile specified in Annex Error: Reference source not found when it contains any of the following:

–	any access unit that does not contain any NAL units with nal_unit_type equal to 1 or 5, but contains one or more NAL units with nal_unit_type equal to 6, 7, 8, 9, or 15;

–	any access unit in which one or more NAL units with nal_unit_type equal to 7, 8, or 15 is present after the last NAL unit in the access unit with nal_unit_type equal to 1 or 5.

The association of VCL NAL units to primary or redundant coded pictures is specified in subclause I.7.4.1.2.5.

The constraints for the detection of the first VCL NAL unit of a primary coded picture are specified in subclause I.7.4.1.2.4.

The constraint expressed in subclause H.Error: Reference source not found on the order of a buffering period SEI message is replaced by the following constraints.

–	When an SEI NAL unit containing a buffering period SEI message is present, the following applies:

–	If the buffering period SEI message is the only buffering period SEI message in the access unit and it is not included in an MVC scalable nesting SEI message or a 3DVC scalable nesting SEI message, the buffering period SEI message shall be the first SEI message payload of the first SEI NAL unit in the access unit.

–	Otherwise (the buffering period SEI message is not the only buffering period SEI message in the access unit or it is included in an MVC scalable nesting SEI message or it is included in a 3DVC scalable nesting SEI message or it is included in a 3DVC texture sub-bitstream HRD nesting SEI message), the following constraints are specified:

–	When a buffering period SEI message that is not included in no one of an MVC scalable nesting SEI message or a 3DVC scalable nesting SEI message or a 3DVC texture sub-bitstream HRD nesting SEI message is present, this buffering period SEI message shall be the only SEI message payload of the first SEI NAL unit in the access unit.

–	An MVC scalable nesting SEI message that includes a buffering period SEI message shall not include any other SEI messages and shall be the only SEI message inside the SEI NAL unit.

–	A 3DVC scalable nesting SEI message that includes a buffering period SEI message shall not include any other SEI messages and shall be the only SEI message inside the SEI NAL unit.

–	A 3DVC texture sub-bitstream HRD nesting SEI message that includes a buffering period SEI message shall not include any other SEI messages and shall be the only SEI message inside the SEI NAL unit.

–	All SEI NAL units that precede an SEI NAL unit that contains an MVC scalable nesting SEI message with a buffering period SEI message as payload, or a 3DVC scalable nesting SEI message with a buffering period SEI message as payload, or a 3DVC texture sub-bitstream HRD nesting SEI message with a buffering period SEI message as payload in an access unit shall only contain buffering period SEI messages or MVC scalable nesting SEI messages with a buffering period SEI message as payload, or 3DVC scalable nesting SEI messages with a buffering period SEI message as payload, or 3DVC texture sub-bitstream HRD nesting SEI messages with a buffering period SEI message as payload.

<< END OF CHANGE 5 >>

<< CHANGE 6 >>

- I.8	3DVC decoding process

...

All sub-bitstreams that can be derived using the sub-bitstream extraction process with depthPresentFlagTarget equal to 0 or 1, pIdTarget equal to any value in the range of 0 to 63, inclusive, tIdTarget equal to any value in the range of 0 to 7, inclusive, viewIdTargetList consisting of any one or more viewIdTarget's identifying the views in the bitstream as inputs as specified in subclause I.8.5 shall result in a set of coded video sequences, with each coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found, Annex Error: Reference source not found and Annex I.

<< END OF CHANGE 6 >>

<< CHANGE 7 >>

- I.8.5.3	Sub-bitstream extraction process

It is requirement of bitstream conformance that any sub-bitstream that is the output of the process specified in this subclause with depthPresentFlagTarget equal to 0 or 1, pIdTarget equal to any value in the range of 0 to 63, inclusive, tIdTarget equal to any value in the range of 0 to 7, inclusive, viewIdTargetList consisting of any one or more values of viewIdTarget identifying the views in the bitstream, shall be conforming to this Recommendation | International Standard.

NOTE 1 – A conforming bitstream contains one or more coded slice NAL units with priority_id equal to 0 and temporal_id equal to 0.

NOTE 2 – It is possible that not all operation points of sub-bitstreams resulting from the sub-bitstream extraction process have an applicable level_idc or level_idc[ i ]. In this case, each coded video sequence in a sub-bitstream must still conform to one or more of the profiles specified in Annex Error: Reference source not found, Annex Error: Reference source not found and Annex I, but may not satisfy the level constraints specified in subclauses Error: Reference source not found, Error: Reference source not found and I.10.2, respectively.

Inputs to this process are:

–	a variable depthPresentFlagTarget (when present),

–	a variable pIdTarget (when present),

–	a variable tIdTarget (when present),

–	a list viewIdTargetList consisting of one or more values of viewIdTarget (when present).

Outputs of this process are a sub-bitstream and a list of VOIdx values VOIdxList.

When depthPresentFlagTarget is not present as input to this subclause, depthPresentFlagTarget is inferred to be equal to 0.

When pIdTarget is not present as input to this subclause, pIdTarget is inferred to be equal to 63.

When tIdTarget is not present as input to this subclause, tIdTarget is inferred to be equal to 7.

When viewIdTargetList is not present as input to this subclause, there shall be one value of viewIdTarget inferred in viewIdTargetList and the value of viewIdTarget is inferred to be equal to view_id of the base view.

The sub-bitstream is derived by applying the following operations in sequential order:

- Let VOIdxList be empty and minVOIdx be the VOIdx value of the base view.

- For each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause Error: Reference source not found for texture views with the viewIdTarget as input.

- If depthPresentFlagTarget is equal to 1, for each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause Error: Reference source not found for depth views with the viewIdTarget as input.

- For each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause Error: Reference source not found for texture views with the value of viewIdTarget as input.

- If depthPresentFlagTarget is equal to 1, for each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause Error: Reference source not found for depth views with the viewIdTarget as input.

- Mark all VCL NAL units and filler data NAL units for which any of the following conditions are true as "to be removed from the bitstream":

–	priority_id is greater than pIdTarget,

–	temporal_id is greater than tIdTarget,

–	anchor_pic_flag is equal to 1 and view_id is not marked as "required for anchor",

–	anchor_pic_flag is equal to 0 and view_id is not marked as "required for non-anchor",

–	nal_ref_idc is equal to 0 and inter_view_flag is equal to 0 and view_id is not equal to any value in the list viewIdTargetList,

–	nal_unit_type is equal to 21 and depthPresentFlagTarget is equal to 0.

- Remove all access units for which all VCL NAL units are marked as "to be removed from the bitstream".

- Remove all VCL NAL units and filler data NAL units that are marked as "to be removed from the bitstream".

- When VOIdxList contains only one value of VOIdx that is equal to minVOIdx, remove the following NAL units:

–	all NAL units with nal_unit_type equal to 14 or 15,

–	all NAL units with nal_unit_type equal to 6 in which the first SEI message has payloadType in the range of 36 to 44, inclusive.

NOTE 3 – When VOIdxList contains only one value of VOIdx equal to minVOIdx, the sub-bitstream contains only the base view or only a temporal subset of the base view.

- Remove all NAL units with nal_unit_type equal to 6 in which the first SEI message has payloadType equal to 0 or 1, or the first SEI message has payloadType equal to equal to 37 (MVC scalable nesting SEI message) and operation_point_flag in the first SEI message is equal to 1.

NOTE – The buffering period SEI and picture timing SEI messages, when not nested or nested in the MVC scalable nesting SEI message, apply for a sub-bitstream obtained with the sub-bitstream extraction process of subclause H.8.5.3, which does not process NAL units of nal_unit_type equal to 21.

- When depthPresentFlagTarget is equal to 0, the following applies.

–	Replace all NAL units with nal_unit_type equal to 6 in which payloadType indicates a 3DVC texture sub-bitstream HRD nesting SEI message with the nal_unit_type equal to 6 with payload consisting of the SEI message nested within the 3DVC texture sub-bitstream HRD nesting SEI message.

–	The following applies for each active texture 3DVC sequence parameter set RBSP.

–	Replace mvc_vui_parameters_extension( ) syntax structure in an active texture 3DVC sequence parameter set RBSPs with the mvc_vui_parameters_extension( ) syntax structure of the 3DVC texture sub-bitstream VUI parameters extension, if both mvc_vui_parameters_extension( ) syntax structures apply to the same views.

–	Otherwise, remove mvc_vui_parameters_extension( ) syntax structure in an active texture 3DVC sequence parameter set RBSP.

–	Remove all NAL units with nal_unit_type equal to 6 in which the first SEI message has payloadType in the range of 48 to 50, inclusive.

- Let maxTId be the maximum temporal_id of all the remaining VCL NAL units. Remove all NAL units with nal_unit_type equal to 6 that only contain SEI messages that are part of an MVC scalable nesting SEI message or 3DVC scalable nesting SEI message with any of the following properties:

–	operation_point_flag is equal to 0 and all_view_components_in_au_flag is equal to 0 and none of sei_view_id[ i ] for all i in the range of 0 to num_view_components_minus1, inclusive, corresponds to a VOIdx value included in VOIdxList,

–	operation_point_flag is equal to 1 and either sei_op_temporal_id is greater than maxTId or the list of sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, is not a subset of viewIdTargetList (i.e., it is not true that sei_op_view_id[ i ] for any i in the range of 0 to num_view_components_op_minus1, inclusive, is equal to a value in viewIdTargetList).

- Let maxTId be the maximum temporal_id of all the remaining VCL NAL units. Remove all NAL units with nal_unit_type equal to 6 that only contain SEI messages that are part of a 3DVC texture sub-bitstream HRD nesting SEI message with any of the following properties:

–	either texture_subbitstream_temporal_id is greater than maxTId or the list of texture_subbitstream_view_id[ i ] for all i in the range of 0 to num_texture_subbitstream_view_components_minus1, inclusive, is not a subset of viewIdTargetList (i.e., it is not true that sei_texture_subbitstream_view_id[ i ] for any i in the range of 0 to num_texture_subbitstream_view_components_minus1, inclusive, is equal to a value in viewIdTargetList).

- Remove each view scalability information SEI message and each operation point not present SEI message, when present.

- When VOIdxList does not contain a value of VOIdx equal to minVOIdx, the view with VOIdx equal to the minimum VOIdx value included in VOIdxList is converted to the base view of the extracted sub-bitstream. An informative procedure that outlines key processing steps to create a base view is described in subclause Error: Reference source not found.

NOTE 4 – When VOIdxList does not contain a value of VOIdx equal to minVOIdx, the resulting sub-bitstream according to the operation steps 1-13 above does not contain a base view that conforms to one or more profiles specified in Annex Error: Reference source not found. In this case, by this operation step, the remaining view with the new minimum VOIdx value is converted to be the new base view that conforms to one or more profiles specified in Annex Error: Reference source not found and Annex H.

<< END OF CHANGE 7 >>

<< CHANGE 8 >>

- I.13.1.2	3DVC scalable nesting SEI message syntax

| 3dv_scalable_nesting( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| operation_point_flag | 5 | u(1) |
| if ( !operation_point_flag ) { |  |  |
| all_view_components_in_au_flag | 5 | u(1) |
| if( !all_view_components_in_au_flag ) { |  |  |
| num_view_components_minus1 | 5 | ue(v) |
| for( i = 0; i <= num_view_components_minus1; i++ ) { |  |  |
| sei_view_id[ i ] | 5 | u(10) |
| sei_view_applicability_flag[ i ] | 5 | u(1) |
| } |  |  |
| } |  |  |
| } else { |  |  |
| num_view_components_op_minus1 | 5 | ue(v) |
| for( i = 0; i <= num_view_components_op_minus1; i++ ) { |  |  |
| sei_op_view_id[ i ] | 5 | u(10) |
| sei_op_view_applicability_flag[ i ] | 5 | u(1) |
| } |  |  |
| sei_op_temporal_id | 5 | u(3) |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| sei_nesting_zero_bit /* equal to 0 */ | 5 | f(1) |
| sei_message( ) | 5 |  |
| } |  |  |

- I.13.2.2	3DVC scalable nesting SEI message semantics

…

sei_op_view_applicability_flag[ i ] equal to 1 indicates that the nested SEI message applies to both the texture view component and the depth view component of the view with view_id equal to sei_op_view_id[ i ]. sei_op_view_applicability_flag[ i ] equal to 0 indicates that the nested SEI message applies only to the depth view component of the view with view_id equal to sei_op_view_id[ i ].

NOTE – MVC scalable nesting SEI message should be used for nesting SEI messages, when depth views may or may not be present in the bitstream, the nested SEI messages apply only to indicated texture view components and the semantics of the nested SEI messages apply when VCL and non-VCL NAL units are classified according to Annex H NAL unit type class of Table 7-1.

NOTE 2 – 3DVC texture sub-bitstream HRD nesting SEI message should be used for nesting buffering period and picture timing SEI messages, when the nested SEI messages apply only to texture views and depth views have been removed using the sub-bitstream extraction process of subclause I.8.5.3 with depthPresentFlagTarget equal to 0.

<< END OF CHANGE 8 >>

<< CHANGE 9 >>

- I.13.1.x	3DVC texture sub-bitstream HRD nesting SEI message syntax

| 3dvc_texture_subbitstream_hrd_nesting( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| num_texture_subbitstream_view_components_minus1 | 5 | ue(v) |
| for( i = 0; i <= num_view_components_op_minus1; i++ ) |  |  |
| texture_subbitstream_view_id[ i ] | 5 | u(10) |
| texture_subbitstream_temporal_id | 5 | u(3) |
| while( !byte_aligned( ) ) |  |  |
| sei_nesting_zero_bit /* equal to 0 */ | 5 | f(1) |
| sei_message( ) | 5 |  |
| } |  |  |

- I.13.2.x	3DVC texture sub-bitstream HRD nesting SEI message semantics

A 3DVC texture sub-bitstream HRD nesting SEI message shall contain one and only one SEI message of payload type 0 or 1 or one and only one MVC scalable nesting SEI message containing one SEI message of payload type 0 or 1. The SEI message included in a 3DVC texture sub-bitstream HRD nesting SEI message and not included in an MVC scalable nesting SEI message is referred to as the nested SEI message.

The semantics of the nested SEI message apply for the sub-bitstream obtained with the sub-bitstream extraction process of subclause I.8.5.3 with depthPresentFlagTarget equal to 0, tIdTarget equal to texture_subbitstream_temporal_id, and viewIdTargetList consisting of texture_subbitstream_view_id[ i ] for all values of i in the range of 0 to num_texture_subbitstream_view_components_ minus1, inclusive, as inputs.

num_texture_subbitstream_view_components_minus1 plus 1 specifies the number of view components of the operation point to which the nested SEI message applies. The value of num_texture_subbitstream_view_components_minus1 shall be in the range of 0 to 1023, inclusive.

texture_subbitstream_view_id[ i ] specifies the view_id of the i-th view component to which the nested SEI message applies.

texture_subbitstream_temporal_id specifies the maximum temporal_id of the bitstream subset to which the nested SEI message applies.

sei_nesting_zero_bit is equal to 0.

<< END OF CHANGE 9 >>

<< CHANGE 10 >>

- I.13.2	SEI message semantics

Depending on payloadType, the corresponding SEI message semantics are extended as follows:

–	If payloadType is equal to 2, 3, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 45 or 47, the following applies:

–	If the SEI message is not included in an MVC scalable nesting SEI message or a 3DVC scalable nesting SEI message, it applies to the texture view component of the current access unit with VOIdx equal to VOIdxMin.

–	Otherwise, if included in an MVC scalable nesting SEI message and not included in a 3DVC scalable nesting SEI message, it applies to all texture view components of the current access unit when all_view_components_in_au_flag is equal to 1, or it applies to all texture view components of the current access unit with view_id equal to sei_view_id[ i ] for any i in the range of 0 to num_view_components_minus1, inclusive, when all_view_components_in_au_flag is equal to 0. When payloadType is equal to 10 for the SEI message that is included in an MVC scalable nesting SEI message, the semantics for sub_seq_layer_num of the sub-sequence information SEI message is modified as follows:

sub_seq_layer_num specifies the sub-sequence layer number of the current picture. When the current picture resides in a sub-sequence for which the first picture in decoding order is an IDR picture, the value of sub_seq_layer_num shall be equal to 0. For a non-paired reference field, the value of sub_seq_layer_num shall be equal to 0. sub_seq_layer_num shall be in the range of 0 to 255, inclusive.

–	Otherwise, if not included in an MVC scalable nesting SEI message and included in an 3DVC scalable nesting SEI message, it applies to all depth view components or view component pairs of the current access unit when all_view_components_in_au_flag is equal to 1, or it applies to all depth view components or view component pairs of the current access unit with view_id equal to sei_view_id[ i ] for any i in the range of 0 to num_view_components_minus1, inclusive, when all_view_components_in_au_flag is equal to 0. When payloadType is equal to 10 for the SEI message that is included in an 3DVC scalable nesting SEI message, the semantics for sub_seq_layer_num of the sub-sequence information SEI message is modified as follows:

sub_seq_layer_num specifies the sub-sequence layer number of the current picture. When the current picture resides in a sub-sequence for which the first picture in decoding order is an IDR picture, the value of sub_seq_layer_num shall be equal to 0. For a non-paired reference field, the value of sub_seq_layer_num shall be equal to 0. sub_seq_layer_num shall be in the range of 0 to 255, inclusive.

–	Otherwise, if payloadType is equal to 41, 42 or 43, the following applies:

–	If the SEI message is not included 3DVC scalable nesting SEI message, it applies to texture views only and NAL units having nal_unit_type equal to 21 are non-VCL NAL units.

–	Otherwise (the SEI message is included in 3DVC scalable nesting SEI message), the SEI message applies to all depth view views or both all texture views and all depth views, depending on the values of the syntax elements of the 3DVC scalable nesting SEI message.

–	Otherwise, if payloadType is equal to 0 or 1, the following applies:

–	If the SEI message is not included in an MVC scalable nesting SEI message or a 3DVC scalable nesting SEI message or a 3DVC texture sub-bitstream HRD nesting SEI message, the following applies. When the SEI message and all other SEI messages with payloadType equal to 0 or 1 not included in an MVC scalable nesting SEI message or a 3DVC scalable nesting SEI message or a 3DVC texture sub-bitstream HRD nesting SEI message are used as the buffering period and picture timing SEI messages for checking the bitstream conformance according to Annex  and the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found is used, the bitstream shall be conforming to this Recommendation | International Standard.

–	Otherwise, if the SEI message is included in an MVC scalable nesting SEI message and not included in a 3DVC scalable nesting SEI message or a 3DVC texture sub-bitstream HRD nesting SEI message, the following applies. When the SEI message and all other SEI messages with payloadType equal to 0 or 1 included in an MVC scalable nesting SEI message with identical values of sei_op_temporal_id and sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, are used as the buffering period and picture timing SEI messages for checking the bitstream conformance according to Annex , the bitstream that would be obtained by invoking the bitstream extraction process as specified in subclause Error: Reference source not found with tIdTarget equal to sei_op_temporal_id and viewIdTargetList equal to sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, shall be conforming to this Recommendation | International Standard.

In the semantics of subclauses Error: Reference source not found and Error: Reference source not found, the syntax elements num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag and the derived variables NalHrdBpPresentFlag, VclHrdBpPresentFlag, and CpbDpbDelaysPresentFlag are substituted with the syntax elements vui_mvc_num_units_in_tick[ i ], vui_mvc_time_scale[ i ], vui_mvc_fixed_frame_rate_flag[ i ], vui_mvc_nal_hrd_parameters_present_flag[ i ], vui_mvc_vcl_hrd_parameters_present_flag[ i ], vui_mvc_low_delay_hrd_flag[ i ], and vui_mvc_pic_struct_present_flag[ i ] and the derived variables VuiMvcNalHrdBpPresentFlag[ i ], VuiMvcVclHrdBpPresentFlag[ i ], and VuiMvcCpbDpbDelaysPresentFlag[ i ].

The values of seq_parameter_set_id's in all buffering period SEI messages included in MVC scalable nesting SEI messages and associated with operation points for which the greatest VOIdx values in the associated bitstream subsets are identical shall be identical.

–	Otherwise, if the SEI message is included in a 3DVC scalable nesting SEI message and not included in an MVC scalable nesting SEI message or a 3DVC texture sub-bitstream HRD nesting SEI message, the following applies. When the SEI message and all other SEI messages with payloadType equal to 0 or 1 included in a 3DVC scalable nesting SEI message with identical values of sei_op_temporal_id and sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, are used as the buffering period and picture timing SEI messages for checking the bitstream conformance according to Annex , the bitstream that would be obtained by invoking the bitstream extraction process as specified in subclause I.8.5 with depthPresentTargetFlag equal to 1, tIdTarget equal to sei_op_temporal_id and viewIdTargetList equal to sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, shall be conforming to this Recommendation | International Standard.

In the semantics of subclauses Error: Reference source not found and Error: Reference source not found, the syntax elements num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag and the derived variables NalHrdBpPresentFlag, VclHrdBpPresentFlag, and CpbDpbDelaysPresentFlag are substituted with the syntax elements vui_mvc_num_units_in_tick[ i ], vui_mvc_time_scale[ i ], vui_mvc_fixed_frame_rate_flag[ i ], vui_mvc_nal_hrd_parameters_present_flag[ i ], vui_mvc_vcl_hrd_parameters_present_flag[ i ], vui_mvc_low_delay_hrd_flag[ i ], and vui_mvc_pic_struct_present_flag[ i ] and the derived variables VuiMvcNalHrdBpPresentFlag[ i ], VuiMvcVclHrdBpPresentFlag[ i ], and VuiMvcCpbDpbDelaysPresentFlag[ i ] for the 3DVC VUI parameters extension.

The values of seq_parameter_set_id's in all buffering period SEI messages included in 3DVC scalable nesting SEI messages and not included in either MVC scalable nesting SEI messages or 3DVC texture sub-bitstream HRD nesting SEI messages and associated with operation points for which the greatest VOIdx values in the associated bitstream subsets are identical shall be identical.

–	Otherwise, if the SEI message is included in a 3DVC texture sub-bitstream HRD nesting SEI message, the following applies. When the SEI message and all other SEI messages included in a 3DVC texture sub-bitstream HRD nesting SEI message with identical values of texture_subbitstream_temporal_id and texture_subbitstream_view_id[ i ] for all i in the range of 0 to num_texture_subbitstream_view_components_minus1, inclusive, are used as the buffering period and picture timing SEI messages for checking the bitstream conformance according to Annex , the bitstream that would be obtained by invoking the bitstream extraction process as specified in subclause I.8.5 with depthPresentTargetFlag equal to 0, tIdTarget equal to texture_subbitstream_temporal_id and viewIdTargetList equal to texture_subbitstream_view_id[ i ] for all i in the range of 0 to num_texture_subbitstream_view_components_minus1, inclusive, shall be conforming to this Recommendation | International Standard.

In the semantics of subclauses Error: Reference source not found and Error: Reference source not found, the syntax elements num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag and the derived variables NalHrdBpPresentFlag, VclHrdBpPresentFlag, and CpbDpbDelaysPresentFlag are substituted with the syntax elements vui_mvc_num_units_in_tick[ i ], vui_mvc_time_scale[ i ], vui_mvc_fixed_frame_rate_flag[ i ], vui_mvc_nal_hrd_parameters_present_flag[ i ], vui_mvc_vcl_hrd_parameters_present_flag[ i ], vui_mvc_low_delay_hrd_flag[ i ], and vui_mvc_pic_struct_present_flag[ i ] and the derived variables VuiMvcNalHrdBpPresentFlag[ i ], VuiMvcVclHrdBpPresentFlag[ i ], and VuiMvcCpbDpbDelaysPresentFlag[ i ] for the 3DVC texture sub-bitstream VUI parameters extension.

The values of seq_parameter_set_id's in all buffering period SEI messages included in 3DVC texture sub-bitstream HRD nesting SEI messages and associated with operation points for which the greatest VOIdx values in the associated bitstream subsets are identical shall be identical.

–	Otherwise (all remaining payloadType values), the corresponding SEI message semantics are not extended.

For the semantics of SEI messages with payloadType in the range of 0 to 23, inclusive, which are specified in subclause Error: Reference source not found, 3DVC sequence parameter set is substituted for sequence parameter set; the parameters of 3DVC sequence parameter set RBSP and picture parameter set RBSP that are in effect are specified in subclauses I.7.4.2.1and I.7.4.2.2, respectively.

Coded video sequences conforming to one or more of the profiles specified in Annex I shall not include SEI NAL units that contain SEI messages with payloadType in the range of 24 to 35, inclusive.

When an SEI NAL unit contains an SEI message with payloadType in the range of 36 to 44, inclusive, it shall not contain any SEI messages with payloadType less than 36 and the first SEI message in the SEI NAL unit shall have payloadType in the range of 36 to 44, inclusive.

When an MVC scalable nesting SEI message (payloadType equal to 37) or a view scalability information SEI message (payloadType equal to 38) or an operation point not present SEI message (payloadType equal to 43) or a 3DVC scalable nesting SEI message (payloadType equal to 48) or a 3DVC texture sub-bitstream HRD nesting SEI message (payloadType equal to 51) is present in an SEI NAL unit, it shall be the only SEI message in the SEI NAL unit.

<< END OF CHANGE 10 >>

<< CHANGE 11 >>

- I.3	Definitions

...

I.3.X	base depth view A depth view that has the minimum value of view order index in a coded video sequence. The base depth view can be decoded independently of other depth views and does not use inter-view prediction. There is only one base depth view in a coded video sequence.

I.3.X	base texture view A texture view that has the minimum value of view order index in a coded video sequence. The base texture view can be decoded independently of other views, does not use inter-view prediction, and contains VCL NAL units only with nal_unit_type equal to 1, 5, or 14. The bitstream subset corresponding to the base texture view conforms to one or more of the profiles specified in Annex Error: Reference source not found. There is only one base texture view in a coded video sequence.

...

- I.7.4.2.13	Depth range update RBSP semantics

pred_direction equal to 0 specifies that the 3DV acquisition variables (the closest and farthest depth value) for the base depth view may be predicted from the respective variables of two pictures. pred_direction equal to 1 specifies that the 3DV acquisition variables for the base depth view may be predicted from the respective variables of one picture. pred_direction equal to 2 specifies that the 3DV acquisition variables for the base depth view are not predicted. The value of pred_direction shall be in the range of 0 to 2, inclusive.

…

- I.8.5.4	Specification of the base texture view bitstream

A bitstream that conforms to one or more profiles as specified in Annex I shall contain a base texture view bitstream that conforms to one or more of the profiles specified in Annex Error: Reference source not found. This base texture view bitstream is derived by invoking the sub-bitstream extraction process as specified in subclause  with no input and the base texture view bitstream being the output.

NOTE – Although all multiview bitstreams that conform to one or more of the profiles specified in this annex contain a base texture view bitstream that conforms to one or more of the profiles specified in Annex Error: Reference source not found, the complete multiview bitstream (prior to operation of the base texture view extraction process specified in this subclause) may not conform to any profile specified in Annex Error: Reference source not found.

…

- I.10.1.1	3D High profile

Bitstreams conforming to the 3D High profile shall obey the following constraints:

–	The base texture view bitstream as specified in subclause Error: Reference source not found shall obey all constraints of the High profile specified in subclause Error: Reference source not found and all active sequence parameter sets shall fulfill one of the following conditions:

…

I.13.2.3.1	Depth acquisition element semantics

...

pred_direction equal to 0 specifies that the 3DV acquisition variables for the base depth view may be predicted from the respective variables of two pictures. pred_direction equal to 1 specifies that the 3DV acquisition variables for the base depth view may be predicted from the respective variables of one picture. pred_direction equal to 2 specifies that the 3DV acquisition variables for the base depth view are not predicted. The value of pred_direction shall be in the range of 0 to 2, inclusive.

<< END OF CHANGE 11 >>

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).