| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0006 |
| --- | --- |

| Title: | Ad hoc on 3D Video Coding High Level Syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Draft Report |  |  |
| Author(s) or Contact(s): | Ying Chen Thomas Rusert, Heiko Schwarz | Tel: Email: | +1-858-845-6589 cheny@qualcomm.com +46 76 7602660 thomas.rusert@ericsson.com heiko.schwarz@hhi.fraunhofer.de |
| Source: | 3DVC HLS Ad hoc |  |  |

_____________________________

# Abstract

This report summarizes the activities of the 3DVC high-level syntax ad hoc group (3DVC HLS AHG) between the 100th MPEG meeting and the 101st MPEG meeting and the 1st JCT2 meeting, and lists the related input documents submitted to this meeting according to their topics.

# Mandates

- Study the HEVC design and HM software with regard to multi-view extensions for high-level syntax, buffering, and other architectural concepts.

- Study high level syntax design extensions of ATM for 3DV functionalities.

- Study the relation of ATM and HTM high-level syntax concepts, also with regard to hybrid architectures.

- Identify commonality of multi-view and scalable high-level extensions of HEVC.

- Seek communication with JCT AHGs on High level syntax and Hooks for scalable coding on these topics.

# Reflector discussions

There was email discussion on the software integration related to High-Level Syntax (HLS) features, such as video parameter sets.

# Working status and comments

Mandates

- Study the HEVC design and HM software with regard to multi-view extensions for high-level syntax, buffering, and other architectural concepts.

There are 8 3D-HEVC HLS input contributions related to e.g., parameter sets, NAL unit header design and random access. They are based on the latest HEVC specification. The AHG has reviewed 4 input documents.

- Study high level syntax design extensions of ATM for 3DV functionalities.

There are 10 input contributions, majority of them are focusing on the MVC based 3DV codec, related to e.g., HRD, sequence parameter set and slice header. The AhG has reviewed 9 out of the 10 input documents.

- Study the relation of ATM and HTM high-level syntax concepts, also with regard to hybrid architectures.

No activities during this meeting on (AVC-HEVC) hybrid codec.

- Identify commonality of multi-view and scalable high-level extensions of HEVC.

Input documents related to video parameter set and sequence parameter set may be applicable to both multi-view and scalable extensions of HEVC.

- Seek communication with JCT AHGs on High level syntax and Hooks for scalable coding on these topics.

Some JCT-VC experts active in the High Level Syntax and extension hooks joined the AhG meeting.  Better communication is expected by having the discussions in JCT2.

# Related documents

High-level syntax related contributions registered by July 11, 2012 to this JCT-2 meeting are listed below, according to the topics. There are overall 20 documents, and they are categorized as follows:

- MVC based 3DV (9)

  - Sequence parameter set

  - HRD and bitstream extraction

  - Slice header

  - Depth range signaling

  - Depth definition

- HEVC based 3DV (8)

  - NAL unit header

  - Video parameter set and sequence parameter set

  - Random access

- AVC based 3DV (1)

  - Reference picture lists

- SEI messages (2)

  - Depth perception SEI

  - Camera parameter signalling

| JCT2 number | Category | MPEG number | Title | Source |
| --- | --- | --- | --- | --- |
| JCT2-A0001 |  | m25277 | Ad hoc on 3D Video Coding | Karsten Müller, Anthony Vetro |
| JCT2-A0006 |  | m25766 | Ad hoc on 3D Video Coding High Level Syntax | Ying Chen, Thomas Rusert, Heiko Schwarz |
| JCT2-A0002 | Type 1: MVC | m25751 | 3DV-ATM HP high-level syntax: HRD parameters for texture sub-bitstreams | M. M. Hannuksela (Nokia) |
| JCT2-A0007 | Type 1: MVC | m25780 | 3D-AVC HLS: Depth Range Parameter Set for MVC Compatible 3D Video Coding | M. LI, P. WU(ZTE) |
| JCT2-A0008 | Type 1: MVC | m25781 | 3D-AVC HLS: Depth Range Parameter Set for AVC Compatible 3D Video Coding | M. LI, P. WU(ZTE) |
| JCT2-A0053 | Type 1: MVC | m25930 | 3DV-ATM HP/EHP HLS：Camera Parameter Set (CPS) for camera parameter signaling | Peng Lu, Yin Zhao, Lu Yu |
| JCT2-A0074 | Type 1: MVC | m26018 | 3DV-ATM HP: Proposed HRD text for ISO/IEC 14496-10:2012/PDAM 2 MVC extensions for inclusion of depth maps | S. Hattori, T. Suzuki (Sony), Y. Chen (Qualcomm), A. Vetro (MERL) |
| JCT2-A0075 | Type 1: MVC | m26019 | 3DV-ATM HP Technical Input : A study on depth range based weighted prediction tool | S. Hattori, T. Suzuki (Sony) |
| JCT2-A0100 | Type 1: MVC | m26055 | 3D-HLS: Signalling number of to-be-decoded texture/depth views for MVC based 3DV | Y. Chen, Y.-K. Wang (Qualcomm) |
| JCT2-A0112 | Type 1: MVC | m26074 | 3DV-ATM HP: Calculation process for parameters of Depth Range-based Weighted Prediction with fixed-point/integer operations | Lulu Chen, Wenyi Su (USTC), Dmytro Rusanovskyy, Miska M. Hannuksela (Nokia) |
| JCT2-A0012 | Type 2: HEVC | m25840 | Tentative working draft text for the HEVC compatible 3D video coding extension | Gerhard Tech, Philipp Merkle, Martin Winken, Heiko Schwarz, Karsten Mueller, Thomas Wiegand |
| JCT2-A0021 | Type 2: HEVC | m26089 | 3D-HEVC HLS: Inter-layer SPS Prediction | Thomas Rusert (Ericsson) |
| JCT2-A0026 | Type 2: HEVC | m26097 | 3D-HLS: On Slice Header and Parameter Set | B. Choi, J. Kim, J. Park (Samsung) |
| JCT2-A0099 | Type 2: HEVC | m26054 | 3D-HLS: Video parameter set for 3D-HEVC | Y.Chen, Y.-K. Wang (Qualcomm) |
| JCT2-A0115 | Type 2: HEVC | m26102 | 3D-HLS: On NAL Unit Header and Video Parameter Set Design for HEVC 3DV | B. Choi, J. Kim, J. Park (Samsung) |
| JCT2-A0117 | Type 2: HEVC | m26105 | 3D-HLS: On Random Access Pictures | B. Choi, J. Kim, J. Park (Samsung) |
| JCT2-A0118 | Type 2: HEVC | m26107 | 3D-HLS: On View Layer Switching | B. Choi, J. Kim, J. Park (Samsung) |
| JCT2-A0121 | Type 2: HEVC | m26214 | 3D-HLS: Design of the Video Parameter Set for 3D-HEVC | Robert Skupin, Valeri George, Thomas Schierl |
| JCT2-A0102 | Type 3: AVC | m26057 | 3D-HLS: On reference picture list construction for AVC based 3DV | Y.-K. Wang, Y. Chen(Qualcomm) |
| JCT2-A0140 | Type 3: AVC | m26138 | 3D-HLS: On Depth Definition | Shinya Shimizu, Shiori Sugimoto, Hideaki Kimata (NTT) |
| JCT2-A0034 | Type 4: SEI | m25897 | 3DV SEI messages on depth perception | Kwan-Jung Oh, Jaejoon Lee, Du Sik Park (Samsung) |
| JCT2-A0054 | Type 4: SEI | m25931 | 3D-HEVC HLS: SEI message for camera parameter signaling | Peng Lu, Yin Zhao, Lu Yu |
| JCT2-A0153 | Type 4: SEI | m26178 | 3D-AVC HLS: a show case of m25256 (view synthesis adaptation to display parameters) | Yin Zhao, Lu Yu |
| JCT2-A0122 | Type 4: SEI | m26225 | Maximum depth discontinuity signaling for view synthesis range determination | Andrey Norkin, Martin Pettersson, Thomas Rusert (Ericsson) |

# Documents listed in different categories

The first meeting for this AHG was held 16:00 pm to 21:15 pm, Sunday before the MPEG plenary. 12 input contribution documents were presented.

## MVC based 3DV

### Sequence parameter set

- JCT2-A0100/m26055 3D-HLS: Signalling number of to-be-decoded texture/depth views for MVC based 3DV [Y. Chen, Y.-K. Wang (Qualcomm)]

In MVC, for each operation point, the number of views to be decoded is signalled in the MVC SPS extension (by the syntax element applicable_op_num_views_minus1[ i ][ j ]). In MVC based 3DV, the semantics of this syntax element becomes ambiguous and insufficient, since the current PDAM allows a depth view to have a different view dependency than that of the texture view. It is proposed that applicable_op_num_views_minus1[ i ][ j ] is used to signal the number of texture views to be decoded while the number of depth views to be decoded for each operation point is further signalled by another syntax element in the subset SPS.

The proposal is to indicate the number of texture and depth view in the SPS MVC extension for 3DV operation points.

The issue with current MVC based 3DV spec.: the same syntax as SPS MVC extension for 3DV which is not clear if the target views are for texture or depth or texture and depth.

The number of depth views is additionally signaled in the 3DV SPS extension, as a bug fix since the number of depth views to be decoded is not clear in the current MVC extension.

3DV SPS extension will include target output view for texture and depth. The new proposed syntax will only be used to indicate operation points containing depth views.

Discussion: there was positive support for the proposal.

Decision: recommend to Adopt with some editorial modifications/improvements in other part of the spec.

### HRD and bitstream extraction

JCT2-A0002/m25751 3DV-ATM HP high-level syntax: HRD parameters for texture sub-bitstreams [M. M. Hannuksela (Nokia)]

It is asserted that the semantics of the MVC video usability information extension and the MVC scalable nesting SEI message used to carry picture timing and buffering period SEI messages rely on the sub-bitstream extraction process specified in subclause H.8.5.3, which treats NAL units of nal_unit_type equal to 21 as non-VCL NAL units and does not perform temporal_id and view_id based extraction for them. Hence, it is asserted that no proper HRD parameters can be conveyed for sub-bitstreams consisting of texture views only and therefore the contribution proposes a mechanism to convey HRD parameters of VUI as well as picture timing and buffering period SEI messages for the sub-bitstream that consists only of texture views while the depth views are removed by the sub-bitstream extraction process of subclause I.8.5.3. It is proposed that the texture sub-bitstream HRD parameters are conveyed in a second instance of mvc_vui_parameters_extension( ) in a 3DVC sequence parameter set and picture timing and buffering period SEI messages are conveyed in a specific 3DVC texture sub-bitstream HRD nesting SEI message. If a texture sub-bitstream is extracted using the sub-bitstream extraction process, these nested HRD parameters and SEI messages replace the respective MVC ones, which, as stated above, assume the presence of NAL units of nal_unit_type 21 as non-VCL NAL units.

In addition, many related smaller changes are proposed including the following:

- Annex C (Hypothetical Reference Decoder) did not refer to Annex I or subclauses of Annex I. Proper references are to Annex I are now added.

- A clarification of the definition of term view, which proposed to be defined as: unless explicitly limited to either texture view or depth view, a texture view and a depth view of the same value of view_id.

- The use of SEI messages with and without the various nesting SEI message is clarified in subclause I.13.2.

- sei_op_view_applicability_flag[ i ] is removed from 3DV scalable nesting SEI message, because the 3DV operation points always include both texture and depth as specified in I.8.

A clarification of the definition of term base view, and an introduction of two new related terms base texture view and base depth view.

MVC extractor doesn’t extract NAL unit type 21 out because the MVC bit-stream extraction process doesn’t extract out non-VCL NAL units. Such non-VCL NAL units include depth slices which have NAL unit type 21.

Syntax elements as part of HRD contained in the SPS 3DVC extension are not accurate.

It is mentioned by the proponent that 3DVC bitstream extractor could be used to extract texture sub-bitsteams.

Two solutions complementary to each other are proposed:

Solution 1: Two sets of HRD parameters in the 3DVC SPS extension

- One set is for the texture ONLY

- The other set is for each 3DV operation point (containing both texture and depth).

Solution 2: A new type of the nesting SEI message for the carriage of picture timing and buffering period for texture only sub-bitstreams is proposed.

It was commented that the solutions seem to solve valid known problems, due to the lack of forward-compatibility of the AVC/MVC design. Revisit.

JCT2-A0074/m26018 3DV-ATM HP: Proposed HRD text for ISO/IEC 14496-10:2012/PDAM 2 MVC extensions for inclusion of depth maps [S. Hattori, T. Suzuki (Sony), Y. Chen (Qualcomm), A. Vetro (MERL)]

This document proposes text on HRD for ISO/IEC 14496-10:2012/PDAM 2 MVC extensions for inclusion of depth maps [MPEG N12731]. The proposal extends current Annex C in AVC for Annex I in MVC extensions for inclusion of depth maps [MPEG N12731] with similar concept of extending process applied for MVC.

The proposed HRD in Annex J follows the following design principles:

- When the decoding process specified in Annex I is applied, the process specified for Annex H in Annex C is invoked for particular texture view and depth view with view order index VOIdx.

- During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered.

- When the coded video sequence conforms to a profile specified in Annex I and the decoding process specified in Annex I is used, the view components of all the target output views of a picture are output at the same time instant and in increasing order of the view order index VOIdx.

- Inside each view component of the current primary coded picture, depth view component if present, is processed after the texture view component within the same view component.

- The same setting of OutputFlag applies to the texture view component and depth view component wit same view order index VOIdx within the same view component.

This proposal in principle applies the same process as in annex H to texture and depth (same view_id) independently in Annex I.

This proposal does not consider the problem when an MVC decoder gets a 3DV bitstream, which is addressed in JCTVC-A0002. Proposal is claimed to be independent of JCTVC-A0002. Revisit.

### Slice header

JCT2-A0075/m26019 3DV-ATM HP Technical Input : A study on depth range based weighted prediction tool [S. Hattori, T. Suzuki (Sony)]

This document reports a comparison between depth range based weighted prediction (DBWP) and explicit weighted prediction (EWP) in AVC. In this document, we compared coding efficiency of both methods with 3D-ATM software. The experimental result shows that coding efficiency of each method is almost the same.

Explicit weighted prediction (as in the AVC) and DBWP (as in the current ATM software) are compared.

When explicit weighted prediction is used, the calculation of the weights can be done only at the encoder as it is now done at the decoder.  It is noted that such calculations of the weights are not needed if explicit weights are signalled.

When explicit weighted prediction is used, depth range parameters in the SPS and depth range update NAL unit are not needed, although the current software implementation still have them present in the bitstream.

Coding loss for GT_fly sequence is about 0.77% for depth only. There is no loss for the other sequences since the depth range of the other sequences keeps unchanged. Results based on synthesized views are reported as 0.08% for GT_fly.

It is commented that the PSNR might be kept unchanged in the proposal compared to the anchor, but this might require some further confirmation from the proponent. Revisit

JCT2-A0112/m26074 Calculation process for parameters of Depth Range-based Weighted Prediction with fixed-point/integer operations Dmytro Rusanovskyy (Nokia)] [late]

Parameters of Depth Range-based Weighted Prediction (DRWP) in 3DV-ATM are currently implemented with floating point operations. This contribution proposes an integer (fixed-point) implementation for this process. Presented in this document results show that proposed implementation can be deployed with insignificant penalty on the coding gain: 0.03% of dBR on total bitrate reduction for GT_Fly test sequence.

This proposal contains two aspects:

The first aspect evaluates the DRWP. 5.6% loss for depth views only was reported when DRWP is disabled, for the GT_fly sequence, which is the only sequence that this technique applies to. The loss leads to 0.2% loss for synthesized views was reported when DRWP is disabled (for GT_fly sequence).

The second aspect simplifies the DRWP with integer based operations. The integer based DRWP introduces only negligible loss.

In the integer based DRWP, all operations, including divisions, can be done with 32-bit accuracy.

Floating point calculations are only done once per slice. However, it is commented that this is the only place in the 3DV spec. that uses the floating point calculations.

It was mentioned that JNB had a comment which suggests changing the floating point to fixed-point calculation during DRWP.

It was commented that this proposal is related to JCT2-A0075.

Decision: Recommend to Adopt after the proposed text is reviewed.

### Depth range signaling

JCT2-A0007/m25780 3D-AVC HLS: Depth Range Parameter Set for MVC Compatible 3D Video Coding [M. Li, P. Wu (ZTE)]

In this document, the concept and operations of parameter set are proposed to be applied to coding depth range parameters for MVC compatible 3D video coding. Therefore, depth range parameters can be effectively shared among different access units by slices’ referring to depth range parameter set (DRPS), which saves bits from repeatedly generating depth range update RBSPs for different access units using the same depth range parameters in decoding.

It was commented that this proposal is related to JCT2-A0075.

Duplications of depth range parameters might be needed when they are identical. It is mentioned that prediction scheme is allowed in the current design, but there might still be slight overhead for NAL unit header and several syntax elements indicating the prediction.

The prediction scheme might not be robust, since the current design enables prediction between access units.

It is proposed that the DRPS can be designed similar to APS (in HEVC).

- A parameter set id is added.

- A reference id of the DRPS is signalled to replace frame_num, so prediction of the DRPSes is still enabled.

- The id equal to zero is reserved for the depth range in SPS. It is commented that the id of the DRPS can be signalled directly in the slice header.

A comment was given that this proposal is giving improvements in terms of error resiliency although detailed design needs to be carefully studied. Revisit.

JCT2-A0053/m25930 3DV-ATM HP/EHP HLS：Camera Parameter Set (CPS) for camera parameter signaling [P. Lu, Y. Zhao, L. Yu (Zhejiang University)]

This proposal modifies the mechanism of camera parameter signaling both in MVC based 3DV and AVC based 3DV. The main characteristics of the proposed method can be summarized as follows:

	This proposal applies the concept of parameter set on camera parameters. Camera parameters are included in Sequence Parameter Set (SPS) 3DVC extension and Camera Parameter Set (CPS).

	Slice can refer to CPS by cam_parameter_set_id in slice header.

	Each CPS contains the camera parameters of one or more access unit.

	The temporal prediction among camera parameters is only carried out within CPS.

	CPS may use the camera parameters in SPS 3DVC extension as references to do temporal prediction among camera parameters.

It is claimed by the proponent that this proposal is similar to JCT2-A0007. It was commented that the proposal might be a bit over optimized for bit-rate saving. It aims to resolve the same problem of error resilience for the signaling of some camera parameters, especially depth range.

JCT2-A0166/m26293. 3DV-ATM High-Level Syntax: Loss Detection of Camera Parameter Sets [M. M. Hannuksela (Nokia)] late

Contributions JCT2-A0007, JCT2-A0008, and JCT2-A0053 propose the use of camera parameter sets to convey information for the closest and farthest depth as well as parameters related to view synthesis prediction. While parameter sets enable loss-resilient transmission of parameters, for example through retransmission possibility, it is asserted in this contribution that the mentioned proposals do not propose means to detect a loss of a camera parameter set. More specifically, if the camera_param_set_id value of the lost camera parameter set NAL unit has been used previously by another camera parameter set NAL unit, it is claimed that in such a case the decoder would continue decoding using the previously received camera parameter set NAL unit, which would be likely to result into clearly visible artifacts. It is asserted that this problem is being discussed also in JCT-VC related to adaptation parameter sets, which may be frequently updated similarly to camera parameter sets. This contribution proposes a parameter set loss detection approach based on constraints on camera_param_set_id value assignment.This proposal is technically identical to JCTVC-0072. It was commented that proposed functionality might be achievable by encoder only method.

JCT2-A0140/m26138 3D-HLS: On Depth Definition [Shinya Shimizu, Shiori Sugimoto, Hideaki Kimata (NTT)]

This contribution proposes a modification on the depth acquisition information SEI message and a definition of depth representation type in order to support many kinds of multiview video plus depth map other than those used in the development. Regarding the depth acquisition information SEI message, it is suggested to support convergence camera setting. The proposed depth representation type includes three kinds of representation: disparity, an inverse of Z value, and Z value. Since there are some coding tools which requires understanding the definition of depth, this contribution proposes to define depth representation type as a part of the sequence parameter set extension. The extension of the depth-range-based weighted prediction, which is one of the coding tool effected by the depth representation type, is also discussed.

It is proposed that many types of depth representations may exist in the marketplace, so the MVC based 3DV standard may support more types such that the current DRWP can be applicable to other depth representations.

It is proposed new types of depth representations are defined in SPS.

- Disparity value

- Z value (instead of 1/Z)

A comment was given that if the group wants to seriously consider new types, it would be good to have test sequences belonging to such types.

It was commented that discussion is needed for the definition of depth. Revisit

## HEVC based 3DV

### NAL unit header

JCT2-A0115/m26102 3D-HLS: On NAL Unit Header and Video Parameter Set Design for HEVC 3DV [B. Choi, J. Kim, J. Park (Samsung)]

NAL unit header and VPS design are proposed for HEVC 3D/scalable extension with consideration of hybrid 3D/scalable CODEC design with non-HEVC video coding standards.

It was noticed that this proposal is almost the same as the JCTVC-J0430.

The reserved_one_5_bits in the NAL unit header of VPS is re-defined to signal the scalability_extension_type.

Type 1 is proposed for 3DV, 1-bit depth_flag and 4-bit view order index.

The motivation of signaling the scalability type only in the VPS NAL unit header was claimed as the reserved_one_5_bits is less useful in the VPS.

This proposal needs to change the base specification.

It was commented that the advantage of having view order index and depth_flag in the NAL unit header versus the generic layer_id is not clear in 3DV, but might be desirable in a harmonized design for 3DV and SVC.

One potential advantage of this proposal is that bitstream extraction can be done in a more flexible way, e.g., extracting MVC operation points by checking only the depth_flag.

The same proposal as in JCTVC-0432 is proposed for the support of multi-standard.

It was noticed that the software package supporting multi-standard stereo video hasn’t been made available yet.

A comment was made that it might be desirable for an AVC NAL decoder to filter out the HEVC NAL units by just checking the NAL unit header.

Encapsulation of heterogeneous view streams (e.g., base AVC, non-base: HEVC) sounds like something that can be taken care of by the system layer.

### Video parameter set and sequence parameter set

JCT2-A0021/m26089 3D-HEVC HLS: Inter-layer SPS Prediction [Thomas Rusert (Ericsson)]

This contribution addresses a redundancy problem that is asserted to be present when separate SPSs are used across layers in layered HEVC extensions. It is reported that re-use of SPSs across layers is not an option if certain SPS parameters are layer-specific, an example being profile_idc and level_idc, thus with the current SPS design a separate SPS has to be used if layer-specific parameters are present. Since many SPS syntax elements may have identical values across layers, those values are duplicated if separate SPSs are sent for different layers.

It is proposed for the layered HEVC extensions to introduce SPS prediction to reduce the redundancy. This implies extending the definition of SPS in the extension specifications. In the proposed SPS prediction scheme, SPS parameters are inherited at the time of SPS activation. The reference SPS for prediction is determined as the active SPS of the reference layer, which is signaled in the referencing SPS. The parameters to be inherited are indicated by flags.

reserved_zero_6bits is to be used as layer_id.

SPS is associated only to layers with the same layer_id, this may lead to cleaner bitstream extraction. So a very flexible SPS design (which doesn’t have to be aligned with the current HEVC SPS design) is possible, meaning it doesn’t have to be compatible to the SPS design in the HEVC base specification.

SPS prediction follows the same dependency as the inter-view dependency used for e.g., texture prediction.

It is mandated to have two separate SPS instances for two layers.

All syntax elements in the SPS are grouped and can be predicted. Whether a group of parameters is to be predicted or not is indicated by a flag in the sequence parameter set (present in enhancement view only).

A comment was made that this design requires activation of the SPS, however in SVC, the IDR pictures are not aligned. De-referencing of SPSs depending on the active SPS for each layer might introduce some complication during random access.

De-referencing was said to be only needed during the activation time. This might have some issue, e.g., the encoder has to be carefully designed such that even when some layers are not present, the prediction can still get the correct reference SPS. This design might complicate the decoder, which needs to track the active sequence parameter sets.

A copy of de-referenced SPS and a NAL unit of each current SPS are needed for each layer.

JCT2-A0026/m26097 3D-HLS: On Slice Header and Parameter Set [B. Choi, J. Kim, J. Park (Samsung)]

Slice header prediction and parameter set sharing are proposed between texture and depth map inside a view component. [The abstract is too short.]

Slice header prediction is proposed between texture and depth.

It was proposed that in some cases, the slice partition for texture and depth picture can be the same, meaning the same slice address for each co-located (overlapped in terms of the covered LCU locations) slice. In this case, the slice address can also be predicted.

Sharing of the parameter set e.g., PPS is proposed.

It was mentioned that SPS sharing is not possible due to the nature of the parameters in SPS. For example syntax elements such as HRD parameters are not the same for texture and depth. Based on that, it is not possible to share the PPS because PPS having the same id must refer to the same SPS.

JCT2-A0099/m26054 3D-HLS: Video parameter set for 3D-HEVC [Y.Chen, Y.-K. Wang (Qualcomm)]

In JCTVC-J0114, VPS syntax design with corresponding changes in SPS (including VUI), is proposed to enable the use of VPS in session negotiation as well as to reduce the number of bits needed for the representation of SPSs. The motivation of JCT2-A0099/JCTVC-J0124 is reportedly to enable more convenient session negotiation for various operation points in the HEVC 3DV extension and better bit efficiency for the coding of parameter sets. A VPS extension design for 3D-HEVC is proposed based on JCTVC-J0114.

One part of the proposal is that dependency between layers/views is signalled in SPS. It was commented, that each layer/view therefore has to use a separate SPS. However, it was asserted by the proponent that since the SPS is small, this design choice causes only a moderate amount of repeated data between SPSes. It was commented that an alternative could be to signal the dependency data in VPS.

It was asked why HRD is located in VPS (instead of SPS). The proponent responded that HRD typically applies over many layers/views, hence suits better VPS (as VPS may be thought to be the place to store cross-layer/view information). It was also commented that fewer SPSes could be needed if HRD parameters are stored in VPS. It was also commented that the relationship between operation points and their HRD parameters can be reportedly mapped more easily with each other if they are stored in VPS.

It was commented that VPS may contain data for layers/views that have been extracted out and are no longer in the bitstream – hence, there could be some redundant information present in VPS.

It was asked what happens if a middlebox reduces the number of layers/views in the middle of a coded video sequence. It was responded that layers/views not present SEI message would be used in such a case and VPS would be left intact.

JCT2-A0121/m26214 3D-HLS: Design of the Video Parameter Set for 3D-HEVC [Robert Skupin, Valeri George, Thomas Schierl (Fraunhofer HHI)] [late]

### Random access

JCT2-A0117/m26105 3D-HLS: On Random Access Pictures [B. Choi, J. Kim, J. Park (Samsung)]

JCT2-A0118/m26107 3D-HLS: On View Layer Switching [B. Choi, J. Kim, J. Park (Samsung)]

### General

- JCT2-A0012/m25840 Tentative working draft text for the HEVC compatible 3D video coding extension [Gerhard Tech, Philipp Merkle, Martin Winken, Heiko Schwarz, Karsten Mueller, Thomas Wiegand (Fraunhofer HHI)]

## AVC based 3DV

JCT2-A0102/m26057 3D-HLS: On reference picture list construction for AVC based 3DV [Y.-K. Wang, Y. Chen (Qualcomm)]

JCT2-A0008/m25781 3D-AVC HLS: Depth Range Parameter Set for AVC Compatible 3D Video Coding [M. Li, P. Wu (ZTE)]

It is technical the same as JCT2-A0007, but proposed to the AVC based 3DV track.

See JCT2-A0007 for detail.

## SEI messages

JCT2-A0034/m25897 3DV SEI messages on depth perception [Kwan-Jung Oh Jaejoon Lee Du Sik Park (Samsung)]

JCT2-A0054/m25931 3D-HEVC HLS: SEI message for camera parameter signaling [Peng Lu, Yin Zhao, Lu Yu (Zhejiang Univ.)]

JCT2-A0153/m26178 3D-AVC HLS: a show case of m25256 (view synthesis adaptation to display parameters) [Yin Zhao, Lu Yu (Zhejiang Univ.)]

JCT2-A0122/m26225 Maximum depth discontinuity signaling for view synthesis range determination [Andrey Norkin, Martin Pettersson, Thomas Rusert (Ericsson)] late