| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0007 MPEG number: m25780 |
| --- | --- |

| Title: | 3D-AVC HLS: Depth Range Parameter Set for MVC Compatible 3D Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ming Li Ping Wu No. 50 Ruanjiandadao Road, ZTE R&D Center District II, Nanjing, China 210012 | Tel: Email: | +86-25-88014644 li.ming42@zte.com.cn ping.wu@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

# Abstract

In this document, the concept and operations of parameter set are proposed to be applied to coding depth range parameters for MVC compatible 3D video coding. Therefore, depth range parameters can be effectively shared among different access units by slices’ referring to depth range parameter set (DRPS), which saves bits from repeatedly generating depth range update RBSPs for different access units using the same depth range parameters in decoding.

# Introduction

In MVC compatible 3D video coding, depth range parameters are conveyed in subset sequence parameter set RBSP and depth range update RBSP [1]. When an access unit contains depth range update RBSP, the depth range parameters in this depth range update RBSP are in effect in decoding the slices in the access unit. To achieve high coding efficiency, temporal compensated predictions are employed in coding depth range parameters. This is carried out with an assistance of maintaining a set of global arrays with entry index derived from frame_num to keep various depth range parameters of different time instants.

However, if a picture employs the same depth range parameters as those of the previously coded picture, the encoder has to repeat the related parameters again by inserting a new depth range update RBSP in current access unit, which wastes bits for coding duplicated information.

In addition, as the entry index for accessing the global arrays are derived from frame_num, the decoder cannot complete the whole operations specified in depth range update RBSP syntax table until it receives a slice header in this access unit and gets the value of frame_num. Therefore, there is somewhat parsing dependency between depth range update RBSP and slice header.

To tackle the above problems in the current design, the concept and operations are proposed to be applied to code depth range parameters with minimum modifications in syntax and decoding process as listed below.

- First, parameter set ID is signalled in the original depth range update RBSP and referred to by a slice, which forms a new parameter set named as depth range parameter set (DRPS).

- Second, a flag is added to slice header to indicate whether the depth range parameter for decoding the current slice are obtained from SPS extension or from a DRPS via the following signalled DRPS ID.

- Third, DRPS ID is used as entry index for the global arrays for temporal predictions among the depth range parameters in different DRPSs, where the same prediction algorithms are used.

- Forth, the initial depth range parameters in SPS extensions are explicitly stored in the first element of every global array. That is, such parameters can be accessed using an entry index of “0” instead of an ambiguous value of “SPS”. In this way, these initial values can be clearly employed as references in temporal predictive coding of depth range parameters.

- Fifth, in the decoding process, variables are calculated with the input values from global arrays indexed by DRPS ID. And then the values of these variables are assigned to the arrays used in reconstruction process with its entry index of [frame_num, i], which minimized the changes in decoding process.

The modified syntax and semantics are presented in section 2, and decoding process, including the activation process of DRPS, in section 3.

# Modified syntax and semantics

For nal_unit_type:

| 16 | Depth range parameter set depth_range_parameter_set_rbsp( ) | 11 | non-VCL | non-VCL | non-VCL |
| --- | --- | --- | --- | --- | --- |

Modified slice header

In 7.3.3,Slice header syntax, replace the syntax table with:

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_mb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| if( separate_colour_plane_flag = = 1 ) |  |  |
| colour_plane_id | 2 | u(2) |
| frame_num | 2 | u(v) |
| …… |  |  |
| if ( nal_unit_type = = 21 && ( slice_type != I && slice_type != SI )) { |  |  |
| depth_weighted_pred_flag | 2 | u(1) |
| if ( (3dv_acquisition_idc != 1) && (depth_weighted_pred_flag = = 1) ) |  |  |
| inherit_depth_range_params_from_sps_flag | 2 | u(1) |
| if (inherit_depth_range_params_from_sps_flag = = 0 ) |  |  |
| depth_range_parameter_set_id | 2 | ue(v) |
| } |  |  |
| } |  |  |
| } |  |  |

Semantics:

inherit_depth_range_params_from_sps_flag equal to 1 specifies that depth range parameters from the seq_parameter_set_3dvc_extension( ) in the sequence parameter set RBSP shall be used for decoding the current slice.  Otherwise, inherit_depth_range_params_from_sps_flag equal to 0 specifies that depth range parameters from the seq_parameter_set_3dvc_extension( ) in the sequence parameter set RBSP shall not be used for decoding the current slice. When inherit_depth_range_params_from_sps_flag is not present, its value is inferred to be equal to 1.

depth_range_parameter_set_id specifies the depth range parameter set in use. The value of depth_range_parameter_set_id shall be in the range of 1 to TBD, inclusive.

Modified sequence parameter set 3DVC extension syntax

| seq_parameter_set_3dvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| depth_range_present_flag | 0 | u(1) |
| if( depth_range_present_flag ) { |  |  |
| 3dv_acquisition_idc | 0 | ue(v) |
| if( ( 3dv_acquisition_idc ) && ( num_views_minus1 > 0 ) ) |  |  |
| for( i = 0; i < num_views_minus1; i++ ) |  |  |
| view_id_3dv[ i ] | 0 | ue(v) |
| if( 3dv_acquisition_idc = = 1 ¦ ¦ 3dv_acquisition_idc = = 2 ) |  |  |
| depth_ranges( 2, SPS ) |  |  |
| if( ( 3dv_acquisition_idc ) && ( num_views_minus1 > 0 ) ) |  |  |
| depth_ranges( 2, 0 ) |  |  |
| } |  |  |
| } |  |  |

Modified semantics:

3dv_acquisition_idc equal to 0 indicates that no 3DV acquisition parameters (the closest and farthest depth value) are present in the sequence parameter set and at least one 3DV parameter update NAL unit shall be present in each access unit. 3dv_acquisition_idc equal to 1 indicates that 3DV acquisition parameters are present in the sequence parameter set and valid for the entire coded video sequence. 3dv_acquisition_idc equal to 2 indicates that 3DV acquisition parameters are present in the sequence parameter set and depth range parameter update NAL units may be present. 3dv_acquisition_idc values greater than 2 are reserved.

Proposed depth range parameter set RBSP syntax

| depth_range_update_rbsp( ) { depth_range_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| depth_range_parameter_set_id | 11 | ue(v) |
| pred_direction | 11 | ue(v) |
| if( pred_direction = = 0 ¦ ¦ pred_direction = = 1 ) { |  |  |
| frame_num_diff0_minus1 | 11 | ue(v) |
| frameNum0 = frame_num – 1 – frame_num_diff0_minus1 |  |  |
| if( frameNum0 < 0 ) frameNum0 += MaxFrameNum |  |  |
| ref_element_id0 | 11 | ue(v) |
| predWeight0 = 64 |  |  |
| } |  |  |
| if( pred_direction = = 0 ) { |  |  |
| frame_num_diff1_minus1 | 11 | ue(v) |
| ref_element_id1 | 11 | ue(v) |
| pred_weight0 | 11 | u(6) |
| frameNum1 = frame_num – 1 – frame_num_diff1_minus1 |  |  |
| if( frameNum1 < 0 ) frameNum1 += MaxFrameNum |  |  |
| predWeight0 = pred_weight0 |  |  |
| } |  |  |
| num_views_minus1 | 11 | ue(v) |
| depth_ranges( pred_direction, frame_num ) |  |  |
| depth_ranges( pred_direction, depth_range_parameter_set_id ) |  |  |
| depth_param_additional_extension_flag | 11 | u(1) |
| if(depth_param_additional_extension_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| depth_param_additional_extension_data_flag | 11 | u(1) |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

Modified semantics:

depth_range_parameter_set_id identifies the depth range parameter set that is referred to in the slice header. The value of depth_range_parameter_set_id shall be in the range of 1 to TBD, inclusive.

ref_element_id0 specifies a first array element index value to be used in prediction of 3DV acquisition variables. The value of ref_element_id0 shall be in the range of 0 to TBD, inclusive.

ref_element_id1 specifies a second array element index value to be used in prediction of 3DV acquisition variables. The value of ref_element_id1 shall be in the range of 0 to TBD, inclusive.

Modified depth ranges syntax

| depth_ranges( predDirection, frameIdc ) { depth_ranges( predDirection, currArrayIndex) { | C | Descriptor |
| --- | --- | --- |
| z_near_flag | 11 | u(1) |
| z_far_flag | 11 | u(1) |
| if( z_near_flag ) |  |  |
| 3dv_acquisition_element( 0, predDirection, 7, 0, ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |  |
| if( z_far_flag ) |  |  |
| 3dv_acquisition_element( 0, predDirection, 7, 0, ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |  |
| } |  |  |

Modified semantics:

I.7.4.2.13.1	Depth ranges semantics

The contents of the syntax structure are controlled through input variables predDirection and frameIdc currArrayIndex the semantics of which are as follows.

–	predDirection equal to 2 specifies that the first loop entry of the element is not predicted and coded in the sign, exponent, and mantissa syntax elements. predDirection equal to 0 or 1 specifies that the first loop entry of the element is predicted and a difference relative to a prediction value is coded in the difference syntax element.

–	frameIdc may be equal to “SPS” indicating a sequence parameter set, or it may be a frame_num value for short-term reference pictures and non-reference pictures. [Ed. frameIdc could also be a long-term picture index for long-term reference pictures.]

–	currArrayIndex equal to 0 is only used in decoding depth ranges in sequence parameter set. Otherwise, currArrayIndex could be a depth_range_parameter_set_id value and used in decoding depth ranges in depth range parameter set.

I.7.4.2.13.2	3DV acquisition element semantics

The syntax structure specifies the value of an element in 3DV acquisition information. The element may contain one or more loop entries i of the order specified by view_id_3dv syntax elements.

The contents of the syntax structure are controlled through input variables predDirection, precMode, and expLen the semantics of which are as follows.

–	deltaFlag equal to 0 specifies that the each loop entry corresponds to a value specific to the view indicated by i. deltaFlag equal to 1 specifies that each loop entry corresponds to a difference of values between the views indicated by i and i + 1.

–	predDirection equal to 2 specifies that the first loop entry of the element is not predicted and coded in the sign, exponent, and mantissa syntax elements. predDirection equal to 0 or 1 specifies that the first loop entry of the element is predicted and a difference relative to a prediction value is coded in the difference syntax element.

–	precMode equal to 0 specifies that the number of bits in the mantissa syntax element.

–	expLen specifies the number of bits in the exponent syntax element.

The syntax structure uses OutSign, OutExp, and OutMantissa variables for both input and output, where each variable is indexed by [ frameIdc, viewIdc ], frameIdc being a frame indicator (either “SPS” or frame_num value) [ currArrayIndex, viewIdc ], currArrayIndex being an indicator (either 0 when decoding depth ranges in sequence parameter set or depth_range_parameter_set_id value when decoding depth range parameter set) and viewIdc being a view indicator (in the order of views for 3DV acquisition parameters).

Modified 3DV acquisition element syntax

| 3dv_acquisition_element( deltaFlag, predDirection, precMode, expLen, OutSign, OutExp, OutMantissa, OutManLen ) { | C | Descriptor |
| --- | --- | --- |
| if( num_views_minus1 + 1 – deltaFlag > 1 ) |  |  |
| element_equal_flag | 11 | u(1) |
| if( element_equal_flag = = 0 ) |  |  |
| numValues = num_views_minus1 + 1 – deltaFlag |  |  |
| else |  |  |
| numValues = 1 |  |  |
| for( i = 0; i < numValues; i++ ) { |  |  |
| if( predDirection = = 2 && i = = 0 ) { |  |  |
| if( precMode = = 0 ) { |  |  |
| matissa_len_minus1 | 11 | u(5) |
| OutManLen[ frameIdc, i ] = manLen = mantissa_len_minus + 1 |  |  |
| OutManLen[ currArrayIndex, i ] = manLen = mantissa_len_minus + 1 |  |  |
| } else |  |  |
| prec | 11 | u(5) |
| } |  |  |
| if( predDirection = = 2 ) { |  |  |
| sign0 | 11 | u(1) |
| OutSign[ frameIdc, i ] = sign0 |  |  |
| OutSign[currArrayIndex, i ] = sign0 |  |  |
| exponent0 | 11 | u(expLen) |
| OutExp[ frameIdc, i ] = exponent0 |  |  |
| OutExp[ currArrayIndex, i ] = exponent0 |  |  |
| if( precMode = = 1 ) { |  |  |
| if( exponent0 = = 0 ) |  |  |
| OutManLen[ frameIdc, i ] = manLen = Max( 0, prec – 30 ) |  |  |
| OutManLen[ currArrayIndex, i ] = manLen = Max( 0, prec – 30 ) |  |  |
| else |  |  |
| OutManLen[ frameIdc, i ] = manLen = Max( 0, exponent0 + prec – 31 ) |  |  |
| OutManLen[ currArrayIndex, i ] = manLen = Max( 0, exponent0 + prec – 31 ) |  |  |
| } |  |  |
| mantissa0 | 11 | u(manLen) |
| OutMantissa[ frameIdc, i ] = mantissa0 |  |  |
| OutMantissa[ currArrayIndex, i ] = mantissa0 |  |  |
| } else { |  |  |
| skip_flag | 11 | u(1) |
| if( skip_flag = = 0 ) { |  |  |
| sign1 | 11 | u(1) |
| OutSign[ frameIdc, i ] = sign1 |  |  |
| OutSign[ currArrayIndex, i ] = sign1 |  |  |
| exponent_skip_flag | 11 | u(1) |
| if( exponent_skip_flag = = 0 ) { |  |  |
| exponent1 | 11 | u(expLen) |
| OutExp[ frameIdc, i ] = exponent1 |  |  |
| OutExp[ currArrayIndex, i ] = exponent1 |  |  |
| } else |  |  |
| OutExp[ frameIdc, i ] = OutExp[ frameNum0, i ] |  |  |
| OutExp[ currArrayIndex, i ] = OutExp[ ref_element_id0, i ] |  |  |
| mantissa_diff | 11 | se(v) |
| mantissaPred = (( OutMantissa[ frameNum0, i ] * predWeight0 + OutMantissa[ frameNum1, i ] * ( 64-predWeight0 ) + 32 ) >> 6 ) |  |  |
| OutMantissa[ frameIdc, i ] = mantissaPred + mantissa_diff |  |  |
| OutManLen[ frameIdc, i ] = OutManLen[ frameNum0, i ] |  |  |
| mantissaPred = (( OutMantissa[ ref_element_id0, i ] * predWeight0 + OutMantissa[ ref_element_id1, i ] * ( 64-predWeight0 ) + 32 ) >> 6 ) |  |  |
| OutMantissa[ currArrayIndex, i ] = mantissaPred + mantissa_diff |  |  |
| OutManLen[ currArrayIndex, i ] = OutManLen[ ref_element_id0, i ] |  |  |
| } else { |  |  |
| OutSign[ frameIdc, i ] = OutSign[ frameNum0, i ] |  |  |
| OutExp[ frameIdc, i ] = OutExp[ frameNum0, i ] |  |  |
| OutMantissa[ frameIdc, i ] = OutMantissa[ frameNum0, i ] |  |  |
| OutManLen[ frameIdc, i ] = OutManLen[ frameNum0, i ] |  |  |
| OutSign[ currArrayIndex, i ] = OutSign[ ref_element_id0, i ] |  |  |
| OutExp[ currArrayIndex, i ] = OutExp[ ref_element_id0, i ] |  |  |
| OutMantissa[ currArrayIndex, i ] = OutMantissa[ ref_element_id0, i ] |  |  |
| OutManLen[ currArrayIndex, i ] = OutManLen[ ref_element_id0, i ] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( element_equal_flag = = 1 ) { |  |  |
| for( i = 1; i < num_views_minus1 + 1 – deltaFlag; i++ ) { |  |  |
| OutSign[ frameIdc, i ] = OutSign[ frameIdc, 0 ] |  |  |
| OutExp[ frameIdc, i ] = OutExp[ frameIdc, 0 ] |  |  |
| OutMantissa[ frameIdc, i ] = OutMantissa[ frameIdc, 0 ] |  |  |
| OutManLen[ frameIdc, i ] = OutManLen[ frameIdc, 0 ] |  |  |
| OutSign[ currArrayIndex, i ] = OutSign[ currArrayIndex, 0 ] |  |  |
| OutExp[ currArrayIndex, i ] = OutExp[ currArrayIndex, 0 ] |  |  |
| OutMantissa[ currArrayIndex, i ] = OutMantissa[ currArrayIndex, 0 ] |  |  |
| OutManLen[ currArrayIndex, i ] = OutManLen[ currArrayIndex, 0 ] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

# Modified decoding process

- I.8.6  Decoding process for depth range parameters

This process is invoked for decoding of the depth range parameters.

When inherit_depth_range_params_from_sps_flag is equal to 1, inputs of this process are the variables obtained from parsing the active sequence parameter set RBSP:

- the variables relative to the closest depth: ZNearSign[ 0, i ], ZNearExp[ 0, i ], ZNearMantissa[ 0, i ], ZNearManLen[ 0, i ];

- the variables relative to the farthest depth: ZFarSign[ 0, i ], ZFarExp[ 0, i ], ZNearMantissa[ 0, i ], ZNearManLen[ 0, i ].

where i is the index to the order of views for 3DV acquisition parameters.

Otherwise, when inherit_depth_range_params_from_sps_flag is equal to 0, inputs of this process are the variables obtained from parsing the active depth range parameter set RBSP:

- the variables relative to the closest depth: ZNearSign[ depth_range_parameter_set_id, i ], ZNearExp[ depth_range_parameter_set_id, i ], ZNearMantissa[ depth_range_parameter_set_id, i ], ZNearManLen[ depth_range_parameter_set_id, i ];

- the variables relative to the farthest depth: ZFarSign[ depth_range_parameter_set_id, i ], ZFarExp[ depth_range_parameter_set_id, i ], ZNearMantissa[ depth_range_parameter_set_id, i ], ZNearManLen[ depth_range_parameter_set_id, i ].

where i is the index to the order of views for 3DV acquisition parameters.

Outputs of this process are

–	the closest depth value ZNear[ frame_num, i ];

–	the farthest depth value ZFar[ frame_num, i ].

where i is the index to the order of views for 3DV acquisition parameters.

The output variables x in Table I-X are derived as follows from the respective variables f, s, e, n, and v indicated in Table I-X.

–	If f is equal to 0 in depth range parameter set RBSP, f in the active sequence parameter set 3DVC extension shall be equal to 1.

- The variables s, e, n, and v are for [arrayElementIndex, i ], where i is index to the order of views for 3DV acquisition parameters, and arrayElementIndex is equal to 0 when inherit_depth_range_params_from_sps_flag is equal to 1 and depth_range_parameter_set_id when inherit_depth_range_params_from_sps_flag is equal to 0.

–	The variable x computed as follows for [ frame_num, i ] where i is index to the order of views for 3DV acquisition parameters:

–	If 0 < e < 127, x = (−1)s * 2e−31 * (1 + n ÷ 2v).

–	Otherwise (e is equal to 0), x = (−1)s * 2−(30+v) * n.

NOTE – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

- Table I-X – Association between depth parameter variables and syntax elements.

| x | f | s | e | n | v |
| --- | --- | --- | --- | --- | --- |
| ZNear | z_near_flag | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | z_far_flag | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |

- I.7.4.1.2.1  Order of 3DVC sequence parameter set RBSPs and picture parameter set RBSPs and their activation

- NOTE 1 – The sequence, picture and depth range parameter set mechanism decouples the transmission of infrequently changing information from the transmission of coded macroblock data. Sequence, picture and depth range parameter sets may, in some applications, be conveyed "out-of-band" using a reliable transport mechanism.

- A depth range parameter set RBSP includes parameters that can be referred to by the coded slice NAL units of one or more depth view components of one or more coded pictures.

- Each depth range parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one depth range parameter set RBSP is considered as the active depth range parameter set RBSP at any given moment during the operation of the decoding process, and when any particular depth range parameter set RBSP becomes the active depth range parameter set RBSP, the previously-active depth range parameter set RBSP (if any) is deactivated.

- In addition to the active depth range parameter set RBSP, zero or more depth range parameter set RBSPs may be specifically active for depth view components (with a particular value of VOIdx less than VOIdxMax) that belong to the target output views or that may be referred to through inter-view prediction in decoding depth view components belonging to the target output views. Such a depth range parameter set RBSP is referred to as the active depth view component depth range parameter set RBSP for the particular value of VOIdx. The restrictions on active depth range parameter set RBSPs also apply to active depth view component depth range parameter set RBSPs for a particular value of VOIdx less than VOIdxMax.

- When a depth range parameter set RBSP (with a particular value of depth_range_parameter_set_id) is not the active depth range parameter set RBSP and it is referred to by a coded slice NAL unit belonging to a depth view component (i.e., with nal_unit_type equal to 21) and with VOIdx equal to VOIdxMax (using that value of depth_range_parameter_set_id), it is activated. This depth range parameter set RBSP is called the active depth range parameter set RBSP until it is deactivated when another depth range parameter set RBSP becomes the active depth range parameter set RBSP. A depth range parameter set RBSP, with that particular value of depth_range_parameter_set_id, shall be available to the decoding process prior to its activation.

- When a depth range parameter set RBSP (with a particular value of depth_range_parameter_set_id) is not the active depth view component depth range parameter set for a particular value of VOIdx less than VOIdxMax and it is referred to by a coded slice NAL unit belonging to a depth view component (i.e., with nal_unit_type equal to 21) and with the particular value of VOIdx (using that value of depth_range_parameter_set_id), it is activated for depth view components with the particular value of VOIdx. This depth range parameter set RBSP is called the active depth view component depth range parameter set RBSP for the particular value of VOIdx until it is deactivated when another depth range parameter set RBSP becomes the active depth view component depth range parameter set RBSP for the particular value of VOIdx. A depth range parameter set RBSP, with that particular value of depth_range_parameter_set_id, shall be available to the decoding process prior to its activation.

- Any depth range parameter set NAL unit containing the value of depth_range_parameter_set_id for the active depth range parameter set RBSP for a coded picture shall have the same content as that of the active depth range parameter set RBSP for this coded picture unless it follows the last VCL NAL unit of this coded picture and precedes the first VCL NAL unit of another coded picture. Any depth range parameter set NAL unit containing the value of depth_range_parameter_set_id for the active depth view component depth range parameter set RBSP for a particular value of VOIdx less than VOIdxMax for a coded picture shall have the same content as that of the active depth view component depth range parameter set RBSP for the particular value of VOIdx for this coded picture unless it follows the last VCL NAL unit of this coded picture and precedes the first VCL NAL unit of another coded picture.

- A picture parameter set RBSP includes parameters that can be referred to by the coded slice NAL units of one or more texture view or depth view components of one or more coded pictures.

……

- A 3DVC sequence parameter set RBSP includes parameters that can be referred to by one or more picture parameter set RBSPs or one or more buffering period SEI messages.

……

For view components with VOIdx equal to VOIdxMax, all constraints that are expressed on the relationship among the values of the syntax elements (and the values of variables derived from those syntax elements) in 3DVC sequence parameter sets, picture parameter sets and depth range parameter sets and other syntax elements are expressions of constraints that apply only to the active 3DVC sequence parameter set, the active picture parameter set and the active depth range parameter set. For view components with a particular value of VOIdx less than VOIdxMax, all constraints that are expressed on the relationship among the values of the syntax elements (and the values of variables derived from those syntax elements) in 3DVC sequence parameter sets, picture parameter sets and depth range parameter sets and other syntax elements are expressions of constraints that apply only to the active view 3DVC sequence parameter set, the active view picture parameter set and the active depth range parameter set for the particular value of VOIdx. If any 3DVC sequence parameter set RBSP having profile_idc equal to the value of one of the profile_idc values specified in Annex Error: Reference source not found or Annex Error: Reference source not found or Annex I is present that is never activated in the bitstream (i.e., it never becomes the active 3DVC sequence parameter set or an active view 3DVC sequence parameter set), its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream. If any picture parameter set RBSP is present that is never activated in the bitstream (i.e., it never becomes the active picture parameter set or an active view picture parameter set), its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream. If any depth range parameter set RBSP is present that is never activated in the bitstream (i.e., it never becomes the active depth range parameter set or an active view depth range parameter set), its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream.

During operation of the decoding process (see subclause I.8), for view components with VOIdx equal to VOIdxMax, the values of parameters of the active depth range parameter set, the active picture parameter set and the active 3DVC sequence parameter set shall be considered in effect. For view components with a particular value of VOIdx less than VOIdxMax, the values of the parameters of the active view depth range parameter set, the active view picture parameter set and the active view 3DVC sequence parameter set for the particular value of VOIdx shall be considered in effect. For interpretation of SEI messages that apply to the entire access unit or the view component with VOIdx equal to VOIdxMax, the values of the parameters of the active depth range parameter set, the active picture parameter set and the active 3DVC sequence parameter set for the same access unit shall be considered in effect unless otherwise specified in the SEI message semantics. For interpretation of SEI messages that apply to view components with a particular value of VOIdx less than VOIdxMax, the values of the parameters of the active view depth range parameter set, the active view picture parameter set and the active view 3DVC sequence parameter set for the particular value of VOIdx for the same access unit shall be considered in effect unless otherwise specified in the SEI message semantics.

- For any active MVC sequence parameter set or active view MVC sequence parameter set, part of the syntax elements in the MVC sequence parameter set extension applies only to the depth views referring to this sequence parameter set, while the some other parts of the syntax elements in the MVC sequence parameter set extension collectively apply to both the depth views referring to this sequence parameter set and the corresponding texture views. More specifically, the view dependency information of the MVC sequence parameter set extension applies only to the depth views, and the level definitions collectively apply to operation points, each of which contains both depth views and their corresponding texture views.

# Conclusions

The concept and operations of parameter set are proposed to be applied to coding depth range parameters. The proposed scheme eliminates potential problems of coding duplicated information and parsing dependency in the original design of depth range update RBSP. The recommendation is to adopt this scheme to MVC extensions for inclusion of depth maps for H.264/AVC.

# References

- Teruhiko Suzuki, Miska M. Hannuksela, Ying Chen, Shinobu Hattori, “Text of ISO/IEC 14496-10:2012/PDAM 2 MVC extensions for inclusion of depth maps”, MPEG Document N12731, Geneva, Switzerland, May 2012.

# Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).