| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0013 |
| --- | --- |

| Title: | 3D-CE5.h related: Simplification of depth-based inter-view prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tadashi Uchiumi Yoshiya Yamamoto Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 uchiumi.tadashi@sharp.co.jp yamamoto.yoshiya@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution presents a coding tool of depth-based inter-view prediction (CE5 related) for HTM and its simulation result. This proposed technique is for PDM-based approach with disparity vector generation. The proposal decreases number of samples needed for deciding disparity vector from a PDM block. The simulation result reports there is no coding loss in average comparing to the anchor HTM3.1.

# Introduction

This proposal is for simplifying the disparity vector generation method using predicted depth map (PDM) introduced in the current HTM.

# Problem Statement

In the current HTM, inter-view motion parameter prediction tool can use predicted depth map (PDM) for disparity vector generation. As the current method for generating a disparity vector, max disparity is calculated in each prediction unit using whole pixels in corresponding PDM block. Complexity of the calculation is rather high especially for large prediction unit.

# Proposed Method

For decreasing the complexity, the proposal introduces sub-sampling method for calculating a max disparity from PDM. That is, the max disparity is calculated from limited pixels in a PDM block instead of all pixels in it.

Figure 1 (b) depicts an example of this method. The method uses only 4 pixels in a PDM block for calculating the max disparity, which the pixels are on the corner of the PDM block, while the anchor method (a) uses all pixels in a PDM block for the calculation. The figure 1 shows an example of 8x8 pixels in a PDM block. The proposed method can decrease the computing steps to 1/16 comparing to the anchor method with that size of PDM, and can eliminate loop-operation required for the anchor method.

In the proposal, the calculation is always performed using 4 pixels in a PDM block even though the PDM block size is smaller than 2x2, in order to avoid an extra judgment. It means that the operation may refer one pixel for two times when a PDM block size is 2x1 or 1x2.

The max disparity derivation process in the proposal is as follows.

maxDepth = predDepthMap [x0][y0]
   if (maxDepth < predDepthMap [x0+w-1][y0] ) maxDepth = predDepthMap [x0+w-1][y0]
   if (maxDepth < predDepthMap [x0][y0+h-1] ) maxDepth = predDepthMap [x0][y0+h-1]
   if (maxDepth < predDepthMap [x0+w-1][y0+h-1] ) maxDepth = predDepthMap [x0+w-1][y0+h-1]

where (x0, y0) specify the top-left sample of the PDM block corresponding to the current prediction unit of texture, and w, h specify width and height of the PDM block.

(a) Anchor method

(b) Proposed method

A block of PDM

corresponding to a target texture block

All pixels are used

for the calculation

Four pixels on the corner are used

for the calculation

Pixels not used for the calculation

Figure 1: Pixels in PDM used for calculating max disparity

# Simulation Results

## Compression Performance

The proposed method was implemented and evaluated on its coding performance in comparison with the anchor HTM3.1. The Simulation was performed based on the common test conditions. The result is shown in Table 1.

As shown in Table 1, the overall average bitrate is the same as the anchor.

Table 1: Rate-distortion results on 3-view coding with depth

| Sequences | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.2% | 0.1% | 0.0% | -0.1% | -0.1% |
| Kendo | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Newspapercc | 0.0% | -0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
| GhostTownFly | 0.0% | -0.3% | -0.2% | -0.1% | -0.1% | -0.1% |
| PoznanHall2 | 0.0% | -0.4% | -0.2% | -0.1% | 0.0% | 0.0% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 1024x768 | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| 1920x1088 | 0.0% | -0.2% | -0.1% | -0.1% | 0.0% | 0.0% |
| average | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% |

## Complexity Analysis

Table 2 shows the ratios of encoding, decoding and rendering time of the proposal in comparison with the anchor. The results indicate the running times are almost same with the anchor.

Table 2: Running time ratio

| Sequences | enc time | dec time | ren time |
| --- | --- | --- | --- |
| Balloons | 100.5% | 99.5% | 100.3% |
| Kendo | 101.1% | 97.0% | 100.6% |
| Newspapercc | 101.4% | 98.2% | 100.8% |
| GhostTownFly | 99.3% | 99.9% | 98.4% |
| PoznanHall2 | 98.4% | 98.1% | 101.0% |
| PoznanStreet | 100.0% | 100.0% | 101.5% |
| UndoDancer | 100.0% | 100.0% | 100.3% |
| 1024x768 | 101.0% | 98.2% | 100.6% |
| 1920x1088 | 99.4% | 99.5% | 100.3% |
| average | 100.1% | 98.9% | 100.4% |

# Conclusion

A simplification method for motion/mode parameter prediction using predicted depth map (PDM) as described in this document is proposed. The proposal can reduce encoding and decoding complexity without any coding loss in comparison with the anchor. In addition, WD description can be more clarified due to loop operations for deciding the max disparity value has been eliminated for each PU. The proposed method should be considered to be adopted in the new HTM for further evaluation.

# References

[1] ISO/IEC JTC1/SC29/WG11, “Test Model under Consideration for HEVC based 3D video coding v3.0”, Doc. N12744, Geneva, CH, May 2012.

[2] ISO/IEC JTC1/SC29/WG11, “Description of Core Experiments in 3D Video Coding - CE5: motion/mode parameter prediction”, Doc. N12746, Geneva, CH, May 2012.

[3] ISO/IEC JTC1/SC29/WG11, “Common Test Conditions for 3DV experimentation”, Doc. N12745, Geneva, Switzerland, May 2012.

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).