| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0014 |
| --- | --- |

| Title: | 3D-CE5.h related: Simplification of AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT2 |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshiya Yamamoto Tomohiro Ikai Tadashi Uchiumi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 yamamoto.yoshiya@sharp.co.jp ikai.tomohiro@sharp.co.jp uchiumi.tadashi@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution presents a simplified motion vector candidate construction for AMVP. The proposed method adds zero vector to the candidate list when the inter-view candidate (derived by Inter-view motion prediction) is not available and changes the pruning combination to avoid the unnecessary candidate motion vector derivation process. It is reported that average BD-rate of the proposal is not changed from that of the anchor (HTM3.1).

# Background

- In the current HTM, motion vector candidate construction process for non-base view is performed. In the process, inter-view candidate derivation is needed due to having pruning process between inter-view candidate and other candidate.

- However, the inter-view candidate may not be available according to situations. In such a case, a couple of derivation might be needed. Then the inter-view candidate derivation process is unnecessary and brings extra burden into motion vector candidate construction process.

# Proposed Method

To simplify the motion vector candidate construction, we propose the following changes of Inter-view motion prediction and pruning.

The proposed method consists of the following elements

- Zero vector is used as the inter-view candidate, when the normal inter-view candidate is not available.

  - Other candidate derivations are not needed in the case of mvp_lX_idx == 0 because the inter-view candidate is always available.

- The pruning combination is always the spatial candidate motion vectors.

  - The unique check of the inter-view candidate is not needed.

Unnecessary candidate motion vector derivation process could be avoided with our proposal. For example, if mvp_lX_idx > 0 (the inter-view candidate is not used), Inter-view motion candidate derivation could be skipped. Moreover, the proposed derivation process except Inter-view motion prediction is exactly the same process in HEVC. Therefore WD of 3D video coding extension will be changed very limitedly and could be kept simple by adopting this proposed scheme.

# Performance Discussion

The proposed method is implemented on 3DV-HTM3.1 and simulation is conducted under the common test conditions [2]. The result is shown in Table 1.

Table 1: Rate-distortion results on 3-view coding with depth

| Sequences | video 0 | video 1 | video 2 | video only | synthesized only |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |
| Kendo | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% |
| Newspapercc | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% |
| GhostTownFly | 0.0% | -0.1% | 0.2% | 0.0% | 0.0% |
| PoznanHall2 | 0.0% | -0.2% | -0.5% | -0.1% | 0.0% |
| PoznanStreet | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| UndoDancer | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% |
| 1920x1088 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| average | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |

# Complexity Assessment

The Encoding and Decoding time are summarized in Table 2.

- The platform is Linux with 24GB RAM.

Table 2: Running time ratio

| Sequences | Encoding time | Decoding time |
| --- | --- | --- |
| Balloons | 99.9% | 100.2% |
| Kendo | 100.4% | 99.6% |
| Newspapercc | 100.9% | 100.5% |
| GhostTownFly | 99.9% | 100.3% |
| PoznanHall2 | 100.0% | 99.4% |
| PoznanStreet | 100.8% | 99.5% |
| UndoDancer | 99.5% | 99.4% |
| 1024x768 | 100.4% | 100.1% |
| 1920x1088 | 100.0% | 99.7% |
| average | 100.2% | 99.9% |

# Conclusions and Remarks

We have proposed a coding technology for 3D Video including depth map as described in this contribution. The proposal simplified the motion vector candidate construction at decoder side. It is reported that the BD-rate for texture and depth case on common test condition is not changed from that of the anchor. We recommend adopting the proposed scheme in the test model for further evaluation.

# References

- ISO/IEC JTC1/SC29/WG11 “Description of 3D Video Technology Proposal by Fraunhofer HHI,” M22571, Geneva, Switzerland, Nov. 2011.

- ISO/IEC JTC1/SC29/WG11, “Common Test Conditions for 3DV experimentation”, N12745, Geneva, Switzerland, May 2012.

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).