| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0015 |
| --- | --- |

| Title: | 3D-CE1.a results on inter-view coding with adaptive luminance compensation by Samsung Cross-check: by NTT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Fartukov I. Kovliga M. Mishurovskiy K. Dubkov Jaejoon Lee | Tel: Email: | +7-495-797-25-00 A.fartukov@samsung.com; i.kovliga@samsung.com; mike.mishky@samsung.com; k.dubkov@samsung.com; jaejoon1.lee@samsung.com |
| Source: | Samsung Moscow Research Center & Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Technology disclosed herein is devoted to an interview coding with adaptive luminance compensation approach. Starting from 100 MPEG meeting, this technology has being studied and optimized to fulfil major requirements claimed to the tools being considered in the scope of 3D video coding standardization. Initial document describing this technology is referred by M24816 document reference number and is available on the MPEG web-site.

It’s well known that natural multi-view sequences and even some synthetic multi-view sequences are featured by inter-view mismatches; in particular - luminance discrepancy between object projections captured by adjacent cameras of 3D capturing system even if they are matched geometrically perfectly.

Proposed technology deals with local estimation of probable luminance discrepancy existing between at least two object projections, captured by multiview system; this estimation is based on an analysis of pixel values of a reference block, so-called template area nearby reference block and finally corresponding template area nearby encoded block. For each unique pixel-value within a reference block an estimation of most probable correction is derived; pixels of a reference block are corrected based on estimations completed before. This method is realized as an auxiliary mode for all partitions starting from P16x16 through to sub-partitions 4x4 for texture P-slices only and successfully works with interview and view synthesis references. This auxiliary mode is signaled by an additional 1 bit-flag right after mb_type at the macroblock level. It was determined that applying of proposed adaptive luminance compensation with an interview reference is likely to be redundant as it does not result in further grow of compression efficiency; that is why finally adaptive luminance compensation is implemented in the view synthesis mode only;

Reported technology is integrated in the 3D-ATM 0.4, measurements were done in accordance with recent CTC (average values): total compression gain (coded PSNR) -1.03% DB-Rate, total compression gain (Synthetic PSNR) -0.59% DB-Rate, encoded time: 102.98%, decoded time: 103.43%. Based on these results it is recommended to adopt this technology into the next version of 3D-ATM.

# Introduction

It’s well known that the majority of natural multi-view sequences and even some of synthesized multi-view sequences usually demonstrate inter-view mismatches in particular, local luminance discrepancy between object projections captured by adjacent cameras of 3D capturing system even if they are matched geometrically perfectly. Importance for compression can be explained easily by inevitable increasing of residuals signal right after classical prediction stage if an inter-view luminance mismatch exists. Classical approach assumes that increased residuals have to be encoded explicitly in the bitstream, decreasing overall compression efficiency.

To cope with this and similar to this issues, several techniques have been proposed by now. Firstly, AVC standard [1] includes weighted prediction that was originally proposed to compensate fade-up/out, flickering or scene-change effects in case of video sequence coding. Weighted prediction technique allows suppressing illumination changes between coding and reference frames. It is applied to the motion compensated prediction at the macroblock level. Weighting factors are the same for all macroblocks of a particular slice, thus they can be considered as global. Weighting factors are determined and stored in a bitstream («explicit» weighted prediction) or calculated during decoding («implicit» weighted prediction). Nevertheless in case of multi-view video sequences there are local illumination and contrast changes between adjacent views which make this technique less effective.

Another well-known approach is an adaptive block-wise illumination compensation technique [2] and modification of this technique “Multi- view One-Step Affine Illumination Compensation” (MOSAIC) [3, 4]. This method combines block-wise illumination compensation and inter prediction techniques. It includes explicit encoding of motion (disparity) vector and some extra parameters that are used in the compensation model; also this method sometimes provides a mechanism for parameters inferring that eliminates necessity to encode parameters explicitly.

Also, it is worth mentioning another realization of similar techniques that is disclosed in the patent application US 2011/0286678. This method of multi-view coding provides an estimation of illumination compensation parameters based on adjacent areas of currently encoded and reference blocks. Estimated parameters are applied to illumination compensation of the reference block. Method also includes reliability estimation stage for adaptive application of luminance compensation procedure. Similar solution is described in the patent application US 2008/0304760. Due-to correction of mean value only and low adaptation of proposed technology for local structure of a frame being encoded efficiency could be reduced.

# Description

## Proposed compensation model of local luminance mismatch

To the beginning, let consider several adjacent frames belonged to a multiview sequence that are captured at the certain moment T. Let consider inter-view encoding for hybrid video compression that includes synthesized frames as additional reference frames (figure 1).

View 0

View 1

View 2

T

Synthesized View 0

View 1

Synthesized View 2

View 2

Capturing Time

Backward View Synthesis

Decoded frame

Camera positions

Forward View Synthesis

Depth 1

Fig. 1 P-I-P coding configuration for 3-view hybrid inter-view coding.

If we consider predicative coding of a certain block of the pixels belonged to the encoded frame, we can identify easily main entities that are considered in respect of encoding / decoding process given in this proposal – figure 2.

Encoded Frame

Reference Frame

X

R

DA

DB

RA

RB

DV: Displacement Vector

Template Width

Fig. 2 Main entities that are used for encoding of a block X with respect of a reference block R

An encoded block “X” has displacement vector associated with him, that consists of two components (dx, dy) and points to a reference block “R” that is used for a prediction of the pixel values of the encoded block. In addition to this, there are adjacent blocks (also called templates) that are located near “X” and “R” blocks. They are denoted as DA, DB and RA, RB and are used together with reference block R for making corrected prediction of an encoded block X.

Proposed technique of an adaptive luminance compensation deals with making adaptive correction of a prediction for each encoded pixel finally resulting in hypothetically smaller residuals without bit-overhead.

Template areas (RA, RB, DA and DB blocks) are available for the decoder as well as a reference block that is explicitly pointed by DV. Thus it’s possible to estimate local changes of intensities between reference and encoded blocks followed by intensities changes compensation process at the decoder side too. If all these procedures are accurate and reliable, more compression ratio can be achieved.

Based on our experiments well-known a 1st-order linear correction model has been adopted. It is applied to each pixel of the reference block, while parameters of the model are determined separately based on geometrical and intensity relations between pixels of a reference block R and templates RA, RB, DA and DB.

For simplicity, RA and RB together are re-denoted as TR(DV) group of pixels; DA and DB are re-denoted as TD group of pixels.

Firstly, our compensation model is approximated by a 1st-order linear model of pixel-wise correction:

Pi,j=αi,j×Ri,jDV+βi,j 	(1)

Here: (i,j) – relative coordinates of a pixel within encoded block X ( and corresponding prediction block R), Pij – means corrected prediction value for the pixel defined by (i,j) coordinates, αi,j and βij are coordinate-depended parameters of the correction model.

In order to evaluate parameters of compensation αi,jandβij, it is assumed that they are estimated as follows:

αi,j=EstDi,jEstRi,j,βij≃0           	(2)

The residual value Δi,jis defined as usually in accordance with (3):

Δi,j=Xij−Pij	(3)

Secondly, we define the valuesEstDi,j, EstRi,j:

EstDi,j=K=0...N−1TDK×0,TDK−TR(DV)K>TH1∨Ri,j(DV)−TR(DV)K>TH2	(4)

EstRi,j=K=0...N−1TRK×0,TDK−TR(DV)K>TH1∨Ri,j(DV)−TR(DV)K>TH2	(5)

Here K is the index that runs over all the pixels of template area which capacity is N pixels. Two thresholds TH1 and TH2 control allowable absolute difference between pixels of a reference block and pixels from corresponding template area TR(DV) and allowable absolute difference between pixels in TD and TR(DV) template areas that have the same relative coordinates but can be differed to each other due-to local geometrical texture distortions. Parameters C1,C2, C3 define the “degree of smoothness” of TD and TR induction in (4) and (5) and together with TH1 and TH2 provide additional flexibility to local texture features. Generally this approach can be applied both to luminance and chrominance channels; also it can use synthesized frames and adjacent (un-warped) frames as well.

## Modified distortion metric for disparity estimation procedure (non-normative)

Besides compensation itself, efficiency of this technique depends on disparity (displacement) vector that defines a reference block. It was found that conventional distortion metrics like Sum of Absolute Difference (SAD) or Sum of Squared Errors (SSE) that are usually used in the motion estimation or disparity estimation procedures, are too sensitive to possible luminance mismatches between two blocks; it results in overestimating of distortion value caused by a non-zero mean difference between these blocks and finally in incorrect disparity vector. In order to cope with this, we proposed to change conventional SAD metric (6) to MR_SAD (7). This approach is quite famous, nevertheless it does not make too much sense for simple video coding as there are no so many luminance discrepancies as in an inter-view case (except special cases with fade in/out effects in video).

SAD(X,R,DV)=i,j|Xij−R(DV)ij|	(6)

MRSAD(X,R,DV)=i,j|Xij−X−R(DV)ij+R(DV)|	(7)

Here X – an encoded block, DV-  a displacement vector, R(DV) – reference block (prediction block) that is used for prediction of X, X - mean value of the encoded block, R(DV) - mean value of reference block.  A bit similar approach is proposed in [5], authors proposed to include in-loop compensation of luminance difference in the motion estimation loop that potentially could increase complexity and limit flexibility of compensation due-to use of template areas by motion estimation procedure.

Mean-removed metric like (7) ignores mean difference R(DV)−Xbetween two blocks thoroughly. It means that if two blocks are quite similar in terms of mean-removed signal but too differed in terms of mean values, MR_SAD metric cannot distinguish them; thus we propose to extend classical MR_SAD metric by means of mean-difference value:

MRSADupdated(X,R,DV)=MRSAD(X,R,DV)+γ×X−R(DV)	(8)

Here γdefines how strong the influence of a mean-difference X−R(DV)onto a metric value is. Finally block-wise modification of a metric that is based on mean-removed idea and idea of separate inclusion of mean-difference into the metric was done. This is important if the sizes of a motion estimation block are greater than the sizes of elementary transformed & corrected blocks (for example – 4x4). In particular mean-removed block-wise SAD with mean-difference update MRBSADupdatedis defined as follows:

MRBSADupdated(X,R,DV)=kMRSADupdated(Xk,Rk(DV)	(9)

Here k – is a serial number of elementary compensated block comprising whole X and R(DV) respectively. In the reported technology elementary compensated block is 4x4 block. Please refer to the figure 3 for the details.

Encoded Frame

Reference Frame

X

R

DA

DB

RA

RB

DV: Displacement Vector

Template Width

16 pixels

16 pixels

Elementary  compensated block 4x4

Fig. 3 Subdivision of whole encoded block 16x16 into elementary compensated blocks 4x4 for mean-removed block-wise SAD metric

It should be noted that complexity of this modified metric is not increased too much due-to almost the same mathematical operations realized as in the case of classical mean-removed SAD. Obviously this modification does not change decoder and thus is non-normative change.

## Adaptive luminance correction in the Skip modes

Proposed technique can be applied both for non-skip and skip modes simultaneously; last case it’s important to correct reference block that is identified by skip rules only if this correction is more “true” than “false”; in other words, as skip mode is very important for compression, any kind of corrections should not decrease quality of predictor and consequently decrease amount of skip macroblocks. In addition to this, it’s important to not apply correction in the skip mode whenever such a correction does not change predictor too much. Main reason is a complexity of coder/decoder. Thus two criteria are proposed here that are used when correction is trying for skip modes:

- Similarity criterion: MNorm

- Dissimilarity criterion: MRNorm

Similarity and dissimilarity are defined for template areas TR(DV) and TD.

Basically similarity means how much TR(DV) and TD are similar in terms of absolute differences of mean values of sub-blocks in TR(DV) and TD. Namely, let assume that TD and TR(DV) are split into M sub-blocks SBi, such that i-th sub-block  SBi of pixels in TD corresponds to i-th sub-block SBi in TR(DV). Then for each SBi the following metric can be defined:

MNorm(SBi)=Ki∈SBiTR(DV)KiSBi−Ki∈SBiTDKiSBi 	(10)

Here |SBi| - number of pixels in SBi, Ki – index of pixel within SBi, TR(DV)Ki means Ki pixel from TR(DV) template, while TDKi means Ki pixel from TD template. Then cumulative similarity criterion is calculated as follows:

MNorm=i=0...M−1MNorm(SBi).

Accordingly to described above, dissimilarity means how much TR(DV) and TD are differed in terms of mean-removed signal of sub-blocks in TR(DV) and TD. Namely, let assume that TD and TR(DV) are also split into M sub-blocks SBi, such that i-th sub-block  SBi of pixels in TD corresponds to i-th sub-block SBi in TR(DV). Then for each SBi mean-removed SAD is calculated according to (11):

MRNorm(SBi)=Ki∈SBiTR(DV)Ki−TDKi−Ki∈SBiTR(DV)KiSBi+Ki∈SBiTDKiSBi 	(11)

Here |SBi| - number of pixels in SBi, Ki – index of pixel within SBi, TR(DV)Ki means corresponding pixel from TR(DV) template, while TDKi means corresponding pixel from TD template. Then a cumulative dissimilarity criterion is calculated as follows:

MRNorm=i=0...M−1MRNorm(SBi).

In case at least one of the (12) and (13) is false, correction is not applied in the skip-mode:

MNorm<TH3	(12)

MRNorm>TH4	(13)

Practically, size of sub-block SBi can be done as small as the width of template; for example if template size is 4, then sub-block SBi is a block 4x4 pixels belonged to the template.

## Adaptive template-size selection

Another aspect of this technology is an adaptive selection of template size depending of applied transform size and partition. In particular template TR(DV) for reference block and TD for encoded block can be local (it means that templates TR(DV) and TD are as close to the reference and encoded blocks as possible) or global (it means that  templates TR(DV) and TD are distantly from reference and encoded blocks). For example, let consider P16x16 mode; in this mode template area can be selected as a global template; in this case template is defined as adjacent areas around 16x16 block; alternatively as whole block 16x16 is predicted and then transformed by transform 4x4 or transform 8x8, it’s possible to use local template for each block 8x8 or 4x4. It can be explained by the figure 4. Template area can also be identified by the size of a block that is immediately surrounded by this template area. The following combinations of templates were identified experimentally:

| Prediction mode | Transform Type: DCT8x8 | Transform Type: DCT4x4 |
| --- | --- | --- |
| 16*16 | Template around 16x16 | Template around 8x8 |
| 16*8 | Template around 16x8 | Template around 4x4 |
| 8*16 | Template around 8x16 | Template around 4x4 |
| 8*8 | Template around 8x8 | Template around 4x4 |
| Skip, MERL-Skip | Template around 16x16 | Template around 16x16 |

Frame

Whole encoded block 8x8

Block 4x4 to be transformed by DCT4x4

Template around 8x8      (“global” for a block 4x4)

Fig. 4.A Example of “global template area” for the blocks 4x4 included into the block 8x8

Frame

Whole encoded block 8x8

Block 4x4 to be transformed by DCT4x4

Template around 8x8      (“local” for a block 4x4)

Fig. 4.B Example of “global template area” for the blocks 4x4 included into the block 8x8

## Integration details

Integration of described technology was done as follows. New syntax element – ALC_FLAG was implemented for all P-Slices that are not depth and that are incompatible with AVC standard. Simplified structure of a bitstream is given in the figure 5.

SKIP FLAG

MB Type

MV & Ref Indexes

CBP

Residuals

SKIP FLAG

MB Type

ALC

FLAG = 1

MV

CBP

Standard bitstream structure ( macroblock level)

Modified bitstream structure ( macroblock level); ALC ON

(With more than 1 reference)

Transform

Flag

ALC

FLAG = 1

MV & Ref Indexes

CBP

Residuals

Transform

Flag

Residuals

Modified bitstream structure ( macroblock level), ALC ON

(With 1 reference only)

SKIP FLAG

MB Type

Modified bitstream structure ( macroblock level); ALC OFF

ALC

FLAG = 0

MV & Ref Indexes

CBP

Residuals

Transform

Flag

SKIP FLAG

MB Type

Transform

Flag

Fig. 5. Modifications of a bitstream structure at the macro-block level with proposed technology

Reported technology is implemented for all partitions and sub-partitions including skip modes. Then  based on experimental runs only those partitions were changed that gave reasonable tradeoff between compression gain and complexity; in particular it was found that sub-partitions like 8x4, 4x8, 4x4 for P8x8 mode were not reasonable to be changed by this technology as compression gain was moderate while complexity of a coder was increased.

Also, we made experiments with inter-view prediction when only synthetic view is used as a reference frame for adaptive luminance compensation and when at least adjacent view and synthetic view are used together, it inevitable lead to explicit encoding of a reference index. It was found that using of an adjacent view and synthetic view together is not efficient as bitrate is increased unreasonably; Thus current integration assumes that coder generates bitstream that looks like 5.B and/or 5.D.

It should be noted that adaptive luminance compensation works in Skip modes also; but turning on/off is determined “implicitly” without any additional signaling in accordance with the description given in 2.3.

# Configuration

Here a list of configuration is provided.

| Parameter | Value | Comment |
| --- | --- | --- |
| C1 | 0.1x0.5 = 0.05 | These parameters can be sequence-dependent |
| C2 | 1 |  |
| C3 | 0 |  |
| TH1 | 30 | These parameters can be sequence-dependent |
| TH2 | 60 |  |
| TH3 | 4 |  |
| TH4 | 3 x ¦Template¦ | ¦Template¦ means total amount of pixels in the template area; for block-size 16x16 and template-width 4 ¦Template¦ = 128 |
| Template size | 4 |  |
| SB Size | 4x4 |  |
| 16x16 for ALC | Supported |  |
| 16x8,8x16 for ALC | Supported |  |
| 8x8 for ALC | Supported |  |
| Sub-partitions for 8x8 for ALC | Supported but Turned OFF | Shown experimentally |
| Search Zone for ME for ALC: Vertical | +-6 QPEL | It was found that this search zone is quite optimal in terms of complexity/gain |
| Search Zone for ME for ALC: Horizontal | +-20 QPEL |  |
| Type of search | Standard full search |  |
| Center of a search-zone for ALC | 0 | Most probable value (prediction) is slightly better |
| Distortion metric | MR_BSAD_updated | See 2.2 |
|  (Distortion metric) | 1 | See 2.2 |
| VSP ALC | Supported | It’s shown to be most effective to use only VSP mode for ALC |
| Inter-view ALC | Supported, Turned OFF | It’s shown to be lest effective to use inter-view and VSP mode for ALC |
| Fixed-point implementation of correction | Supported | 32bit fixed point implementation |
| Coding of ALC flag | CABAC | Initialization probabilities: TBD |

| Prediction mode | Transform Type: DCT8x8 | Transform Type: DCT4x4 |
| --- | --- | --- |
| 16*16 | Template around 16x16 | Template around 8x8 |
| 16*8 | Template around 16x8 | Template around 4x4 |
| 8*16 | Template around 8x16 | Template around 4x4 |
| 8*8 | Template around 8x8 | Template around 4x4 |
| Skip, MERL-Skip | Template around 16x16 | Template around 16x16 |

# Syntax changes and config file amendments

## Formal description of syntax changes

- Flag which enables/disables Adaptive Luminance Compensation tool in the «Subset sequence parameter set RBSP syntax»

| subset_seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| seq_parameter_set_data( ) | 0 |  |
| if( profile_idc = = 83 ¦ ¦ profile_idc = = 86 ) { |  |  |
| … |  |  |
| } else if ( profile_idc = = 118 ¦ ¦ profile_idc = = 128 ¦ ¦ profile_idc = = 138 ¦ ¦ profile_idc = = 139) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvc_extension( ) /* specified in Annex H */ | 0 |  |
| mvc_vui_parameters_present_flag | 0 | u(1) |
| if ( mvc_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) /* specified in Annex H */ | 0 |  |
| enable_alc_encoder_flag | 0 | u(1) |
| } |  |  |
| if( profile_idc = = 138 ¦ ¦ profile_idc = = 139 ) { |  |  |
| … |  |  |
| } |  |  |
| … |  |  |
| } |  |  |

Notes:

The table above shows current version of integration and can be changed in accordiance with any further recommendations related with harmonization of high-level syntax. For example, enable_alc_encoder_flag can be moved into seq_parameter_set_mvc_extension( )  or seq_parameter_set_3dvc_extension( ).

- Flag which indicates that Adaptive Luminance Compensation tool is applied for the current macroblock.

| macroblock_layer( ) { | C | Descriptor |
| --- | --- | --- |
| mb_type | 2 | ue(v) ¦ ae(v) |
| if (slice_type == P ¦¦ entropy_coding_mode_flag ¦¦ !is_depth ¦¦ !base_view ¦¦ !is_intra(mb_type)) { |  |  |
| mb_alc_flag | 2 | ae(v) |
| } |  |  |
| … |  |  |

Notes:

- is_depth – Boolean variable which equals to 1 if current macroblock belongs to a depth sequence, and equals to 0 otherwise.

- base_view – Boolean variable which equals to 1 if current macroblock belongs to a base view, and equals to 0 otherwise.

- is_intra(mb_type) – function which returns 1 if current macroblock is intra macroblock; and returns 0 otherwise.

## Integration description – additional parameters in the encoder configuration file

- AdaptiveLuminanceCompensation

Class: Numeric (Integer)

Description: Enable Adaptive Luminance Compensation Tool in P slices for non-base views (1: enable, 0 disable/default).

- AlcSearchRangeX

Class: Numeric (Integer)

Description: Sets allowable search range along X axis for Displacement Estimation in Adaptive Luminance Compensation method.

- AlcSearchRangeY

Class: Numeric (Integer)

Description: Sets allowable search range along Y axis for Displacement Estimation in Adaptive Luminance Compensation method.

Results

The following results are reported:

| Compression efficiency: Search Zone for ALC: 6/20 Center of search zone for ALC: 0 ALC: in VSP mode only ( see 5.B for the reference) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Seq. | Texture Coding |  | Total Coding ( coded PSNR) |  | Total Coding ( synthesized PSNR) |  |
|  | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB |
| S01 | -0.87 | 0.03 | -0.81 | 0.03 | -0.43 | 0.02 |
| S02 | -0.34 | 0.01 | -0.36 | 0.01 | 0.23 | -0.01 |
| S03 | 0.04 | 0.00 | 0.03 | 0.00 | 0.09 | 0.00 |
| S04 | 0.19 | -0.01 | 0.18 | -0.01 | -0.03 | 0.00 |
| S05 | -2.71 | 0.14 | -2.34 | 0.11 | -1.77 | 0.08 |
| S06 | -2.34 | 0.12 | -2.22 | 0.11 | -1.40 | 0.06 |
| S08 | -1.75 | 0.08 | -1.67 | 0.07 | -0.85 | 0.03 |
| Average | -1.11 | 0.05 | -1.03 | 0.05 | -0.59 | 0.03 |

| Seq. | Complexity |  |
| --- | --- | --- |
|  | Encoder Time | decoder Time |
| S01 | 95.74 | 102.86 |
| S02 | 101.83 | 103.66 |
| S03 | 102.67 | 103.45 |
| S04 | 104.71 | 101.33 |
| S05 | 103.00 | 104.77 |
| S06 | 104.90 | 105.13 |
| S08 | 108.05 | 102.82 |
| Average | 102.98 | 103.43 |

# Conclusions

This document provides comprehensive description of an adaptive luminance compensation algorithm for the inter-view coding. Reported technology allows compensating possible local luminance mismatches between at least two object projections captured by adjacent cameras of a 3D capturing system even if they are matched geometrically perfectly. Description includes technical details of mathematical grounds of the technology, details of integration into recent test model of 3D video standard (3D-ATM v. 0.4), provides list of best parameters settings and finally measurements results.

In spite of the fact that mathematical grounds include floating point operations, this technology is implemented with fixed-point arithmetic operations (32bit) than makes implementation of the technology more useful for practical realizations.

The following advantages could be summarized and highlighted:

- Total compression gain is -1.03% DBR (total bitrate: views + depths, coded PSNR)

- Texture compression gain is -1.1% DBR (total bitrate: views, coded PSNR)

- Total compression gain is -0.58% DBR (total bitrate: views + depths, synthesized PSNR)

- Complexity: Encoder-part: 102.98%

- Complexity: Decoder-part: 103.43%

- Fixed-point realization: yes, 32bit arithmetic

- Possibility to speed-up: yes, by parallel implementation due-to independent correction of pixels of a reference block.

Based on this, its’ recommended to adopt this technology into next version of 3D-ATM.

# Patent rights declaration(s)

Samsung Moscow Research Center and Samsung Electronics Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).