| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0016 |
| --- | --- |

| Title: | 3DV-CE2.h adaptive quantization for depth map (AQD) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam, Sunmi Yoo, Hyomin Choi, Hyunho Jo, Donggyu Sim, Gun Bang, Won-Sik Cheong, and Namho Hur Kwangwoon University 447-1, Wolgye-Dong, Nowon-Gu, Seoul, Korea Electronics and Telecommunications Research Institute, 138, Gajeongno, Yuseong-Gu, Daejeon, Korea | Tel: Email: | +82-(02) 940-5470 {qejixfyza, youni333, hyomin06, idjhh, dgsim}@kw.ac.kr {gbang, wscheong, namho}@etri.re.kr |
| Source: | ETRI and Kwangwoon University (KWU) |  |  |

_____________________________

# Abstract

This contribution reported the results of CE2.h for evaluating adaptive quatization for depth map (AQD) proposed by ETRI and KWU’s proposal. The AQP is implemented on HTM3.1. Experimental results are shown that the proposed method achieves approximately bitrate increase about 0.1% and 0.1% for three-view and two-view in terms of synthesized quality, respectively.

# Introduction

CE2.h was continually established to evaluate depth representation and coding based 3D-HTM. In the previous meetings, we proposed adaptive quantization for depth map coding (AQD) [1-2]. In this contribution, we evaluate performance of the proposed tool at 3DV-HTM3.1 version. The AQD tool for each TU adaptively applies quantization with transformation or quantization without transformation. In the spatial domain, the quantization without any transformation can yield better RD performance for some regions. The proposed method can preserve sharp edges of depth maps and improve subjective quality of synthesized videos for some sequences.

# Simulation results

Simulation was conducted according to the common test conditions described in [3]. Table I and Table II show the overall results for three-view and two-view case respectively. As shown in Table I and Table II, the results achieve loss of subjective quality about 0.1% and 0.1% in three-view and two-view, respectively. The complexity of the encoder and the decoder are silightly increased about 2% and 1%, respectively.

Table I. Overall simulation result for three-view case

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.5% | 100.8% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 98.5% | 101.4% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 103.4% | 101.3% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | -0.4% | -0.2% | 103.6% | 101.1% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.7% | 0.7% | 105.7% | 100.6% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.2% | 104.5% | 100.6% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 102.2% | 100.8% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.4% | 101.2% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 104.0% | 100.8% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 102.4% | 101.0% |

Table II. Overall simulation result for two-view case

|  | video 0 | video 1 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Newspapercc | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | -0.3% | -0.1% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.7% | 0.7% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.2% | 0.2% |
| UndoDancer | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% |
| average | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |

# Conclusions

This contribution evaluated adaptive quantization for depth maps. The proposed AQD tool can improve subjective quality of synthesized videos for some sequences. The complexity of encoder and decoder are comparable with anchor.

# References

[1] G. Bang, J. Nam, H. Choi, S. Yoo, W. Lim, W. Cheong, N. Hur, and D. Sim, “3D-HEVC-CE4 results on adaptive quantization for depth map,” The 99th MPEG meeting, M23621, San Jose, CA, Feb. 2012.

[2] J. Nam, S. Yoo, H. Choi, W. Lim, D. Sim, G. Bang, W. Cheong, and N. Hur, “3DV-CE2.h results on adaptive quantization for depth map,” The 100th MPEG meeting, M24907, Geneva, CH, May 2012.

[3] H. Schwarz and D. Rusanovskyy, “Common test conditions for 3DV experimentation,” The 100th MPEG meeting, N12745, Geneva, CH, May 2012.

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).