| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0018 |
| --- | --- |

| Title: | 3D-CE1.h Results on View Synthesis Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Shimizu, Shiori Sugimoto, Hideaki Kimata, (NTT) Dong Tian, Feng Zou, Anthony Vetro (MERL) | Tel: Email: | +81-46-859-2703, shimizu.shinya@lab.ntt.co.jp, +1-617-621-7556 tian@merl.com |
| Source: | NTT Corporation and Mitsubishi Electric Research Laboratories (MERL) |  |  |

_____________________________

# Abstract

View synthesis prediction (VSP) is a technique to remove inter-view redundancies when coding multiview video information, which uses a synthetic picture as a reference picture to predict the current picture. This contribution first proposes inserting synthetic pictures to reference picture lists, which shows an average bitrate saving of -2.6% and -0.4% for the dependent views, with maximum gains of -9.9% and -5.8% for these views. This contribution further proposes a VSP skip mode which enables the skip mode relative to the synthetic reference picture. However, this mode has not been fully implemented and requires further study. The benefit of VSP for depth coding has also been investigated, but the result is not yet conclusive. Lastly, when VSP is enabled, this contribution proposes to simplify the rendering process in HTM, which would reduce the decoding time by 4% if VSP is only applied to texture, or 20% if VSP is applied to both texture and depth.

# Introduction

As a technique to utilize inter-view redundancies, view synthesis prediction (VSP) was adopted and under further development within the AVC compatible 3D video extension (ATM). In HEVC compatible 3D video extension (HTM), however, VSP has not been explored as substantial as in AVC categories until some initial study of VSP within the framework of HEVC was performed at the last MPEG meeting in Geneva [1]. As a result, the exploration of VSP was being placed in the CE1.h schedule. In this contribution, we make further investigation on VSP related techniques in the scope of CE1.h.

# View Synthesis Prediction

## Basic VSP scheme

Within a basic implementation of VSP, a synthetic picture is first generated using a decoded texture image and a decoded depth image. The rendering module in HTM is re-used to generate the synthetic picture in our current implementation. For a synthetic picture, POC and view_id values are set to equal to the picture being coded.

For the second step, the synthetic picture is proposed to be appended to the reference picture lists (LIST_0 and LIST_1). In principle, the synthetic picture can be inserted at the end of the reference picture lists or configured at any position in the lists. In such a way, the synthetic pictures could be selected to predict the current picture in the same manner as temporal and inter-view reference pictures.

In the current implementation, the synthetic picture is placed at the end of the reference picture list for both LIST_0 and LIST_1. For bi-predictive pictures, the same synthetic picture is added to the two lists to allow a flexible combination. More advanced placing synthetic pictures is subject to future work.

The above VSP prediction can to be applied to both texture and depth, and it is configurable in our implementation.

## VSP skip mode

On top of the basic VSP scheme, VSP skip/direct [2] was proposed to enable the use of synthetic picture for skip/direct mode even when the synthetic picture is not placed at the beginning of the reference picture list. An adaptive signaling method was further proposed in [3] by re-defining the meaning of the skip flags.

In the context of HTM framework, we propose to copy the above principals of ATM designs. However, due to the limitation of short time, we only implemented the VSP skip part as of this meeting. A flag is encoded to indicate the type of the skip mode. Value 0 specifies a non-synthetic reference and value 1 specifies a synthetic reference. Alternatively, the merging candidate list may be modified to signal the VSP skip mode.

## Simplification of in-loop view synthesis process

It is noticed that the decoding complexity is increased by the introduction of VSP prediction because of the rendering process for the synthetic picture. Some work to address the problem has been performed in ATM development [4] and we propose to re-use them into HTM framework.

The proposed simplification is applied on the warping process; no interpolation is applied on the disparity filed while bi-linear interpolation is applied in the VSRS-1D-Fast to generate subjectively better synthesis results.

## Issue for encoder control without VSP for depth coding

In our current implementation, we enable VSP for both texture and depth by default. In order to evaluate the performance of VSP for texture coding only, we need to disable VSP for depth coding, which is realized by a macro in source code. Since motion vector inheritance (MVI) requires having the same configuration on the reference picture lists with those on texture coding, the current implementation forces the encoder not to select MVI mode at the PUs where corresponding PUs on texture refer VSP picture. This means one bit flag which indicate whether MVI is applied or not is wasted in the current implementation.

# Simulation Results

The proposed methods are built on the top of the reference software 3DV-HTM v3.1. The experiments are conducted according to the common test condition [5]. In order to evaluate each technique, the test cases we tested are summarized in TABLE 1. The experimental results are summarized in TABLE 2, and TABLE 3. The details are available in the attached EXCEL files.

TABLE 1: Test cases

|  | VSP for texture | VSP for depth | VSP Skip mode | Synthesis algorithm |
| --- | --- | --- | --- | --- |
| Anchor | N/A | N/A | N/A | N/A |
| Case 0 | X | X | X | VSRS-1D-Fast |
| Case 1 | X |  | X | VSRS-1D-Fast |
| Case 2 | X |  |  | VSRS-1D-Fast |
| Case 3 | X | X |  | VSRS-1D-Fast |
| Case 4 | X |  | X | Simplified |
| Case 5 | X |  |  | Simplified |
| Case 6 | X | X |  | Simplified |

## Coding performance evaluation

The coding performance is evaluated for the 7 test cases and the results are shown in TABLE 2. Our observations are summarized as below.

- When enable all VSP related tools in our implementation (case 0), -0.5% coding gain is found for video1; unfortunately, a loss 1.3% is found for video2. It is unclear to show the overall benefits of our proposal as a single package.

- We then turn off VSP for depth, but keep all VSP tools for texture (case 1). -0.1% coding gain is found for video1; still a loss 1.4% is found for video2. The benefits of the proposed VSP tool package for texture only are unfortunately unclear.

- We further turn off VSP skip mode in our current implementation (case 2). -2.6% and -0.4% coding gain are observed for video1 and video2, respectively. The gain can be up to -9.9% and -5.8% for video1 and video2. This is clear evidence that the coding of video can benefit from adding VSP picture to the reference picture lists.

- We test another combination: enable basic VSP for both texture and depth, but turn off VSP skip (case 3). It is still unclear to demonstrate the VSP technique for depth.

- Test case 4, 5, and 6 are designed as counter party for case 1, 2, and 3, respectively by replacing the synthesis procedure with a simplified version.

  - When comparing case 4 with 1 (all VSP tools for texture only). Some coding gains are even identified, which is a very good indication that the simplification of view synthesis should be acceptable.

  - When comparing case 5 with 2 (basic VSP for texture only). There are some coding loss for video1 and some gain for video2. It still favorites the simplified rendering.

  - When comparing case 6 with 3 (basic VSP for both texture and depth). There are some coding loss for video1 and some gain for video2. It still favorites the simplified rendering.

It is noted that for all test cases, the coding gains for HD sequences are clearly high; especially for the basic VSP scheme for texture only (test case 2). Unfortunately, the results for XGA sequences are not satisfactory. The clear gap between the two categories sequences was not present in ATM development. A major difference between ATM and HTM framework is that the depth is coded at half resolution in ATM, but at full resolution in HTM. We believe the down-sampling and up-sampling process in ATM would smooth out many noises in the depth images, and the depth maps in XGA sequences contain more noises than HD sequences.

Last, from the nature of VSP prediction, it is supposed to improve the prediction efficiency of the target components. For example, when applying VSP for texture components only (test case 2), it is clearly the video coding can benefit from VSP, however, it is not understandable that the synthesis results won't show benefits from the improved video coding. The VSO may be of the reason, because we found the depth coding results were also changed even no VSP enabled for depth coding. So we suggest focusing only on the coding gain reported for video, and not distracted by the synthesis results. The synthesis issue need be further discussed during the meeting.

TABLE 2: Experimental Results

- Case 0 ( both texture and depth, VSP skip mode)

- Case 1 (only texture, VSP skip mode)

- Case 2 (only texture)

- Case 3 (both texture and depth)

- Case 4 (only texture, VSP skip mode, simplified warping)

- Case 5 (only texture, simplified warping)

- Case 6 (both texture and depth, simplified warping)

## Complexity evaluation

From TABLE 2, it could be found that the decoder complexity is significantly increased due to the generation of VSP picture. Though this increase might be allowed because the generation of VSP picture can be parallelized with the entropy decoding on coding views since only decoded texture and depth map on the reference view are required, we would try to reduce the decoding complexity.

In TABLE 3 we compare the encoding and decoding time changes from the proposed rendering algorithm when VSP is turned on. In (case4, case1) and (case 5, case2), the VSP is only used for texture, which show about 4% decoding time reduction. In (case6, case3), the VSP is used for both texture and depth, which shows about 24% decoding time reduction.

TABLE 3: Encoding /Decoding time

# Conclusions

Based on the observed results from our extensive tests, we recommend 1) adopt the basic VSP scheme by adding VSP picture to reference picture lists for texture (and depth) into HTM; 2) adopt the simplified rendering process into HTM; 3) continue to study VSP skip in CE1.h; 4) further simplification of rendering process.

# Patent rights declaration(s)

NTT Corporation and Mitsubishi Electric Research Laboratories may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Danillo Graziosi, Dong Tian, and Anthony Vetro, "3D-CE1.h related proposal for view synthesis prediction", ISO/IEC JTC1/SC29/WG11 MPEG2012/M24933, April 2012, Geneva, Switzerland.

- Dong Tian, Danillo Graziosi, and Anthony Vetro, "3D-AVC-CE1 results on VSP skip /direct mode by Mitsubishi", ISO/IEC JTC1/SC29/WG11 MPEG2012/M23915, February 2012, San Jose, USA.

- Jin Young Lee, Jaejoon Lee, and Du Sik Park, "3D-CE1.a results on a context-based adaptive skip flag positioning method by Samsung", ISO/IEC JTC1/SC29/WG11 MPEG2012/M24819, April 2012, Geneva, Switzerland.

- Shinya Shimizu and Hideaki Kimata, "3D-CE1.a related: Simplification of in-loop view synthesis with fractional-pel disparity", ISO/IEC JTC1/SC29/WG11 MPEG2012/M24796, April 2012, Geneva, Switzerland.

- ISO/IEC JTC1/SC29/WG11, “Common test conditions for 3DV experimentation”, ISO/IEC JTC1/SC29/WG11 N12745, May 2012, Geneva, Switzerland.