| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0021 |
| --- | --- |

| Title: | 3D-HEVC HLS: Inter-layer SPS Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thomas Rusert Ericsson Research 164 80 Stockholm Sweden | Tel: Email: | +46 76 7602660 thomas.rusert@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution addresses a redundancy problem that is asserted to be present when separate SPSs are used across layers in layered HEVC extensions. It is reported that re-use of SPSs across layers is not an option if certain SPS parameters are layer-specific, an example being profile_idc and level_idc, thus with the current SPS design a separate SPS has to be used if layer-specific parameters are present. Since many SPS syntax elements may have identical values across layers, those values are duplicated if separate SPSs are sent for different layers.

It is proposed for the layered HEVC extensions to introduce SPS prediction to reduce the redundancy. This implies extending the definition of SPS in the extension specifications. In the proposed SPS prediction scheme, SPS parameters are inherited at the time of SPS activation. The reference SPS for prediction is determined as the active SPS of the reference layer, which is signaled in the referencing SPS. The parameters to be inherited are indicated by flags.

The document is an update of MPEG-m24817 / JCTVC-I0535, considering comments received at the last Geneva meeting as well as the latest draft HEVC specification.

# Introduction

In the Test Model under Consideration for HEVC-based 3D Video Coding [MPEG-N12744], each video and depth view is associated with a separate sequence parameter set (SPS). With that design, for the case of a 2-view (3-view) video + depth representation, a total of 4 (6) complete SPSs must be sent for each random access point. The SPSs associated with the different video and depth views have very high similarity, since the video dimensions are identical and typically the same coding algorithms and parameters are used across views. By duplicating this information in several SPSs, unnecessary redundancy, typically at least around 300 bits per SPS, and potentially more, is introduced.

The scope of this contribution is to reduce the redundancy and thus bit rate overhead associated with such use of multiple SPSs in the HEVC 3D extension. The proposed solution is applicable for both 3D and scalable extensions of HEVC. For multiview texture and depth coding, SPS parameters are very similar across views. Likewise, for spatial scalability or SNR scalability or potentially other forms of scalability, when different layers are associated with dedicated SPSs, the SPSs typically have high similarities across layers. Since the proposed approach is broadly applicable, even though the proposal is made for the 3D extension to HEVC at this time, we will occasionally refer to the more general term “layer” rather than “video and/or depth view”.

One approach for reducing SPS overhead is to re-use SPSs across layers, i.e. to activate the same SPS for more than one layer. The effectiveness of that approach is limited since even though many SPS parameters typically have identical values across layers, some SPS parameters still have different values. One example are the syntax elements profile_idc and level_idc: For example, for 2-view video + depth representations, it is likely that at least three different profiles will be signaled, e.g. a HEVC main profile associated with the base video view, a stereo profile associated with base and enhancement video views, and a 2-view video + depth profile, each of them being associated with a different value of profile_idc. Thus at least three different SPSs are required. Similarly, for 3-view video + depth representations, it may be desirable to include the abovementioned three profiles in order to support mono-capable decoders, stereo-capable (video only, and video + depth) decoders, and to add at least one additional profile for 3-view video and depth, thus a total of at least four different values of profile_idc have to be signaled, i.e. at least four different SPSs are required. Even if it is not necessary to signal a new profile with each layer, it may be desirable to signal different level requirements for the different layers. For the same reasons, for spatial and/or SNR scalability, different profiles and/or levels will be associated with the different layers, and many SPS parameters will be identical across layers. Thus, even if some SPSs are re-used across layers, significant redundancy remains.

This contribution proposes adding prediction of SPS parameters across layers. The proposed prediction mechanism uses a simple reference mechanism without any reference chains. At activation time, the syntax elements for a current SPS associated with a current layer are conditionally predicted from the active SPS of a referenced layer below the current layer. The presence of such layer references in the SPS is indicated by flags that are present for layers above the base layer (layer_id_plus1 > 1). Thus no changes to the base specification are required.

# Proposed scheme

Proposed syntax and semantics as well as updated text for SPS activation are provided further below. The definitions are based on the latest HEVC specification draft [JCTVC-I1003_d6]. All changes are highlighted in yellow.

In the proposal, layers are identified by a value of layer_id_plus1, which is signaled in the NAL unit header, using the reserved_one_5bits fields reserved in the base specification. Each layer has up to one active SPS. An SPS NAL unit and the layer the SPS is active for have the same value of layer_id_plus1.

The presence of the proposed layer referencing mechanism in the SPS is conditioned on the value of layer_id_plus1, i.e. the layer referencing can only be present in SPSs associated with enhancement layers (layer_id_plus1 > 1). The SPS of the base video view is defined according to the HEVC core specification, which is not modified. In SPSs associated with layers other than the base layer, the presence of SPS prediction is indicated using ref_layer_present_flag. If present, a reference to another SPS is signaled by means of a reference to a reference layer, using delta_ref_layer_id_minus1. Furthermore, the SPS parameters are grouped into four groups, and use of SPS prediction is indicated by flags (rep_format_inheritance_flag, temp_layer_param_inheritance_flag, coding_param_inheritance_flag, ref_pic_sets_inheritance_flag). If one of those flags is equal to 1, the SPS parameters associated with the respective group are omitted in the SPS, and SPS prediction is used instead.

If a reference to a reference layer is present in a given SPS, then during the activation of the SPS for a given layer, values of syntax elements that are not present in the given SPS are inferred from the active SPS in the reference layer, and thus remain active in the given layer.

With the proposed mechanism, the SPS overhead in the HEVC-based 3D video coding test model can be significantly reduced. It is expected that savings between around 1000 bits and several thousand bits per random access point can be realized.

## SPS syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_space | u(3) |
| profile_idc | u(5) |
| constraint_flags | u(16) |
| level_idc | u(8) |
| for( i = 0; i < 32; i++ ) |  |
| profile_compatability_flag[ i ] | u(1) |
| seq_parameter_set_id | ue(v) |
| video_parameter_set_id | ue(v) |
| if( layer_id_plus1 > 1 ) { */ not base layer */ |  |
| ref_layer_present_flag | u(1) |
| if( ref_layer_present_flag = = 1 ) { |  |
| delta_ref_layer_id_minus1 | ue(v) |
| rep_format_inheritance_flag | u(1) |
| temp_layer_param_inheritance_flag | u(1) |
| coding_param_inheritance_flag | u(1) |
| ref_pic_sets_inheritance_flag | u(1) |
| } |  |
| } |  |
| if( rep_format_inheritance_flag = = 0 ) { |  |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| } |  |
| if( temp_layer_param_inheritance_flag = = 0 ) |  |
| sps_max_temporal_layers_minus1 | u(3) |
| if( rep_format_inheritance_flag = = 0 ) { |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| [Ed. (BB): chroma bit depth present in HM software but not used further ] |  |
| } |  |
| if( coding_param_inheritance_flag = = 0 ) |  |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| } |  |
| if( temp_layer_param_inheritance_flag = = 0 ) |  |
| for( i = 0; i <= sps_max_temporal_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering[ i ] | ue(v) |
| sps_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase[ i ] | ue(v) |
| } |  |
| if( coding_param_inheritance_flag = = 0 ) { |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| } |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| transform_skip_enabled_flag | u(1) |
| seq_loop_filter_across_slices_enabled_flag | u(1) |
| asymmetric_motion_partitions_enabled_flag | u(1) |
| nsrqt_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| } |  |
| if( temp_layer_param_inheritance_flag = = 0 ) |  |
| sps_temporal_id_nesting_flag | u(1) |
| [Ed. (BB): x y padding syntax missing here, present in HM software ] |  |
| if( ref_pic_sets_inheritance_flag = = 0 ) { |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| } |  |
| if( coding_param_inheritance_flag = = 0 ) { |  |
| long_term_ref_pics_present_flag | u(1) |
| sps_temporal_mvp_enable_flag | u(1) |
| } |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## SPS semantics

ref_layer_present_flag equal to 1 indicates that sequence parameter set prediction is used. If ref_layer_present_flag is not present, it is inferred to be equal to 0.

delta_ref_layer_id_minus1 + 1 indicates the difference of the value of layer_id_plus1 and the value of layer_id_plus1 associated with the reference layer used for sequence parameter set prediction. The value of delta_view_ref_layer_id_minus1 shall be in the range of 0 to layer_id_plus1 - 2, inclusive.

rep_format_inheritance_flag equal to 1 indicates that representation format related parameters are inherited from the sequence parameter set active for the layer identified by layer_id_plus1 equal to layer_id_plus1 - delta_ref_layer_id_minus1 – 1. If rep_format_inheritance_flag is not present, it is inferred to be equal to 0.

temp_layer_param_inheritance_flag equal to 1 indicates that temporal layering related parameters are inherited from the sequence parameter set active for the layer identified by layer_id_plus1 equal to layer_id_plus1 - delta_ref_layer_id_minus1 – 1. If temp_layer_param_inheritance_flag is not present, it is inferred to be equal to 0.

coding_param_inheritance_flag equal to 1 indicates that coding algorithm related parameters are inherited from the sequence parameter set active for the layer identified by layer_id_plus1 equal to layer_id_plus1 - delta_ref_layer_id_minus1 – 1. If coding_param_inheritance_flag is not present, it is inferred to be equal to 0.

ref_pic_sets_inheritance_flag equal to 1 indicates that reference parameter sets are inherited from the sequence parameter set active for the layer identified by layer_id_plus1 equal to layer_id_plus1 - delta_ref_layer_id_minus1 – 1. If ref_pic_sets_inheritance_flag is not present, it is inferred to be equal to 0.

## SPS activation

- A sequence parameter set RBSP includes parameters that can be referred to by one or more picture parameter set RBSPs or one or more SEI NAL units containing a buffering period SEI message. Each sequence parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one sequence parameter set RBSP is considered active for a layer at any given moment during the operation of the decoding process, and the activation of any particular sequence parameter set RBSP for a layer results in the deactivation of the previously-active sequence parameter set RBSP (if any) for that layer.

- When a sequence parameter set RBSP (with a particular value of seq_parameter_set_id) having layer_id_plus1 equal to LIdPlus1 is not already active and it is referred to by activation of a picture parameter set RBSP (using that value of seq_parameter_set_id) or is referred to by an SEI NAL unit containing a buffering period SEI message (using that value of seq_parameter_set_id), it is activated for the layer having layer_id_plus1 equal to LIdPlus1. This sequence parameter set RBSP is called the active sequence parameter set RBSP for the layer having layer_id_plus1 equal to LIdPlus1 until it is deactivated by the activation of another sequence parameter set RBSP. A sequence parameter set RBSP, with that particular value of seq_parameter_set_id and layer_id_plus1 equal to LIdPLus1 shall be available to the decoding process prior to its activation, included in at least one access unit with temporal_id equal to 0, unless the sequence parameter set is provided through external means. An activated sequence parameter set RBSP shall remain active for the entire coded video sequence.

- NOTE 2 – Because an IDR or BLA access unit begins a new coded video sequence and an activated sequence parameter set RBSP must remain active for the entire coded video sequence, a sequence parameter set RBSP can only be activated by a buffering period SEI message when the buffering period SEI message is part of an IDR or BLA access unit.

- Any sequence parameter set NAL unit having layer_id_plus1 equal to LIdPlus1 and containing the value seq_parameter_set_id for the active sequence parameter set RBSP having layer_id_plus1 equal to LIdPlus1 for a coded video sequence shall have the same content as that of the active sequence parameter set RBSP for the layer having layer_id_plus1 equal to LIdPlus1 for the coded video sequence unless it follows the last access unit of the coded video sequence and precedes the first VCL NAL unit and the first SEI NAL unit containing a buffering period SEI message (when present) of another coded video sequence.

- When a sequence parameter set RBSP is activated for a layer having layer_id_plus1 equal to LIdPlus1 and delta_ref_layer_id_minus1 is present in the sequence parameter set RBSP, the values of all syntax elements present in the active sequence parameter set RBSP for the layer having layer_id_plus1 equal to LIdPlus1 - delta_view_ref_layer_id_minus1 - 1 and not present in the sequence parameter set RBSP that is activated for the layer having layer_id_plus1 equal to LIdPlus1 are inferred to be active for the layer having layer_id_plus1 equal to LIdPlus1.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).