| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0022 |
| --- | --- |

| Title: | CE1.a results on adaptive illumination compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Shimizu, Shiori Sugimoto, Hideaki Kimata | Tel: Email: | +81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

This contribution presents the simulation results of the adaptive illumination compensation technique proposed in m22616. In the proposed method, the parameters for the illumination compensation are derived locally at the decoder side by minimizing the differences between the decoded picture and the prediction picture. Two compensation models, a linear function and a 2D filter with offset, are evaluated in the experiments. Experimental results are provided with 3DV-ATM v4.0 as anchor for the common test condition. For the texture coding, average BD-rate improvements of 0.25% and 0.04% were obtained with the linear function and 2D filter with offset, respectively. The maximum gain was 0.98% for the sequence "Kendo" with the linear function.

# Introduction

Adaptive illumination compensation was proposed in order to improve the performance of view synthesis prediction (VSP) as a part of NTT’s response to the Call for Proposal on 3D video coding technologies [1]. In this contribution, we also evaluate the performance by extending the concept to the general inter-view prediction.

# Adaptive Illumination Compensation

The proposed method compensates illumination differences between prediction picture and coded picture in a localized manner. Although one compensation model is utilized for all blocks, their parameters are decided block by block. For example, a linear function like the weighted prediction is assumed as the compensation function, different scaling factor and offset can be used for each block.

These compensation parameters are derived by minimizing the differences between the prediction picture and decoded picture on the already decoded neighbouring blocks. The prediction picture is generated by using the prediction mode for the coding target block, not the neighbouring block. This minimization problem can be solved by the least square method. The same derivation process can be performed at the decoder side without any additional information, so that no compensation parameter is encoded.

Since the minimization is conducted at the neighbouring pixels, it is not guaranteed that the compensation can always reduce prediction errors on the coding block. This means that it is necessary to make a decision on whether the compensation is applied or not in order to achieve a better coding performance. One of the simple ways is to introduce a flag, but it brings an increase of bitrate and requires syntax changes at the MB level. Therefore, in this contribution, another method is investigated. In the proposed method, the illumination compensation is turn on if the reliability of the derived compensation parameters is high. The reliability is expressed by calculating how much the compensation can reduce prediction errors on the neighbouring blocks. In the experiment, the compensation is applied when prediction errors are reduced to less than half of those without the compensation.

# Simulation Results

The proposed methods are built on the top of the reference software 3DV-ATM v4.0. The experiments are conducted according to the common test condition [5]. Since the proposed method is applied to texture coding only, the performances on texture coding are summarized. The overall results can be found in the attachment.

## Compensation on different type of reference pictures

The first experiment was to evaluate the coding performance of the proposed method on different type of reference pictures. TABLE 1 shows the coding performances with activating the proposed method only when the type of reference pictures is VSP picture, inter-view picture or both of them. Unfortunately, the coding gains are not significant in any cases, but the results show that it is possible to improve the coding performance by using illumination compensation technique since the positive effects can be found on the best case, i.e. when activating only for VSP picture. It is also noted that small improvements for some sequences are good indication of the effectiveness of illumination compensation on inter-view prediction.

TABLE 1: Coding results on different type of reference pictures

|  | Only VSP |  | Only inter-view |  | Both VSP and inter-view |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dBR, % | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -0.26 | 0.01 | -0.21 | 0.01 | 0.27 | -0.01 |
| S02 | -0.09 | 0.00 | 0.13 | 0.00 | 0.21 | -0.01 |
| S03 | 0.01 | 0.00 | 0.22 | -0.01 | 0.42 | -0.01 |
| S04 | 0.10 | 0.00 | 0.14 | -0.01 | 0.50 | -0.02 |
| S05 | -0.98 | 0.05 | -0.15 | 0.01 | -0.36 | 0.02 |
| S06 | -0.36 | 0.02 | -0.03 | 0.00 | 0.12 | -0.01 |
| S08 | -0.19 | 0.01 | 0.26 | -0.01 | 0.28 | -0.01 |
| Average | -0.25 | 0.01 | 0.05 | 0.00 | 0.21 | -0.01 |

## Evaluation of different compensation model

The second experiment was to investigate the compensation model. In this experiments, two compensation models were tested. One is a linear function like the weighted prediction. In this case, there are two compensation parameters; scaling factor and offset. Another is a square-shaped 5x5 filter with offset. No symmetry is assumed, so that there are 26 parameters. Note that our software supports any other compensation models. In this experiments, the proposed compensation is performed only for VSP pictures.

TABLE 2 shows the results. As it can be seen, the linear function is better than 5x5 filter with offset. Since the parameters are derived by optimizing on the neighboring blocks, compensation models with large number of parameters tend to have less performance. However, looking into the details, there are many blocks which have less errors than using the linear function model. Therefore, if it is possible to select suitable compensation model block-by-block, more improvement can be achieved.

TABLE 2: Coding results with different compensation model

|  | Linear function |  | 5x5 filter with offset |  |
| --- | --- | --- | --- | --- |
|  | dBR, % | dBR, % | dBR, % | dPSNR,dB |
| S01 | -0.26 | 0.01 | -0.06 | 0.00 |
| S02 | -0.09 | 0.00 | 0.04 | 0.00 |
| S03 | 0.01 | 0.00 | 0.13 | 0.00 |
| S04 | 0.10 | 0.00 | 0.07 | 0.00 |
| S05 | -0.98 | 0.05 | -0.24 | 0.01 |
| S06 | -0.36 | 0.02 | -0.11 | 0.01 |
| S08 | -0.19 | 0.01 | -0.11 | 0.00 |
| Average | -0.25 | 0.01 | -0.04 | 0.00 |

# Conclusions

We propose adaptive illumination compensation technique where block-adaptive compensation parameters are derived at the decoder side. Unfortunately, the achieved gain is not so large, but the experiments show that this technique has a potential to achieve more efficient coding. Therefore, we recommend continuing the CE for further improvements.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Shinya Shimizu and Hideaki Kimata, "Description of 3D video coding technology proposal by NTT", ISO/IEC JTC1/SC29/WG11 MPEG2011/M22616, October 2011, Geneva, Switzerland.

- ISO/IEC JTC1/SC29/WG11, “Common test conditions for 3DV experimentation”, ISO/IEC JTC1/SC29/WG11 N12745, May 2012, Geneva, Switzerland.