| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0023 |
| --- | --- |

| Title: | CE1.a related results on the modification of in-loop view synthesis |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Shimizu, Shiori Sugimoto, Hideaki Kimata | Tel: Email: | +81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

This contribution presents the simulation results of the simplified warping process on view synthesis prediction. This technique is proposed in m25195 and used in the view synthesis reference software (VSRS-1D-Fast). In the proposed method, the processing direction of the pixel-wise warping is inverted. Besides, the hole-filling and inpainting is also simplified. Experiment on the common test condition showed that no complexity reduction on the decoding time, it is reported that the proposed method can reduce memory requirements since no z-buffering is necessary. No performance loss was reported. On the contrary, average BD-rate improvement of 0.53% were also obtained on texture coding since more accurate disparities become available to be utilized. The maximum gain was 1.12% for "Undo_Dancer" sequence.

# Introduction

At the 100th MPEG meeting, the in-loop view synthesis with a quarter resolution depth map was adopted. In the m25915[1], changing the processing direction on the pixel-wise warping was also proposed as a harmonization technique between m24796[2] and m24941[3]. The details and benefits were not fully understood and postponed the decision on this technique. In this contribution, we provide the details and clear benefits.

# Current Design on In-loop View Synthesis

In the WD2 [4], the pixel-wise warping process for the generation of VSP picture is defined to process right to left pixel when generating a right-side view of the reference view, and to process left to right pixel when generating a left-side views. In this case, it could be happen a foreground object is warped first, and then a background object is warped to the same pixel as illustrated in Figure 1. Therefore, it is necessary to check the occlusion. In other words, it is necessary to check which is a foreground object, already assigned pixel or newly warped pixel. This check is called as z-buffering.

Figure 1: Warping process in WD2

When using quarter resolution depth map, the z-buffering is defined to be performed in the chroma sample space. As the results, the warping of luma sample should be aligned to the chroma sample. This means that the amount of warped position for luma sample should be even pixels. As the results, as shown in Figure 2, when combining mxxx with mxxx, more than (or up to) one pixel shift on the reference view is introduced since the concept of mxxxx is to keep the total disparity value. This corresponds to the assumption that a pixel which is more than (up to) one pixel apart from the reference pixel has the same depth value with the reference pixel. This assumption is so strong that the quality of the synthesized picture is degraded.

Figure 2: Sub-pixel conversion with quarter resolution depth

One solution to overcome this problem is to allocate separate z-buffer between luma and chroma components. However, this solution requires more working memory. In the case of HD sequence, at least, additional 1920 bytes are required with 8-bit depth map. This may not be preferable.

# Proposed warping process

The proposal is to inverse the processing direction; defining the pixel-wise warping process to process left to right pixel when generating a right-side view of the reference view, and to process right to left pixel when generating a right-side views. When defining the pixel-wise warping process in this direction, it becomes often happen that the a foreground object is warped to the pixel where a background object has already assigned while it is rarely happen that the background object is warped to the pixel where a foreground object has already assigned. In this case, z-buffering is unnecessary anymore since a foreground object can always over-write the pixel. Figure 3 shows an example. As the results, we can save the memory for z-buffering and utilize more loose assumption which brings a better quality on synthesized pictures.

Figure 3: Proposed warping process

It is also noted that the hole-filling and inpainting is also simplified. The hole-filling and inpainting can be performed by fulfilling pixels between the current warped pixel and the last warped pixel by the same pixel values with the current warped pixel as illustrated in Figure 4.

Figure 4: Hole-filling and inpainting on the proposed method

The same concept with the proposed warping is used in the view synthesis reference software VSRS-1D-Fast. The difference is whether the disparity field is interpolated or not. The proposed method re-uses disparity value at the integer-pixel position in order to minimize the computational complexity with keeping the coding performances, while VSRS-1D-Fast interpolates disparity values in order to achieve subjectively better synthesis.

The proposed specification is attached.

# Simulation Results

The proposed method has already built in the reference software 3DV-ATM v4.0 although the implementation is switched off on the compiler level in the default setting. The proposed implementation is activated by setting NTT_VSP_HARMONIZATION_M25195 to 1. The experiments are conducted according to the common test condition [5]. TABLE 1 shows the summary of the experimental results. The detailed results are found in the attached EXCEL file.

TABLE 1: Summary of the experimental results

|  | Texture Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Enc. Time, % | Dec. Time, % |
| S01 | -0.61 | 0.02 | -0.54 | 0.02 | -0.57 | 0.02 | 100.06 | 100.29 |
| S02 | -0.18 | 0.01 | -0.17 | 0.00 | -0.14 | 0.00 | 101.06 | 100.07 |
| S03 | -1.12 | 0.04 | -1.04 | 0.04 | -0.84 | 0.02 | 101.32 | 99.16 |
| S04 | -0.32 | 0.01 | -0.30 | 0.01 | -0.27 | 0.01 | 101.03 | 100.45 |
| S05 | -0.46 | 0.02 | -0.36 | 0.02 | -0.38 | 0.02 | 100.07 | 100.41 |
| S06 | -0.65 | 0.03 | -0.56 | 0.03 | -0.56 | 0.02 | 100.51 | 100.25 |
| S08 | -0.37 | 0.02 | -0.30 | 0.01 | -0.29 | 0.01 | 100.43 | 102.66 |
| Average | -0.53 | 0.02 | -0.47 | 0.02 | -0.43 | 0.01 | 100.64 | 100.47 |

Unfortunately, no complexity reduction can be identified in this measure. However, in the proposed method, the number of operation is actually reduced because there is no z-buffering, which requires the comparison operation. And memory reduction is also achieved. When using quarter resolution 8-bit depth map on a full HD sequence, the memory reduction is at least 960 bytes.

Besides the complexity reduction, the proposed method always achieves some improvement on the coding efficiency. The maximum gain on texture coding is 1.12% for Undo_Dancer sequence.

# Conclusions

Based on the results that complexity reductions with some performance improvements, we recommend adopting the proposed warping process into 3DV-ATM and WD.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Shinya Shimizu, Hideaki Kimata, Li Zhang, Yu Yang, Ying Chen, Jin Young Lee, and Jaejoon Lee, "3D-CE1.a related: results on joint proposal of NTT, Qualcomm and Samsung", ISO/IEC JTC1/SC29/WG11 MPEG2012/M25915, April 2012, Geneva, Switzerland.

- Shinya Shimizu and Hideaki Kimata, "3D-CE1.a related: Simplification of in-loop view synthesis with fractional-pel disparity," ISO/IEC JTC1/SC29/WG11 MPEG2012/M24796, April 2012, Geneva, Switzerland.

- Li Zhang, Ying Chen, Yang Yu, Karthic Veera, and Marta Karczewicz, "3D-CE1.a related: View synthesis based on asymmetric texture and depth resolutions," ISO/IEC JTC1/SC29/WG11 MPEG2012/M24941, April 2012, Geneva, Switzerland.

- ISO/IEC JTC1/SC29/WG11, "Working Draft 2 of AVC compatible video with depth", ISO/IEC JTC1/SC29/WG11 N12732, May 2012, Geneva, Switzerland.

- ISO/IEC JTC1/SC29/WG11, “Common test conditions for 3DV experimentation”, ISO/IEC JTC1/SC29/WG11 N12745, May 2012, Geneva, Switzerland.