| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0025 |
| --- | --- |

| Title: | 3D-CE1.a related: Cross-check report of JCT2-A0036 on dilation based view synthesis prediction without upsampling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Shinya Shimizu 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, 2390837, Japan | Tel: Email: | +81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the dilation based view synthesis prediction without upsampling in JCT2-A0036. The verification task was done successfully, and the coding results matched those proposed by Samsung. The simulation result reportedly shows the bitrate savings of 0.21% in terms of texture coding over all the sequences.

# Introduction

The contribution JCT2-A0036 proposed a dilation based view synthesis prediction without upsampling in the AVC-based 3D video coding category. In this cross-check report, the simulations are conducted with the same software used in [1].

# Cross-check results

The simlations are conducted on the common test condiciton [2]. The provided source code was compiled on 64-bit Windows 7 environment by using Microsoft Visual Studio 2008 Standard Edition, and the experiments were performed on the cluster of PCs with 64-bit Windows 7.

No mismatch is identified between encoder and decoder. The results matched those provided by the proponents. The obtained results are summarized in Error: Reference source not found.

TABLE 1: Results of cross-check simulation

|  | Texture Coding |  | Total (Coded PSNR) |  | Total (Synthesized PSNR) |  | Complexity (ratio to anchor) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Enc. Time, % | Dec. Time, % |
| S01 | -0.33 | 0.01 | -0.30 | 0.01 | -0.25 | 0.01 | 100.63 | 102.92 |
| S02 | -0.28 | 0.01 | -0.24 | 0.01 | -0.29 | 0.01 | 100.46 | 103.73 |
| S03 | 0.12 | 0.00 | 0.12 | 0.00 | -0.06 | 0.00 | 100.97 | 103.43 |
| S04 | -0.46 | 0.02 | -0.43 | 0.02 | -0.46 | 0.01 | 99.50 | 102.51 |
| S05 | -0.19 | 0.01 | -0.15 | 0.01 | -0.16 | 0.01 | 99.56 | 104.20 |
| S06 | -0.24 | 0.01 | -0.19 | 0.01 | -0.24 | 0.01 | 100.65 | 104.34 |
| S08 | -0.06 | 0.00 | -0.05 | 0.00 | -0.06 | 0.00 | 100.48 | 106.84 |
| Ave. | -0.21 | 0.01 | -0.18 | 0.01 | -0.22 | 0.01 | 100.32 | 103.99 |

# Conclusion

The results obtained during this cross-check match those provided by the proponents. Experimental result reportedly shows that the bitrate savings in terms of texture coding over all the sequences is -0.21%. Source code was not reviewed in details due to the lack of time.

# References

- Seok Lee, Seungsin Lee, Ho-Cheon Wey, Jin Young Lee, and Jaejoon Lee, "3D-CE1.a related results on dilation based view synthesis prediction without upsampling", JCT2-A0036, Stockholm, July 2012.

- ISO/IEC JTC1/SC29/WG11, “Common test conditions for 3DV experimentation”, ISO/IEC JTC1/SC29/WG11 N12745, Ganeva, May 2012.