| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0026 |
| --- | --- |

| Title: | On Slice Header and Parameter Set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Jaehyun Kim Jeonghoon Park | Email: | b.d.choi@samsung.com jhgim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Slice header prediction and parameter set sharing are proposed between texture and depth map inside a view component. Additionally, we propose the parameter set sharing between the texture and the depth map.

# Introduction

In the MVD format, the parameter sets and slice header can be predicted from the texture to the depth due to their dependency and similarity. In the last meeting, the short slice header for depth map was proposed in m24872. The short slice header may share syntax elements of the texture slice header and the realization of slice header prediction is similar to the slice header of an entropy slice. In HEVC, the lightweight design of the entropy slice header has proved its efficiency in terms of coding gain. In this proposal, the syntax elements sharing methods are proposed for both slice header and parameters.

# Proposal

## Slice header prediction

The slice header design has more impacts on the coding efficiency than other parameters, because the slice headers are very frequently signaled as mandatory. The design philosophy is almost same with the lightweight slice header of the entropy slice in HEVC. The syntax elements in the depth map slice header, such as DPB status (RPS), the reference index, slice QP delta and so on, can be predicted from the texture slice header in the same view and the same access unit. short_slice_header_flag indicates whether the value of slice header syntax elements is present or not. When short_slice_header_flag equal to 1, the syntax element in a depth map slice header is inferred to be equal to the value of slice header syntax elements in the corresponding texture slice. As an additional constraint, the slices of the depth map picture shall be equally partitioned with the slices of the texture picture. Thus, the partitioning size of CTUs in depthmap slices shall be equal to the partitioning size of the corresponding texture slices. It is because the depth map data is supplementary data for decoding and rendering the intermediate views. If the slice partitioning is not aligned, the progress decoding of pictures could be impossible.

## Parameter set prediction from texture to depth

Instead of syntax element prediction from the parameter set of texture to the parameter set of depth, the parameter set sharing seems more practical. In most cases, the value of syntax elements in a depth map parameter set are equal to the value of syntax elements in the corresponding texture parameter set. If the pic_parameter_set_id of a depth map slice is equal to the pic_parameter_set_id of a texture map slice, it means that both texture and depth map slices are referring to the same picture parameter set. If the separation of the parameter set reference between texture and depth map for independent parameter parsing is desired, depth map slices and parameter sets should not refer to the texture parameter set. However, all texture slices and parameters cannot refer to the parameter sets of depth map NAL units for the texture only extraction.

## Parameter set prediction from base view to non-base views

The syntax elements prediction between view components is not encouraged for independent parsing and decoding of each view component. In the practical implementation, the syntax element prediction between view components needs more complicated hardware design than the reconstructed picture referencing. Especially, the parameter set prediction between view components makes the reuse of the legacy HEVC chip for the simple multiview extension difficult.

| slice_header_mvd ( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| if(depth_flag) { |  |
| short_slice_header_flag | u(1) |
| } |  |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| if( !(entropy_slice_flag ¦¦ short_slice_header_flag) ) { |  |
| pic_parameter_set_id | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } else { |  |
| pic_order_cnt_lsb | u(v) |
| ……………… |  |
| } |  |
| if( cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| if( !(entropy_slice_flag ¦¦ short_slice_header_flag) ) { |  |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| ……………… |  |
| } |  |

depth_flag equal to 1 specifies that this slice represents a depth map. depth_flag equal to 0 specifies that this slice represents a texture.

short_slice_header_flag equal to 1 specifies that the value of slice header syntax elements not present is inferred to be equal to the value of slice header syntax elements in the corresponding texture slice which has the same view_id. When not present, depth_flag shall be inferred to be 0.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).