| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0035 |
| --- | --- |

| Title: | 3D-CE1.a results on dilation based view synthesis prediction with upsampling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seok Lee, Seungsin Lee, Hocheon Wey, Jaejoon Lee San 14, Nongseo-dong, Giheung-gu, Yongin-si, Gyeonggi-do | Tel: Email: | 82-31-280-6696 lee.seok@samsung.com |
| Source: | Samsung Electronics Co., Ltd . |  |  |

_____________________________

# Abstract

Dilation based view synthesis prediction with upsampling is proposed and its performance evaluation result when integrated on 3D-ATM 0.4 is reported.

# Algorithm description

Proposed depth resampling is applied to in-loop filtering part, where upsampled depth is used to view synthesis prediction or depth based motion vector prediction. In proposed method, first depth is upsampled to texture resolution with linear interpolation and then 3x3 grey scale dilation filter is applied to the upsampled result (fig. 2.1). This can be done by taking maximum value of neighbouring pixels. Most synthesis artifacts are occurred in edge region where foreground and background are met. Especially when depth is upsampled by linear interpolation, the edge of upsampled depth is varies smoothly and this can result synthesis artifact at edge regions. By applying dilation filter, foreground region of depth is extended to background regions, and synthesis artifact occurs out of boundary edge. Thus objective and subjective quality of view synthesis result is improved.

Fig. 2.1 Proposed resampling using dilation filter

# Test results

Test results are summarized in Table 2.1. These tables show BD-rate gain in terms of PSNR of decoded views over texture + depth bit rate and BD-rate gain in terms of PSNR of synthesized views over texture + depth bit rate, respectively when Samsung’s in-loop resampling is enabled. In current ATM v0.4 software, if asymmetric view synthesis prediction from Qualcomm is turned on, depth upsampling is skipped in VSP process. Thus to apply inloop depth upsampling, asymmetric VSP is turned off in this test. The ATM v0.4 result with CTC where asymmetric VSP on is used as anchor.

Table 2.1 Test results for proposed dilation based depth resampling

| Sequences | BD-rate with PSNR of decoded views over texture + depth bit rate | BD-rate with PSNR of synthesized views over texture + depth bit rate |
| --- | --- | --- |
| Poznan Hall2 | 0.02 | -0.05 |
| Poznan Street | -0.44 | -0.44 |
| Undo Dancer | -0.61 | -0.68 |
| GT Fly | -0.01 | -0.05 |
| Kendo | -0.15 | -0.19 |
| Balloons | -0.39 | -0.39 |
| Newspaper | -0.03 | -0.01 |
| Average | -0.23 | -0.26 |
| Encoding Time (%) | 100.53% |  |
| Decoding Time (%) | 152.60% |  |

# Summary

Dilation based view synthesis prediction with upsampling was integrated on 3D ATM 0.4 and performance evaluation was shown. Proposed method achieves -0.23% BD-rate gain in terms of decoded texture PSNR, and -0.26% BD-rate gain in terms of synthesized PSNR. The decoding time is 152.6% which include proposed resampling complexity and full resolution warping complexity when asymmetric VSP off.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).