| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0038 |
| --- | --- |

| Title: | 3D-CE3.a results on dilation filter for depth post processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seok Lee, Seungsin Lee, Hocheon Wey, Jaejoon Lee San 14, Nongseo-dong, Giheung-gu, Yongin_si, Gyeonggi-do | Tel: Email: | 82-31-280-6696 lee.seok@samsung.com |
| Source: | Samsung Electronics Co., Ltd . |  |  |

_____________________________

# Abstract

A depth post filtering with the dilation filter is proposed and its performance evaluation results when integrated on 3D-ATM 0.4 are reported.

# Algorithm description

The proposed post dilation filtering is applied to post processing part, where decoded depth is upsampled to texture resolution for output depth writing. The proposed filter first upsamples depth with linear interpolation within the test model and then applies grey scale dilation filter to the upsampled result (fig. 2.1). The dilation filter is implemented by taking a maximum value among neighbouring pixels. Most synthesis artifacts are occurred in edge regions where foreground and background areas are met. Especially when a depth image is upsampled by a linear interpolation method, edges in the upsampled depth image vary smoothly, which results in synthesis artifacts around edge regions.

By applying the proposed dilation filter, foreground regions of depth are extended to background regions, and synthesis artifacts occur out of boundary edge. Hence, the objective and subjective quality of view synthesis result is improved. In the experiment, 3x3 dilation filter are used for all of HD sequences and SD sequences. The filter size may be determined by input resolution in encoder and decoder respectively and no additional information is transmitted.

Fig. 1.1 Proposed post filtering using dilation filter

# Test results

Test results are summarized in Table 3.1 to Table 3.2. These tables show BD-rate gains in terms of PSNR of synthesized views over texture + depth bit rate and BD-rate gain in terms of PSNR of decoded views over texture + depth bit rate respectively when the proposed post filtering is enabled. The decoding complexity is measured with YUV file writing processes turned on for all test cases. Note that the post filter is disabled if the YUV output file writing process is turned off.

Though it increases the overall decoding time as reported in the tables, it does not implicate that the proposed post filter increases the complexity of normative decoding processes. That is because the post process is implemented in the output writing routine after the depth is completely decoded. The post filtering process is separated from the in-loop and normative decoding processes. The BD-rate of decoded views proves that the post filter is independent of the normative decoding processes.

Table 3.1 Test results for dilation filter with delocking filter on

| Sequences | BD-rate with PSNR of synthesized views over texture + depth bit rate | BD-rate with PSNR of decoded views over texture + depth bit rate |
| --- | --- | --- |
| Poznan Hall2 | -0.37 | 0.00 |
| Poznan Street | -1.97 | 0.00 |
| Undo Dancer | -19.25 | 0.00 |
| GT Fly | -7.95 | 0.00 |
| Kendo | -0.51 | 0.00 |
| Balloons | -0.03 | 0.00 |
| Newspaper | -1.17 | 0.00 |
| Average | -4.47 | 0.00 |
| Encoding Time (%) | 100 % |  |
| Decoding Time (%) | 110.21% |  |

Table 3.2 Test results for dilation filter with delocking filter off

| Sequences | BD-rate with PSNR of synthesized views over texture + depth bit rate | BD-rate with PSNR of decoded views over texture + depth bit rate |
| --- | --- | --- |
| Poznan Hall2 | -0.11 | 0.00 |
| Poznan Street | -1.85 | 0.00 |
| Undo Dancer | -19.05 | 0.00 |
| GT Fly | -8.35 | 0.00 |
| Kendo | -0.31 | 0.00 |
| Balloons | 0.03 | 0.00 |
| Newspaper | -0.77 | 0.00 |
| Average | -4.35 | 0.00 |
| Encoding Time (%) | 100% |  |
| Decoding Time (%) | 108.73% |  |

# Examples of subjective evaluation

The dilation-based post filter improves visual quality of synthesized views around edge regions. Fig. 4.1 and 4.2 demonstrate examples of subjective evaluations when the proposed post filter is applied to the decoded depth maps, and they show synthesized views at viewpoint of 4 of Undo dancer R4 for anchor and proposed results. The artifacts around head and arm in (a) are reduced in proposed result in (b).

(a) anchor 	(b) proposed post filter applied

Fig. 4.1 Examples of subjective evaluation at the 94th frame of R4 rate point (Deblocking On)

(a) anchor 	(b) proposed post filter applied

Fig. 4.1 Examples of subjective evaluation at the 53th frame of R4 rate point (Deblocking Off)

# Conclusion

A depth post filtering using dilation filter was integrated on 3D ATM 0.4 and performance evaluation was reported. The proposed post filter achieves -4.47% BD-rate gain in terms of synthesized texture PSNR with deblocking filter turned on, and -4.35% BD-rate gain with deblocking filter off. The post filtering with dilation filter also improves visual quality of synthesized views at boundary regions.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] ISO/IEC JTC1/SC29/WG11 “3D-CE3 results on in-loop depth resampling by Samsung,” M24821, May. 2012.

[2] ISO/IEC JTC1/SC29/WG11 “Common Test Conditions for 3DV experimentation,” N12745, May 2012.

[3] ISO/IEC JTC1/SC29/WG11 “Test Model for AVC based 3D video coding,” N12558, March 2012.