| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0039 |
| --- | --- |

| Title: | 3D-CE3.a related results on depth hybrid post filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungsin Lee, Seok Lee, Hocheon Wey, Jaejoon Lee San 14, Nongseo-dong, Giheung-gu, Yongin_si, Gyeonggi-do | Tel: Email: | 82-31-280-6687 Seungsin.lee@samsung.com |
| Source: | Samsung Electronics Co., Ltd . |  |  |

_____________________________

# Abstract

A related result on depth post filtering is reported using a depth hybrid post filter. The post filter is implemented by a hybrid method utilizing a dilation filter and a filter based on a local segmentation. The report also demonstrates that coding performance can be improved by identifying the type of depth maps using a proper method and by applying the proposed hybrid filter.

# Algorithm description

The depth hybrid post filter is mainly composed of two parts. It first determines the type of a decoded depth map. The depth type detector selects a proper filtering option by detecting a type of the input depth map. According to the detected depth type, the hybrid post filter applies a different filtering approach. The hybrid filter proposed here uses the dilation filter suggested in M24821 ([1]) and the segmentation filter.

Fig. 1.1 Proposed post filtering using dilation filter

2.1. Depth type detector

When a 3D MVD sequence is encoded, the encoder compares the first color texture frame and depth map frame in the input sequence. Using the correlation between edge maps of color textures and depth maps, it computes whether the edges are well matched or not. If the edge map between color textures and depth maps are highly correlated, it considers the depth map is closed to the ground truth depth map. Such  a depth map might have been synthesized from the 3D graphic models as in S03 and S04 of test sequences. In that case, the encoder set a depth type flag to one. When a decoder encounters the corresponding flag with one, it applies the dilation filter as the post filter.

On the contrary, depth maps estimated from color texture images as in test sequences other than S03 and S04, encoder sets the depth type flag to zero. Then, the decoder applies the proposed segmentation filter instead of the dilation filter.

2.2. Dilation filter

The dilation post filter was first introduced in M24821 ([1]).It is implemented by taking a maximum value among neighbouring pixels. Most synthesis artifacts are occurred in edge regions where foreground and background areas are met. Especially when a depth image is upsampled by a linear interpolation method, edges in the upsampled depth image vary smoothly, which results in synthesis artifacts around edge regions.

2.3. Segmentation filter

The segmentation filter employs a simple clustering algorithm in a local region around a filtered depth pixel. Depth maps without noise usually have less texture, and clear edges around objects. Within a small size of a local block, the depth map can be clearly divided into a small number of sub- regions.

The proposed segmentation filter divides a local block into a given number of sub-regions by applying a simple clustering algorithm such as k-mean algorithm. The overall algorithm is briefly depicted in Fig. 1. The representative values of each sub-region may be a mean pixel value as used in the proposed algorithm. Because the filter segments a local region into a given number of sub-regions around edges, it preserves edges. Furthermore, since it is able to merge sub-regions according to the differences between adjacent sub-regions, it is possible to reduce noise around edges without blurring object edges.

In the proposed filter, the segmentation algorithm is applied on 1-D local block vertically, and horizontally to reduce the computational complexity as illustrated in Fig. 2. The 1-D segmentation first is applied horizontally with 1x5 1-D local block. Initially, it divides the local block into two sub-regions, and calculates the mean values of the sub regions. When the iteration is completed, the current pixel is substituted by a closer mean value. If the difference between the two sub-region mean values is less than a given threshold, the two regions is merged, and the filtered pixel is changed into a mean value of the whole 1-D local block. The last process is to reduce excessive segmentation which can be caused by coding noise such as block noise. The same procedure is implemented again vertically.

The filtering approach is somewhat similar to the approach suggested by Qualcomm in M23810 ([1]). However, in the suggested segmentation filter approach, it is able to reduce the noise at the same time while enhancing object edges by applying the segmentation filter with merging process. The merging process also reduces the necessary of additional smoothing filter to remove the ringing noise. The computational complexity increased by applying the suggested filter on all the pixels instead of edges only can be reduced by employing separable 1D filter structure as in Fig. 2.

Set a local Block around a filtered pixel

Calculate the distance between each pixel value in the bloc and the representative value

Classify the pixels into a sub-region which has the minimum distance

Do the representative values converge?

Does iteration number reach a given number?

If a difference between the representative values is smaller than a given threshold, merge the sub-regions into one

Initialize the number of sub-regions and their representative value

No

Yes

No

Replace the current pixel value into the closest representative value

Recalculate the representative values of sub-regions.

Fig. 1. Overall block diagram of the segmentation filter.

2 . Mean value computation

1. Segmentation

3. Pixel substitution

Row-wise Process

Column-wise Process

Output

1. Segmentation

2 . Mean value computation

3. Pixel substitution

Fig. 2. 1-D separable segmentation filter

# Test results

The results reported in the following tables suggest that the hybrid filtering approach can improve the performance with additional depth type identification.

Table 3.1 Test results for hybrid filter with delocking filter on

| Sequences | BD-rate with PSNR of synthesized views over texture + depth bit rate | BD-rate with PSNR of decoded views over texture + depth bit rate |
| --- | --- | --- |
| Poznan Hall2 |  |  |
| Poznan Street |  |  |
| Undo Dancer |  |  |
| GT Fly |  |  |
| Kendo |  |  |
| Balloons |  |  |
| Newspaper |  |  |
| Average |  |  |
| Encoding Time (%) |  |  |
| Decoding Time (%) |  |  |

Table 3.2 Test results for hybrid filter with delocking filter off

| Sequences | BD-rate with PSNR of synthesized views over texture + depth bit rate | BD-rate with PSNR of decoded views over texture + depth bit rate |
| --- | --- | --- |
| Poznan Hall2 |  |  |
| Poznan Street |  |  |
| Undo Dancer |  |  |
| GT Fly |  |  |
| Kendo |  |  |
| Balloons |  |  |
| Newspaper |  |  |
| Average |  |  |
| Encoding Time (%) |  |  |
| Decoding Time (%) |  |  |

# Examples of subjective evaluation

# Conclusion

A depth post filtering using the hybrid filter was integrated on 3D ATM 0.4 and performance evaluation was reported.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] ISO/IEC JTC1/SC29/WG11 “3D-CE3 results on in-loop depth resampling by Samsung,” M24821, May. 2012.

[2] ISO/IEC JTC1/SC29/WG11 “Common Test Conditions for 3DV experimentation,” N12745, May 2012.

[3] ISO/IEC JTC1/SC29/WG11 “3D-AVC-CE10 on Depth In-loop Filtering by Qualcomm,” M23810, February 2012.