| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0039 |
| --- | --- |

| Title: | 3D-CE3.a Related results on depth hybrid post filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungsin Lee, Seok Lee, Hocheon Wey, Jaejoon Lee San 14, Nongseo-dong, Giheung-gu, Yongin_si, Gyeonggi-do | Tel: Email: | 82-31-280-6687 Seungsin.lee@samsung.com |
| Source: | Samsung Electronics Co., Ltd . |  |  |

_____________________________

# Abstract

Related results on the depth post filtering is reported using a depth hybrid post filter. The post filter is implemented by a hybrid method utilizing a dilation filter and a filter based on local segmentation. The report also demonstrates that coding performance can be improved by identifying the type of depth maps using a proper method and by applying the proposed hybrid filter.

# Algorithm description

The depth hybrid post filter is mainly composed of two parts. The first it determines the type of a decoded depth map. The depth type detector selects a proper filtering option by detecting a type of the input depth map. According to the detected depth type, the hybrid post filter applies different filtering approaches. The hybrid filter proposed here uses the dilation filter suggested in M24821 ([1]) and the segmentation filter.

Fig. 1.1 Proposed post filtering using dilation filter

2.1. Depth type detector

The depth data estimated by depth estimation algorithms using color images or measured from depth sensors such as TOF cameras have different characteristics from the graphics depth data. For graphics data the exact depth information is known and edge correlation between texture and depth images is very high. On the other hand, accuracy of the estimated and measured depth is lower than graphics data, and the edge is not well matched with corresponding texture data. In addition, the depth estimation is generally implemented based on disparity whose degree of accuracy is usually half pel or quarter pel. The lack of accuracy of the depth estimation results in discrete changes of depth values even in flat regions unlike graphics depth data which continuously varies in non edge regions.

For these different characteristics of depth data, it is helpful to synthesize higher quality virtual views if a decoder knows the type of transferred depth maps. In the proposed post filter, the depth type is classified in encoder and a different filter is applied for post processing. In the encoder, edge maps are obtained from the first texture frame and depth frame of the base view of an input sequence. After then the normalized cross correlation between the texture edge map and depth edge map is calculated. Using a proper threshold for the correlation, the input data is classified to flag 0 (measured or estimated depth) or flag 1 (graphics depth). This flag bit is sent to decoder in SPS. Accordingly, the test sequence S03 and S04 are set to be depth type 1, and others are set to be type 0. In decoder side, the dilation filter is applied to type 1 depth maps, and the proposed segmentation filter is applied to type 0 depth maps. This flag information of depth can also be send as SEI messages.

2.2. Dilation filter

The dilation post filter was first introduced in M24821 ([1]). It is implemented by taking a maximum value among neighbouring pixels. Most synthesis artifacts are occurred in edge regions where foreground and background areas are met. Especially when a depth image is upsampled by a linear interpolation method, edges in the upsampled depth image vary smoothly, which results in synthesis artifacts around edge regions. By applying the proposed dilation filter, foreground regions of depth are extended to background regions, and synthesis artifacts occur out of boundary edge. Hence, the objective and subjective quality of view synthesis results is improved.

2.3. Segmentation filter

The segmentation filter employs a simple clustering algorithm in a local region around a filtered depth pixel. Depth maps without noise usually have less texture, and clear edges around objects. Within a small size of a local block, the depth map can be clearly divided into a small number of sub- regions.

The proposed segmentation filter divides a local block into a given number of sub-regions by applying a simple clustering algorithm such as k-means algorithm. The overall algorithm is briefly depicted in Fig. 1. The representative values of each sub-region may be a mean pixel value as used in the proposed algorithm. Because the filter divides a local region into a given number of sub-regions around edges, it can preservs and enhance blur edges. Furthermore, since the proposed segmentation filter is able to merge sub-regions according to the differences between adjacent sub-regions, it is possible to reduce noise around edges without blurring object edges.

In the proposed filter, the segmentation algorithm can be applied on 1-D local block vertically and horizontally as well as 2D. By this separable operation it is possible to reduce the computational complexity as illustrated in Fig. 2. The 1-D segmentation first is applied horizontally with 1x5 1-D local block. Initially, it divides the local block into two sub-regions, and calculates the mean values of the sub regions. When the iteration is completed, the current pixel is substituted by a closer mean value. If the difference between the two sub-region mean values is less than a given threshold, the two regionssare merged, and the filtered pixel is changed intoathe mean value of the whole 1-D local block. The last process is to reduce excessive segmentation which can be caused by coding noise such as block noise. The same procedure is implemented again vertically.

In order to reduce computational complexity further, the 1-D segmentation filter can be applied on the horizontal direction only as used for this report. That is a reasonable option for the current view synthesis reference software using horizontal disparity only.

The filtering approach is similar to the approach suggested by Qualcomm in M23810 ([3]) in some respects because it also uses a clustering algorithm. Unlike the Qualcomm's approach, the suggested segmentation filter in this report mainly uses 1D separable filtering approach as described in Fig. 2, and is able to reduce the computational complexity by choosing row-wise filtering only mode. It is also possible to reduce noise while enhancing object edges with merging process. The merging process also reduces the necessary of the additional smoothing filter to remove the ringing noise as used in [3].

Set a local bBlock around a filtered pixel

Calculate the distance between each pixel value in the block and the representative value

Classify the pixels into a sub-region which hass the minimum distance

Do the representative values converge?

Does iteration number reach a given number?

If athe difference between the representative values is smaller than a given threshold, merge the sub-regions into one

Initialize the number of sub-regions and their representative value

No

Yes

No

Replace the current pixel value into the closest representative value

Recalculate the representative values of sub-regions.

Fig. 1. Overall block diagram of the segmentation filter.

2 . Mean value computation

1. Segmentation

3. Pixel substitution

Row-wise Process

Column-wise Process

Output

1. Segmentation

2 . Mean value computation

3. Pixel substitution

Fig. 2. 1-D separable segmentation filter

# Test results

The results reported in the following tables suggest that the hybrid filtering approach can improve the performance with additional depth type identification.

Table 3.1 Test results for hybrid filter with delocking filter on

| Sequences | BD-rate with PSNR of synthesized views over texture + depth bit rate | BD-rate with PSNR of decoded views over texture + depth bit rate |
| --- | --- | --- |
| Poznan Hall2 | -2.49 | 0.00 |
| Poznan Street | -1.86 | 0.00 |
| Undo Dancer | -19.25 | 0.00 |
| GT Fly | -7.95 | 0.00 |
| Kendo | -1.85 | 0.00 |
| Balloons | -2.28 | 0.00 |
| Newspaper | -4.16 | 0.00 |
| Average | -5.69 | 0.00 |
| Encoding Time (%) | - |  |
| Decoding Time (%) | 193.7% |  |

Table 3.2 Test results for hybrid filter with delocking filter off

| Sequences | BD-rate with PSNR of synthesized views over texture + depth bit rate | BD-rate with PSNR of decoded views over texture + depth bit rate |
| --- | --- | --- |
| Poznan Hall2 | -2.49 | 0.00 |
| Poznan Street | -1.86 | 0.00 |
| Undo Dancer | -19.05 | 0.00 |
| GT Fly | -8.35 | 0.00 |
| Kendo | -1.71 | 0.00 |
| Balloons | -2.27 | 0.00 |
| Newspaper | -4.08 | 0.00 |
| Average | -5.69 | 0.00 |
| Encoding Time (%) | - |  |
| Decoding Time (%) | 177.6% |  |

- Examples of subjective evaluation

The hybrid post filter improves visual quality of synthesized views around edge regions. Fig. 3.1 and 3.2 demonstrate examples of subjective evaluations when the proposed post filter is applied to the decoded depth maps, and they show synthesized views at viewpoint of 4 of Undo dancer R4 for anchor and proposed results. The artifacts around head and arm in (a) are reduced in proposed result in (b). The results is the same as the results in Samsung' report ([4]) because the depth type selector chooses the dilation filter for this specific sequence.

(a) anchor 	(b) proposed post filter applied

Fig. 3.1 Examples of subjective evaluation at the 94th frame of R4 rate point (Deblocking On)

(a) anchor 	(b) proposed post filter applied

Fig. 3.1 Examples of subjective evaluation at the 53th frame of R4 rate point (Deblocking Off)

# Conclusion

A depth post filtering using a hybrid filter was integrated on 3D ATM 0.4 and performance evaluation was reported. The proposed post filter achieves -5.69% BD-rate gain in terms of synthesized texture PSNR with deblocking filter turned on, and -5.69% BD-rate gain with deblocking filter off.  About 193.7% and 177.6 % of decoder complexity was observed respectively.

- Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] ISO/IEC JTC1/SC29/WG11 “3D-CE3 results on in-loop depth resampling by Samsung,” M24821, May. 2012.

[2] ISO/IEC JTC1/SC29/WG11 “Common Test Conditions for 3DV experimentation,” N12745, May 2012.

[3] ISO/IEC JTC1/SC29/WG11 “3D-AVC-CE10 on Depth In-loop Filtering by Qualcomm,” M23810, February 2012.

[4] ISO/IEC JTC1/SC29/WG11 “3D-CE3.a Results on dilation filter for depth post processing by Samsung,” JCT2-A0038, July 2012.