| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0040 |
| --- | --- |

| Title: | 3D-CE5.a related results on improved skip and direct motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee Jaejoon Lee Du-Sik Park Tadashi Uchiumi Yoshiya Yamamoto | Email: | jinyoung79.lee@samsung.com jaejoon1.lee@samsung.com uchiumi.tadashi@sharp.co.jp |
| Source: | Samsung Electronics Co. Ltd. and Sharp Corporation |  |  |

_____________________________

# Abstract

A depth-based motion vector prediction method (M22552) was integrated on the current 3DV-ATM with several improvements (M24824). A competition-based skip and direct motion vector prediction method (M24847) was proposed at the last meeting. This proposal presents an improved skip and direct motion vector prediction on the top of the current depth-based motion vector prediction and the new competition-based skip and direct motion vector prediction. It is demonstrated that the proposed method improves the coding efficiency and reduces the decoding complexity.

# Algorithm Description

- To find a motion vector of skip and direct modes in the current depth-based motion vector prediction, the corresponding depth map is used. The skip and direct motion vector is decided to the motion vector which has the minimum disparity SAD through competition of the neighboring motion vectors around the current block Cb. SAD is calculated between the disparities converted from the depth pixels within the corresponding depth block Disp(Cb) and the motion compensated block Disp(Cb, MVi) from the neighboring motion vector (i=A, B, or C).

SAD=y=0,x=0y=15,x=15(Dispy,x(Cb)−Dispy,x(Cb,MVi))

where y and x indicate vertical and horizontal locations on a macroblock, respectively.

For the simple operation, the proposed method calculates SAD between the disparities as follows:

SAD=y=0,x=0y=15,x=15(Dispy,x(Cb)−Dispy,x(Cb,MVi) =y=0,x=0y=15,x=15(TempDispCoeff⋅Dy,x(Cb)−TempDisp⋅Coeff⋅Dy,x(Cb,MVi) =y=0,x=0y=15,x=15(TempDispCoeff⋅(Dy,x(Cb)−Dy,x(Cb,MVi)) ≈y=0,x=0y=15,x=15((Dy,x(Cb)−Dy,x(Cb,MVi)>>DispCoeff)

In the 1D camera setting, the disparity can be converted from the depth as follows:

Disp(Cb)=TempDispCoeff⋅D(Cb)TempDispCoeff=f⋅L255⋅(1Znear−1Zfar)

where f is the focal length, L is the baseline between the current and the intermediate view, Znear and Zfar are the values of the nearest and farthest depth of the scene, respectively. In order to use the simple SHIFT operation, we converted TempDispCoeff to DispCoeff as follow:

DispCoeff=(Int)(log21TempDispCoeff+0.5)

Also, the proposed method uses a max disparity instead of an average disparity associated with Cb, if the final inter-view motion vector for skip and direct modes is a zero motion vector.

- MediaTek’s proposal (JCT2-A0045) replaces the current disparity-based motion vector competition for skip and direct modes (M22552). It employs a disparity vector, which is derived from the neighboring blocks or the corresponding depth blocks, to bring motion information for skip and direct motion vector from the inter-view reference frame. If the neighboring blocks do not have an inter-view motion vector, a disparity for a middle sample in the corresponding depth block is used. The proposed method, which uses a max disparity for skip and direct modes, was applied to MediaTek’s proposal to verify its performance.

# Test Results in M24941-on condition

Test results are summarized in Table 1. The proposed method reduces the decoding time up to 95.94% on average with the small coding gain, compared to the anchor.

Table 1. Summary results

# Test Results in M24941-off condition

Since view synthesis based on asymmetric texture and depth resolutions (M24941) that was implemented in the current software includes a bug, an additional test was performed in the M24941-off condition. The test results are summarized in Table 2. The proposed method reduces the decoding time by 96.62% on average with the bit saving of about -0.2%, compared to the anchor.

Table 2. Summary results

# Test Results on the top of JCT2-A0045

An additional test was performed on the top of MediaTek’s program (JCT2-A0045). The test results are summarized in Table 3. The proposed method achieves the bitrate reduction of about -0.2% on average, when MediaTek’s proposal (MVP list size=1) is an anchor.

Table 3. Summary results

# Conclusion

The improved skip and direct motion vector prediction was proposed. By integrating the proposed method on the current 3DV-ATM, it achieved the reduced decoding time with the small coding gain. In addition, it was verified that the proposed method can be well applicable to MediaTek’s proposal (JCT2-A0045), which is the new prediction method for skip and direct modes.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).