| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0044 |
| --- | --- |

| Title: | Depth Quadtree Prediction for HTM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Jung, K. Viswanathan 38 Rue Général Leclerc 92130 Issy-les-Moulineaux France | Tel: Email: | +33145295366 joelb.jung@orange.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

This document presents a modification of the depth quadtree decoding process. For a given CTU, the quadtree of the depth is linked to the collocated CTU quadtree in the texture, in a way that a given CU of the depth cannot be split more than its collocated CU in the texture. The quadtree of the depth is predictively encoded relatively to the quadtree of the texture. A 62% runtime is reported compared to the reference, while BDR gains of 0.1% and 0.9% are observed for synthetized only and coded + synthesized cases respectively, under common test conditions.

# Introduction

HEVC splits a given CTB into smaller CUs, based on RD optimized decisions. A corresponding quadtree (QT) is obtained for the texture, and another one for the depth. The HTM3.0 makes full investigation of every possible configuration, for both texture and depth, and encodes both QTs separately. A modification of the depth quadtree decoding process is proposed to save bits and encoding runtime.

# Proposed method

The proposed method performs a prediction of the depth QT, using the texture QT. Figure 1 shows a simple example of QT partitioning for depth and texture. Table 1 shows how both QTs are represented, before being sent to the entropy coder.

Figure 1: example of depth (left) and texture (right) QTs.

| Texture | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Depth | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  | 1 | 0 | 0 | 0 | 0 |

Table 1: representation of depth and texture QTs corresponding to figure 1.

The proposed method forces the encoder to limit the partitioning of the depth at the same level as the partitioning of the texture. For a given CTU, the quadtree of the depth is linked to the collocated CTB quadtree in the texture, so that a given CU of the depth cannot be split more than its collocated CU in the texture.

The proposed method applies the convention from Table 2 to predict depth from texture. The basic idea is to send nothing if the CU of the texture is not further split, and send one bit to indicate if depth is split or not when the current partition of the texture is further split.

| Texture | Depth | Residual Depth |
| --- | --- | --- |
| 1 | 1 | 1 |
| 0 | 0 | - |
| 1 | 0 | 0 |

Table 2: computation of the residual depth.

| Residual Depth | 1 | 1 |  |  | 1 |  |  |  | 0 |  |  | 0 |  |  |  |  | 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 3: residual depth for figure 1.

# Experimental results

Table 4 and Table 5 report the experimental results of the method under common test conditions, in HTM3.0. The method is disabled on Intra slices.

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Poznan_Hall2 | 0,0% | 0,0% | 0,0% | 0,0% | -0,9% | -1,5% | 62,6% | 104,2% | 100,7% |
| Poznan_Street | 0,0% | 0,0% | 0,0% | 0,0% | -0,5% | -0,8% | 56,8% | 101,2% | 95,1% |
| Dancer | 0,0% | 0,0% | 0,0% | 0,0% | -1,3% | -1,4% | 64,6% | 103,2% | 106,5% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,0% | -0,5% | -0,8% | 63,4% | 105,4% | 103,5% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | -0,6% | 65,6% | 98,2% | 94,9% |
| Balloons | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | -0,5% | 69,0% | 98,6% | 99,9% |
| Newspaper | 0,0% | 0,0% | 0,0% | 0,0% | 0,3% | -0,8% | 54,5% | 104,2% | 102,4% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | -0,6% | 61,2% | 102,8% | 100,7% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,0% | -0,8% | -1,1% | 62,9% | 101,5% | 100,1% |
| Average | 0,0% | 0,0% | 0,0% | 0,0% | -0,4% | -0,9% | 62,2% | 102,1% | 100,4% |

Table 4: experimental results - 3 views test case (negative values are gains)

|  | video 0 | video 1 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- |
| Poznan_Hall2 | 0,0% | 0,0% | 0,0% | -0,6% | -1,3% |
| Poznan_Street | 0,0% | 0,0% | 0,0% | -0,4% | -0,6% |
| Dancer | 0,0% | 0,0% | 0,0% | -1,0% | -1,1% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | -0,1% | -0,6% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,0% | -0,5% |
| Balloons | 0,0% | 0,0% | 0,0% | 0,4% | -0,4% |
| Newspaper | 0,0% | 0,0% | 0,0% | 0,9% | -0,5% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 0,4% | -0,5% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | -0,5% | -0,9% |
| average | 0,0% | 0,0% | 0,0% | -0,1% | -0,7% |

Table 5: experimental results - 2 views test case (negative values are gains)

Table 6 and Table 7 report the experimental results of the method under common test conditions, when the predictive coding of the QT is not applied. The method is disabled on Intra slices. Accurate decoding runtimes should be equal to 100%.

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Poznan_Hall2 | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | -0,8% | 58,5% | 104,4% | 105,2% |
| Poznan_Street | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | -0,4% | 66,0% | 102,0% | 100,0% |
| Dancer | 0,0% | 0,0% | 0,0% | 0,0% | -0,6% | -0,8% | 66,8% | 102,8% | 106,3% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,0% | -0,2% | -0,5% | 50,8% | 99,6% | 93,5% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,0% | 0,4% | -0,1% | 50,9% | 91,9% | 83,2% |
| Balloons | 0,0% | 0,0% | 0,0% | 0,0% | 0,6% | 0,0% | 52,6% | 91,3% | 93,6% |
| Newspaper | 0,0% | 0,0% | 0,0% | 0,0% | 0,7% | -0,3% | 49,5% | 93,4% | 93,6% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 0,0% | 0,6% | -0,2% | 63,6% | 103,0% | 103,8% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,0% | -0,3% | -0,6% | 50,9% | 94,0% | 90,9% |
| Average | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | -0,4% | 56,0% | 97,8% | 96,2% |

Table 6: experimental results without predictive coding- 3 views test case (negative values are gains)

|  | video 0 | video 1 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- |
| Poznan_Hall2 | 0,0% | 0,0% | 0,0% | 0,1% | -0,7% |
| Poznan_Street | 0,0% | 0,0% | 0,0% | -0,1% | -0,3% |
| Dancer | 0,0% | 0,0% | 0,0% | -0,4% | -0,6% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,1% | -0,3% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,4% | -0,1% |
| Balloons | 0,0% | 0,0% | 0,0% | 0,7% | 0,0% |
| Newspaper | 0,0% | 0,0% | 0,0% | 1,0% | -0,3% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 0,7% | -0,1% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | -0,1% | -0,5% |
| average | 0,0% | 0,0% | 0,0% | 0,3% | -0,3% |

Table 7: experimental results without predictive coding - 2 views test case (negative values are gains)

# Conclusion

# A modification of the depth quadtree decoding process is proposed. For a given CTU, the quadtree of the depth is linked to the collocated CTB quadtree in the texture, so that a given CU of the depth cannot be split more than its collocated CU in the texture. The quadtree of the depth is predictively encoded. 62% runtime is achieved compared to the reference, while BDR gains of 0.1% and 0.9% are observed for synthetized only and coded + synthesized cases respectively, under common test conditions.

Given the significant runtime improvement of the encoder jointly with BDR gains, we propose this method for adoption in the HTM and in the working draft.

# References

# Patent rights declaration(s)

Orange Labs may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).