| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0045 |
| --- | --- |

| Title: | 3D-CE5.a results on the motion vector competition-based Skip/Direct mode with explicit signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Lin Chang, Yu-Pao Tsai, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, yulin.chang, yupao.tsai, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of tests T01, T02 and T03 in CE5.a. The goal of this CE is to evaluate the performance of the motion prediction tool proposed in m24847 and the depth-based motion vector competition (DMC) in 3DV-ATM v.4. The anchor used in the CE5.a tests is 3DV-ATM v.4 with DMC off for the skip and direct modes, which falls back to the median-based derivation of the motion vector predictor (MVP) as AVC/H.264. The results are summarized as follows. In experiments T01 and T02, the MV competition-based skip/direct mode with explicit signaling of the MVP index as proposed in m24847 is tested with the size of the MVP list fixed to 1 and 2, respectively. In T01, which fixes the size of the MVP list to 1, 9.2% BD-Rate reduction can be achieved for texture coding and 7.2% BD-Rate reduction can be achieved for synthesized pictures over the total bit-rate (texture + depth). In T02, which fixes the size of the MVP list to 2, BD-Rate reductions of 5.1% and 3.2% are achieved for texture coding and synthesized results, respectively. T03 is conducted to evaluate the performance of DMC for the skip and direct modes. The results show that DMC for the skip and direct modes brings 2.5% and 2.3% BD-Rate reduction for texture coding and synthesized results, respectively.

# Introduction

The goal of this Core Experiment (CE) 5.a [1] is to investigate the motion/mode parameter prediction for texture and depth which is applied to the AVC-based 3D video coding, 3DV-ATM v.4 [2]. The tool proposed by MediaTek (m24847 [3]) and the depth-based motion vector competition (DMC) in 3DV-ATM v.4 are both evaluated according to their impact on the compression efficiency and the computational complexity.

# Proposed method

In m24847 [3], an MVC scheme with explicit signaling of the optimal MVP index to the decoder for the skip and direct modes is proposed. In a predefined derivation order, the candidates set includes an additional inter-view candidate and the three spatial candidates derived from the neighboring blocks A, B, and C (D is used only when C is unavailable) as shown in Figure 1. The size of the MVP list is fixed to a positive integer number N to avoid parsing problems. Only the first N non-redundant candidates are included in the MVP candidate list. If the number of non-redundant candidates is less than N, the MVP candidate list will be filled to N candidates with zero MV. The encoder selects one candidate from the MVP candidate list and explicitly sends the index of the selected candidate to the decoder. Thus, the decoder does not need to repeat the computations to find the best MVP. Moreover, the computation for depth matching and additional memory access for depth fetching in DMC could both be removed. Note that, in the case when N is equal to 1, the signaling of the MVP index is not required.

Figure 1. Inter-view MV and spatial MV derivation.

On the decoder side, the motion compensation is performed according to the motion information of that selected candidate. The motion information includes the prediction direction (uni-direction prediction or bi-direction prediction), the reference picture type (temporal prediction, virtual prediction, or inter-view prediction), and the reference picture index.

Inter-view MV candidate derivation is also shown in Figure 1. The central point of the current block in the dependent view and its disparity vector are used to find the corresponding point in the base view. After that, the MV of the block including the corresponding point in the base view is used as the inter-view candidate of the current block. The disparity vector can be derived from both the neighboring blocks and the depth value of the central point. Specifically, if one of the neighboring blocks has disparity vector (DV), e.g. DVA in Figure 1, the DV is used as the disparity. Otherwise, the depth-based disparity vector which is derived using the depth value of the central point and camera parameters is used. Compared to only using the depth-based disparity vector, DVs from spatial neighbours can partially reduce error propagation in case that the depth value of the central point is unavailable, e.g. the depth image is lost.

# Simulation results

According to the CE5.a descriptions in [1], tests T01 and T02 are conducted to evaluate the performance of the proposed method with MVP list size equal to 1 and 2, respectively. T03 is conducted to compare the performance between DMC on and DMC off. All simulations are conducted under the common test conditions [4] and the anchor used in the CE5.a tests is 3DV-ATM v.4 with DMC off for the skip and direct modes, which falls back to the median-based derivation of the motion vector predictor (MVP) as AVC/H.264. Since the proposed scheme does not affect HP conditions, the tests are only carried out for EHP conditions. The results for the three tests are summarized in TABLE 1. The detailed results are shown in TABLE2, TABLE 3 and TABLE 4.

By comparing T01 and T03, we can see the proposed method outperforms DMC in terms of BD-Rate savings and run time. Additional comparison results between the proposed method and DMC are also provided as shown in TABLE 5. The proposed method (with MVP list size equal to 1) could provide about 7% coding gain over DMC in 3DV-ATM v.4 while the computation for depth matching and additional memory access for depth fetching could both be removed.

TABLE 1 Summarized results (BD-Rate savings) of three tests

|  | Tested method | BD-Ratei | BD-Rateii | Encoding time | Decoding time |
| --- | --- | --- | --- | --- | --- |
| T01 | Proposed method with MVP list size equal to 1 | -9.2% | -7.2% | 96.2% | 98.4% |
| T02 | Proposed method with MVP list size equal to 2 | -5.1% | -3.2% | 99.5% | 96.5% |
| T03 | 3DV-ATM v.4 (DMC is on) | -2.5% | -2.3% | 100.4% | 101.1% |

- Coded texture views over the bitrates of texture

- Coded and Synthesized pictures over the total bitrates (texture + depth)

Table 2 Test results of test T01

Table 3 Test results of test T02

Table 4 Test results of test T03

Table 5 Additional comparison results

* reference: 3DV-ATM v.4 (DMC is on); tested: the proposed method with MVP list size equal to 1.

# Conclusion

An MVC scheme with explicit signaling of the optimal MVP index for the skip and direct modes is proposed to avoid the complexity and memory access bandwidth issue caused by the depth-based motion vector competition (DMC) in 3DV-ATM v.4. The MVC scheme was integrated into 3DV-ATM v.4 and applied to the texture coding. When replacing the DMC by the proposed MVC scheme with the size of the MVP list fixed to 1, the results show that it achieves about 7% BD-rate reduction for texture coding and 5% BD-rate reduction for synthesized pictures over the total bit-rate.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Anthony Vetro and Karsten Müller, “Description of Core Experiments in 3D Video Coding”, ISO/IEC JTC1/SC29/WG11 MPEG2011/N12746, May 2012, Geneva, Switzerland.

[2] 3DV-ATM v.4, http://mpeg3dv.research.nokia.com/svn/mpeg3dv/trunk/

[3] J.-L. Lin, Y.-W. Chen, X. Guo, Y.-L. Chang, Y.-P. Tsai, Y.-W. Huang, S. Lei, “3D-CE5.a related motion vector competition-based Skip/Direct mode with explicit signaling”, ISO/IEC JTC1/SC29/WG11 MPEG2011/M24847, May 2012, Geneva, Switzerland.

[4] Heiko Schwarz, and Dmytro Rusanovskyy, “Common Test Conditions for 3DV experimentation”, ISO/IEC JTC1/SC29/WG11 MPEG2011/N12745, May 2012, Geneva, Switzerland.