Decoding Process:

    - 3DVC inter prediction, inter-view prediction, and view synthesis prediction process

Processes described in this subclause are invoked when decoding P and B macroblock types, and when nal_unit_type is equal to 21.

The function UniformDepth( depthSample, zN, zF ) is specified to return the zValue specified as follows.

zValue = 1 ÷ ( depthSample ÷ 255 * ( 1 ÷ zN – 1 ÷ zF ) + 1 ÷ zF )	(I.8-XX)

The function dispQPel( depthSample, zN, zF, f, dTX, dPX) is specified to return dispQPel specified as follows.

dispQPel = Floor( ( f * dTX ÷ UniformDepth( depthSample, zN, zF ) + dPX ) * 4 )	(I.8-XX)

The function Disparity( depthSample, zN, zF, f, dTX ) is specified to return disparityValue specified as follows.

disparityValue = dispQPel( depthSample, zN, zF, f, dTX, 0)	(I.8-XX)

[Ed.note: depthSample, camera parameters, like dTX, and zN, zF need satisfy certain constraints. Check the following among others and modify the equations above, if needed. dTX is always non-negative? The depth to be encoded, and zN, zF are provided in a right-hand coordinate system? WarpDir should be drived from camer parameters than index?]

When DepthFlag is equal to 0 and dmvp_flag is equal to 1, the variables DepthRefPicList0, DepthRefPicList1 for B slices, and DepthCurrPic are specified as follows. The variable DepthRefPicList0 is specified to consist of the upsampled depth view components of the view component pairs for which the texture view components are in RefPicList0 in the order that RefPicList0[ i ] and DepthRefPicList0[ i ] form a view component pair for any value of i = 0.. num_ref_idx_l0_active_minus1. The variable DepthRefPicList1 is specified for B slices to consist of the upsampled depth view components of the view component pairs for which the texture view components are in RefPicList1 in the order that RefPicList1[ i ] and DepthRefPicList1[ i ] form a view component pair for any value of i = 0.. num_ref_idx_l1_active_minus1. The variable DepthCurrPic is specified to be the upsampled decoded sample array of the depth view component of the view component pair for which the texture view component is the current texture view component. To obtain an upsampled depth view component, the in-loop depth resampling process specified in Error: Reference source not found is invoked.

Subclause H.8.4 is applied with the following the following changes.

–	The following paragraph is added after the paragraph starting with “Outputs of this process are …”:

When nal_unit_type is equal to 21 and DepthFlag is equal to 0, subclause Error: Reference source not found is invoked to decode the sample arrays for each synthetic reference picture included in RefPicListX for X equal to 0 and, if the current slice is a B slice, 1.

–	When nal_unit_type is equal to 21, DepthFlag is equal to 0, and dmvp_flag is equal to 1, subclause J.2 is invoked instead of subclause 8.4.1.

–	When nal_unit_type is equal to 21, DepthFlag is equal to 1, and mb_ivmp_flag is equal to 1, subclause Error: Reference source not found is invoked instead of subclause 8.4.1.

–	The following additional step applied between steps 3 and 4 in subclause 8.4.

–	When (nal_unit_type is equal to 21 and depth_weighted_pred_flag is equal to 1 and (slice_type % 5) is equal to 0 or 3) or (depth_weighted_bipred_flag is equal to 1 and (slice_type % 5) is equal to 1), the derivation process for prediction weights in depth-range-based weighted prediction in subclause I.8.3.1 is invoked.

      - Derivation process for motion vector components and reference indices

Inputs to this process are:

–	a macroblock partition mbPartIdx,

–	a sub-macroblock partition subMbPartIdx.

Outputs of this process are:

–	   luma motion vectors mvL0 and mvL1 and when ChromaArrayType is not equal to 0, the chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags predFlagL0 and predFlagL1,

–	a motion vector count variable subMvCnt.

The specifications of subclause 8.4.1 apply with the following changes.

–	The following paragraphs in subclause 8.4.1:

For the derivation of the variables mvL0 and mvL1 as well as refIdxL0 and refIdxL1, the following applies.

–	If mb_type is equal to P_Skip, the derivation process for luma motion vectors for skipped macroblocks in P and SP slices in subclause 8.4.1.1 is invoked with the output being the luma motion vectors mvL0 and reference indices refIdxL0, and predFlagL0 is set equal to 1. mvL1 and refIdxL1 are marked as not available and predFlagL1 is set equal to 0. The motion vector count variable subMvCnt is set equal to 1.

–	Otherwise, if mb_type is equal to B_Skip or B_Direct_16x16 or sub_mb_type[ mbPartIdx ] is equal to B_Direct_8x8, the derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 in B slices in subclause 8.4.1.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the motion vector count variable subMvCnt, and the prediction utilization flags predFlagL0 and predFlagL1.

are replaced by:

For the derivation of the variables mvL0 and mvL1 as well as refIdxL0 and refIdxL1, the following applies:

–	If mb_type is equal to P_Skip, the following applies:

–	If MbVSSkipFlag is equal to 0, the following applies:

–	If nal_unit_type is equal to 21 and DepthFlag is equal to 0 and dmvp_flag is equal to 1, the depth-based derivation process for luma motion vectors for skipped macroblock in P and SP slices in subclause J.8.3.1.1 is invoked with the output being the luma motion vectors mvL0 and refernece indices refIdxL0, and predFlagL0 is set equal to 1.

–	Otherwise (nal_unit_type is not equal to 21 or DepthFlag is equal to 1 or dmvp_flag is equal to 0), the derivation process for luma motion vectors for skipped macroblock in P and SP slices in subclause 8.4.1.1 is invoked with the output being the luma motion vectors mvL0 and refernece indices refIdxL0, and predFlagL0 is set equal to 1.

–	Otherwise (MbVSSkipFlag is equal to 1), the derivation process for luma motion vectors for VSP skipped macroblock in P and SP slices in subclause J.8.3.1.2 is invoked with the output being the luma motion vectors mvL0 and reference indices refIdxL0, and predFlagL0 is set equal to 1.

–	mvL1 and refIdxL1 are marked as not available and predFlagL1 is set equal to 0. The motion vector count variable subMvCnt is set equal to 1.

–	Otherwise, if mb_type is equal to B_Skip or B_Direct_16x16 or sub_mb_type[ mbPartIdx ] is equal to B_Direct_8x8, the following applies.

–	The variable vspFlag is specified as follows:

vspFlag = ! ( ( mb_type  = =  B_Skip  &&  MbVSSkipFlag  = =  0 )  | |  ( ( mb_type  = =  B_Direct_16x16  | |  sub_type[ mbPartIdx ]  = =  B_Direct_8x8 )  &&  !mb_direct_type_flag ) )

–	If vspFlag is equal to 0 and nal_unit_type is equal to 21 and DepthFlag is equal to 0 and dmvp_flag is equal to 1, the depth-based derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 in B slices in subclause J.8.3.1.3 is invoked with mbPartIdx and subMbPartIdx as the input and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the motion vector count variable subMvCnt, and the prediction utilization flags predFlagL0 and predFlagL1.

–	Otherwise, if vspFlag is equal to 0 either nal_unit_type is not equal to 21 or DepthFlag is equal to 1 or dmvp_flag is equal to 0, the derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 in B slices in subclause 8.4.1.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the motion vector count variable subMvCnt, and the prediction utilization flags predFlagL0 and predFlagL1.

–	Otherwise (vspFlag is equal to 1), the derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 in B slices in subclause J.8.3.1.4 is invoked.

–	The following sentence in subclause 8.4.1:

4.	When predFlagLX is equal to 1, the derivation process for luma motion vector prediction in subclause 8.4.1.3 is invoked with mbPartIdx subMbPartIdx, refIdxLX, and currSubMbType as the inputs and the output being mvpLX.

is replaced by

4.	When predFlagLX is equal to 1 and DepthFlag is equal to 0 and dmvp_flag is equal to 1, the derivation process for luma motion vector prediction in subclause J.8.3.1.5 is invoked with mbPartIdx subMbPartIdx, refIdxLX, and currSubMbType as the inputs and the output being mvpLX. When predFlagLX is equal to 1 and either DepthFlag is equal to 1 or dmvp_flag is equal to 0, the derivation process for luma motion vector prediction in subclause 8.4.1.3 is invoked with mbPartIdx subMbPartIdx, refIdxLX, and currSubMbType as the inputs and the output being mvpLX.

        - Derivation process for luma motion vectors for skipped macroblocks in P and SP slices

This process is invoked when mb_type is equal to P_Skip, nal_unit_type is equal to 21, DepthFlag is equal to 0, dmvp_flag is equal to 1 and MbVSSkipFlag is equal to 0.

Outputs of this process are:

–	the motion vector mvL0,

–	the reference index refIdxL0.

For the derivation of the motion vector mvL0 and refIdxL0 of a P_Skip macroblock type, the following ordered steps are specified:

- The process specified in subclause 8.3.1.x is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType set equal to "na", and listSuffixFlag = 0  as input and the output is assigned to the motion vectors mvCand [ 0 ] and the reference indices refIdxCand[ 0 ].

- The process specified in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType set equal to "na", and listSuffixFlag set equal to 0 as input and with mbAddrN\mbPartIdxN\subMbPartIdxN, reference indices refIdxCand[ i ] and the motion vectors mvCand[ i ] as outputs with i equal to 1, 2, and 3 corresponding to neighbouring partition A, B, and C, respectively.

- When refIdxCand[ i ] is equal to -1 for all values of i = 0 .. 3, the variables mvL0 and refIdxL0 are specified as follows

refIdxCand[ 0 ] = 0		(J.8-XX)
mvCand[ 0 ] [ 0 ] = 0		(J.8-XX)
mvCand[ 0 ] [ 1 ] = 0		(J.8-XX)

- refIdxL0 is initialized as -1

MvL0 [0] is initialized as 0

MvL0 [1] is initialized as 0

- for( cIdx = 0;cIdx<4; refIdxL0 < 0; cIdx ++)
  mvL0 = mvCandL[ cIdx ],

refIdxL0 = refIdxCandL[ cIdx ]

- If refIdxL0 is a reference index to an inter-view reference component or an inter-view only reference component, mvL0[ 0 ] is equal to 0, and mvL0[ 1 ] is equal to 0, the following steps are applied.

–	The inverse macroblock scanning process as specified in subclause Error: Reference source not found is invoked with CurrMbAddr as the input and the output is assigned to ( dbx1, dby1 ).

–	The modification process of inter-view motion vector as specified in subclause J.8.3.1.8 is invoked with depthPic being equal to DepthRefPicList0[ refIdxL0 ], dbx1, dby1, and mv being equal to mvL0 as inputs and the output assigned to mvL0.

Otherwise, the depth-based motion vector candidate derivation process specified in subcaluse J.8.3.1.6 is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, numMVCand set equal to 3, mvCand, refIdxCand, and depthRefPicList set to DepthRefPicList0 as inputs and mvp assigned to mvL0 and refIdx assigned to refIdxL0 as outputs.

        - Derivation process for luma motion vectors for VSP skipped macroblocks in P and SP slices

This process is invoked when mb_type is equal to P_Skip, nal_unit_type is equal to 21, DepthFlag is equal to 0, and MbVSSkipFlag is equal to 1.

Outputs of this process are the motion vector mvL0 and the reference index refIdxL0.

The reference index refIdxL0 for a VSP skipped macroblock is derived as the synthetic picture that appears first in RefPicList0.

The motion vector mvL0 is set equal to zero motion vector.

        - Derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8

Inputs to this process are current macroblock partition index mbPartIdx and subMbPartIdx.

Outputs of this process are the reference indices refIdxL0, refIdxL1, the motion vectors mvL0 and mvL1, the motion vector count variable subMvCnt, and the prediction list utilization flags, predFlagL0 and predFlagL1.

For the derivation of output, the following ordered steps are specified:

- Let the variable currSubMbType be set equal to sub_mb_type[ mbPartIdx ].

- The process specified in subclause 8.3.1.x is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType and listSuffixFlag = 0  as input and the output is assigned to the motion vectors mvCandL0[ 0 ] and the reference indices refIdxCandL0[ 0 ].

- The process specified in subclause 8.3.1.x is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType and listSuffixFlag = 1  as input and the output is assigned to the motion vectors mvCandL1[ 0 ] and the reference indices refIdxCandL1[ 0 ].

- The process specified in subclause Error: Reference source not found is invoked with mbPartIdx = 0, subMbPartIdx = 0, currSubMbType, and listSuffixFlag = 0 as inputs and the output is assigned to the motion vectors mvCandL0[ i ] and the reference indices refIdxCandL0[ i ] with i equal to 1, 2, and 3 corresponding to neighbouring partition A, B, and C, respectively.

- The process specified in subclause Error: Reference source not found is invoked with mbPartIdx = 0, subMbPartIdx = 0, currSubMbType, and listSuffixFlag = 1 as inputs and the output is assigned to the motion vectors mvL1N and the reference indices refIdxL1N with N being replaced by A, B, or C.

NOTE 1 – The motion vectors mvL0N, mvL1N and the reference indices refIdxL0N, refIdxL1N are identical for all 4x4 sub-macroblock partitions of a macroblock.

5  When refIdxCandL0[ i ] and refIdxCandL1[ i ] are all equal to -1 for all values of i = 0 .. 3, the variables mvL0 and refIdxL0 are specified as follows

refIdxCandL0[ 0 ] = 0		(J.8-XX)
mvCandL0 [ 0 ][ 0 ] = 0		(J.8-XX)
mvCandL0 [ 0 ][ 1 ] = 0		(J.8-XX)

refIdxCandL1[ 0 ] = 0		(J.8-XX)
mvCandL1[ 0 ][ 0 ] = 0		(J.8-XX)
mvCandL1[ 0 ][ 1 ] = 0		(J.8-XX)

- refIdxL0 is initialized as -1

MvL0 [0] is initialized as 0

MvL0 [1] is initialized as 0

refIdxL1 is initialized as -1

MvL1 [0] is initialized as 0

MvL1 [1] is initialized as 0

- for( cIdx = 0;cIdx<4; (refIdxL0 < 0 && refIdxL1 < 0 ); cIdx ++)
    mvL0 = mvCandL0[ cIdx ]

refIdxL0 = refIdxCandL0[ cIdx ]

mvL1 = mvCandL1[cIdx ]

refIdxL1 = refIdxCandL1[cIdx ]

- Let mvCandLXDY[ i ] be an array of motion vectors and refIdxCandLXDY be an array of reference indices, where X is replaced by 0 for reference picture list 0 and 1 for reference picture list 1, and Y is replaced by 0, 1, or 2. mvCandLXDY[ i ] and refIdxCandLXDY[ i ] are specified as follows.

numMVCandLXD0 = 0
numMVCandLXD1 = 0
numMVCandLXD2 = 0
for( i = 0; i < 3; i++ )
	if( refIdxCandLX[ i ] refers to an inter-view or inter-view only reference component ) {
		mvCandLXD1[ numMVCandLXD1 ] = mvCandLX[ i ]
		refIdxCandLXD1[numMVCandLXD1++ ] = refIdxCandLX[ i ]
	} else if ( refIdxCandLX[ i ] refers to a synthetic reference component ) {
		mvCandLXD2[ numMVCandLXD2 ] = mvCandLX[ i ]
		refIdxCandLXD2[numMVCandLXD2++ ] = refIdxCandLX[ i ]
	} else if( refIdxCandLX[ i ]  >= 0 ) {
		mvCandLXD0[ numMVCandLXD0 ] = mvCandLX[ i ]
		refIdxCandLXD0[numMVCandLXD0++ ] = refIdxCandLX[ i ]
}

- The depth-based motion vector candidate derivation process specified in subcaluse J.8.3.1.6is invoked for each combination of X = 0..1 and Y = 0..2 with mbPartIdx being equal to 0, subMbPartIdx being equal to 0, numMVCand being equal to numMVCandLXDY, mvCand being equal to mvCandLXDY, refIdxCand being equal to refIdxCandLXDY, depthRefPicList being equal to DepthRefPicListX as inputs and mvp assigned to mvCandLXDY, refIdx assigned to refIdxCandLXDY, and costMVP assigned to costCandLXDY as outputs.

- The variable costCandDY is derived for each value of Y equal to 0..2 as follows

if( numMVCandL0DY > 0  &&  numMVCandL1DY > 0 )
	costCandDY = costCandL0DY + costCandL1DY
else
	costCandDY = "not available"

- If costCandDY is equal to "not available" for all values of Y = 0..2, the variables refIdxLX and mvLX are set as follows:

refIdxL0 = 0
refIdxL1 = 0
mvL0[ 0 ] = mvL0[ 1 ] = mvL1[ 0 ] = mvL1[ 1 ] = 0

Otherwise, the following applies.

–	Let Z be the value of Y = 0..2 for which costCandDY is the smallest and not equal to "not available".

–	The outputs are specified as follows:

refIdxL0 = refIdxCandL0DZ
refIdxL1 = refIdxCandL1DZ
mvL0 = mvCandL0DZ
mvL1 = mvCandL1DZ

- The prediction utilization flags predFlagL0 and predFlagL1 are derived as specified using Table 8–9.

- The variable subMvCnt is derived as follows:

–	If subMbPartIdx is not equal to 0, subMvCnt is set equal to 0.

–	Otherwise (subMbPartIdx is equal to 0), subMvCnt is set equal to predFlagL0 + predFLagL1.

J.8.3.1.y Derivation process for the inter-view reference picture and the disparity vector

Inputs to this process are :

- depth reference view component depthPic,

- the location of a top-left sample ( dbx1, dby1 ) of a partition and

- listSuffixFlag

Outputs of this process are :

- the picture InterViewPic,

- an offset vector dv and

- An variable InterViewAvailable

Set InterViewAvailable equal to 0

Variable names that include the string "ListX" are interpreted with the X being equal to listSuffixFlag.

Variable names that include the string "ListY" are interpreted with the Y being equal to 1 - listSuffixFlag.

The following procedure is conducted to find a inter-view reference picture InterViewPic:

for( cIdx = 0;cIdx<num_ref_idx_l0_active_minus1 + 1; InterViewAvailable == 0; cIdx ++)
	When view_id of RefPicListX[ cIdx ] is equal to view_id of Base view, the following applied:

InterViewPic = RefPicListX[ cIdx ]

InterViewAvailable = 1

for( cIdx = 0;cIdx<num_ref_idx_l1_active_minus1 + 1; InterViewAvailable == 0; cIdx ++)
	When view_id of RefPicListY[ cIdx ] is equal to view_id of Base view, the following applied:

InterViewPic = RefPicListY[ cIdx ]

InterViewAvailable = 1

When InterViewAvailable is equal to 1, the following ordered steps apply:

Set the variable DvAvailable equal to 0

- The process specified in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType set equal to "na", and listSuffixFlag set equal to 0 as input and with reference indices refIdxCandL0[ i ] and the motion vectors mvCandL0[ i ] as outputs with i equal to 0, 1, and 2 corresponding to neighbouring partition A, B, and C, respectively.

- The process specified in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType set equal to "na", and listSuffixFlag set equal to 1 as input and with reference indices refIdxCandL1[ i ] and the motion vectors mvCandL1[ i ] as outputs with i equal to 0, 1, and 2 corresponding to neighbouring partition A, B, and C, respectively.

- for( cIdx = 0;cIdx<3; DvAvailable == 0; cIdx ++)
When view_id of RefPicListX[ refIdxCandL0[ cIdx ] ] is equal to view_id of Base view, the following applied:

dv[ 0 ] = mvCandL0[ cIdx ][ 0 ]

dv[ 1 ] = mvCandL0[ cIdx ][ 1 ]

DvAvailable = 1

- for( cIdx = 0;cIdx<3; DvAvailable == 0; cIdx ++)
When view_id of RefPicListY[ refIdxCandL1[ cIdx ] ] is equal to view_id of Base view, the following applied:

dv[ 0 ] = mvCandL1[ cIdx ][ 0 ]

dv[ 1 ] = mvCandL1[ cIdx ][ 1 ]

DvAvailable = 1

- When DvAvailable is equal to 0, the following applied:

depth = depthPic[dbx1 +(partWidth << 1) -1, dby1+ (partHeight << 1) -1 ]

index = ViewIdTo3DVAcquisitionParamIndex( view_id of the current view )

refIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the InterViewPic )

dv[ 0 ] = 0
dv[ 1 ] = Disparity( NdrInverse[depth], ZNear[ frame_num, index ], ZFar[ frame_num, index ],
  FocalLengthX[ frame_num, index ], AbsTX[ index ] – AbsTX[ refIndex ] )

8.3.1.x  Derivation process for the inter-view motion vector in inter-view reference picture

Inputs to this process are :

- mbPartIdx and

- subMbPartIdx

- the list suffix flag listSuffixFlag

Outputs of this process are :

- the motion vector mvCorrespond and

- the reference index refIdxCorrespond

Variable names that include the string "LX" are interpreted with the X being equal to listSuffixFlag.

Inter-view reference picture InterViewPic and an offset vector dv are derived as specified by the following ordered steps:

- The inverse macroblock scanning process as specified in subclause 6.4.1is invoked with CurrMbAddr as the input and the output is assigned to ( x1, y1 ).

- The inverse macroblock partition scanning process specified in subclause 6.4.2.1 is invoked with mbPartIdx as the input and the output assigned to ( dx1, dy1 ).

- The inverse sub-macroblock partition scanning process specified in subclause 6.4.2.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output assigned to ( dx2, dy2 ).

- The process specified in subclause 8.3.1.y is invoked with depthPic being equal to DepthCurrPic, dbx1 being equal to x1 + dx1 + dx2, dby1 being equal to y1 + dy1 + dy2 and listSuffixFlag as input and InterViewPic, an offset vector dv and an variable InterViewAvailable as outputs.

If InterViewAvailable is equal to 0, the following applies.

refIdxCorrespond = -1

mvCorrespond [ 0 ] = 0

mvCorrespond [ 1 ] = 0

Otherwise, the following applies.

The variable luma4x4BlkIdx is derived as  (4 * mbPartIdx + subMbPartIdx).

The inverse 4x4 luma block scanning process as specified in subclause Error: Reference source not found is invoked with luma4x4BlkIdx as the input and ( x, y ) as the output.

( xCorrespond, yCorrespond ) is set equal to ( x + ( dv[ 0 ]>>4 ), y + ( dv [ 1 ]>>4 ) )       (8-XXX)

mbAddrCorrespond is set equal to ( ( CurrMbAddr / PicWidthInMbs ) + ( dv[1] >>6 ) ) * PicWidthInMbs + ( CurrMbAddr % PicWidthInMbs ) + ( dv[0] >>6 )      	(8-XXX)

Let mbTypeCorrespond be the syntax element mb_type of the macroblock with address mbAddrCorrespond inside the picture InterViewPic and, when mbTypeCorrespond is equal to P_8x8, P_8x8ref0, or B_8x8, let subMbTypeCorrespond be the syntax element list sub_mb_type of the macroblock with address mbAddrCorrespond inside the picture InterViewPic.

Let mbPartIdxCorrespond be the macroblock partition index of the corresponding partition and subMbPartIdxCorrespond the sub-macroblock partition index of the corresponding sub-macroblock partition. The derivation process for macroblock and sub-macroblock partition indices as specified in subclause Error: Reference source not found is invoked with the luma location                       ( xCorrespond, yCorrespond ), the macroblock type mbTypeCorrespond, and, when mbTypeCorrespond is equal to P_8x8, P_8x8ref0, or B_8x8, the list of sub-

macroblock types subMbTypeCorrespond as the inputs and the outputs are the macroblock partition index mbPartIdxCorrespond and the sub-macroblock partition index subMbPartIdxCorrespond.

The motion vector mvCorrespond and the reference index refIdxCorrespond are derived as follows.

–	If the macroblock mbAddrCorrespond is coded in an Intra macroblock prediction mode, both components of mvCorrespond are set equal to 0 and refIdxCorrespond is set equal to –1.

–	Otherwise (the macroblock mbAddrCorrespond is not coded in an Intra macroblock prediction mode), the prediction utilization flags predFlagLXCorrespond is set equal to PredFlagLX[ mbPartIdxCorrespond ], which is the prediction utilization flag that has been assigned to the macroblock partition mbAddrCorrespond\mbPartIdxCorrespond inside the picture InterViewPic and the following applies.

–	When predFlagLXCorrespond is equal to 1, the motion vector mvCorrespond and the reference index refIdxCorrespond are set equal to MvLX[ mbPartIdxCorrespond ][ subMbPartIdxCorrespond ] and RefIdxLX[ mbPartIdxCorrespond ], respectively, which are the motion vector mvLX and the reference index refIdxLX that have been assigned to the (sub-)macroblock partition mbAddrCorrespond\mbPartIdxCorrespond\subMbPartIdxCorrespond inside the picture InterViewPic.

        - Derivation process for luma motion vectors for VSP skipped/direct macroblocks in B slices

Outputs of this process are the motion vector mvL0, mvL1 and the reference index refIdxL0, refIdxL1.

The reference index refIdxLX for a VSP skipped/direct macroblock is derived as the synthetic reference component that appears first in the reference picture list X, with X being replaced by 0 or 1. When there is no synthetic picture in the reference picture list X, refIdxLX is set equal to 0.

The motion vector mvLX is set equal to zero motion vector, with X being replaced by 0 or 1.

        - Derivation process for luma motion vector prediction

Inputs to this process are:

–	the macroblock partition index mbPartIdx,

–	the sub-macroblock partition index subMbPartIdx,

–	the reference index of the current partition refIdxLX (with X being 0 or 1),

–	the variable currSubMbType.

Output of this process is the prediction mvpLX of the motion vector mvLX (with X being 0 or 1).

The specifications of subclause 8.4.1.3 apply with the following changes.

–	The following additional sentence is applied

If refIdxLX is not equal to refIdxLXN for any N = A, B, or C and X equal to 0 or 1, the following applies:

mbAddrN\mbPartIdxN\subMbPartIdxN is marked as not available
refIdxLXN = -1
mvLXN[ 0 ] = 0
mvLXN[ 1 ] = 0

after the following paragraph in subclause 8.4.1.3:

The derivation process for the neighbouring blocks for motion data in subclause Error: Reference source not found is invoked with mbPartIdx, subMbPartIdx, currSubMbType, and listSuffixFlag = X (with X being 0 or 1 for refIdxLX being refIdxL0 or refIdxL1, respectively) as the input and with mbAddrN\mbPartIdxN\subMbPartIdxN, reference indices refIdxLXN and the motion vectors mvLXN with N being replaced by A, B, or C as the output.

–	The following additional sentence is applied

–	Otherwise, If refIdxLX is a reference index to an inter-view reference component or an inter-view only reference component, the depth-based derivation process for median luma motion vector prediction in subclause J.8.3.1.7 is invoked with mbAddrN\mbPartIdxN\subMbPartIdxN, mvLXN, refIdxLXN with N being replaced by A, B, or C, and refIdxLX as the inputs and the output is assigned to the motion vector predictor mvpLX.

after the following paragraph in subclause 8.4.1.3

–	Otherwise, if MbPartWidth( mb_type ) is equal to 8, MbPartHeight( mb_type ) is equal to 16, mbPartIdx is equal  mvpLX = mvLXCto 1, and refIdxLXC is equal to refIdxLX, the motion vector predictor mvpLX is derived by:

mvpLX = mvLXC

        - Depth-based motion vector candidate derivation process

Inputs to this process are

–	the macroblock partition index mbPartIdx,

–	the sub-macroblock partition index subMbPartIdx,

–	the number of motion vector candidates numMVCand,

–	motion vectors mvCand[ i ] and reference indices refIdxCand[ i ], where i = 0 .. ( numMVCand – 1 ),

–	a reference picture list for depth view components depthRefPicList.

Outputs of this process are:

–	motion vector prediction mvp,

–	reference indices refIdx,

–	the sum of absolute differences costMVP associated with the motion vector prediction mvp.

For the derivation of the outputs the following ordered steps are applied.

- The inverse macroblock scanning process as specified in subclause 6.4.1is invoked with CurrMbAddr as the input and the output is assigned to ( x1, y1 ).

- The inverse macroblock partition scanning process specified in subclause 6.4.2.1 is invoked with mbPartIdx as the input and the output assigned to ( dx1, dy1 ).

- The inverse sub-macroblock partition scanning process specified in subclause 6.4.2.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output assigned to ( dx2, dy2 ).

- The the cost derivation process of two depth blocks is invoked for each value of i = 0 .. ( numMVCand – 1 ) for which refIdxCand[ i ] is greater than or equal to 0 with costi being the output and with inputs set as follows:

–	depthPic1 = DepthCurrPic

–	depthPic2 = depthRefPicList[ refIdxCand[ i ] ]

–	dbx1 = x1 + dx1 + dx2

–	dby1 = y1 + dy1 + dy2

–	dbx2 = dbx1 + mvCand[ i ][ 0 ]

–	dby2 = dby1 + mvCand[ i ][ 1 ]

–	depthBlockWidth = SubMbPartWidth( subMbPartIdx )

–	depthBlockHeight = SubMbPartHeight( subMbPartIdx )

- The variable j is selected so that costj is the smallest among all values of costi, for any value of i = 0 .. ( numMVCand – 1 ) for which refIdxCand[ i ] is greater than or equal to 0. The variable mvp is set to mvCand[ j ], the variable refIdx is set to refIdxCand[ j ], and the variable costMVP is set to costj.

          - Cost derivation process for two depth blocks

Inputs to this process are

–	sample arrays of decoded depth view components depthPic1 and depthPic2

–	the top-left corner [ dbx1, dby1 ] of a block within depthPic1 and the top-left corner [ dbx2, dby2 ] of a block within depthPic2

–	the horizontal and vertical extents depthBlockWidth and depthBlockHeight a depth block

Output of this process is the disparity-based sum of absolute differences dispSumOfAbsDiff converted from the sum of absolute differences between the the depth blocks.

The variable dispSumOfAbsDiff is specified as follows.

index = ViewIdTo3DVAcquisitionParamIndex( view_id of the current view )
baseIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the base view )

dispCoeff=Max( FocalLengthX[ frame_num, index ] ÷ 255* ( AbsTX[ index ] – AbsTX[ baseIndex ] ) * ( 1 ÷ ZNear[ frame_num, index ] – 1 ÷ ZFar[ frame_num, index ] ), 0 )

dispSumOfAbsDiff = 0
for( j = 0; j < depthBlockHeight; j++ )
	for( i = 0; i < depthBlockWidth; i++ )		(I-8-XX)
		dispSumOfAbsDiff += 
			Round( Abs( NdrInverse[ depthPic1[ dbx1 + i, dby1 + j ] ] – NdrInverse[ depthPic2[ dbx2 + i, dby1 + j ] ] ) * dispCoeff )

        - Depth-based derivation process for median luma motion vector prediction

Inputs to this process are:

–	the neighbouring partitions mbAddrN\mbPartIdxN\subMbPartIdxN (with N being replaced by A, B, or C),

–	the motion vectors mvLXN (with N being replaced by A, B, or C) of the neighbouring partitions,

–	the reference indices refIdxLXN (with N being replaced by A, B, or C) of the neighbouring partitions,,

–	the reference index refIdxLX of the current partition.

Output of this process is the motion vector prediction mvpLX.

When either partition mbAddrN\mbPartIdxN\subMbPartIdxN is not available or refIdxLXN is not equal to refIdxLX, mvLXN is derived as specified by the following ordered steps:

1.	The inverse macroblock scanning process as specified in subclause 6.4.1is invoked with CurrMbAddr as the input and the output is assigned to ( x1, y1 ).

2.	The inverse macroblock partition scanning process specified in subclause 6.4.2.1 is invoked with mbPartIdx as the input and the output assigned to ( dx1, dy1 ).

3.	The inverse sub-macroblock partition scanning process specified in subclause 6.4.2.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output assigned to ( dx2, dy2 ).

4.	The modification process of inter-view motion vector in median luma motion vector prediction as specified in subclause J.8.3.1.7.1 is invoked with depthPic being equal to DepthRefPicList0[ refIdxL0 ], dbx1 being equal to x1 + dx1 + dx2, dby1 being equal to y1 + dy1 + dy2, and mv being equal to mvL0 as inputs and the output is assigned to the motion vector mvLXN.

Each component of the motion vector prediction mvpLX is given by the median of the corresponding vector components of the motion vector mvLXA, mvLXB, and mvLXC:

mvpLX[ 0 ] = Median( mvLXA[ 0 ], mvLXB[ 0 ], mvLXC[ 0 ] )                 (J8-XX)

mvpLX[ 1 ] = Median( mvLXA[ 1 ], mvLXB[ 1 ], mvLXC[ 1 ] )                 (J8-XX)

          - Modification process for inter view motion vector in median luma motion vector prediction

Inputs to this process are

–	depth reference view component depthPic,

–	the location of a top-left sample ( dbx1, dby1 ) of a partition,

–	a motion vector mv,

Outputs of this process are:

–	the motion vector mv.

Let refViewId be the view_id value of depthPic.

The following ordered steps apply:

- Let numSamples be partWidth * partHeight.

- The variable maxDepth is specified as follows:

maxDepth = INT_MIN
for( j = 0; j < partHeight; j++ )
	for( i = 0; i < partWidth; i++ )		(J-8-XX)
		if( depthPic[ dbx1 + i, dby1 + j ] > maxDepth ) maxDepth = depthPic[ dbx1 + i, dby1 + j ]

- The variable mv is specified as follows:

index = ViewIdTo3DVAcquisitionParamIndex( view_id )
refIndex = ViewIdTo3DVAcquisitionParamIndex( refViewId )

mv[ 0 ] = Disparity( NdrInverse[ maxDepth ], ZNear[ frame_num, index ], ZFar[ frame_num, index ],
	FocalLengthX[ frame_num, index ], AbsTX[ index ] – AbsTX[ refIndex ] )
mv[ 1 ] = 0

        - Modification process for inter-view motion vector

Inputs to this process are

–	depth reference view component depthPic,

–	the location of a top-left sample ( dbx1, dby1 ) of a partition,

–	a motion vector mv,

Outputs of this process are:

–	the motion vector mv.

Let refViewId be the view_id value of depthPic.

The following ordered steps apply:

- Let numSamples be partWidth * partHeight.

- The variable averageDepth is specified as follows:

depthSum = 0
for( j = 0; j < partHeight; j++ )
	for( i = 0; i < partWidth; i++ )		(I-8-XX)
		depthSum += depthPic[ dbx1 + i, dby1 + j ]
averageDepth = ((depthSum << 1) + 1) >> ( Log2( numSamples ) + 1 )

- The variable devDepth is specified as follows:

devDepth = 0
for( j = 0; j < partHeight; j++ )
	for( i = 0; i < partWidth; i++ )		(I-8-XX)
		devDepth += Abs( depthPic[ dbx1 + i, dby1 + j ] – averageDepth )

- If devDepth is smaller than ( 12 * numSamples ), the variable mv is specified as follows:

index = ViewIdTo3DVAcquisitionParamIndex( view_id )
refIndex = ViewIdTo3DVAcquisitionParamIndex( refViewId )
mv[ 0 ] = 0
mv[ 1 ] = Disparity( NdrInverse[ averageDepth ], ZNear[ frame_num, index ], ZFar[ frame_num, index ],
	FocalLengthX[ frame_num, index ], AbsTX[ index ] – AbsTX[ refIndex ] )