Decoding Process:

J.8.3.1.7.1 Modification process for inter view motion vector in median luma motion vector prediction

Inputs to this process are

–	depth reference view component depthPic,

–	the location of a top-left sample ( dbx1, dby1 ) of a partition,

–	a motion vector mv,

Outputs of this process are:

–	the motion vector mv.

Let refViewId be the view_id value of depthPic.

The following ordered steps apply:

- Let numSamples be partWidth * partHeight.

- The variable maxDepth is specified as follows:

maxDepth = INT_MIN
for( j = 0; j < partHeight; j+=( partHeight -1) )
	for( i = 0; i < partWidth; i+=( partWidth -1) )		(J-8-XX)
		if( depthPic[ dbx1 + i, dby1 + j ] > maxDepth ) maxDepth = depthPic[ dbx1 + i, dby1 + j ]

- The variable mv is specified as follows:

index = ViewIdTo3DVAcquisitionParamIndex( view_id )
refIndex = ViewIdTo3DVAcquisitionParamIndex( refViewId )

mv[ 0 ] = Disparity( NdrInverse[ maxDepth ], ZNear[ frame_num, index ], ZFar[ frame_num, index ],
	FocalLengthX[ frame_num, index ], AbsTX[ index ] – AbsTX[ refIndex ] )
mv[ 1 ] = 0