| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0046 |
| --- | --- |

| Title: | 3D-CE5.a related: Simplification on the disparity vector derivation for AVC-based 3D video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the AVC-based 3D video coding, 3DV-ATM v.4, the direction-separated MVP is utilized for the temporal and inter-view motion vector predictions in the inter mode. When the reference picture is an inter-view prediction picture, the inter-view motion vectors of the adjacent blocks are employed for the inter-view prediction. If an inter-view motion vector is unavailable, it is replaced by a disparity vector derived from a maximum depth value within the depth block associated with current block. This contribution proposes a simplification to derive the disparity vector from a maximum depth value of four corner depth samples instead of all depth samples within the associated depth block. With this simplification, the number of the depth samples to be accessed can be significantly reduced from 256 to 4 and the number of the required comparisons can also be reduced from 255 to 3. The experimental results reportedly show that this proposed simplification does not cause any coding loss.

# Introduction

In the 3DV-ATM v.4 [1], the direction-separate motion vector prediction is utilized for the temporal and inter-view motion vector predictions in the inter mode. If the target reference picture is a temporal prediction picture, the temporal motion vectors of the adjacent blocks around the current block Cb such as A, B, and C in Figure 1 are employed in the derivation of the motion vector prediction. If a temporal motion vector is unavailable, a zero vector is used. The temporal motion vector prediction is then derived as the median of the motion vectors of the adjacent blocks A, B, and C.

On the contrary, if the target reference picture is an inter-view prediction picture, the inter-view motion vectors of the neighboring blocks are employed for the inter-view prediction. If an inter-view motion vector is unavailable, a disparity vector converted from a maximum depth value within the associated depth block is used. The inter-view motion vector predictor is then derived as the median of the inter-view motion vectors of the adjacent blocks A, B, and C.

Figure 1. The direction-separated motion vector prediction for the inter mode [3].

# Proposed Methods

As shown in Figure 2(a), in 3DV-ATM v.4, deriving the maximum disparity vector needs to search all the depth samples within the associated depth block. In 3DV-ATM v.4, the largest partition size is 16x16 and the associated block is also 16x16. To find out the maximum disparity value, it needs to access 256 depth samples within the associated block and perform 255 comparisons.

In this contribution, we propose a simplified disparity vector derivation. The disparity vector is derived from the maximum depth value of four corner depth samples instead of all depth samples within the associated depth block as shown in Figure 2(b). Based on this simplification, it only needs to access 4 depth samples and perform 3 comparisons.

Compared to 3DV-ATM v.4, for the worst cases when a 16x16 macroblock partition is coded, the number of the depth samples to be accessed can be reduced from 256 to 4 and the number of the required comparisons can also be reduced from 255 to 3.

| (a) | (b) |
| --- | --- |

Figure 2. Derivation of the maximum depth value within the associated depth block: (a) 3DV-ATM v0.4, (b) proposed simplification.

# Experiments

The proposed simplified maximum disparity value derivation is integrated into 3DV-ATM v.4 [1] and the simulations are run under the common test conditions [2]. Table 1 shows the coding results compared to the anchor, 3DV-ATM v.4.

The results show that the proposed simplification doesn’t cause any coding loss, while 1:64 complexity reduction can be achieved. For more detail, please refer to JCT2-A0046_Results.xls.

Table 1 Results of the proposed simplification

# Conclusion

In this contribution, a simplified derivation of the maximum disparity vector is proposed. With this simplification, the number of the depth samples to be accessed can be significantly reduced from 256 to 4 and the number of the required comparisons can also be reduced from 255 to 3 when a 16x16 macroblock partition is coded. The experimental results show that this simplification introduces no loss of coding performance.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] 3DV-ATM v.4, http://mpeg3dv.research.nokia.com/svn/mpeg3dv/trunk/

[2] Heiko Schwarz, and Dmytro Rusanovskyy, “Common Test Conditions for 3DV experimentation”, ISO/IEC JTC1/SC29/WG11 MPEG2011/N12745, May 2012, Geneva, Switzerland.

[3] J.-Y. Lee, T. Uchiumi, J. Lee, Y. Yamamoto and D.-S. Park, “3D-CE5.a results on joint proposal for an improved depth-based motion vector prediction method by Samsung and Sharp”, ISO/IEC JTC1/SC29/WG11 MPEG2011/m24824, May 2012, Geneva, Switzerland.