| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0047 |
| --- | --- |

| Title: | 3D-CE5.h related: Simplification on disparity vector derivation for HEVC-based 3D video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the HEVC-based 3D video coding, a disparity vector derived from the depth values is used to locate a corresponding block for the inter-view motion prediction in the merge, skip and inter modes. In HTM-3.1, the disparity vector is derived from a maximum depth value within the depth block associated with the current PU. In this contribution, a simplification is proposed to derive the disparity vector from a maximum depth value of four corner depth samples instead of all depth samples within the associated depth block. With this simplification, the number of the depth samples to be accessed can be significantly reduced from 256 to 4 and the number of the required comparisons can also be reduced from 255 to 3 when a 64x64 PU is coded. The experimental results also reportedly show that this proposed simplification can even achieve slight coding gain (0.2% BD-rate reduction for view 1).

# Introduction

In the HTM-3.1 [1], the inter-view motion prediction is employed to derive the inter-view motion vector predictor (MVP) candidate for the inter mode and the inter-view merging candidate for the merge and skip modes. A disparity vector converted from a maximum depth value within the associated depth block is used to locate a corresponding block for the inter-view motion prediction, as shown in Figure 1. To derive the disparity vector, it needs to search all the samples within the associated depth block to find out the maximum depth value. If the corresponding block is coded using the motion-compensated prediction, the associated motion parameters can be used as a candidate for the current block for the merge, skip and inter modes. The derived disparity vector can also be directly used as a candidate for disparity-compensated prediction.

Figure 1. The inter-view motion prediction using a max disparity [3]

# Proposed Method

As shown in Figure 2(a), in HTM-3.1, deriving the maximum disparity vector needs to search all the samples within the associated depth block. In the common test conditions [2], the largest PU size is 64x64 and the associated block is 16x16. To find out the maximum disparity value, it needs to access 256 depth samples within the associated block and perform 255 comparisons.

In this contribution, we propose a simplified disparity vector derivation. The disparity vector is derived from the maximum depth value of four corner depth samples instead of all depth samples within the associated depth block as shown in Figure 2(b). Based on this simplification, it only needs to access 4 depth samples and perform 3 comparisons.

Compared to HTM-3.1, for the worst cases when the 64x64 PU is coded, the number of the depth samples to be accessed can be reduced from 256 to 4 and the number of the required comparisons can also be reduced from 255 to 3.

| (a) | (b) |
| --- | --- |

Figure 2.  Derivation of the maximum depth value within the associated depth block: (a) HTM-3.1, (b) proposed simplification

# Experiments

The proposed simplified maximum disparity value derivation is integrated into HTM-3.1 [1] and the simulations are run under the common test conditions. Table 1 shows the coding results compared to the anchor, HTM-3.1.

The results show that the proposed simplification doesn’t cause any coding loss. For view 1, this simplified maximum disparity value derivation can even lead to 0.2% BD-rate saving. For more detail, please refer to JCT2-A0047_Results.xls.

Table 1 Results of the proposed simplification

# Conclusion

In this contribution, a simplified derivation of the maximum disparity vector is proposed. With this simplification, the number of depth samples to be accessed can be significantly reduced from 256 to 4 and the number of the required comparisons can be reduced from 255 to 3 for the worst cases when the 64x64 PU is coded. The experimental results also show that this simplification could also achieve slight coding gain (0.2% BD-rate saving for view 1).

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] HTM-3.1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-3.1/

[2] Heiko Schwarz, and Dmytro Rusanovskyy, “Common Test Conditions for 3DV experimentation”, ISO/IEC JTC1/SC29/WG11 MPEG2011/N12745, May 2012, Geneva, Switzerland.

[3] J.-Y. Lee, J. Lee, and D.-S. Park, “3D-CE5.h related results on an improved motion parameter prediction method by Samsung”, ISO/IEC JTC1/SC29/WG11 MPEG2011/m24829, May 2012, Geneva, Switzerland.