| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0048 |
| --- | --- |

| Title: | 3D-CE5.h related: Pruning process for inter-view candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the HEVC-based 3D video coding, a number of parallelizable motion information comparisons between the spatial merging candidates are performed for the redundancy removal in the candidate list of the merge and skip modes. This contribution proposes to make one additional parallelizable motion information comparison between the inter-view candidate and the first spatial candidate in the pruning process. This contribution also proposes to remove the check on the number of available candidates before the pruning process in the inter mode for simplification. The experimental results reportedly show BD-rate savings of 0.2% and 0.4% are achieved for texture view 1 and view 2, respectively, with this simple candidate comparison.

# Introduction

In the HEVC-based 3D video coding, HTM-3.1 [1], a number of parallelizable motion information comparisons between the spatial merging candidates are performed for the removal of redundant candidates in the merging candidate list of the merge and skip modes. The temporal and inter-view merging candidates are exempted from the pruning process. The spatial merging candidates are inserted into the merging candidate list depending on the following specific conditions.

As shown in Figure 1, the conditions for each spatial merging candidate to be excluded from the merging candidate list are as follows:

•	A1: No change (which means A1 is always inserted as long as it is available.)

•	B1: B1 has the same motion information as A1

•	B0: B0 has the same motion information as B1

•	A0: A0 has the same motion information as A1

•	B2: B2 has the same motion information as A1 or has the same motion information as B1 (Note that   B2 is considered only if any of A1, B1, B0 or A0 is excluded from the candidate list.)

Figure 1. Spatial neighbors which are the potential candidates for the merging candidate list. Arrows indicate which spatial merging candidate(s) are to be compared.

As for the inter mode, the temporal motion vector predictor (MVP) candidate is exempted from the pruning process. The number of available spatial and inter-view MVP candidates is checked first. Only when the number of available spatial and inter-view MVP candidates equals 2, the pruning process is applied to comparing these two MVP candidates and removing the redundant one. After that, only the first three available MVP candidates are included in the candidate list. If the number of available MVP candidates is smaller than three, the zero candidate is inserted.

# Proposed Method

The derived motion vectors of the spatial and inter-view merging candidates are most likely to be the same due to the observations that the spatial neighbor and the inter-view neighbor are most probably associated with same object and the motion vectors for the spatial merging candidate and the inter-view merging candidate will not be scaled during the derivation (Unlike the temporal merging candidate, the motion vector is scaled during the derivation).

Since the derivation of the inter-view merging candidate takes more cycles than the derivation of spatial merging candidates, the first spatial merging candidate in the candidate list (A1) is already available when the inter-view merging candidate is derived. Without reducing the parallel ability to derive the candidate list, in this contribution, we propose to add one additional comparison between the inter-view merging candidate and the first spatial merging candidate, A1, in the pruning process.

In the proposed pruning process for deriving the merging candidate list, the temporal merging candidate is still exempted from the pruning process. The spatial or inter-view merging candidates are inserted into the merging candidate list through pruning process depending on the following conditions:

As shown in Figure 2, the conditions for each spatial or inter-view merging candidate to be excluded from the merging candidate list are as follows:

•	A1: No change (which means A1 is always inserted as long as it is available.)

•	B1: B1 has the same motion information as A1

•	B0: B0 has the same motion information as B1

•	A0: A0 has the same motion information as A1

•	B2: B2 has the same motion information as A1 or has the same motion information as B1 (Note that B2 is considered only if any of A1, B1, B0 or A0 is excluded from the merge list.)

•	I: I has the same motion information as A0 (I is the inter-view merging candidate)

Figure 2. Spatial and inter-view neighbors which are the potential candidates for the merge candidate set. Arrows indicate which spatial candidate(s) are to be compared.

In the pruning process for the inter mode, we propose to remove the check on the number of available spatial and inter-view MVP candidates before applying the pruning process. The inter-view MVP candidate and the second spatial MVP candidate, if available, are both directly compared to the first spatial MVP candidate. At most two parallelizable motion information comparisons are performed for the removal of redundant candidates in the MVP candidate list.

# Experiments

The proposed methods are integrated into HTM-3.1 [1] and the simulations are run under the common test conditions [2]. In Test1, the inter-view candidate is included in the pruning process of the merge and skip modes. The results compared to the anchor, HTM-3.1, is shown in Table 1. Adding one additional parallelizable motion information comparison between the inter-view candidate and the first spatial candidate in the pruning process for the merge and skip modes can achieve 0.2% and 0.3% coding gains for view 1 and view 2, respectively.

In Test2, the inter-view candidate is included in the pruning process of the merge, skip and inter modes and the check on the number of available MVP candidates before the pruning process of the inter mode is also removed. Table 2 shows that 0.2% and 0.4% coding gains are achieved for view 1 and view 2, respectively. For the results of all texture views over the bitrates of all texture views, 0.2% coding gain is observed. For more detail of test1 and test2, please refer to JCT2-A0048_Test1_Results.xls and JCT2-A0048_Test2_Results.xls.

Table 1 Experimental results of Test1

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 99.8% | 97.3% | 95.1% |
| Kendo | -0.4% | -0.3% | -0.2% | -0.2% | -0.2% | 97.8% | 97.9% | 99.0% |
| Newspapercc | -0.3% | -0.2% | -0.1% | 0.1% | 0.0% | 99.8% | 97.3% | 100.0% |
| GhostTownFly | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 100.2% | 103.0% | 100.0% |
| PoznanHall2 | -0.5% | -0.9% | -0.4% | -0.2% | -0.3% | 100.1% | 98.8% | 100.0% |
| PoznanStreet | -0.1% | -0.2% | -0.1% | 0.0% | -0.1% | 101.6% | 97.5% | 97.5% |
| UndoDancer | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% | 101.1% | 99.3% | 97.5% |
| 1024x768 | -0.3% | -0.2% | -0.1% | 0.0% | -0.1% | 99.1% | 97.5% | 98.0% |
| 1920x1088 | -0.2% | -0.3% | -0.2% | -0.1% | -0.1% | 100.8% | 99.6% | 98.7% |
| average | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% | 100.1% | 98.7% | 98.4% |

Table 2 Experimental results of Test2

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% | 99.4% | 99.7% | 103.6% |
| Kendo | -0.3% | -0.4% | -0.2% | -0.1% | -0.1% | 99.2% | 99.9% | 100.7% |
| Newspapercc | -0.2% | -0.3% | -0.1% | -0.2% | -0.2% | 100.2% | 98.3% | 97.6% |
| GhostTownFly | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.8% | 99.1% | 96.3% |
| PoznanHall2 | -0.5% | -1.3% | -0.5% | -0.3% | -0.3% | 99.1% | 101.7% | 102.7% |
| PoznanStreet | 0.1% | -0.4% | -0.1% | -0.1% | -0.1% | 98.8% | 93.2% | 90.1% |
| UndoDancer | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 99.6% | 98.1% | 95.0% |
| 1024x768 | -0.2% | -0.3% | -0.2% | -0.1% | -0.1% | 99.6% | 99.3% | 100.6% |
| 1920x1088 | -0.2% | -0.5% | -0.2% | -0.1% | -0.1% | 99.6% | 98.0% | 95.9% |
| average | -0.2% | -0.4% | -0.2% | -0.1% | -0.1% | 99.6% | 98.5% | 97.9% |

# Conclusion

In this contribution, we propose to add one additional motion information comparison between the inter-view candidate and the spatial candidate in the pruning process for the merge, skip and inter modes. We further propose to remove the check on the number of available candidates before the pruning process in the inter mode. Since the derivation of inter-view candidate takes more cycles than spatial candidate, the proposed scheme does not reduce the parallel ability to derive the candidate list. The simulation results show that BD-rate savings of 0.2% and 0.4% are achieved for view 1 and view2, respectively, by simply adding one comparison for the inter-view candidate.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] HTM-3.1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-3.1/

[2] Heiko Schwarz, and Dmytro Rusanovskyy, “Common Test Conditions for 3DV experimentation”, ISO/IEC JTC1/SC29/WG11 MPEG2011/N12745, May 2012, Geneva, Switzerland.