| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0050 |
| --- | --- |

| Title: | 3D-CE1.a related: Inter-view Skip mode with sub-partition scheme |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chi-Ling Wu, Yu-Lin Chang, Yu-Pao Tsai, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chiling.wu, yulin.chang, yupao.tsai, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, an inter-view Skip mode is reported. In AVC-based 3D video coding, View Synthesis Prediction (VSP) is a tool to use the reference from the synthetic frame to improve coding efficiency. However, it is costly in terms of number of computations and amount of required storage to perform per-pixel warping. The inter-view Skip mode is proposed to eliminate full frame view synthesis for reducing the computational complexity, memory bandwidth, and buffer. When turning off VSP but with the proposed inter-view Skip mode active, the experimental results reportedly show an average of 10% decoding time saving with 0.21% BD-rate reduction for synthesized pictures over the total bit-rate.

# Introduction

In 3D video coding, View Synthesis Prediction (VSP) provides an inter-view reference from the synthetic frames. In 3DV-ATM version 4.0, VSP is implemented as: provided that the texture and depth pair of a first view is coded and decoded, a second view can be predicted by warping the first view to the second view position [1]. Also, a VSP skip/direct mode [2] and a context-based adaptive skip flag positioning method [3] were proposed to use a skip_type flag to adaptively select a synthetic reference or a non-synthetic reference according to the skip status of neighbouring blocks.

However, VSP is quite expensive due to the warping process of every pixel to generate a synthetic picture. In the MPEG 100th meeting, some contributions were proposed to reduce the complexity, such as [4], [5], and [6], which were integrated in 3DV-ATM v4.0. In [4], the subsampling was applied only when the pixels in the warped frame are located at sub-pixel positions. The decoding time was reduced to 80% with minor texture coding loss in 3DV-ATM v0.3. In [5], a method of low complexity block-based VSP implementation was proposed. The decoding time was reduced to 70% with 30% encoding time increase and minor texture coding loss. In [6], VSP was directly applied on the lower resolution of texture and depth. The decoding time was reduced to 80% with minor texture coding loss in 3DV-ATM v0.3. Compared to these three proposals, which were a part of 3DV-ATM v4.0, the proposed scheme achieves higher decoding time saving and some total coding gain in 3DV-ATM v4.0 without full frame view synthesis, which requires more computations, larger memory bandwidth and storage.

The rest of this contribution is organized as follows. The proposed method is described in Section 2, and the simulation results are shown in Section 3. Finally, Section 4 gives the conclusions.

# Algorithm description

In this contribution, an inter-view Skip mode without full frame view synthesis is proposed. Macroblocks with the inter-view Skip mode provides a good inter-view reference without warping the base view to the dependent views. In the inter-view Skip mode, the macroblock is partitioned into blocks, and their corresponding blocks are obtained in the other view as the reference without warping the other view to the current view pixel by pixel.

Figure 1 shows the details of the inter-view Skip mode. Note that View 0 is coded or decoded before View 1 and the texture (Texture 1) is coded after the depth (Depth 1) in View 1. When coding or decoding a macroblock in Texture 1, the macroblock is first partitioned into 16 4x4 blocks. For each 4x4 block, the corresponding block in Texture 0 is obtained from the disparity converted from the depth value. In 3DV-ATM v4.0, the depth value of the block is the maximum depth value among the samples in the collocated block in Depth 1. The inter-view reference of the current block in Texture 1 is obtained by collecting 16 4x4 corresponding blocks in Texture 0 without warping Texture 0 to View 1. One flag is needed to indicate the inter-view Skip mode, which is the same as the VSP Skip/Direct mode in [2].

Figure 1  Proposed inter-view Skip and Direct mode

# Test results

The proposed method is implemented on the test model for AVC-based 3D video coding (3DV-ATM v4.0) [7], and all the simulations are done following the common test conditions [8]. Since the proposed scheme does not affect HP condidtions, the tests are only carried out for EHP conditions. The tables below show BD-rate results of the proposed scheme, using 3DV-ATM v4.0 CTC as the anchor. In the tests, VSP is turned off from the configuration settings, and the VSP Skip/Direct mode is modified to the proposed inter-view Skip mode.

Table 1 shows the summary of the experimental results. Compared with 3DV-ATM v4.0, the proposed method reduces about 10% decoding time with minor texture coding loss and small total (synthesized+coded) gain, which are 0.07% loss and 0.21% gain in BD rates, respectively. Note that the decoding time saving mainly comes from turning VSP off.

Table 1 Test results of the proposed scheme with VSP off

|  | Texture Coding |  | Depth Coding |  | Total (Coded + Synthesed) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 1.38% | -0.04 | 0.00% | 0.00 | 0.81% | -0.03 | 91.60 | 87.14 | 98.15 |
| S02 | 0.57% | -0.02 | 0.00% | 0.00 | 0.21% | -0.01 | 96.87 | 90.19 | 106.08 |
| S03 | 0.46% | -0.02 | 0.00% | 0.00 | -0.02% | 0.00 | 96.39 | 95.90 | 102.98 |
| S04 | 0.70% | -0.03 | 0.00% | 0.00 | 0.35% | -0.01 | 103.76 | 91.83 | 100.06 |
| S05 | -0.67% | 0.03 | 0.00% | 0.00 | -0.81% | 0.03 | 96.98 | 93.91 | 104.23 |
| S06 | -1.52% | 0.07 | 0.00% | 0.00 | -1.68% | 0.07 | 88.08 | 85.06 | 99.45 |
| S08 | -0.47% | 0.02 | 0.00% | 0.00 | -0.34% | 0.01 | 97.04 | 86.47 | 97.32 |
| Average | 0.07% | 0.00 | 0.00% | 0.00 | -0.21% | 0.01 | 95.82 | 90.07 | 101.18 |

# Conclusion

An inter-view Skip mode is proposed to avoid the high complexity and huge memory bandwidth caused by full frame view synthesis. The inter-view Skip mode is tested on 3DV-ATM v4.0 and applied on the texture coding with VSP off. When turning off VSP and modifying the VSP Skip/Direct mode to the proposed inter-view Skip mode, the results show that it achieves an average of 10% decoding time saving with minor BD-rate loss (0.07%) for texture coding and small BD-rate gain (0.21%) for synthesized pictures over the total bit-rate.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- D. Tian, D. Graziosi, Y. Wang, N. Cheung, A. Vetro, “Mitsubishi Response to MPEG Call for Proposal on 3D Video Coding Technology”, ISO/IEC JTC1/SC29/WG11 document M22663, Geneva, Switzerland, November 2011.

- ISO/IEC JTC1/SC29/WG11 “3D-AVC-CE1 results on VSP skip/direct mode by Mitsubishi,” M23915, Feb. 2012.

- ISO/IEC JTC1/SC29/WG11 “3D-CE1.a results on a context-based adaptive skip flag positioning method by Samsung,” M24819, May 2012.

- ISO/IEC JTC1/SC29/WG11 “3D-CE1.a related: Simplification of in-loop view synthesis with fractional-pel disparity,” M24796, May 2012.

- ISO/IEC JTC1/SC29/WG11 “3D-CE1.a related: 3DV-CE1.a-related: Low complexity block-based View Synthesis Prediction,” M24915, May 2012.

- ISO/IEC JTC1/SC29/WG11 “3D-CE1.a related: View synthesis based on asymmetric texture and depth resolutions,” M24941, May 2012.

- Miska M. Hannuksela, ”Test Model for AVC based 3D video coding”, ISO/IEC JTC1/SC29/WG11 document N12558, San Jose, USA, February 2012.

- Heiko Schwarz, Dmytro Rusanovskyy, ” Common Test Conditions for AVC and HEVC-based 3DV”, ISO/IEC JTC1/SC29/WG11 document N12560, San Jose, USA, February 2012.