| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0053 |
| --- | --- |

| Title: | 3DV-ATM HP/EHP HLS：Camera Parameter Set (CPS) for camera parameter signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peng Lu, Yin Zhao and Lu Yu Zhejiang University Zheda Road 38 Hangzhou, Zhejiang, China | Tel: Email: | +86-571-87953107 yiqiu8932@gmail.com yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This proposal modifies the mechanism of camera parameter signaling both in MVC compatible track and AVC compatible track. The main characteristics of the proposed method can be summarized as follows:

- This proposal applies the concept of parameter set on camera parameters. Camera parameters are included in Sequence Parameter Set (SPS) 3DVC extension and Camera Parameter Set (CPS).

- Slice can refer to CPS by cam_parameter_set_id in slice header.

- Each CPS contains the camera parameters of one or more access unit.

- The temporal prediction among camera parameters is only carried out within CPS.

- CPS may use the camera parameters in SPS 3DVC extension as references to do temporal prediction among camera parameters.

# Problem Statement

In 3D Video Coding, camera parameters are essential for depth-range-based weighted prediction (both in MVC compatible track and AVC compatible track) and view synthesis prediction (in AVC compatible track) at decoder. In “PDAM 2 MVC extensions for inclusion of depth maps”[1] and “Working Draft 2 of AVC compatible video with depth information”[2], SPS 3DVC extension is used to transmit the camera parameters of one access unit and depth range update NAL Unit is used to update the camera parameters if the camera parameters change. Each depth range update NAL Unit comprises the camera parameters of one access unit. To exploit the correlation among camera parameters, temporal prediction is used. However, there are two problems in this mechanism.

(1) One depth range update NAL unit corresponds to one access unit. There is no syntax element in depth range update NAL unit or slice header to indicate the connection between the depth range update NAL unit and the access unit. The connection is determined by the arrived order of the NAL unit, which is not explicit.

(2) Camera parameters will be decoded into global parameter arrays and depth range update NAL unit updates the parameter arrays. The depth range update NAL unit may use the parameters in parameter arrays to do prediction and the parameters derived from the depth range update NAL unit may be used to predict other parameters. The error of one 3DV parameters update NAL unit may propagate.

To solve these problems, this proposal applies the concept of parameter set on camera parameters while keeping the original structure, syntax and semantics in [1] and [2] as much as possible.

# Proposed Method

In this proposal, the camera parameters may be included in SPS 3DVC extension, which is same with [1] and [2]. Depth range update NAL unit is replaced by the camera parameter set (CPS). Whether the camera parameters of one slice are in SPS 3DVC extension or in CPS is indicated by syntax element in SPS 3DVC extension and slice header.

The mainly differences between depth range update NAL unit and CPS are:

(1) Each CPS has a cam_parameter_set_id syntax element. If the slice refers to CPS, the cam_parameter_set_id in slice header indicates which CPS is referred to, which is similar to the relationship between PPS and slice header, see Fig 3-1.

Fig 3-1 the relationship among slice and different parameter sets.

(2) Each depth range update NAL unit contains the camera parameters of only one access unit, while each CPS comprises the camera parameters of one or more access unit. If there are T types of camera parameters and each access unit is consisting of V views, then each access unit corresponds to TxV camera parameters. We stored the TxV camera parameters in a camera parameter subset. Therefore, one CPS may include one or more camera parameter subsets. If one slice refers to a CPS which comprises more than one camera parameter subsets, there will also be a syntax element cam_parameter_subset_idx in slice header to indicate which camera parameter subset in CPS corresponds to this slice, see Fig 3-2.

Fig 3-2 the function of cam_parameter_set_id and cam_parameter_subset_idx

(3) The temporal prediction among camera parameters is only carried out within CPS, which means the prediction between two different CPSs is forbidden. But the CPS may use the camera parameters in SPS 3DVC extension as references to do temporal prediction among camera parameters.

Through the proposed method, the problems described in section 1 can be solved:

(1) The reference between slice and CPS is determined by the cam_parameter_set_id and cam_parameter_subset_idx in slice header. Comparing to the connection between the 3DV parameters update NAL unit and the access unit, this mechanism is more reliable.

(2) There is no prediction between different CPSs, which brings better error resiliency.

The proposed method modifies both MVC compatible track and AVC compatible track with the same concept but different types of camera parameters. ZNear and Zfar are coded both in MVC compatible track and AVC compatible track, while FocalLengthX, FocalLengthY, PrincipalPointX, PrincipalPointY and Translation are coded only in AVC compatible track.

The modified syntax and semantics of AVC compatible track will be described in next section, based on reference [2]. Changes by this proposal are indicated with yellow highlighting. More details about the proposed changes to MVC compatible track and AVC compatible track can be found in the two attachments of this proposal.

# Syntax and Semantics Changes

## Slice header

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_mb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| if( nal_unit_type = = 21 && DepthFlag ) { |  |  |
| if( depth_info_present_flag ) { |  |  |
| if( 3dv_acquisition_idc = = 0 ) { |  |  |
| camera_parameter_set_id | 2 | ue(v) |
| if(num_subsets_minus1 != 0 ) { |  |  |
| camera_parameter_subset_idx | 2 | u(v) |
| } |  |  |
| } |  |  |
| else if( 3dv_acquisition_idc = = 2 ) { |  |  |
| camera_parameter_refer_to_sps_flag | 2 | u(1) |
| if( !camera_parameter_refer_to_sps_flag ) { |  |  |
| camera_parameter_set_id | 2 | ue(v) |
| if( num_subsets_minus1 != 0 ) { |  |  |
| camera_parameter_subset_idx | 2 | u(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| …… |  |  |

camera_parameter_set_id specifies the camera parameter set in use. The value of camera_parameter_set_id shall be in the range of 0 to 31, inclusive.

camera _parameter _subset_idx specifies the index of camera parameter subset which contains the camera parameter of the slice in the camera parameter set in use and shall be represented by log2_subsets_num_minus1 + 1 bits in the bitstream.

camera_parameter_refer_to_sps_flag equal to 0 specifies that the camera parameters of the slice are not in the sequence parameter set in use. camera_parameter_refer_to_sps_flag equal to 1 specifies that the camera parameters of the slice are present in the sequence parameter set in use.

## Sequence parameter set 3DVC extension

| seq_parameter_set_3dvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| depth_info_present_flag | 0 | u(1) |
| if( depth_info_present_flag ) { |  |  |
| 3dv_acquisition_idc | 0 | ue(v) |
| if( ( 3dv_acquisition_idc ) && ( num_views_minus1 > 0 ) ) |  |  |
| for( i = 0; i < num_views_minus1; i++ ) |  |  |
| view_id_3dv[ i ] | 0 | ue(v) |
| if( 3dv_acquisition_idc = = 1 ¦ ¦ 3dv_acquisition_idc = = 2 ) { |  |  |
| depth_ranges( 2, SPS, 0 ) |  |  |
| if( profile_idc = = 139 ) |  |  |
| vsp_param( 2, SPS, 0 ) |  |  |
| } |  |  |
| } |  |  |
| if( profile_idc = = 139 && depth_info_present_flag ) { |  |  |
| jvf_idc | 0 | u(2) |
| if( jvf_idc = = 1 ) |  |  |
| jvf_threshold | 0 | ue(v) |
| reduced_resolution_flag | 0 | u(1) |
| slice_header_prediction_idc | 0 | u(2) |
| inside_view_mvp_flag | 0 | u(1) |
| } |  |  |
| if( profile_idc = = 139 && !depth_info_present_flag ) |  |  |
| seq_view_synthesis_flag | 0 | u(1) |
| } |  |  |
| } |  |  |

3dv_acquisition_idc equal to 0 indicates that no camera parameters are present in the sequence parameter set and each coded slice extension for a depth view component shall refer to one camera parameter set. 3dv_acquisition_idc equal to 1 indicates that camera parameters are present in the sequence parameter set and valid for the entire coded video sequence. 3dv_acquisition_idc equal to 2 indicates that camera parameters are present in the sequence parameter set and each coded slice extension for a depth view component may refer to one camera parameter set. 3dv_acquisition_idc values greater than 2 are reserved.

## Camera parameter set

| depth_range_update_rbsp( ) { camera_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| camera_parameter_set_id | 11 | ue(v) |
| log2_subsets_num_minus1 | 11 | ue(v) |
| num_subsets_minus1 | 11 | u(v) |
| num_views_minus1 | 11 | ue(v) |
| vsp_param_flag | 11 | u(1) |
| for( i=0; i<=num_subsets_minus1; i++ ) { |  |  |
| pred_direction | 11 | ue(v) |
| if( pred_direction = = 0 ¦ ¦ pred_direction = = 1 ) { |  |  |
| frame_num_diff0_minus1 | 11 | ue(v) |
| if( i = = 0 ){ |  |  |
| subset0_in_sps_flag | 11 | u(1) |
| subset0inSpsFlag = subset0_in_sps_flag |  |  |
| } else |  |  |
| subset0inSpsFlag = 0 |  |  |
| if( subset0inSpsFlag ) |  |  |
| refSubsetIdx0 = SPS |  |  |
| else { |  |  |
| ref_subset_idx_diff0_minus1 | 11 | ue(v) |
| refSubsetIdx0 = i – 1 – ref_subset_idx_diff0_minus1 |  |  |
| } |  |  |
| frameNum0 = frame_num – 1 – frame_num_diff0_minus1 |  |  |
| if( frameNum0 < 0 ) frameNum0 += MaxFrameNum |  |  |
| predWeight0 = 64 |  |  |
| } |  |  |
| if( pred_direction = = 0 ) { |  |  |
| frame_num_diff1_minus1 | 11 | ue(v) |
| ref_subset_ idx_diff1_minus1 | 11 | ue(v) |
| refSubsetIdx1 = i – 1 – ref_subset_idx_diff1_minus1 |  |  |
| pred_weight0 | 11 | u(6) |
| frameNum1 = frame_num – 1 – frame_num_diff1_minus1 |  |  |
| if( frameNum1 < 0 ) frameNum1 += MaxFrameNum |  |  |
| predWeight0 = pred_weight0 |  |  |
| } |  |  |
| num_views_minus1 | 11 | ue(v) |
| depth_ranges( pred_direction, frame_num camera_parameter_set_id, i ) |  |  |
| vsp_param_flag | 11 | u(1) |
| if( vsp_param_flag ) |  |  |
| vsp_param( pred_direction, frame_num camera_parameter_set_id, i ) |  |  |
| } |  |  |
| depth_param_additional_extension_flag | 11 | u(1) |
| if(depth_param_additional_extension_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| depth_param_additional_extension_data_flag | 11 | u(1) |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

cam_parameter_set_id identifies the camera parameter set that is referred to in the slice header. The value of cam_parameter_set_id shall be in the range of 0 to 31, inclusive.

log2_subsets_num_minus1 plus 1 specifies the number of bits of the syntax element num_subsets_minus1 in camera parameter set and the number of bits of the syntax element cam_parameter_subset_idx in slice header. The value of log2_subsets_num_minus1 shall be in the range of 0 to TBD, inclusive.

num_subsets_minus1 plus 1 specifies the number of camera parameter subsets in this camera parameter set and shall be represented by log2_subsets_num_minus1 + 1 bits in the bitstream.

num_views_minus1 + 1 specifies the number of views for which camera parameters are specified in the included depth_ranges() and vsp_param()structure.

vsp_param_flag equal to 0 specifies that the vsp parameters are not present in this camera parameter set. vsp_param_flag equal to 0 specifies that the vsp parameters are present in this camera parameter set.

pred_direction equal to 0 specifies that the 3DV acquisition variables  may be predicted from the respective variables of two subsets. pred_direction equal to 1 specifies that the 3DV acquisition variables may be predicted from the respective variables of one subset. pred_direction equal to 2 specifies that the 3DV acquisition variables are not predicted. The value of pred_direction shall be in the range of 0 to 2, inclusive.

subset0_in_sps_flag equal to 0 specifies that the first subset to be used in prediction of 3DV acquisition variables is not in the sequence parameter set. subset0_in_sps_flag equal to 1 specifies that the first subset to be used in prediction of 3DV acquisition variables is in the sequence parameter set.

ref_subset_idx_diff0_minus1 + 1 specifies a first subset_idx value to be used in prediction of 3DV acquisition variables relative to the subset_idx value of the current subset.

ref_subset_idx_diff1_minus1 + 1 specifies a second subset_idx value to be used in prediction of 3DV acquisition variables relative to the subset_idx value of the current subset.

pred_weight0 specifies a weight associated with the a first frame_num value derived from frame_num_diff0_minus1 and used for predicting 3DV acquisition variables.

depth_param_additional_extension_flag equal to 0 indicates that no additional data follows within the camera parameter set RBSP prior to the RBSP trailing bits. The value of depth_param_additional_extension_flag shall be equal to 0. The value of 1 for depth_param_additional_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follows the value of 1 for depth_param_additional_extension_flag in a camera parameter set RBSP.

## Depth range

| depth_ranges( predDirection, frameIdc cpsId, subsetIdx) { | C | Descriptor |
| --- | --- | --- |
| z_near_flag | 11 | u(1) |
| z_far_flag | 11 | u(1) |
| ZNearFlag[cpsId, subsetIdx] = z_near_flag |  |  |
| ZFarFlag[cpsId, subsetIdx] = z_far_flag |  |  |
| if( z_near_flag ) |  |  |
| 3dv_acquisition_element( 0, predDirection, 7, 0, ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |  |
| if( z_far_flag ) |  |  |
| 3dv_acquisition_element( 0, predDirection, 7, 0, ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |  |
| } |  |  |

The contents of the syntax structure are controlled through input variables predDirection and subsetIdx the semantics of which are as follows.

–	predDirection equal to 2 specifies that the first loop entry of the element is not predicted and coded in the sign, exponent, and mantissa syntax elements. predDirection equal to 0 or 1 specifies that the first loop entry of the element is predicted and a difference relative to a prediction value is coded in the difference syntax element.

–	cpsId specifies the id of the current camera parameter set. cpsId may be equal to “SPS” indicating a sequence parameter set, or it may be a camera_parameter_set_id value for camera parameter set.

–	subsetIdx specifies the subset_idx of the current subset.

z_near_flag equal to 0 specifies that the syntax elements specifying the closest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the closest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

## 3DV acquisition element

| 3dv_acquisition_element( deltaFlag, predDirection, precMode, expLen, OutSign, OutExp, OutMantissa, OutManLen ) { | C | Descriptor |
| --- | --- | --- |
| if( num_views_minus1 + 1 – deltaFlag > 1 ) |  |  |
| element_equal_flag | 11 | u(1) |
| if( element_equal_flag = = 0 ) |  |  |
| numValues = num_views_minus1 + 1 – deltaFlag |  |  |
| else |  |  |
| numValues = 1 |  |  |
| for( i = 0; i < numValues; i++ ) { |  |  |
| if( predDirection = = 2 && i = = 0 ) { |  |  |
| if( precMode = = 0 ) { |  |  |
| matissa_len_minus1 | 11 | u(5) |
| OutManLen[ cpsId, frameIdc subsetIdx, i ] = manLen = mantissa_len_minus + 1 |  |  |
| } else |  |  |
| prec | 11 | u(5) |
| } |  |  |
| if( predDirection = = 2 ) { |  |  |
| sign0 | 11 | u(1) |
| OutSign[ cpsId, frameIdc subsetIdx, i ] = sign0 |  |  |
| exponent0 | 11 | u(expLen) |
| OutExp[ cpsId, frameIdc subsetIdx, i ] = exponent0 |  |  |
| if( precMode = = 1 ) { |  |  |
| if( exponent0 = = 0 ) |  |  |
| OutManLen[ cpsId, frameIdc subsetIdx, i ] = manLen = Max( 0, prec – 30 ) |  |  |
| else |  |  |
| OutManLen[ cpsId, frameIdc subsetIdx, i ] = manLen = Max( 0, exponent0 + prec – 31 ) |  |  |
| } |  |  |
| mantissa0 | 11 | u(manLen) |
| OutMantissa[ cpsId, frameIdc subsetIdx, i ] = mantissa0 |  |  |
| } else { |  |  |
| skip_flag | 11 | u(1) |
| if( skip_flag = = 0 ) { |  |  |
| sign1 | 11 | u(1) |
| OutSign[ cpsId, frameIdc subsetIdx, i ] = sign1 |  |  |
| exponent_skip_flag | 11 | u(1) |
| if( exponent_skip_flag = = 0 ) { |  |  |
| exponent1 | 11 | u(expLen) |
| OutExp[ cpsId, frameIdc subsetIdx, i ] = exponent1 |  |  |
| } else |  |  |
| OutExp[ cpsId, frameIdc subsetIdx, i ] = OutExp[ cpsId, frameNum0 refSubsetIdx0, i ] |  |  |
| mantissa_diff | 11 | se(v) |
| mantissaPred = (( OutMantissa[ cpsId, frameNum0 refSubsetIdx0, i ] * predWeight0 + OutMantissa[ cpsId, frameNum0 refSubsetIdx0, i ] * ( 64-predWeight0 ) + 32 ) >> 6 ) |  |  |
| OutMantissa[ cpsId, frameIdc subsetIdx, i ] = mantissaPred + mantissa_diff |  |  |
| OutManLen[ cpsId, frameIdc subsetIdx, i ] = OutManLen[ cpsId, frameNum0 refSubsetIdx0, i ] |  |  |
| } else { |  |  |
| OutSign[ cpsId, frameIdc subsetIdx, i ] = OutSign[ cpsId, frameNum0 refSubsetIdx0, i ] |  |  |
| OutExp[ cpsId, frameIdc subsetIdx, i ] = OutExp[ cpsId, frameNum0 refSubsetIdx0, i ] |  |  |
| OutMantissa[ cpsId, frameIdc subsetIdx, i ] = OutMantissa[ cpsId, frameNum0 refSubsetIdx0, i ] |  |  |
| OutManLen[ cpsId, frameIdc subsetIdx, i ] = OutManLen[ cpsId, frameNum0 refSubsetIdx0, i ] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( element_equal_flag = = 1 ) { |  |  |
| for( i = 1; i < num_views_minus1 + 1 – deltaFlag; i++ ) { |  |  |
| OutSign[ cpsId, frameIdc subsetIdx, i ] = OutSign[ cpsId, frameIdc subsetIdx, 0 ] |  |  |
| OutExp[ cpsId, frameIdc subsetIdx, i ] = OutExp[ cpsId, frameIdc subsetIdx, 0 ] |  |  |
| OutMantissa[ cpsId, frameIdc subsetIdx, i ] = OutMantissa[ cpsId, frameIdc subsetIdx, 0 ] |  |  |
| OutManLen[ cpsId, frameIdc subsetIdx, i ] = OutManLen[ cpsId, frameIdc subsetIdx, 0 ] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

The syntax structure specifies the value of an element in 3DV acquisition information. The element may contain one or more loop entries i of the order specified by view_id_3dv syntax elements.

The contents of the syntax structure are controlled through input variables predDirection, precMode, and expLen the semantics of which are as follows.

–	deltaFlag equal to 0 specifies that the each loop entry corresponds to a value specific to the view indicated by i. deltaFlag equal to 1 specifies that each loop entry corresponds to a difference of values between the views indicated by i and i + 1.

–	predDirection equal to 2 specifies that the first loop entry of the element is not predicted and coded in the sign, exponent, and mantissa syntax elements. predDirection equal to 0 or 1 specifies that the first loop entry of the element is predicted and a difference relative to a prediction value is coded in the difference syntax element.

–	precMode equal to 0 specifies that the number of bits in the mantissa syntax element.

–	expLen specifies the number of bits in the exponent syntax element.

The syntax structure uses OutSign, OutExp, and OutMantissa variables for both input and output, where each variable is indexed by [ cps_id, subsetIdx, viewIdc ], cps_id being a camera parameter indicator (either “SPS” or camera_parameter_set_id value), subsetIdx being a subset indicator and viewIdc being a view indicator (in the order of views for 3DV acquisition parameters).

element_equal_flag equal to 0 specifies that the sign, exponent, and mantissa may not be identical to respective values for any two loop entries i and j. element_equal_flag equal to 1 specifies that the sign, exponent, and mantissa are identical to respective values for any two loop entries i and j.

mantissa_len_minus1 + 1 specifies the number of bits in the mantissa syntax element. The value of mantissa_len_minus1 shall be in the range of 0 to 31, inclusive.

prec specifies the exponent of the maximum allowable truncation error for the value represented by the sign, exponent, and mantissa given by 2−prec. The value of prec shall be in the range of 0 to 31, inclusive.

sign0 equal to 0 indicates that the sign of the value provided in the loop entry is positive. sign0 equal to 1 indicates that the sign is negative.

exponent0 specifies the exponent of the value provided by the loop entry. The value of exponent0 shall be in the range of 0 to 2expLen – 2, inclusive. The value 2expLen – 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 2expLen – 1 as indicating an unspecified value.

mantissa0 specifies the mantissa of the value provided by the loop entry.

skip_flag equal to 0 specifies that syntax elements sign1, exponent_skip_flag and mantissa_diff are present for the loop entry. skip_flag equal to 1 specifies that elements sign1, exponent_skip_flag and mantissa_diff are not present for the loop entry.

sign1 equal to 0 indicates that the sign of the value provided in the loop entry is positive. sign1 equal to 1 indicates that the sign is negative.

exponent1, if present, specifies the exponent of the value provided by the loop entry. The value of exponent1 shall be in the range of 0 to 2expLen – 2, inclusive. The value 2expLen – 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 2expLen – 1 as indicating an unspecified value.

mantissa_diff specifies the difference of the mantissa of the value provided by the loop entry relative to its prediction value.

## View synthesis parameters

| vsp_param( predDirection, frameIdccpsId, subsetIdx ) { | C | Descriptor |
| --- | --- | --- |
| focal_length_x_flag | 11 | u(1) |
| focal_length_y_flag | 11 | u(1) |
| principal_point_x_flag | 11 | u(1) |
| principal_point_y_flag | 11 | u(1) |
| translation_flag | 11 | u(1) |
| FocalLengthXFlag[ cpsId, subsetIdx ] = focal_length_x_flag |  |  |
| FocalLengthYFlag[ cpsId, subsetIdx ] = focal_length_y_flag |  |  |
| PrincipalPointXFlag[ cpsId, subsetIdx ] = principal_point_x_flag |  |  |
| PrincipalPointYFlag[ cpsId, subsetIdx ] = principal_point_y_flag |  |  |
| TranlastionFlag[ cpsId, subsetIdx ] = translation_flag |  |  |
| if( focal_length_x_flag ) |  |  |
| 3dv_acquisition_element( 0, predDirection, 6, 1, FocalLengthXSign, FocalLengthXExp, FocalLengthXMantissa, FocalLengthXManLen ) |  |  |
| if( focal_length_y_flag ) |  |  |
| 3dv_acquisition_element( 0, predDirection, 6, 1, FocalLengthYSign, FocalLengthYExp, FocalLengthYMantissa, FocalLengthYManLen ) |  |  |
| if( principal_point_x_flag ) |  |  |
| 3dv_acquisition_element( 0, predDirection, 6, 1, PrincipalPointXSign, PrincipalPointXExp, PrincipalPointXMantissa, PrincipalPointXManLen ) |  |  |
| if( principal_point_y_flag ) |  |  |
| 3dv_acquisition_element( 0, predDirection, 6, 1, PrincipalPointYSign, PrincipalPointYExp, PrincipalPointYMantissa, PrincipalPointYManLen ) |  |  |
| if( translation_flag ) |  |  |
| 3dv_acquisition_element( 1, predDirection, 6, 1, TXSign, TXExp, TXMantissa, TXManLen ) |  |  |
| } |  |  |

The contents of the syntax structure are controlled through input variables predDirection and frameIdc the semantics of which are as follows.

–	predDirection equal to 2 specifies that the first loop entry of the element is not predicted and coded in the sign, exponent, and mantissa syntax elements. predDirection equal to 0 or 1 specifies that the first loop entry of the element is predicted and a difference relative to a prediction value is coded in the difference syntax element.

–	cpsId specifies the id of the current camera parameter set. cpsId may be equal to “SPS” indicating a sequence parameter set, or it may be a camera_parameter_set_id value for camera parameter set.

–	cpsId specifies the id of the current camera parameter set.

focal_length_x_flag equal to 0 specifies that the syntax elements specifying the horizontal focal length are not present in the syntax structure. focal_length_x_flag equal to 1 specifies that the syntax elements specifying the horizontal focal length are present in the syntax structure.

focal_length_y_flag equal to 0 specifies that the syntax elements specifying the vertical focal length are not present in the syntax structure. focal_length_y_flag equal to 1 specifies that the syntax elements specifying the vertical focal length are present in the syntax structure.

principal_point_x_flag equal to 0 specifies that the syntax elements specifying the horizontal principal point are not present in the syntax structure. principal_point_x_flag equal to 1 specifies that the syntax elements specifying the horizontal principal point are present in the syntax structure.

principal_point_y_flag equal to 0 specifies that the syntax elements specifying the vertical principal point are not present in the syntax structure. principal_point_y_flag equal to 1 specifies that the syntax elements specifying the vertical principal point are present in the syntax structure.

translation_flag equal to 0 specifies that the syntax elements specifying the horizontal translation between views are not present in the syntax structure. translation_flag equal to 1 specifies that the syntax elements specifying the horizontal translation between views are present in the syntax structure.

# References

[1] Teruhiko Suzuki, Miska M. Hannuksela, Ying Chen, Shinobu Hattori, “Text of ISO/IEC 14496-10:2012/PDAM 2 MVC extensions for inclusion of depth maps”, ISO/IEC JTC1/SC29/WG11 document N12731, May 2012.

[2] Miska M. Hannuksela, Ying Chen, “Working Draft 2 of AVC compatible video with depth information”, ISO/IEC JTC1/SC29/WG11 document N12732, May 2012.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).