| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0055 m25932 |
| --- | --- |

| Title: | 3D-CE1.a results on flexible rendering precision |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao and Lu Yu #38 Zheda Road Zhejiang University Hangzhou, China | Tel: Email: | +86-571-87953107 {zhaoyin,yul}@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This document presents a method of flexible rendering precision in VSP for different texture view components. Based on NTT’s simplified view synthesis with fractional-pel disparity, integer, half-pel, and quarter-pel precision are supported simply with three different fractional-pel disparity look-up tables. The precision values for views using VSP are decided by a RD-cost-based encoder optimization technique (which evaluates the frame-level RD cost of using different rendering precision at the first access unit), and are signalled in SPS.

With flexible rendering precision, the encoder may choose lower rendering precision which results in similar or even better RD performance compared with always using quarter-pel precision. On average, this proposed method keeps the same RD performance (0.00% BD-rate saving in terms of coded texture PSNR and coded texture rate), while reducing 5% ~ 10% decoding time by using lower-precision rendering.

# Introduction

In m24898, it was reported that view synthesis in VSP generally results in better RD performance with higher rendering precision. However, higher precision also means more complexity. Thus, the synthesis process in VSP was recommended to have flexible rendering precision, which allows further encoder optimization on a trade-off between RD performance and decoding complexity of the VSP module.

In NTT’s proposal m24794 on VSP with fractional-pel disparity (as shown in Fig. 1), an integer pixel A is warped to target view, and its warping position is rounded to 1/4 sample raster. Then, a sub-pel pixel between A and B is warped to the integer position C, by which the fractional-pel disparity of A is compensated. To support quarter-pel rendering precision, half-pel pixels are generated with the 6-tap AVC half-pel interpolation filter, and quarter-pel pixels are interpolated with AVC quarter-pel interpolation filter.

Based on m24794, intger, half-pel and quarter-pel rendering precision can be realized by simply using different fractional-pel disparity look-up tables (which quantize the warped pixel position to 1/N sample raster, and N refers to 1, 2, and 4 for integer, half-pel, and quarter-pel precision). Accordingly, for half-pel precision, only the half-pel interpolation is used; for integer precision, the complexity on half-pel and quarter-pel interpolation can be saved.

Fig. 1: NTT’s warping process.

# The Proposed Method

2.1 Implementation of flexible rendering precision

The flexible rendering precision is supported by using different fractional-pel disparity look-up tables. More specifically, for quarter-pel precision, C can be filled by one pixel from pixels A, 1, 2 and 3, as shown in Fig. 1; for half-pel precision, C can be filled by one pixel from pixels A and 2; and for integer precision, C can only be filled by A. Apart from the precision-guided initialization of the look-up table, the core rendering process remains unchanged.

The precision information is signalled in subset SPS. The related syntax and semantics are illustrated in the following two subsections.

2.1.1 Syntax

Sequence parameter set 3DV extension syntax

| seq_parameter_set_3dvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| if( profile_idc = = 139 && !depth_info_present_flag ) |  |  |
| seq_view_synthesis_flag | 0 | u(1) |
| if(seq_view_synthesis_flag){ |  |  |
| for(i=1; i<=num_views_minus1; i++) |  |  |
| seq_view_synthesis_precision_idc[ i ] | 0 | u(2) |
| } |  |  |
| } |  |  |

2.1.2 Semantics

seq_view_synthesis_precision_idc[i] equal to 0 indicates that the texture view component of VOIdx = i referring to this SPS uses integer-pixel rendering precision in view synthesis prediction. seq_view_synthesis_precision_idc[i] equal to 1 indicates that that the texture view component of VOIdx = i referring to this SPS uses half-pixel rendering precision in view synthesis prediction. seq_view_synthesis_precision_idc[i] equal to 2 indicates that that the texture view component of VOIdx = i referring to this SPS uses quarter-pixel rendering precision in view synthesis prediction. seq_view_synthesis_precision_idc shall be in the range from 0 to 2, inclusive.

2.2 Encoder optimization on deciding the rendering precision

A simple encoder optimization technique is employed to decide the rendering precision for each non-base view. For any non-base view in the first access unit of a sequence, the texture is encoded with all the three rendering precision first. Then, the bits and SSE for encoding the texture view component are converted to a frame-based RD cost:

Cost(prec) = SSE(prec) + lambda(QP) * bits(prec)

where prec indicates the rendering precision, and QP is the average QP of the encoding the frame with precision prec.

The rendering precision for the view is selected as the one that results in the minimum RD cost.

# Experimental Results

The flexible rendering precision (with the simple encoder optimization) leads to slight coding gain in S05 and S06, and on average keeps the same RD performance as that by always using 1/4 precision, as listed in Table I. Besides, by using lower rendering precision (as listed in Table II), the decoding time is reduced by around 5% ~10 % on average as indicated by two decoding tests: Test 1, decoding all ATM-bitstreams and the proposed bitstreams in two parallel processes; Test 2, decoding all bitstreams one by one.

Table I RD performance of the proposal method

|  |  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Decoder Time, % |  |
|  |  |  |  |  |  |  |  |  |  | Test1 | Test2 |
| Hall2 | S01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.05 | 0.00 | 0.856 | 1.018 |
| Street | S02 | 0.05 | 0.00 | 0.00 | 0.00 | 0.04 | 0.00 | 0.05 | 0.00 | 0.953 | 0.987 |
| Dancer | S03 | 0.22 | -0.01 | 0.00 | 0.00 | 0.20 | -0.01 | 0.20 | 0.00 | 0.994 | 0.845 |
| GT | S04 | 0.03 | 0.00 | 0.00 | 0.00 | 0.03 | 0.00 | 0.20 | -0.01 | 1.059 | 1.016 |
| Kendo | S05 | -0.09 | 0.01 | 0.00 | 0.00 | -0.08 | 0.00 | -0.07 | 0.00 | 1.072 | 0.945 |
| Balloons | S06 | -0.24 | 0.01 | 0.00 | 0.00 | -0.21 | 0.01 | -0.11 | 0.01 | 0.906 | 0.716 |
| News | S08 | 0.03 | 0.00 | 0.00 | 0.00 | 0.03 | 0.00 | 0.03 | 0.00 | 0.805 | 0.708 |
|  | Average | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.05 | 0.00 | 0.95 | 0.89 |

Table II. Precision values selected by encoder (1: integer, 2: half-pel, 4: quarter-pel)

| Sequence |  | R4 |  | R3 |  | R2 |  | R1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L | R | L | R | L | R | L | R |
| Hall2 | S01 | 1 | 4 | 2 | 1 | 1 | 2 | 2 | 1 |
| Street | S02 | 4 | 4 | 4 | 4 | 4 | 1 | 4 | 2 |
| Dancer | S03 | 4 | 4 | 4 | 4 | 4 | 1 | 2 | 4 |
| GT | S04 | 4 | 2 | 4 | 4 | 2 | 2 | 4 | 1 |
| Kendo | S05 | 1 | 1 | 2 | 2 | 2 | 4 | 2 | 2 |
| Balloons | S06 | 4 | 2 | 2 | 1 | 4 | 2 | 4 | 1 |
| News | S08 | 4 | 4 | 1 | 2 | 2 | 2 | 2 | 4 |

# Conclusion

Allowing flexible rendering precision brings negligible decoder implementation complexity (which is only on the initilization of the fractional-pel disparity look-up table). It can save decoding time (especially for a software decoder) when lower precision is chosen. A RD-cost-based encoder optimization is also proposed to decide the rendering precision values for the non-base views, which resutls in the same RD performance as ATM4 on average while reducing decoding time by 5% ~ 10%.

We suggest adopting VSP with flexible rendering precision (one from integer-, half-, and quater-pixel precision) and using sequence-level indices to signal the rendering precision values.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).